# Hex ↔ Binary Conversion with the Top-Row Bin/Hex Buttons and LEDs

A Beboputer / PY-DIYCALCULATOR assembly-language tutorial

# 1. What You Will Build

Press Hex, then type one hex digit, and the display shows its 4-bit binary pattern while the same pattern lights up on the Calculator’s top-row LEDs. Press Bin, then type four bits, and the LEDs light up live as you go before the display shows the equivalent hex digit:

Hex   A     ->  shows  A=1010          (LEDs: . X . X)

Hex   7     ->  shows  7=0111          (LEDs: . X X X)

Bin   1010  ->  shows  1010=A          (LEDs light as you type)

Bin   0111  ->  shows  0111=7

The source file is tutorial/14_hex_bin_conversion.asm. Clear or CE resets to the start and turns the LEDs off; pressing Hex or Bin again after an answer begins the next conversion.

# 2. The Top Row: Three Blank Buttons, Then Bin, Dec, Hex

Above the main keypad sits a strip of six LED+button pairs (tools/calculator.py’s _top_led_row): three blank/unassigned buttons, then Bin, then Dec, then Hex, each with its own LED directly beside it. All six LEDs are driven by one port, $F032 — bit 5 is the leftmost LED, bit 0 the rightmost — exactly the port exercise 07 introduced. This exercise only uses Bin and Hex; Dec is left as an extension idea in Section 9. Bin and Hex send $43 and $45 respectively — see the BIN_KEY/HEX_KEY constants in the source listing.

# 3. Program Design

A keypad-polling state machine, same shape as every earlier exercise, with two independent entry modes instead of one:

| STAGE | The program is... |
| --- | --- |
| 0 | Idle — waiting for Hex or Bin to choose a direction. |
| 1 | Hex mode — waiting for one hex digit. |
| 2 | Bin mode — collecting four bits. |
| 3 | Showing an answer — Hex or Bin starts the next one. |

# 4. Hex → Bin: the Digit Already Is the Nibble

Because digit and hex-letter buttons already arrive as their raw nibble value (0–15), there is nothing to decode — KEYVAL already holds the exact 4-bit value the digit represents. Writing it straight to LEDS lights the matching bits immediately:

ST_HEXENTRY:

LDA     [KEYVAL]

STA     [DISP]           # echo the digit as typed

LDA     [KEYVAL]

AND     $0F              # already 0-15; kept for safety

STA     [NIBBLE]

JMP     [SHOW_HEX_TO_BIN]

SHOW_HEX_TO_BIN then prints the four binary characters one at a time, most-significant bit first, by shifting a saved copy right by 3, then 2, then 1, then 0 places and masking off everything but the bottom bit with AND $01:

LDA     [TEMP]

SHR

SHR

SHR                      # bit 3 is now in bit 0's position

AND     $01

OR      $30              # 0 or 1 -> ASCII '0' or '1'

STA     [DISP]

# 5. Bin → Hex: Building a Nibble One Bit at a Time

Typing bits most-significant-first is exactly how you’d read them off a page, and it turns into a shift-and-OR each keypress: shift the nibble-so-far left one place (making room, and guaranteeing the new bottom bit is 0), then OR in whichever of 0 or 1 was just typed:

BIN_BIT:

LDA     [KEYVAL]

STA     [DISP]           # echo the '0' or '1' as typed

LDA     [NIBBLE]

SHL                      # make room; new bottom bit is 0

OR      [KEYVAL]         # merge in the bit just typed

STA     [NIBBLE]

STA     [LEDS]           # live feedback - light it now

LDA     [BITCNT]

INCA

STA     [BITCNT]

CMPA    $04

JZ      [SHOW_BIN_TO_HEX]

JMP     [WAIT]

After four bits, NIBBLE holds exactly the value that many binary digits represent — 1,0,1,0 becomes 0001 → 0010 → 0101 → 1010 as each SHL-then-OR runs, ending at decimal 10.

# 6. A Familiar Three-Way Test, in a New Job

Turning that nibble (0–15) into a displayable character needs a digit ‘0’–‘9’ for 0–9, but a letter ‘A’–‘F’ for 10–15 — and the boundary between them needs the same three-way CMPA/JC/JZ test earlier tutorials use for “greater than or equal to” comparisons, just repurposed here to classify a value instead of compare two variables. A lone JC (checking only “strictly greater than 10”) would miss NIBBLE landing exactly on 10 — which is precisely the input “1010” this exercise most wants to get right, since that’s the whole point of typing four 1-or-0 digits to spell out the letter A:

SHOW_BIN_TO_HEX:

LDA     $3D              # '='

STA     [DISP]

LDA     [NIBBLE]

CMPA    $0A

JC      [BTH_LETTER]     # C=1: NIBBLE > 10

JZ      [BTH_LETTER]     # Z=1: NIBBLE == 10 - this is 'A'

LDA     [NIBBLE]         # falls through: NIBBLE < 10

OR      $30

STA     [DISP]

JMP     [RESET_STATE]

BTH_LETTER:

LDA     [NIBBLE]

SUB     $0A              # 0-5

ADD     $41              # 'A'-'F'

STA     [DISP]

JMP     [RESET_STATE]

Without the JZ line, typing “1010” would render “:” (0x3A) instead of “A”, because OR $30 applied directly to 10 produces the character right after ‘9’ in ASCII, not a letter.

# 7. Verified Against the Real CPU Emulator

This program was assembled with the project’s own das.py and run against the project’s own CPU class with simulated keypresses. Every one of the 16 possible hex digits was converted to binary and checked — both the displayed text and the actual byte written to the LED port ($F032) — and every one of the 16 possible 4-bit entries was converted back to a hex digit and checked the same way. A representative sample:

| Direction | Input | Result | LEDs (bit5→bit0) |
| --- | --- | --- | --- |
| Hex → Bin | Hex, 0 | 0=0000 | 000000 |
| Hex → Bin | Hex, 7 | 7=0111 | 000111 |
| Hex → Bin | Hex, A | A=1010 | 001010 |
| Hex → Bin | Hex, F | F=1111 | 001111 |
| Bin → Hex | Bin, 0101 | 0101=5 | 000101 |
| Bin → Hex | Bin, 1010 | 1010=A | 001010 |
| Bin → Hex | Bin, 1111 | 1111=F | 001111 |

Behavior around Clear, CE, and re-entry was also checked: Clear mid-entry correctly restarts a Hex or Bin conversion from scratch and blanks the LEDs; CE partway through typing four bits does the same; and typing digit “2” or hex-letter “A” while idle is correctly ignored rather than being misread as Bin or Dec (Bin/Dec were moved to $43/$44 specifically so a plain digit or hex letter can never be mistaken for one of them).

# 8. Try It Yourself

Assemble tutorial/14_hex_bin_conversion.asm, Load → CPU, click Run. Press Hex, then a hex digit — watch the LEDs and the binary digits appear together. Press Clear, then Bin, then type four bits (e.g. 1, 0, 1, 0) — watch each LED light up as you go, then see “=A” appear once the fourth bit lands.

Open Display → Port Map Status alongside the exercise to see $F011 receive $45 (Hex) or $43 (Bin) the instant you press them, and each bit land on $F011 as a plain $00/$01 while typing in Bin mode. Watch $F032 change live as each LED updates.

# 9. Ideas to Extend This Program

- Wire up Dec too: after a Hex or Bin conversion, pressing Dec could show the same nibble’s decimal value (0–15) instead.

- Extend this to a full byte (two hex digits / eight bits) instead of one nibble — you’ll need a second LED port or to show the byte across two conversions, since this Calculator only has six LEDs to work with.

- Add input validation: right now, typing anything other than 0 or 1 in Bin mode is silently ignored rather than flagged.

# Appendix: Complete Source Listing

tutorial/14_hex_bin_conversion.asm

# ================================================================

# 14_hex_bin_conversion.asm

# Beboputer Hands-On Tutorial — Bonus Exercise (Calculator)

#

# Convert a hex digit to binary, or four typed bits back to a hex

# digit, using the Bin/Hex buttons and the LEDs in the Calculator's

# top row.

#

# The top row: three blank buttons, then Bin, Dec, Hex

# --------------------------------------------------------

#   Above the main keypad sits a strip of six LED+button pairs (see

#   tools/calculator.py's _top_led_row): three blank/unassigned

#   buttons, then Bin, then Dec, then Hex, each with its own LED

#   directly beside it. All six LEDs are driven by one port, $F032 -

#   bit 5 is the leftmost LED, bit 0 the rightmost - exactly the

#   port exercise 07 introduced. This exercise only uses Bin and

#   Hex; Dec is left for you in "Try this next".

#

# A button-code collision this exercise ran into (and fixed)

# ---------------------------------------------------------------

#   Bin and Dec used to default to codes $02 and $0A. Digit and hex-

#   letter buttons don't send their ASCII code - tools/diy_button.py

#   converts them down to a raw nibble first (0-9 for digits, 10-15

#   for A-F, see exercise 10's note on this) - which meant digit "2"

#   arrives as plain byte $02, identical to Bin's old code, and hex

#   letter "A" arrives as byte $0A, identical to Dec's old code. A

#   program watching for "did the user press Bin?" by comparing

#   against $02 could not tell that apart from someone simply typing

#   the digit 2. This is the same class of bug exercise 10 found

#   between CE and digit "1" - and the fix is the same: Bin, Dec,

#   and Hex were moved to $43/$44/$45 in _BUTTON_DEFAULTS

#   (tools/calculator.py), all outside the 0-15 nibble range, so

#   they can never again collide with a digit or hex-letter

#   keypress. This exercise watches for those new codes below.

#

# What this program does

# -----------------------

#   Press Hex, then type one hex digit (0-9 or A-F): the display

#   shows the digit, then its 4-bit binary pattern, and the same

#   pattern lights up on the LEDs.

#       Hex   A     ->  shows  A=1010          (LEDs: . X . X)

#       Hex   7     ->  shows  7=0111          (LEDs: . X X X)

#

#   Press Bin, then type exactly four bits using the 0 and 1 keys,

#   most-significant bit first: the LEDs light up live as each bit

#   lands, and once the fourth bit arrives the display shows the

#   equivalent hex digit.

#       Bin   1010  ->  shows  1010=A          (LEDs light as you type)

#       Bin   0111  ->  shows  0111=7

#

#   Clear or CE resets to the beginning at any point, and turns the

#   LEDs off. Pressing Hex or Bin again after an answer starts a new

#   conversion (any other key while an answer is showing is

#   ignored - press Hex or Bin to begin the next one).

#

# How it works

# -------------

#   A keypad-polling state machine, same shape as every earlier

#   exercise, with two independent entry modes instead of one:

#

#     STAGE 0   idle - waiting for Hex or Bin to choose a direction

#     STAGE 1   Hex mode - waiting for one hex digit

#     STAGE 2   Bin mode - collecting four bits

#     STAGE 3   answer is showing - Hex or Bin starts the next one

#

# Hex -> Bin: the digit already IS the nibble

# -----------------------------------------------

#   Because digit and hex-letter buttons already arrive as their raw

#   nibble value (0-15, per the note above), there is nothing to

#   decode - KEYVAL already holds the exact 4-bit value the digit

#   represents. Writing it straight to LEDS lights the matching bits

#   immediately:

#

#     ST_HEXENTRY:

#             LDA     [KEYVAL]

#             STA     [DISP]           # echo the digit as typed

#             LDA     [KEYVAL]

#             AND     $0F              # already 0-15; kept for safety

#             STA     [NIBBLE]

#             JMP     [SHOW_HEX_TO_BIN]

#

#   SHOW_HEX_TO_BIN then prints the four binary characters one at a

#   time, most-significant bit first, by shifting a saved copy right

#   by 3, then 2, then 1, then 0 places and masking off everything

#   but the bottom bit with AND $01:

#

#     LDA     [TEMP]

#     SHR

#     SHR

#     SHR                      # bit 3 is now in bit 0's position

#     AND     $01

#     OR      $30              # 0 or 1 -> ASCII '0' or '1'

#     STA     [DISP]

#

# Bin -> Hex: building a nibble one bit at a time

# ----------------------------------------------------

#   Typing bits most-significant-first is exactly how you'd read

#   them off a page, and it turns into a shift-and-OR each keypress:

#   shift the nibble-so-far left one place (making room, and

#   guaranteeing the new bottom bit is 0), then OR in whichever of 0

#   or 1 was just typed:

#

#     BIN_BIT:

#             LDA     [KEYVAL]

#             STA     [DISP]           # echo the '0' or '1' as typed

#             LDA     [NIBBLE]

#             SHL

#             OR      [KEYVAL]

#             STA     [NIBBLE]

#             STA     [LEDS]           # live feedback - light it now

#             LDA     [BITCNT]

#             INCA

#             STA     [BITCNT]

#             CMPA    $04

#             JZ      [SHOW_BIN_TO_HEX]

#             JMP     [WAIT]

#

#   After four bits, NIBBLE holds exactly the value that many binary

#   digits represent - 1,0,1,0 becomes 0001 -> 0010 -> 0101 -> 1010

#   as each SHL-then-OR runs, ending at decimal 10.

#

# A familiar three-way test, in a new job

# -------------------------------------------

#   Turning that nibble (0-15) into a displayable character needs a

#   digit '0'-'9' for 0-9, but a letter 'A'-'F' for 10-15 - and the

#   boundary between them needs the same three-way CMPA/JC/JZ test

#   exercises 10-13 use for "greater than or equal to" comparisons,

#   just repurposed here to classify a value instead of compare two

#   variables. A lone JC (checking only "strictly greater than 10")

#   would miss NIBBLE landing exactly on 10 - which is precisely the

#   input "1010" that this exercise most wants to get right, since

#   that's the whole point of typing four 1-or-0 digits to spell out

#   the letter A:

#

#     SHOW_BIN_TO_HEX:

#             LDA     $3D              # '='

#             STA     [DISP]

#             LDA     [NIBBLE]

#             CMPA    $0A

#             JC      [BTH_LETTER]     # C=1: NIBBLE > 10

#             JZ      [BTH_LETTER]     # Z=1: NIBBLE == 10 - this is 'A'

#             LDA     [NIBBLE]         # falls through: NIBBLE < 10

#             OR      $30

#             STA     [DISP]

#             JMP     [RESET_STATE]

#     BTH_LETTER:

#             LDA     [NIBBLE]

#             SUB     $0A              # 0-5

#             ADD     $41              # 'A'-'F'

#             STA     [DISP]

#             JMP     [RESET_STATE]

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run. Press Hex, then a hex digit -

#   watch the LEDs and the binary digits appear together. Press

#   Clear, then Bin, then type four bits (e.g. 1, 0, 1, 0) - watch

#   each LED light up as you go, then see "=A" appear once the

#   fourth bit lands. Try 1010 specifically to confirm the boundary

#   case above renders "A", not ":" (which is what OR $30 applied to

#   10 directly would wrongly produce).

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status alongside this exercise to see

#   $F011 receive $45 (Hex) or $43 (Bin) the instant you press them,

#   and each bit land on $F011 as a plain $00/$01 while typing in Bin

#   mode. Watch $F032 change live as each LED updates.

#

# Try this next

# ---------------

#   - Wire up Dec too: after a Hex or Bin conversion, pressing Dec

#     could show the same nibble's decimal value (0-15) instead.

#   - Extend this to a full byte (two hex digits / eight bits)

#     instead of one nibble - you'll need a second LED port or to

#     show the byte across two conversions, since this Calculator

#     only has six LEDs to work with.

#   - Add input validation: right now, typing anything other than 0

#     or 1 in Bin mode is silently ignored rather than flagged.

# ================================================================

KEY:      .EQU    $F011

DISP:     .EQU    $F031

LEDS:     .EQU    $F032

BIN_KEY:  .EQU    $43        # code the Bin button sends (post-fix)

HEX_KEY:  .EQU    $45        # code the Hex button sends (post-fix)

.ORG    $4000

START:  LDA     $00

STA     [STAGE]        # begin idle, waiting for Hex or Bin

# ---------------------------------------------------------------

# Main loop - poll the keypad, dispatch on STAGE

# ---------------------------------------------------------------

WAIT:   LDA     [KEY]

CMPA    $FF             # $FF = idle, no key waiting

JZ      [WAIT]

STA     [KEYVAL]        # stash it - this is the ONLY read of KEY

CMPA    $1B             # Clear key

JZ      [DO_CLEAR]

CMPA    $7F             # CE key

JZ      [DO_CLEAR]

LDA     [STAGE]

CMPA    $00

JZ      [ST_IDLE]

CMPA    $01

JZ      [ST_HEXENTRY]

CMPA    $02

JZ      [ST_BINENTRY]

JMP     [ST_ANSWER]      # only remaining value is STAGE 3

# ---------------------------------------------------------------

# STAGE 3 -> 0 : answer was showing - Hex/Bin starts a new one

# ---------------------------------------------------------------

ST_ANSWER:

LDA     $1B              # clear code - wipes the old expression

STA     [DISP]

JMP     [ST_IDLE]

# ---------------------------------------------------------------

# STAGE 0 : idle - waiting for Hex or Bin to pick a direction

# ---------------------------------------------------------------

ST_IDLE:

LDA     [KEYVAL]

CMPA    HEX_KEY

JZ      [START_HEX]

CMPA    BIN_KEY

JZ      [START_BIN]

JMP     [WAIT]           # anything else is ignored

START_HEX:

LDA     $01

STA     [STAGE]

JMP     [WAIT]

START_BIN:

LDA     $00

STA     [NIBBLE]

LDA     $00

STA     [BITCNT]

LDA     $02

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 1 : Hex mode - waiting for exactly one hex digit

# ---------------------------------------------------------------

ST_HEXENTRY:

LDA     [KEYVAL]

STA     [DISP]           # echo the digit as typed

LDA     [KEYVAL]

AND     $0F              # already 0-15; kept for safety

STA     [NIBBLE]

JMP     [SHOW_HEX_TO_BIN]

SHOW_HEX_TO_BIN:

LDA     $3D              # '='

STA     [DISP]

LDA     [NIBBLE]

STA     [LEDS]           # light the matching bits immediately

STA     [TEMP]           # keep a copy - the shifts below destroy ACC

LDA     [TEMP]

SHR

SHR

SHR                      # bit 3 into bit 0's position

AND     $01

OR      $30

STA     [DISP]

LDA     [TEMP]

SHR

SHR                      # bit 2 into bit 0's position

AND     $01

OR      $30

STA     [DISP]

LDA     [TEMP]

SHR                      # bit 1 into bit 0's position

AND     $01

OR      $30

STA     [DISP]

LDA     [TEMP]           # bit 0 is already in position

AND     $01

OR      $30

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# STAGE 2 : Bin mode - collecting four bits, MSB first

# ---------------------------------------------------------------

ST_BINENTRY:

LDA     [KEYVAL]

CMPA    $00

JZ      [BIN_BIT]

CMPA    $01

JZ      [BIN_BIT]

JMP     [WAIT]           # anything but 0 or 1 is ignored

BIN_BIT:

LDA     [KEYVAL]

STA     [DISP]           # echo the '0' or '1' as typed

LDA     [NIBBLE]

SHL                      # make room; new bottom bit is 0

OR      [KEYVAL]         # merge in the bit just typed

STA     [NIBBLE]

STA     [LEDS]           # live feedback - light it now

LDA     [BITCNT]

INCA

STA     [BITCNT]

CMPA    $04

JZ      [SHOW_BIN_TO_HEX]

JMP     [WAIT]

SHOW_BIN_TO_HEX:

LDA     $3D              # '='

STA     [DISP]

LDA     [NIBBLE]

CMPA    $0A

JC      [BTH_LETTER]     # C=1: NIBBLE > 10

JZ      [BTH_LETTER]     # Z=1: NIBBLE == 10 - this is 'A'

LDA     [NIBBLE]         # falls through: NIBBLE < 10

OR      $30

STA     [DISP]

JMP     [RESET_STATE]

BTH_LETTER:

LDA     [NIBBLE]

SUB     $0A              # 0-5

ADD     $41              # 'A'-'F'

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Explicit Clear / CE handling - also blanks the LEDs

# ---------------------------------------------------------------

DO_CLEAR:

LDA     $1B

STA     [DISP]

LDA     $00

STA     [STAGE]

LDA     $00

STA     [LEDS]

JMP     [WAIT]

RESET_STATE:

LDA     $03              # STAGE 3: Hex/Bin starts a new conversion

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# Variables (reserved bytes - see exercise 09 for this pattern)

# ---------------------------------------------------------------

STAGE:   .BYTE

NIBBLE:  .BYTE

BITCNT:  .BYTE

KEYVAL:  .BYTE

TEMP:    .BYTE

.END    $4000