# Lab 2: Beboputer Fundamentals

A Beboputer / PY-DIYCALCULATOR assembly-language tutorial

# 1. What You Will Build

Six short, standalone programs, each introducing exactly one new idea, building from the simplest possible Beboputer program up to reading the keypad, driving the LEDs, and printing a message:

| Lab | Introduces | What it does |
| --- | --- | --- |
| 2a | The bare minimum | Clears the display using raw numbers only, then stops. |
| 2b | .EQU labels | Identical to 2a, rewritten with named constants. |
| 2c | DECA / JNZ loops | Counts down from 9 to 1 on the display. |
| 2d | Reading the keypad | Echoes digit keys 0–9 to the display; ignores everything else. |
| 2e | SHR and the LEDs | Walks a single lit LED across the six-LED row, forever. |
| 2f | Indexed addressing | Prints “HELLO WORLD!” one character at a time from a table. |

The source files are tutorial/15a_lab2a_clear_display.asm through tutorial/15f_lab2f_hello_world.asm — annotated versions of the original Data/lab2a.asm–lab2f.asm example programs, with the same code but a full explanation of what each line does and why.

# 2. Lab 2a: The Shortest Possible Program

Four instructions, no labels, nothing but raw numbers:

LDA     $10        # Load accumulator with clear code

STA     [$F031]    # Store accumulator to address $F031

JMP     [$0000]    # Jump to address $0000

$10 is the byte this Calculator treats as “clear the display” (tools/calculator.py’s write_display also accepts $0D and $1B for the same purpose). $F031 is the display’s output port.

The closing jump is worth pausing on. This CPU has no “halt and stay halted” state you fall into automatically — the Program Counter just keeps advancing unless told otherwise. Address $0000 is always zeroed on power-on and stays that way unless a program is deliberately loaded there, and a zeroed byte decodes as opcode $00 — NOP, “do nothing, move on.” So JMP [$0000] sends the CPU into an address full of NOPs, where it keeps fetching NOP after NOP forever — not a true halt, but a program that has, for all practical purposes, stopped. (This CPU does have a real HALT instruction, opcode $3C — the Terminal Print Message exercise uses it. Lab 2a’s “jump into zeroed memory” trick is the older, equally valid way of getting the same practical result, and it turns up again in Labs 2c, 2d, and 2f.)

# 3. Lab 2b: Naming Your Constants

The exact same program as Lab 2a, byte for byte, rewritten with .EQU:

CLRCODE:  .EQU  $10       # Special code to clear the main display

MAINDISP: .EQU  $F031     # Address of output port for main display

LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Store accumulator to main display

JMP     [$0000]    # Jump to address $0000

.EQU gives a name to a constant value at assembly time — every place that name appears gets replaced with the value during assembly, so the CPU never sees “CLRCODE” or “MAINDISP,” only $10 and $F031. This is the same trick every later exercise relies on (KEY, DISP, LEDS, and so on): it costs nothing at runtime and makes a program far easier to read once it grows past a handful of lines. Assembling both 2a and 2b produces identical bytes at $4000 onward — labels are a convenience for the person writing the program, not something the assembled program itself retains any trace of.

# 4. Lab 2c: Your First Loop — Counting Down

The first loop in this series, and the shape almost every later loop in this project is built from:

LDA     $09        # Load the accumulator with $09

LOOP:      STA     [MAINDISP] # Store accumulator to the main display

DECA               # Decrement the accumulator

JNZ     [LOOP]     # Jump to LOOP if ACC isn't zero

ACC starts at 9. Each pass through LOOP shows the current value before decrementing it, so the display sees 9, then 8, then 7, and so on. DECA subtracts 1 and sets the Zero flag exactly when the result lands on 0; JNZ keeps the loop going for every value except that last one — so the sequence is show 9 → decrement to 8 → loop; show 8 → decrement to 7 → loop; … ; show 1 → decrement to 0 → Zero flag set → JNZ does not jump. The loop ends right after 1 is shown; 0 itself is never displayed.

$09 down to $01 are raw values, not ASCII text, but they appear as the digit characters “9” through “1” anyway: raw bytes $00–$09 are shown as the digit characters ‘0’–‘9’ by the display port’s write_display method, on top of its normal handling of printable ASCII. Every Calculator exercise that echoes a keypad digit straight to the display without converting it first relies on this same convention.

# 5. Lab 2d: Reading the Keypad

A busy-wait loop reading the keypad port ($F011), guarded by two range checks before anything reaches the display:

LOOP:      LDA     [KEYPAD]   # Load the accumulator from the keypad

JN      [LOOP]     # Jump to LOOP if N flag is set

CMPA    $09        # Compare accumulator to code $09

JC      [LOOP]     # Jump to LOOP if C flag is set

STA     [MAINDISP] # ... else store accumulator to display

JMP     [LOOP]     # Jump to LOOP

First check — JN (“jump if Negative”): the keypad port sits at $FF (“idle, nothing pressed”) whenever no key is down, and $FF has its top bit set, which LDA’s N flag reports as “negative.” Any byte $80–$FF trips this same check, which conveniently screens out any button code that happens to live in that range too, with one read and one branch.

Second check — CMPA $09 then JC (“jump if Carry”): this CPU’s Carry flag after CMPA means “the accumulator is strictly greater than the operand” (see the 4-Function Calculator tutorial for the long version, and why this trips up naive “is A >= B” tests elsewhere). JC firing means the keypress was 10 or higher. Digit and hex-letter buttons arrive as their raw nibble value, 0–15, so this single check is exactly what separates plain digits 0–9 from hex letters A–F and everything else.

Only a keypress that survives both checks reaches the STA that puts it on the display — and because raw bytes $00–$09 render as their matching digit characters, whatever digit was pressed appears exactly as itself.

# 6. Lab 2e: A Chasing LED Pattern

$F032 lights the six LEDs directly from the bits of whatever byte is stored there — bit 5 is the leftmost LED, bit 0 the rightmost. This program starts with only bit 5 set and shifts that single lit bit rightward one place per loop:

LOOPA:     LDA     $20        # Load accumulator with $20

LOOPB:     STA     [SIXLEDS]  # Store accumulator to six LEDs

SHR                # Shift accumulator 1 bit to the right

JNZ     [LOOPB]    # Jump to LOOPB if Z flag not set

JMP     [LOOPA]    # ..else jump to LOOPA

$20 is 00100000 in binary. SHR shifts every bit one place right, discarding whatever falls off the bottom and filling the new top bit with 0:

| Value | Binary | LED lit |
| --- | --- | --- |
| $20 | 00100000 | leftmost |
| $10 | 00010000 |  |
| $08 | 00001000 |  |
| $04 | 00000100 |  |
| $02 | 00000010 |  |
| $01 | 00000001 | rightmost |
| $00 | 00000000 | (none — SHR of $01 shifts the bit off the end) |

Each value is written to SIXLEDS and shown before the next SHR runs, so the lit LED visibly walks left to right. The instant the shift produces $00, JNZ stops firing — SHR sets the Zero flag on a zero result exactly like every other instruction in this project — and the loop falls through to JMP [LOOPA], reloading $20 and starting the sweep over. There is no exit condition; Stop or Power Off is the only way out.

# 7. Lab 2f: Printing a Message

This is the same table-walking idea the Terminal Print Message exercise uses — that exercise is built on exactly this pattern. The message is stored as plain ASCII bytes right after the code, and the Index register (X) walks through them:

BLDX        0       # Load the index register with 0

LOOP:      LDA     [MESSAGE,X] # Load accumulator with character

JZ      [$0000]     # If character = $00 jump to $0000

STORE:     STA     [MAINDISP]  # ... else write character to display

INCX                # Increment the index register

JMP     [LOOP]      # Jump back to LOOP

MESSAGE:  .BYTE $48, $45, $4C, $4C, $4F, $20

.BYTE $57, $4F, $52, $4C, $44, $21, $00

[MESSAGE,X] is indexed addressing: “read the byte at MESSAGE’s address, plus whatever X currently holds.” With X starting at 0, the first pass reads MESSAGE+0 ($48, ‘H’); INCX makes it 1 for the next pass; and so on down the table. JZ checks the character just loaded before doing anything else with it — the moment the terminating $00 byte turns up, JZ jumps to $0000 and the program stops (see Section 2’s note on why that works). Every character before the $00 gets written to the display first.

The message is split across two .BYTE lines under one MESSAGE label — the assembler doesn’t care how many .BYTE directives a table is split across, only that they run one after another with no other code between them.

# 8. Try It Yourself

Assemble each file in turn, Load → CPU, click Run. For 2a/2b/2c/2f, just watch the display. For 2d, press digit keys and confirm only 0–9 show up. For 2e, watch the LED row and try Step instead of Run to see each SHR happen one at a time.

Open Display → Port Map Status alongside any of these to watch $F011/$F031/$F032 change live, and the Registers panel to watch the accumulator, Index register, and flags change instruction by instruction.

# 9. Ideas to Extend These Programs

- 2a/2c/2f: replace JMP [$0000] with HALT ($3C) and compare — the display ends up the same either way; only the Registers panel shows the difference.

- 2c: count up instead of down, or add a delay loop between the STA and the DECA so each digit is visible for a moment instead of flashing past.

- 2d: also accept hex letters A–F by comparing against $0F instead of $09, or add a Clear-key check the way every later Calculator exercise’s DO_CLEAR does.

- 2e: make the sweep bounce instead of restart (switch to SHL once you reach $01 and walk back up before switching back to SHR), or light two LEDs at once.

- 2f: change MESSAGE to your own text (remembering the final $00 terminator), or replace JZ [$0000] with a proper HALT.

# Appendix: Complete Source Listings

## tutorial/15a_lab2a_clear_display.asm

# ================================================================

# 15a_lab2a_clear_display.asm

# Beboputer Hands-On Tutorial — Lab 2a

#

# The shortest possible Beboputer program: clear the Calculator's

# main display and stop.

#

# What this program does

# -----------------------

#   Writes the display's "clear" code ($10) to the display port

#   ($F031), then jumps to address $0000 and stays there.

#

# How it works

# -------------

#   Four instructions, no labels, nothing but raw numbers - as bare

#   as a Beboputer program gets:

#

#     LDA  $10        load the accumulator with the literal value

#                      $10 - the byte this Calculator treats as

#                      "clear the display" (see tools/calculator.py's

#                      write_display: $0D, $10, and $1B all clear it)

#     STA  [$F031]    store the accumulator to address $F031 - the

#                      Calculator's main display port

#     JMP  [$0000]    jump to address $0000

#

#   That last jump is worth pausing on. This CPU has no HALT-and-

#   stay-halted state you fall into automatically - once the last

#   instruction runs, the Program Counter simply keeps advancing

#   into whatever comes next unless you tell it otherwise. Address

#   $0000 is always zeroed out on power-on and stays that way unless

#   a program is deliberately loaded there, and a zeroed byte

#   decodes as opcode $00 - NOP, "do nothing, move to the next

#   instruction." So JMP [$0000] sends the CPU into an address full

#   of NOPs, where it will keep fetching NOP after NOP forever - not

#   a true halt, but a program that has, for all practical purposes,

#   stopped doing anything. (This CPU does have a real HALT

#   instruction, opcode $3C - exercise 09 uses it. Lab 2a's "jump

#   into zeroed memory" trick is the more old-fashioned way of

#   getting the same practical result, and is worth recognizing

#   since you'll see it again in Lab 2c/2d/2f below.)

#

#   Notice this program has no .EQU declarations and no labels at

#   all - $4000, $10, and $F031 are typed as plain numbers every

#   time. That is deliberate: Lab 2b takes this exact program and

#   rewrites it with named constants, so you can compare the two

#   side by side and see directly what labels buy you.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run. The Calculator's display goes

#   blank (or stays blank, if it already was) and the program then

#   sits idle.

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status and watch $F031 receive $10 the

#   moment you click Run, then step through with Step to see the

#   Program Counter land on $0000 and stay in the 0000-0000

#   neighborhood, incrementing through NOPs.

#

# Try this next

# ---------------

#   - Replace JMP [$0000] with HALT ($3C) and compare - Display ->

#     Port Map Status and the Registers panel should look identical

#     either way, since both leave the display showing the same

#     thing; the difference only shows up in the Registers panel

#     (HALT stops the Program Counter dead; JMP [$0000] leaves it

#     ticking through NOPs).

#   - Change $10 to a different value and see what else the display

#     does with it (see tools/calculator.py's write_display for the

#     full list of what each byte range means).

# ================================================================

.ORG     $4000	# Set program's origin to address $4000

LDA     $10        # Load accumulator with clear code

STA     [$F031]    # Store accumulator to address $F031

JMP     [$0000]    # Jump to address $0000

.END                # This is the end of the program

## tutorial/15b_lab2b_labeled_constants.asm

# ================================================================

# 15b_lab2b_labeled_constants.asm

# Beboputer Hands-On Tutorial — Lab 2b

#

# The exact same program as Lab 2a - clear the display and stop -

# rewritten using named constants instead of raw numbers.

#

# What this program does

# -----------------------

#   Identical behavior to 15a_lab2a_clear_display.asm: writes the

#   display's clear code to the display port, then stops. Assemble

#   both and compare the bytes - they produce the same program.

#

# How it works

# -------------

#   .EQU gives a name to a constant value at assembly time. Every

#   place that name appears later gets replaced with the value

#   during assembly - the CPU never sees "CLRCODE" or "MAINDISP",

#   only the numbers $10 and $F031 they stand for:

#

#     CLRCODE:  .EQU  $10       # Special code to clear the main display

#     MAINDISP: .EQU  $F031     # Address of output port for main display

#

#   With those two names defined, the body of the program reads

#   like a sentence instead of a string of addresses:

#

#     LDA     CLRCODE     # Load accumulator with clear code

#     STA     [MAINDISP]  # Store accumulator to main display

#     JMP     [$0000]     # Jump to address $0000

#

#   Compare that to Lab 2a's LDA $10 / STA [$F031] - same two

#   instructions, same two numbers, but "CLRCODE" and "MAINDISP"

#   tell you what those numbers mean without needing a comment to

#   explain it. This is exactly the same trick exercises 01 onward

#   use everywhere (SW1, LED8, KEY, DISP, and so on) - .EQU costs

#   nothing at runtime and makes every later exercise far easier to

#   read than Lab 2a's style would be once a program gets longer

#   than a handful of lines.

#

#   Note that MAINDISP is used with the indirect-address brackets,

#   [MAINDISP], exactly the way [$F031] was written directly in Lab

#   2a - a label stands in for a number, so it follows the same

#   addressing-mode rules as the number it replaces.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run - you should see exactly the

#   same result as Lab 2a. Then open both files side by side and

#   compare: everywhere Lab 2a has a raw number, Lab 2b has a name

#   that explains it.

#

# Watch it happen

# -----------------

#   Open the Memory Walker after assembling and look at the bytes

#   at $4000 onward - they are identical to Lab 2a's, byte for byte.

#   Labels are a convenience for you while writing the program; the

#   assembled program itself doesn't know they ever existed.

#

# Try this next

# ---------------

#   - Add a third .EQU, e.g. EXITADDR: .EQU $0000, and use JMP

#     [EXITADDR] instead of JMP [$0000] - now every address in the

#     program has a name.

# ================================================================

CLRCODE:  .EQU     $10        # Special code to clear the main display

MAINDISP: .EQU     $F031      # Address of output port for main display

.ORG     $4000	# Set program's origin to address $4000

LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Store accumulator to main display

JMP     [$0000]    # Jump to address $0000

.END                # This is the end of the program

## tutorial/15c_lab2c_countdown_loop.asm

# ================================================================

# 15c_lab2c_countdown_loop.asm

# Beboputer Hands-On Tutorial — Lab 2c

#

# Clear the display, then count down from 9 to 1 on it.

#

# What this program does

# -----------------------

#   Clears the display (as in Labs 2a/2b), then shows 9, 8, 7, 6,

#   5, 4, 3, 2, 1 in quick succession before stopping - too fast to

#   read each digit individually at full speed, but easy to follow

#   with Step.

#

# How it works

# -------------

#   The first loop this series introduces - decrement-and-test,

#   the same shape almost every later exercise's loops are built

#   from:

#

#     LDA     $09        # Load the accumulator with $09

#     LOOP:      STA     [MAINDISP] # Store accumulator to the main display

#                DECA               # Decrement the accumulator

#                JNZ     [LOOP]     # Jump to LOOP if ACC isn't zero

#

#   ACC starts at 9. Each pass through LOOP shows the CURRENT value

#   before decrementing it, so the display sees 9 first, then 8,

#   then 7, and so on. DECA subtracts 1 and sets the Zero flag when

#   the result lands on exactly 0; JNZ ("jump if not zero") keeps

#   the loop going for every value except that last one. So the

#   sequence is: show 9, decrement to 8, still nonzero, loop; show

#   8, decrement to 7, loop; ...; show 1, decrement to 0, Zero flag

#   set, JNZ does NOT jump - the loop ends right after 1 was shown,

#   which is exactly the "9 downto 1" the header promises (0 is

#   never displayed, since the loop stops as soon as ACC reaches it,

#   before another STA can run).

#

#   $09 down to $01 are raw values, not ASCII text - but they show

#   up as the digit characters "9" through "1" anyway, because the

#   display port has a special case for exactly this range: raw

#   bytes $00-$09 are shown as the digit characters '0'-'9' (see

#   tools/calculator.py's write_display), on top of its normal

#   handling of printable ASCII. This is the same convention every

#   Calculator exercise relies on when it echoes a keypad digit

#   straight to the display without converting it to ASCII first.

#

#   Like Lab 2a, this program ends with JMP [$0000] rather than

#   HALT - see 15a's note for why that works.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run - the display flashes through

#   9 down to 1 almost instantly. Switch to Step instead of Run to

#   watch each digit land one at a time, and watch the accumulator

#   count down in the Registers panel in lockstep.

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status and Step through the program -

#   you'll see $F031 receive 9, then 8, then 7, ... down to 1, one

#   value per trip through LOOP.

#

# Try this next

# ---------------

#   - Count up instead of down: start ACC at $01, INCA instead of

#     DECA, and stop the loop once ACC passes 9 (CMPA $09 then a

#     conditional jump, the way later exercises test ranges).

#   - Insert a delay loop between the STA and the DECA so each digit

#     is visible for a moment instead of flashing past - Lab 2e's

#     inner SHR loop is a good model for a simple busy-wait.

# ================================================================

CLRCODE:  .EQU     $10        # Special code to clear the main display

MAINDISP: .EQU     $F031      # Address of output port for main display

.ORG     $4000	# Set program's origin to address $4000

LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Store accumulator to main display

LDA     $09        # Load the accumulator with $09

LOOP:      STA     [MAINDISP] # Store accumulator to the main display

DECA               # Decrement the accumulator

JNZ     [LOOP]     # Jump to LOOP if ACC isn't zero

JMP     [$0000]    # Jump to address $0000

.END                # This is the end of the program

## tutorial/15d_lab2d_keypad_echo_digits.asm

# ================================================================

# 15d_lab2d_keypad_echo_digits.asm

# Beboputer Hands-On Tutorial — Lab 2d

#

# Clear the display, then echo digit keys 0-9 to it - and quietly

# ignore everything else.

#

# What this program does

# -----------------------

#   Clears the display, then sits in a loop reading the keypad

#   port. Press a digit key (0-9) and it appears on the display.

#   Press anything else - an operator, Clear, a hex letter, Sin,

#   whatever - and nothing happens; the program just keeps waiting.

#

# How it works

# -------------

#   A busy-wait loop reading one port ($F011, the keypad), guarded

#   by two range checks before anything reaches the display:

#

#     LOOP:      LDA     [KEYPAD]   # Load the accumulator from the keypad

#                JN      [LOOP]     # Jump to LOOP if N flag is set

#                CMPA    $09        # Compare accumulator to code $09

#                JC      [LOOP]     # Jump to LOOP if C flag is set

#                STA     [MAINDISP] # ... else store accumulator to display

#                JMP     [LOOP]     # Jump to LOOP

#

#   First check - JN ("jump if Negative"): the keypad port sits at

#   $FF ("idle, nothing pressed") whenever no key is down, and $FF

#   has its top bit set, which LDA's N flag reports as "negative."

#   Any byte $80-$FF - not just $FF itself - trips this same check,

#   which conveniently also screens out any button whose code

#   happens to live up in that range, with one read and one branch.

#   As long as nothing is pressed, JN keeps firing and the loop just

#   spins on this line.

#

#   Second check - CMPA $09 then JC ("jump if Carry"): once a key

#   with its top bit clear gets through the first check, this one

#   asks "is it bigger than 9?" This CPU's Carry flag after CMPA

#   means "the accumulator is STRICTLY GREATER THAN the operand"

#   (see exercise 10 for the long version of this, and why it trips

#   up naive "is A >= B" tests elsewhere) - so JC firing means the

#   keypress was 10 or higher, and gets looped past just like an

#   idle read did. Digit and hex-letter buttons arrive as their raw

#   nibble value, 0-15 (see exercise 10's note on tools/diy_button.py)

#   - so this check is exactly what separates plain digits 0-9 from

#   hex letters A-F (10-15) and everything else, without needing to

#   list out ten separate values.

#

#   Only a keypress that survives both checks - not negative, and

#   not greater than 9 - reaches the STA that puts it on the

#   display. Because raw bytes $00-$09 show up as the digit

#   characters '0'-'9' on this display (the same convention Lab 2c

#   and every Calculator exercise relies on), whatever digit you

#   pressed appears exactly as itself.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, switch the Calculator on if it

#   isn't already, and press digit keys - each one appears on the

#   display. Press Clear, an operator, or a letter key and watch

#   nothing happen.

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status and watch $F011 as you press

#   different keys - notice it always briefly shows the byte you

#   pressed and then reverts to $FF (see exercise 05's note on this

#   read-clear-latch behavior), and that only digit keys make it to

#   $F031.

#

# Try this next

# ---------------

#   - Also accept hex letters A-F: change the second check to

#     compare against $0F instead of $09.

#   - Add a Clear-key check ($1B) that resets the display, the way

#     every later Calculator exercise's DO_CLEAR does.

# ================================================================

CLRCODE:  .EQU     $10        # Special code to clear the main display

MAINDISP: .EQU     $F031      # Address of output port for main display

KEYPAD:   .EQU     $F011      # Address of input port for keypad

.ORG     $4000	# Set program's origin to address $4000

LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Store accumulator to main display

LOOP:      LDA     [KEYPAD]   # Load the accumulator from the keypad

JN      [LOOP]     # Jump to LOOP if N flag is set

CMPA    $09        # Compare accumulator to code $09

JC      [LOOP]     # Jump to LOOP if C flag is set

STA     [MAINDISP] # ... else store accumulator to display

JMP     [LOOP]     # Jump to LOOP

.END                # This is the end of the program

## tutorial/15e_lab2e_led_chase_pattern.asm

# ================================================================

# 15e_lab2e_led_chase_pattern.asm

# Beboputer Hands-On Tutorial — Lab 2e

#

# Walk a single lit LED across the Calculator's six-LED row, one

# position at a time, forever.

#

# What this program does

# -----------------------

#   Clears the display, then lights the leftmost LED alone, then

#   the next one, then the next, all the way to the rightmost -

#   then starts over from the left again. A classic "chase" or

#   "scanner" pattern, built from one instruction most exercises

#   before this one haven't needed yet: SHR.

#

# How it works

# -------------

#   Exercise 07 showed that $F032 lights the six LEDs directly from

#   the bits of whatever byte you store there - bit 5 is the

#   leftmost LED, bit 0 the rightmost. This program starts with only

#   bit 5 set and shifts that single lit bit rightward one place

#   per trip through the inner loop:

#

#     LOOPA:     LDA     $20        # Load accumulator with $20

#     LOOPB:     STA     [SIXLEDS]  # Store accumulator to six LEDs

#                SHR                # Shift accumulator 1 bit to the right

#                JNZ     [LOOPB]    # Jump to LOOPB if Z flag not set

#                JMP     [LOOPA]    # ..else jump to LOOPA

#

#   $20 is 00100000 in binary - bit 5 set, every other bit clear.

#   SHR shifts every bit one place to the right, discarding whatever

#   falls off the bottom and filling the new top bit with 0. Watch

#   what that does across six passes through LOOPB:

#

#       $20 = 00100000   (leftmost LED)

#       $10 = 00010000

#       $08 = 00001000

#       $04 = 00000100

#       $02 = 00000010

#       $01 = 00000001   (rightmost LED)

#       $00 = 00000000   <- SHR of $01 - the bit finally shifts off

#                            the end entirely, and nothing replaces it

#

#   Each of those six values gets written to SIXLEDS and shown

#   before the next SHR runs, so the lit LED visibly walks from left

#   to right. The moment the shift produces $00, JNZ ("jump if Z

#   flag not set") stops firing - SHR sets the Zero flag exactly

#   when its result is zero, the same flag every other instruction

#   in this project sets on a zero result - so the loop falls

#   through to JMP [LOOPA], which reloads $20 and starts the whole

#   sweep over from the left again. The result is a lit LED that

#   walks all the way across and then jumps back to the start,

#   repeating forever (there is no exit condition in this program -

#   Stop or Power Off on the Calculator is the only way out).

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, and watch the LED row - one

#   LED lit at a time, sweeping left to right, restarting at the

#   left each time it reaches the far right.

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status and watch $F032's value cycle

#   through $20, $10, $08, $04, $02, $01, then jump back to $20 -

#   Step through slowly to see SHR's effect on the accumulator in

#   the Registers panel one bit at a time.

#

# Try this next

# ---------------

#   - Make it bounce instead of restart: once you reach $01, switch

#     to SHL and walk back up to $20 before switching back to SHR,

#     instead of jumping straight back to LOOPA.

#   - Light two LEDs at once (e.g. start at $22 = 00100010) and

#     watch both march across in step.

#   - Slow it down: exercise 15c's note on adding a delay loop

#     applies here too - without one, the sweep runs faster than the

#     eye can separate each individual step at full emulator speed.

# ================================================================

CLRCODE:  .EQU     $10        # Special code to clear the main display

MAINDISP: .EQU     $F031      # Address of output port for main display

SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs

.ORG     $4000	# Set program's origin to address $4000

LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Store accumulator to main display

LOOPA:     LDA     $20        # Load accumulator with $20

LOOPB:     STA     [SIXLEDS]  # Store accumulator to six LEDs

SHR                # Shift accumulator 1 bit to the right

JNZ     [LOOPB]    # Jump to LOOPB if Z flag not set

JMP     [LOOPA]    # ..else jump to LOOPA

.END                # This is the end of the program

## tutorial/15f_lab2f_hello_world.asm

# ================================================================

# 15f_lab2f_hello_world.asm

# Beboputer Hands-On Tutorial — Lab 2f

#

# Print "HELLO WORLD!" on the Calculator's main display.

#

# What this program does

# -----------------------

#   Clears the display, then writes out the message "HELLO WORLD!"

#   one character at a time and stops.

#

# How it works

# -------------

#   This is the same table-walking idea exercise 09 uses for its own

#   message - in fact, if this looks familiar, it's because exercise

#   09 is built on exactly this pattern. The message is stored as

#   plain ASCII bytes right after the code, and the Index register

#   (X) walks through them one at a time:

#

#     BLDX        0       # Load the index register with 0

#     LOOP:      LDA     [MESSAGE,X] # Load accumulator with character

#                JZ      [$0000]     # If character = $00 jump to $0000

#     STORE:     STA     [MAINDISP]  # ... else write character to display

#                INCX                # Increment the index register

#                JMP     [LOOP]      # Jump back to LOOP

#

#     MESSAGE:  .BYTE $48, $45, $4C, $4C, $4F, $20

#                #      H    E    L    L    O  SPACE

#                .BYTE $57, $4F, $52, $4C, $44, $21, $00

#                #      W    0    R    L    D    !  NUL

#

#   [MESSAGE,X] is indexed addressing: "read the byte at MESSAGE's

#   address, plus whatever X currently holds." With X starting at 0,

#   the first pass reads MESSAGE+0 ($48, 'H'); INCX makes it 1 for

#   the next pass (MESSAGE+1, $45 'E'); and so on down the table.

#   JZ checks the character that was just loaded before doing

#   anything else with it - the moment a $00 byte turns up (the

#   very last entry in the table, deliberately placed there as a

#   terminator), JZ jumps to $0000 and the program stops (see Lab

#   2a's note on why "jump to zeroed memory" works as a stopping

#   point). Every character before that $00 gets written to the

#   display and the loop continues.

#

#   Notice the message is written as two separate .BYTE lines under

#   one MESSAGE label - the assembler doesn't care how many .BYTE

#   directives you split a table across, only that they all come one

#   after another with no other code between them; the label just

#   marks where the very first byte lands, and INCX walks past the

#   line break exactly the same as it would if it were all one line

#   (see exercise 09 for the same technique used with a shorter

#   message).

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run - "HELLO WORLD!" appears on the

#   display, one character at a time (fast enough that it looks

#   instantaneous at full run speed - switch to Step to watch each

#   character land individually).

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status and Step through the program to

#   watch $F031 receive each character in turn, and watch the Index

#   register count up 0, 1, 2, ... in the Registers panel as X walks

#   the table.

#

# Try this next

# ---------------

#   - Change MESSAGE to your own text - remember the final $00

#     terminator, or the loop will walk straight past the end of the

#     table into whatever bytes happen to follow it.

#   - Replace JZ [$0000] with a proper HALT ($3C), the way exercise

#     09 does, and compare the two approaches directly.

# ================================================================

CLRCODE:  .EQU     $10         # Special code to clear the main display

MAINDISP: .EQU     $F031       # Address of main display output port

.ORG     $4000	 # Set program's origin to address $4000

LDA     CLRCODE     # Load accumulator with clear code

STA     [MAINDISP]  # Store accumulator to main display

BLDX        0       # Load the index register with 0

LOOP:      LDA     [MESSAGE,X] # Load accumulator with character

JZ      [$0000]     # If character = $00 jump to $0000

STORE:     STA     [MAINDISP]  # ... else write character to display

INCX                # Increment the index register

JMP     [LOOP]      # Jump back to LOOP

MESSAGE:  .BYTE $48, $45, $4C, $4C, $4F, $20

#      H    E    L    L    O  SPACE

.BYTE $57, $4F, $52, $4C, $44, $21, $00

#      W    0    R    L    D    !  NUL

.END