# Beboputer — Raspberry Pi Installation Guide

PY-DIYCALCULATOR: Virtual 8-bit Computer — Deployment Steps

# 1. Overview

This guide covers installing Beboputer (PY-DIYCALCULATOR) on a Raspberry Pi. There are three supported installation methods, described in order of recommendation: the .deb package (recommended), the direct one-shot install script, and a per-user install script for machines without sudo access.

| Requirement | Details |
| --- | --- |
| Python | 3.8 or later (installed automatically if missing) |
| PyQt5 | python3-pyqt5 (installed automatically if missing) |
| Supported Pi models | Pi 1, 2, 3, 4, 5, Zero, Zero 2 W — any Raspberry Pi OS / Raspbian / Debian-based OS |
| Package architecture | all (pure Python source — no compilation needed) |

# 2. Option A — Install via .deb Package (Recommended)

This is the cleanest method: it registers the app with dpkg/apt so it can be listed, upgraded, and removed like any other Debian package.

## 2.1 Build the package

The package is built once (on a development machine) and then copied to any number of Pi devices. It can be built on Windows using WSL, or directly on Linux:

wsl bash bin/beboputer_v7/RPI_INSTALL/build_deb.sh

or, run directly on Linux:

bash bin/beboputer_v7/RPI_INSTALL/build_deb.sh

This produces:

dist/beboputer_7.0.0_all.deb

Note: build_deb.sh stages the package under /tmp so dpkg-deb can set correct Unix file permissions — this step fails if run against a Windows-mounted path (e.g. /mnt/c/...), so always run it from WSL's own Linux filesystem or a native Linux machine.

## 2.2 Copy the package to the Pi

Transfer the .deb file to the Raspberry Pi using a USB drive, SCP, or any file transfer method of your choice.

## 2.3 Install

On the Raspberry Pi, run:

sudo dpkg -i beboputer_7.0.0_all.deb
sudo apt-get install -f

The second command resolves and installs any missing dependencies (Python 3, python3-pyqt5) that dpkg could not satisfy on its own.

## 2.4 Uninstall

sudo apt-get remove beboputer

# 3. Option B — Direct Install Script (No Build Required)

This method installs straight from Python source with a single script, system-wide, without building a package first.

- Copy the entire Bebop_python project folder to the Raspberry Pi.

- From the project root, run:

sudo bash bin/beboputer_v7/RPI_INSTALL/install_rpi.sh

The script must be run as root (via sudo) — it checks for this and exits with an instruction message if not.

install_rpi.sh performs the following steps automatically:

- Verifies Python 3 is installed; installs it via apt if missing.

- Verifies PyQt5 is importable; installs python3-pyqt5 via apt if missing.

- Copies bin/, BITMAPS/, Config/, Data/, databook/, and WorkInProgress/ into /usr/share/beboputer.

- Creates a launcher at /usr/local/bin/beboputer.

- Creates a desktop menu entry (PY-DIYCALCULATOR, under Education / Emulator).

# 4. Option C — Per-User Install (No sudo Required)

install_linux.sh installs Beboputer into the current user's home directory instead of system-wide paths, for Pi users who do not have (or do not want to use) sudo access. This is a separate install location from Options A and B — do not mix installation methods on the same machine without removing the previous one first.

chmod +x install_linux.sh
./install_linux.sh

This script:

- Installs Python 3 and PyQt5 if not already present (using sudo apt-get for these two package installs only).

- Copies the app to ~/.local/share/beboputer.

- Creates a launcher script at ~/.local/bin/beboputer.

- Creates a .desktop file so the app appears in the application menu.

Note: If ~/.local/bin is not already on your PATH, the script prints the line to add to ~/.bashrc or ~/.profile.

# 5. What Gets Installed — File Locations

| Location | Contents |
| --- | --- |
| /usr/share/beboputer/ | Application files: bin/, BITMAPS/, Config/, Data/, databook/, WorkInProgress/ (Options A & B) |
| /usr/local/bin/beboputer | Launcher script — type beboputer in a terminal to run (Options A & B) |
| ~/.local/share/beboputer | Application files (Option C, per-user install) |
| ~/.local/bin/beboputer | Launcher script (Option C) |
| /usr/share/applications/beboputer.desktop | Application-menu entry, listed as “PY-DIYCALCULATOR” |

# 6. Launching the Application

After any of the three install methods, Beboputer can be started either of two ways:

- From a terminal: type beboputer and press Enter.

- From the desktop: open the application menu and look for “PY-DIYCALCULATOR” under the Education or Emulator category.

# 7. Troubleshooting

## PyQt5 fails to install via apt

On some Debian-based systems apt may not have python3-pyqt5 available. As a fallback, install it with pip:

pip3 install pyqt5 --break-system-packages

## “beboputer” command not found after install (Option C)

Add ~/.local/bin to your PATH as instructed by the installer output, then reload your shell:

export PATH="$HOME/.local/bin:$PATH"
source ~/.bashrc

## Permission errors saving files

When installed system-wide (Options A or B), /usr/share/beboputer is root-owned, so saving compiled programs or work-in-progress files there will fail for a normal user. The application automatically falls back to a writable folder at ~/beboputer/ when this happens — no manual action is required.

## Building the .deb fails with a dpkg-deb error

This almost always means build_deb.sh was run against a Windows-mounted path. Re-run it from a native Linux filesystem path inside WSL (not /mnt/c/...), or build directly on a Linux machine.

# Appendix: Quick-Launch Script (Development Use)

RPI_INSTALL/run_beboputer.sh is a minimal alternative launcher intended for development use, not for the packaged installs above. It assumes the project has been copied to ~/beboputer/Bebop_python on the Pi:

bash ~/run_beboputer.sh

Caveat: This script hard-codes the path ~/beboputer/Bebop_python/bin/run_beboputer_v7.py. It only works if the project was copied there manually — it is unrelated to, and not updated by, the installers in Options A–C above.