# Beboputer Hands-On Tutorial

Exercising the Workbench, Calculator, Keyboard, and Terminal with real assembly programs

# 1. Before You Start

This tutorial gives you working, tested assembly-language programs for each of the four interactive peripherals in PY-DIYCALCULATOR: Workbench 1 (switches, LEDs, 7-segment displays), the Calculator (hex keypad, LCD display, indicator LEDs), the on-screen Keyboard (QWERTY typing), and the Terminal (text output). It also covers the three tools you will lean on constantly while working through the exercises: the Memory Walker, the I/O Ports Display, and the System Clock. Every example uses the real memory-mapped ports and instruction set implemented in the emulator, and every program has been traced instruction-by-instruction against the CPU's actual opcode behaviour.

## 1.1 The One Rule That Trips Everyone Up: Power On First

Important: Turning the Calculator's On/Off button ON purges all 64KB of RAM and resets the CPU. If you assemble and load a program before powering on, switching the Calculator on afterward will wipe it out. Always power on the Calculator BEFORE you assemble and load your program.

Here is why: the Calculator's On/Off button is wired to more than just the calculator screen. Powering on also enables the Terminal, enables the Workbench's switches and displays, and (the surprising part) automatically purges RAM, resets the CPU, and starts it running. So the safe sequence for every exercise in this tutorial is:

- Launch PY-DIYCALCULATOR. The Calculator window opens automatically (powered off).

- Click the Calculator's On/Off button to power everything on.

- Open whichever panels the exercise needs (Workbench 1, Keyboard, Terminal) from the Tools and Display menus.

- Open Tools -> Assembler / Editor, type in the example program, and click Assemble.

- Click Load -> CPU (this button becomes active once assembly succeeds).

- Use the Calculator's Step or Run buttons, or the Memory Walker's own STEP column, to execute your program.

Also note: Run, Step, and Reset only exist as buttons on the Calculator's bottom bar - there is no separate menu item for them. Even a Workbench-only or Terminal-only exercise still needs the Calculator open and powered on, because that is the only place to start your program running (the Memory Walker can single-step independently, but Run still lives on the Calculator).

## 1.2 Menu Reference

| Panel / Tool | Menu Path |
| --- | --- |
| Calculator | Tools -> Calculator... |
| Workbench 1 | Tools -> Workbench 1... |
| Keyboard | Tools -> Keyboard... |
| Terminal | Display -> Terminal |
| Memory Walker | Memory -> Memory Walker (already open by default at startup) |
| I/O Ports Display | Display -> Port Map Status |
| System Clock | Tools -> System Clock... |
| Assembler / Editor | Tools -> Assembler / Editor... |
| CPU Registers | Display -> CPU Registers |
| Purge RAM | File -> Purge RAM... |

## 1.3 Assembler Syntax Quick Reference

| Element | Syntax | Example |
| --- | --- | --- |
| Comment | # to end of line | # read the switches |
| Label | NAME: in column 1 | LOOP: |
| Constant | NAME: .EQU value | SW1: .EQU $F000 |
| Load address | .ORG address | .ORG $4000 |
| Reserve bytes | .BYTE v1, v2, ... | MSG: .BYTE $48, $49, $00 |
| Immediate operand | bare hex value | LDA $FF |
| Direct (memory) operand | [address or label] | LDA [SW1] |
| Indexed operand | [address or label,X] | LDA [MSG,X] |
| Indirect operand | [[address or label]] | JMP [[$4000]] |
| Pre-indexed indirect | [[address or label,X]] | LDA [[$4000,X]] |
| End of source | .END address | .END $4000 |

All the examples in this tutorial write hex values with a $ prefix ($4000, $0A, and so on), which is the form used consistently throughout the built-in Assembler / Editor's own Insert menu snippets.

## 1.4 Instruction Set Used in This Tutorial

| Mnemonic | What it does |
| --- | --- |
| LDA | Load the Accumulator (immediate, direct, indexed, indirect, or pre-indexed indirect) |
| STA | Store the Accumulator to memory (direct, indexed, indirect, pre-indexed indirect - no immediate form) |
| ADD / SUB | Add / subtract into the Accumulator |
| CMPA | Compare the Accumulator with a value. Sets the Zero flag when equal. Unusually, Carry is set only when the Accumulator is strictly greater than the operand. |
| AND / OR / XOR | Bitwise logic against the Accumulator (immediate, direct, or indexed) |
| SHL | Shift the Accumulator left one bit (carry out captures the vacated bit) |
| BLDX | Load the 16-bit Index register (immediate or direct) |
| INCX / DECX | Increment / decrement the Index register |
| JMP | Unconditional jump (direct, indirect, or pre-indexed indirect) |
| JZ / JNZ | Jump if the Zero flag is / is not set |
| JC / JNC | Jump if Carry is / is not set |
| JSR / RTS | Call a subroutine / return from one |
| HALT | Stop the CPU |

Tip: LDA already sets the Zero and Negative flags from the value it loads. In the message-printing exercise later in this tutorial, a loop tests for a zero byte with a plain JZ right after the LDA - no separate CMPA is needed.

# 2. Tools You Will Use While Debugging

These three tools are not tied to any one exercise - you will reach for all of them constantly while working through this tutorial. Get comfortable with them now.

## 2.1 The Memory Walker

The Memory Walker is already open when the application starts (it is one of the three panels shown by default, alongside the Message Display and the Calculator), and it can also be reopened from Memory -> Memory Walker. It shows a scrollable, live 256-row window of RAM with four columns: BP, STEP, ADDRESS, and DATA.

| Control | What it does |
| --- | --- |
| Address field + GO | Type a 4-digit hex address (e.g. 4000) and click GO to jump the view there. This locks the view in place - it will not auto-follow the Program Counter until you step from inside the Memory Walker again. |
| Click a BP cell | Toggles a breakpoint (shown as a red dot) at that row's address. |
| Click a STEP cell | Executes exactly one CPU instruction, independently of the Calculator's Step button, and updates the status line with the mnemonic that ran and the resulting Accumulator value. |
| RUN to BP | Runs the CPU until it hits any breakpoint, executes HALT, or reaches the safety limit of 500,000 steps. |
| Clear BPs | Removes every breakpoint. |
| Double-click a DATA cell | Lets you type a new hex byte directly into that RAM address - a live memory editor. |

While your program is running or single-stepping, an amber triangle marks the row at the current Program Counter, and the view automatically scrolls to keep it in sight (unless you have manually navigated there with GO).

Try this: Load the switches-to-LEDs program from the Workbench exercise below, then click the BP cell next to the LOOP label's address before pressing Run. Execution will stop the instant it reaches that address, and you can then use the STEP column to advance one instruction at a time and watch the DATA column change as STA writes to $F022.

Also useful: After assembling a program that includes a .BYTE table (see the message-printing exercise later in this tutorial), type the table's address into the Memory Walker and click GO to confirm the assembler laid out exactly the bytes you expect.

## 2.2 The I/O Ports Display

Open with Display -> Port Map Status. Unlike the Memory Walker, this panel does not show general RAM - it is a dedicated live monitor for exactly three fixed addresses: the ones the Calculator itself uses.

| Address | What is shown |
| --- | --- |
| $F031 | The last byte written to the Calculator's display, in hex and as a printable character/annotation. |
| $F032 | The last byte written to the Calculator's LEDs, in hex and as an 8-bit binary pattern. |
| $F011 | The current key code (hex + annotation) and the previous key code (hex + annotation), so you can see a keypress transition even though the port auto-clears back to $FF. |

Important: The I/O Ports Display does NOT show the Workbench's ports ($F000, $F001, $F021-$F024) or the Terminal's port ($F028) - only the three addresses above. For the Workbench exercises, watch the Workbench panel itself; the I/O Ports Display is most useful for the Calculator and Keyboard exercises.

The "I/P from Buttons" field is editable: type a hex value (e.g. 07) into it and press Enter, and it writes that value directly into RAM at $F011 - simulating a keypress without touching the Calculator or the on-screen Keyboard at all. This is a convenient way to feed a specific test value into a WAIT/poll loop while you are debugging it.

## 2.3 The System Clock

Open with Tools -> System Clock... This dialog sets how fast the emulated CPU runs, from 1 Hz up to 10,000 Hz (the default is 100 Hz). Type a new value and click OK; if Run is currently active, the change takes effect immediately.

Try this: Slow the clock down to around 2-5 Hz before running one of the WAIT-and-poll loops in this tutorial, so you can watch the Memory Walker's PC marker and the I/O Ports Display update on every single pass of the loop. Speed it back up to the default 100 Hz (or higher) once you just want the program to run.

# 3. Exercise 1 - Workbench 1

Open Tools -> Workbench 1... Workbench 1 gives you two 8-switch input banks and four different output displays, all wired to fixed ports:

| Port | Device |
| --- | --- |
| $F000 | Switch Bank 1 (input) |
| $F001 | Switch Bank 2 (input) |
| $F021 | 7-Segment, undecoded (bits 0-6 = segments a-g) |
| $F022 | 8-bit LED bar |
| $F023 | 7-Segment, decoded (low nibble = hex digit 0-F) |
| $F024 | Dual 7-Segment, decoded (high nibble = left digit, low nibble = right digit) |

## 3.1 Mirror the Switches on the LED Bar

The simplest possible I/O program: whatever Switch Bank 1 reads, the LED bar shows.

SW1:    .EQU    $F000
LED8:   .EQU    $F022
 
        .ORG    $4000
 
LOOP:   LDA     [SW1]
        STA     [LED8]
        JMP     [LOOP]
 
        .END    $4000

Assemble, Load -> CPU, then click Run on the Calculator. Flip switches in Switch Bank 1 and watch the LED bar respond immediately.

Debug it: Set a breakpoint on the LOOP address in the Memory Walker, click RUN to BP, then use the STEP column to advance one instruction at a time - you'll see the DATA byte at $F022 change the moment STA executes.

## 3.2 Show a Switch Value as a Hex Digit

This masks Switch Bank 1 down to its low nibble and shows it as a single hex digit (0-F) on the decoded 7-segment display.

SW1:    .EQU    $F000
SEG2:   .EQU    $F023
 
        .ORG    $4000
 
LOOP:   LDA     [SW1]
        AND     $0F        # keep only the low nibble
        STA     [SEG2]
        JMP     [LOOP]
 
        .END    $4000

## 3.3 Drive the Undecoded 7-Segment Directly

$F021 is not decoded - each of its low 7 bits drives one segment (a through g) directly. Any 7-bit pattern from the switches becomes a (probably nonsensical, and genuinely fun to explore) segment pattern.

SW1:    .EQU    $F000
SEG1:   .EQU    $F021
 
        .ORG    $4000
 
LOOP:   LDA     [SW1]
        AND     $7F        # keep bits 0-6 (segments a-g)
        STA     [SEG1]
        JMP     [LOOP]
 
        .END    $4000

## 3.4 Combine Both Switch Banks on the Dual 7-Segment Display

This one is a bit more advanced: it builds a single byte where the low nibble of Switch Bank 1 becomes the left digit and the low nibble of Switch Bank 2 becomes the right digit, using four SHL instructions to move a nibble into the high position.

SW1:    .EQU    $F000
SW2:    .EQU    $F001
SEG3:   .EQU    $F024
 
        .ORG    $4000
 
LOOP:   LDA     [SW1]
        AND     $0F        # low nibble of bank 1 -> left digit
        SHL
        SHL
        SHL
        SHL                # shift it into the high nibble
        STA     [TEMP]
        LDA     [SW2]
        AND     $0F        # low nibble of bank 2 -> right digit
        OR      [TEMP]     # combine hi(bank1) | lo(bank2)
        STA     [SEG3]
        JMP     [LOOP]
 
TEMP:   .BYTE   $00         # scratch byte (never executed - JMP always skips it)
 
        .END    $4000

Debug it: Navigate the Memory Walker to the TEMP label's address (use GO) to watch the intermediate shifted value land there on every pass.

# 4. Exercise 2 - The Calculator

Open Tools -> Calculator... Every key on the Calculator writes a code to port $F011 (the same port the on-screen Keyboard uses - see the note in the next exercise). Digit keys 0-9 and hex keys A-F are translated to their raw nibble value (0-15) before being written, which lines up perfectly with the decoded 7-segment display's own low-nibble convention.

## 4.1 Show the Last Key Pressed on a 7-Segment Display

Open Workbench 1 as well for this one, so you have somewhere to see the result. Press any digit or hex key (0-9, A-F) on the Calculator and watch the decoded 7-segment display echo it.

KEY:    .EQU    $F011
SEG2:   .EQU    $F023
 
        .ORG    $4000
 
WAIT:   LDA     [KEY]      # read the keypad port
        CMPA    $FF        # $FF means idle - no key waiting
        JZ      [WAIT]     # keep waiting
        AND     $0F        # keep the low nibble (0-9, A-F)
        STA     [SEG2]     # show it
        JMP     [WAIT]
 
        .END    $4000

How this works: Port $F011 is a read-clear latch: the moment your program reads a non-$FF value, the underlying byte snaps back to $FF so the same keypress is never processed twice. Your LDA still gets the real key value in the Accumulator before that happens - only the copy in RAM is cleared.

Watch it happen: Open the I/O Ports Display (Display -> Port Map Status) alongside this exercise. Its $F011 box shows both the current and previous key code with annotations, which makes the read-clear behaviour easy to see in real time. You can also type a hex value straight into its "I/P from Buttons" field to feed the loop a test key without touching the Calculator at all.

## 4.2 Write Text to the Calculator's Own Display

The Calculator's LCD is itself a memory-mapped output device at $F031. Printable ASCII (32-126) appends a character; codes $0D, $10, or $1B clear the display.

DISP:   .EQU    $F031
 
        .ORG    $4000
 
        LDA     $1B        # clear code
        STA     [DISP]
 
        LDA     $48        # 'H'
        STA     [DISP]
        LDA     $49        # 'I'
        STA     [DISP]
 
        HALT
 
        .END    $4000

Assemble, Load -> CPU, then Step through it a few instructions at a time (rather than Run) so you can watch each character land on the display before moving to the next. The I/O Ports Display's $F031 box will show each character the instant STA executes.

## 4.3 Light Up the Calculator's LEDs

$F032 drives the Calculator's six indicator LEDs - bit 5 is the leftmost, bit 0 the rightmost.

LEDS:   .EQU    $F032
 
        .ORG    $4000
 
        LDA     $2A        # 00101010 -> alternating LEDs on
        STA     [LEDS]
 
        HALT
 
        .END    $4000

# 5. Exercise 3 - The Keyboard and Terminal

Open Tools -> Keyboard... and Display -> Terminal. The on-screen Keyboard writes raw ASCII codes to the same port the Calculator uses, $F011 (letters and digits as their normal ASCII values - unlike the Calculator's hex keys, the Keyboard tool does not translate digits to raw nibbles). This exercise polls that port and echoes each keystroke to the Terminal at $F028.

Worth knowing: The on-screen Keyboard tool already echoes every keystroke straight to the Terminal itself, independent of any program you write - it is a direct convenience wire built into the tool. That means once your own program is also polling $F011 and writing to $F028, each keystroke will appear TWICE (once from the Keyboard tool's own wire, once from your program). This is expected - it is not a bug in your assembly code. Watching this doubling happen is actually a good way to confirm your program really is reading and reacting to each keypress.

## 5.1 Echo Typed Characters to the Terminal

This program also demonstrates translating one device's codes into another's: the Keyboard tool's ENTER key sends code $05, but the Terminal only recognises $0A as a newline, so the program converts one into the other before sending it on.

KEY:    .EQU    $F011
TERM:   .EQU    $F028
 
        .ORG    $4000
 
WAIT:   LDA     [KEY]      # read the keyboard port
        CMPA    $FF
        JZ      [WAIT]     # idle - keep waiting
 
        CMPA    $05        # is it the Keyboard tool's ENTER code?
        JNZ     [SEND]     # no - send the byte as typed
        LDA     $0A        # yes - translate to the Terminal's newline code
SEND:   STA     [TERM]
        JMP     [WAIT]
 
        .END    $4000

Assemble, Load -> CPU, click Run, then type on the on-screen Keyboard. Letters and digits appear in the Terminal (doubled, per the note above); pressing ENTER moves to a new line instead of printing the raw $05 byte.

Slow it down: If characters fly by too fast to follow, drop the System Clock to a few Hz (Tools -> System Clock...) so each poll of WAIT is easy to watch on the Memory Walker's PC marker.

# 6. Exercise 4 - Printing a Stored Message on the Terminal

This exercise does not need any input device - it prints a fixed message stored in the program itself, using the Index register to walk through a table of bytes. It is the best example in this tutorial for seeing .BYTE data, indexed addressing, and a loop-with-terminator pattern all at once.

TERM:   .EQU    $F028
 
        .ORG    $4000
 
        BLDX    $0000      # IX = 0 (index into MSG)
 
LOOP:   LDA     [MSG,X]    # load the next character
        JZ      [DONE]     # the $00 terminator ends the message
        STA     [TERM]     # send it to the Terminal
        INCX               # move to the next character
        JMP     [LOOP]
 
DONE:   HALT
 
MSG:    .BYTE   $48, $45, $4C, $4C, $4F, $2C, $20, $42
        .BYTE   $45, $42, $4F, $50, $55, $54, $45, $52
        .BYTE   $21, $0A, $00
 
        .END    $4000

MSG spells out "HELLO, BEBOPUTER!" followed by a newline ($0A) and a $00 terminator that ends the loop. Assemble, Load -> CPU, and click Run (or Step through it to watch IX count up and PC walk the loop on the CPU Registers panel).

Check your work: After assembling, look at the assembly listing in the output panel to find MSG's address, then type it into the Memory Walker's address field and click GO. The DATA column should show exactly the 19 bytes above, in order - a quick way to confirm the table assembled the way you expect before you even run the program.

Try this next: Change the .BYTE table to your own message. Remember every line must end with a $00 terminator for the JZ to find, and each .BYTE line can hold as many comma-separated values as you like (the Assembler / Editor's own Insert String helper wraps them at 8 per line, purely for readability).

# 7. Challenge: Put It All Together

Once each exercise above is working on its own, try combining them into one program: poll the Keyboard at $F011, echo each character to the Terminal at $F028 (translating ENTER to newline as in Exercise 3), and also show the low nibble of the last key code on the Workbench's decoded 7-segment display at $F023 (as in Exercise 2). All three pieces reuse the same WAIT-and-poll structure - the only new work is writing to a second and third output port inside the same loop body before jumping back to WAIT. Set a breakpoint at WAIT in the Memory Walker and use RUN to BP to step through the combined loop one keypress at a time.

# Appendix A: Full Port Reference

Every memory-mapped port used across the four exercises in this tutorial, in one place.

| Port | Device | Direction / Notes |
| --- | --- | --- |
| $F000 | Workbench Switch Bank 1 | Input - 8 toggle switches, bit 7 = leftmost |
| $F001 | Workbench Switch Bank 2 | Input - 8 toggle switches, bit 7 = leftmost |
| $F011 | Keypad (Calculator keys AND on-screen Keyboard) | Input - last key code; idle value is $FF; auto-clears to $FF the instant your program reads a non-$FF value |
| $F021 | Workbench 7-Segment (undecoded) | Output - bits 0-6 drive segments a-g directly |
| $F022 | Workbench 8-Bit LED Bar | Output - one bit per LED |
| $F023 | Workbench 7-Segment (decoded) | Output - low nibble (0-F) selects the digit shape |
| $F024 | Workbench Dual 7-Segment (decoded) | Output - high nibble = left digit, low nibble = right digit |
| $F028 | Terminal | Output - ASCII 32-126 prints the character; $0A prints a newline; everything else is ignored |
| $F031 | Calculator Display | Output - ASCII 32-126 appends to the display; $0D, $10, $1B clear it |
| $F032 | Calculator LEDs | Output - bit 5 = leftmost LED ... bit 0 = rightmost LED |

## A.1 The $F011 Keypad Port Has Two Personalities

Both the Calculator and the on-screen Keyboard write to the same port, but they write different kinds of values:

| Source | What lands in $F011 |
| --- | --- |
| Calculator digit keys (0-9) | Raw nibble 0-9 (translated from the button's ASCII code before writing) |
| Calculator hex keys (A-F) | Raw nibble 10-15 (translated from the button's ASCII code before writing) |
| Calculator operator/function keys (+, -, =, Clear, ...) | The button's own configured code, unmodified |
| On-screen Keyboard letters, digits, punctuation | Standard ASCII value, unmodified |
| On-screen Keyboard ENTER / BSpace / arrows | Fixed control codes ($05, $04, $07-$0A) - not standard ASCII |

Because of this, a single polling loop cannot reliably tell which physical control sent a value just by looking at the byte. Each exercise in this tutorial is written for one input device at a time to keep the examples unambiguous - Exercise 2 assumes the Calculator's digit/hex keys, Exercise 3 assumes the on-screen Keyboard.

# Appendix B: Troubleshooting

- Program does not seem to run at all: check that the Calculator is powered on (its On/Off button) - Run, Step, and Reset all silently do nothing while it is off.

- My program vanished after I turned the Calculator on: powering on purges RAM. Always power on first, then Assemble and Load -> CPU.

- Assemble succeeded but nothing happens on Run: remember Assemble does not load the program by itself - click Load -> CPU afterward. The button is disabled until assembly succeeds.

- Reloading a second program does not clear the display/LEDs from the first one: Load -> CPU clears the program area in RAM (including the I/O port bytes) before writing your new bytes, but the on-screen displays only repaint when your program writes to them again - so a stale LED or 7-segment value can stay on screen until your new program overwrites that port.

- A key doesn't seem to register: check WAIT loops correctly test for the $FF idle value with CMPA $FF and JZ - if the loop condition is inverted (JNZ instead of JZ, or vice-versa) it will either spin forever or fall straight through without waiting. The I/O Ports Display's $F011 box (current + previous value) is the fastest way to confirm whether a keypress actually reached the port.

- The Terminal or Workbench displays stay blank even though my program is clearly running: confirm the panel is actually open (Display -> Terminal / Tools -> Workbench 1) and that the Calculator is powered on - both devices silently discard writes while powered off.

- Everything runs too fast to follow: lower the clock speed with Tools -> System Clock... (1 Hz is easiest for tracing a loop one pass at a time; the default is 100 Hz).

- Not sure what a program actually assembled to: use the Memory Walker's GO field to jump to the address in question and read the DATA column directly, or double-click a DATA cell to poke a test value in by hand.