# Beboputer Hands-On Tutorial

Exercising the Workbench, Calculator, Keyboard, and Terminal with real assembly programs

# Before You Start

This tutorial gives you working, tested assembly-language programs for each of the four interactive peripherals in PY-DIYCALCULATOR: Workbench 1 (switches, LEDs, 7-segment displays), the Calculator (hex keypad, LCD display, indicator LEDs), the on-screen Keyboard (QWERTY typing), and the Terminal (text output). It also covers the three tools you will lean on constantly while working through the exercises: the Memory Walker, the I/O Ports Display, and the System Clock. Every example uses the real memory-mapped ports and instruction set implemented in the emulator, and every program has been traced instruction-by-instruction against the CPU's actual opcode behaviour.

## The One Rule That Trips Everyone Up: Power On First

Important: Turning the Calculator's On/Off button ON purges all 64KB of RAM and resets the CPU. If you assemble and load a program before powering on, switching the Calculator on afterward will wipe it out. Always power on the Calculator BEFORE you assemble and load your program.

Here is why: the Calculator's On/Off button is wired to more than just the calculator screen. Powering on also enables the Terminal, enables the Workbench's switches and displays, and (the surprising part) automatically purges RAM, resets the CPU, and starts it running. So the safe sequence for every exercise in this tutorial is:

- Launch PY-DIYCALCULATOR. The Calculator window opens automatically (powered off).

- Click the Calculator's On/Off button to power everything on.

- Open whichever panels the exercise needs (Workbench 1, Keyboard, Terminal) from the Tools and Display menus.

- Open Tools -> Assembler / Editor, type in the example program, and click Assemble.

- Click Load -> CPU (this button becomes active once assembly succeeds).

- Use the Calculator's Step or Run buttons, or the Memory Walker's own STEP column, to execute your program.

Also note: Run, Step, and Reset only exist as buttons on the Calculator's bottom bar - there is no separate menu item for them. Even a Workbench-only or Terminal-only exercise still needs the Calculator open and powered on, because that is the only place to start your program running (the Memory Walker can single-step independently, but Run still lives on the Calculator).

## Menu Reference

Table 4 maps each panel and tool to its menu path.

Table 4: Menu Reference

| Panel / Tool | Menu Path |
| --- | --- |
| Calculator | Tools -> Calculator... |
| Workbench 1 | Tools -> Workbench 1... |
| Keyboard | Tools -> Keyboard... |
| Terminal | Display -> Terminal |
| Memory Walker | Memory -> Memory Walker (already open by default at startup) |
| I/O Ports Display | Display -> Port Map Status |
| System Clock | Tools -> System Clock... |
| Assembler / Editor | Tools -> Assembler / Editor... |
| CPU Registers | Display -> CPU Registers |
| Purge RAM | File -> Purge RAM... |

## Assembler Syntax Quick Reference

Table 5 gives the full assembler syntax reference used in this tutorial.

Table 5: Assembler Syntax Quick Reference

| Element | Syntax | Example |
| --- | --- | --- |
| Comment | # to end of line | # read the switches |
| Label | NAME: in column 1 | LOOP: |
| Constant | NAME: .EQU value | SW1: .EQU $F000 |
| Load address | .ORG address | .ORG $4000 |
| Reserve bytes | .BYTE v1, v2, ... | MSG: .BYTE $48, $49, $00 |
| Immediate operand | bare hex value | LDA $FF |
| Direct (memory) operand | [address or label] | LDA [SW1] |
| Indexed operand | [address or label,X] | LDA [MSG,X] |
| Indirect operand | [[address or label]] | JMP [[$4000]] |
| Pre-indexed indirect | [[address or label,X]] | LDA [[$4000,X]] |
| End of source | .END address | .END $4000 |

All the examples in this tutorial write hex values with a $ prefix ($4000, $0A, and so on), which is the form used consistently throughout the built-in Assembler / Editor's own Insert menu snippets.

## Instruction Set Used in This Tutorial

Table 6 lists the instruction set used in this tutorial.

Table 6: Instruction Set Used in This Tutorial

| Mnemonic | What it does |
| --- | --- |
| LDA | Load the Accumulator (immediate, direct, indexed, indirect, or pre-indexed indirect) |
| STA | Store the Accumulator to memory (direct, indexed, indirect, pre-indexed indirect - no immediate form) |
| ADD / SUB | Add / subtract into the Accumulator |
| CMPA | Compare the Accumulator with a value. Sets the Zero flag when equal. Unusually, Carry is set only when the Accumulator is strictly greater than the operand. |
| AND / OR / XOR | Bitwise logic against the Accumulator (immediate, direct, or indexed) |
| SHL | Shift the Accumulator left one bit (carry out captures the vacated bit) |
| BLDX | Load the 16-bit Index register (immediate or direct) |
| INCX / DECX | Increment / decrement the Index register |
| JMP | Unconditional jump (direct, indirect, or pre-indexed indirect) |
| JZ / JNZ | Jump if the Zero flag is / is not set |
| JC / JNC | Jump if Carry is / is not set |
| JSR / RTS | Call a subroutine / return from one |
| HALT | Stop the CPU |

Tip: LDA already sets the Zero and Negative flags from the value it loads. In the message-printing exercise later in this tutorial, a loop tests for a zero byte with a plain JZ right after the LDA - no separate CMPA is needed.

# Tools You Will Use While Debugging

These three tools are not tied to any one exercise - you will reach for all of them constantly while working through this tutorial. Get comfortable with them now.

## The Memory Walker

The Memory Walker is already open when the application starts (it is one of the three panels shown by default, alongside the Message Display and the Calculator), and it can also be reopened from Memory -> Memory Walker. It shows a scrollable, live 256-row window of RAM with four columns: BP, STEP, ADDRESS, and DATA.

Table 7 describes the Memory Walker's controls.

Table 7: Memory Walker Controls

| Control | What it does |
| --- | --- |
| Address field + GO | Type a 4-digit hex address (e.g. 4000) and click GO to jump the view there. This locks the view in place - it will not auto-follow the Program Counter until you step from inside the Memory Walker again. |
| Click a BP cell | Toggles a breakpoint (shown as a red dot) at that row's address. |
| Click a STEP cell | Executes exactly one CPU instruction, independently of the Calculator's Step button, and updates the status line with the mnemonic that ran and the resulting Accumulator value. |
| RUN to BP | Runs the CPU until it hits any breakpoint, executes HALT, or reaches the safety limit of 500,000 steps. |
| Clear BPs | Removes every breakpoint. |
| Double-click a DATA cell | Lets you type a new hex byte directly into that RAM address - a live memory editor. |

While your program is running or single-stepping, an amber triangle marks the row at the current Program Counter, and the view automatically scrolls to keep it in sight (unless you have manually navigated there with GO).

Try this: Load the switches-to-LEDs program from the Workbench exercise below, then click the BP cell next to the LOOP label's address before pressing Run. Execution will stop the instant it reaches that address, and you can then use the STEP column to advance one instruction at a time and watch the DATA column change as STA writes to $F022.

Also useful: After assembling a program that includes a .BYTE table (see the message-printing exercise later in this tutorial), type the table's address into the Memory Walker and click GO to confirm the assembler laid out exactly the bytes you expect.

## The I/O Ports Display

Open with Display -> Port Map Status. Unlike the Memory Walker, this panel does not show general RAM - it is a dedicated live monitor for exactly three fixed addresses: the ones the Calculator itself uses.

Table 8 shows what the I/O Ports Display shows at each address.

Table 8: I/O Ports Display Address Map

| Address | What is shown |
| --- | --- |
| $F031 | The last byte written to the Calculator's display, in hex and as a printable character/annotation. |
| $F032 | The last byte written to the Calculator's LEDs, in hex and as an 8-bit binary pattern. |
| $F011 | The current key code (hex + annotation) and the previous key code (hex + annotation), so you can see a keypress transition even though the port auto-clears back to $FF. |

Important: The I/O Ports Display does NOT show the Workbench's ports ($F000, $F001, $F021-$F024) or the Terminal's port ($F028) - only the three addresses above. For the Workbench exercises, watch the Workbench panel itself; the I/O Ports Display is most useful for the Calculator and Keyboard exercises.

The "I/P from Buttons" field is editable: type a hex value (e.g. 07) into it and press Enter, and it writes that value directly into RAM at $F011 - simulating a keypress without touching the Calculator or the on-screen Keyboard at all. This is a convenient way to feed a specific test value into a WAIT/poll loop while you are debugging it.

## The System Clock

Open with Tools -> System Clock... This dialog sets how fast the emulated CPU runs, from 1 Hz up to 10,000 Hz (the default is 100 Hz). Type a new value and click OK; if Run is currently active, the change takes effect immediately.

Try this: Slow the clock down to around 2-5 Hz before running one of the WAIT-and-poll loops in this tutorial, so you can watch the Memory Walker's PC marker and the I/O Ports Display update on every single pass of the loop. Speed it back up to the default 100 Hz (or higher) once you just want the program to run.

# Exercise: Workbench 1

Open Tools -> Workbench 1... Workbench 1 gives you two 8-switch input banks and four different output displays, all wired to fixed ports:

Table 9 lists Workbench 1's ports and devices.

Table 9: Workbench 1 Port Reference

| Port | Device |
| --- | --- |
| $F000 | Switch Bank 1 (input) |
| $F001 | Switch Bank 2 (input) |
| $F021 | 7-Segment, undecoded (bits 0-6 = segments a-g) |
| $F022 | 8-bit LED bar |
| $F023 | 7-Segment, decoded (low nibble = hex digit 0-F) |
| $F024 | Dual 7-Segment, decoded (high nibble = left digit, low nibble = right digit) |

## Mirror the Switches on the LED Bar

The simplest possible I/O program: whatever Switch Bank 1 reads, the LED bar shows.

SW1:    .EQU    $F000
LED8:   .EQU    $F022
 
        .ORG    $4000
 
LOOP:   LDA     [SW1]
        STA     [LED8]
        JMP     [LOOP]
 
        .END    $4000

Assemble, Load -> CPU, then click Run on the Calculator. Flip switches in Switch Bank 1 and watch the LED bar respond immediately.

Debug it: Set a breakpoint on the LOOP address in the Memory Walker, click RUN to BP, then use the STEP column to advance one instruction at a time - you'll see the DATA byte at $F022 change the moment STA executes.

## Show a Switch Value as a Hex Digit

This masks Switch Bank 1 down to its low nibble and shows it as a single hex digit (0-F) on the decoded 7-segment display.

SW1:    .EQU    $F000
SEG2:   .EQU    $F023
 
        .ORG    $4000
 
LOOP:   LDA     [SW1]
        AND     $0F        # keep only the low nibble
        STA     [SEG2]
        JMP     [LOOP]
 
        .END    $4000

## Drive the Undecoded 7-Segment Directly

$F021 is not decoded - each of its low 7 bits drives one segment (a through g) directly. Any 7-bit pattern from the switches becomes a (probably nonsensical, and genuinely fun to explore) segment pattern.

SW1:    .EQU    $F000
SEG1:   .EQU    $F021
 
        .ORG    $4000
 
LOOP:   LDA     [SW1]
        AND     $7F        # keep bits 0-6 (segments a-g)
        STA     [SEG1]
        JMP     [LOOP]
 
        .END    $4000

## Combine Both Switch Banks on the Dual 7-Segment Display

This one is a bit more advanced: it builds a single byte where the low nibble of Switch Bank 1 becomes the left digit and the low nibble of Switch Bank 2 becomes the right digit, using four SHL instructions to move a nibble into the high position.

SW1:    .EQU    $F000
SW2:    .EQU    $F001
SEG3:   .EQU    $F024
 
        .ORG    $4000
 
LOOP:   LDA     [SW1]
        AND     $0F        # low nibble of bank 1 -> left digit
        SHL
        SHL
        SHL
        SHL                # shift it into the high nibble
        STA     [TEMP]
        LDA     [SW2]
        AND     $0F        # low nibble of bank 2 -> right digit
        OR      [TEMP]     # combine hi(bank1) | lo(bank2)
        STA     [SEG3]
        JMP     [LOOP]
 
TEMP:   .BYTE   $00         # scratch byte (never executed - JMP always skips it)
 
        .END    $4000

Debug it: Navigate the Memory Walker to the TEMP label's address (use GO) to watch the intermediate shifted value land there on every pass.

# Exercise: The Calculator

Open Tools -> Calculator... Every key on the Calculator writes a code to port $F011 (the same port the on-screen Keyboard uses - see the note in the next exercise). Digit keys 0-9 and hex keys A-F are translated to their raw nibble value (0-15) before being written, which lines up perfectly with the decoded 7-segment display's own low-nibble convention.

## Show the Last Key Pressed on a 7-Segment Display

Open Workbench 1 as well for this one, so you have somewhere to see the result. Press any digit or hex key (0-9, A-F) on the Calculator and watch the decoded 7-segment display echo it.

KEY:    .EQU    $F011
SEG2:   .EQU    $F023
 
        .ORG    $4000
 
WAIT:   LDA     [KEY]      # read the keypad port
        CMPA    $FF        # $FF means idle - no key waiting
        JZ      [WAIT]     # keep waiting
        AND     $0F        # keep the low nibble (0-9, A-F)
        STA     [SEG2]     # show it
        JMP     [WAIT]
 
        .END    $4000

How this works: Port $F011 is a read-clear latch: the moment your program reads a non-$FF value, the underlying byte snaps back to $FF so the same keypress is never processed twice. Your LDA still gets the real key value in the Accumulator before that happens - only the copy in RAM is cleared.

Watch it happen: Open the I/O Ports Display (Display -> Port Map Status) alongside this exercise. Its $F011 box shows both the current and previous key code with annotations, which makes the read-clear behaviour easy to see in real time. You can also type a hex value straight into its "I/P from Buttons" field to feed the loop a test key without touching the Calculator at all.

## Write Text to the Calculator's Own Display

The Calculator's LCD is itself a memory-mapped output device at $F031. Printable ASCII (32-126) appends a character; codes $0D, $10, or $1B clear the display.

DISP:   .EQU    $F031
 
        .ORG    $4000
 
        LDA     $1B        # clear code
        STA     [DISP]
 
        LDA     $48        # 'H'
        STA     [DISP]
        LDA     $49        # 'I'
        STA     [DISP]
 
        HALT
 
        .END    $4000

Assemble, Load -> CPU, then Step through it a few instructions at a time (rather than Run) so you can watch each character land on the display before moving to the next. The I/O Ports Display's $F031 box will show each character the instant STA executes.

## Light Up the Calculator's LEDs

$F032 drives the Calculator's six indicator LEDs - bit 5 is the leftmost, bit 0 the rightmost.

LEDS:   .EQU    $F032
 
        .ORG    $4000
 
        LDA     $2A        # 00101010 -> alternating LEDs on
        STA     [LEDS]
 
        HALT
 
        .END    $4000

# Exercise: The Keyboard and Terminal

Open Tools -> Keyboard... and Display -> Terminal. The on-screen Keyboard writes raw ASCII codes to the same port the Calculator uses, $F011 (letters and digits as their normal ASCII values - unlike the Calculator's hex keys, the Keyboard tool does not translate digits to raw nibbles). This exercise polls that port and echoes each keystroke to the Terminal at $F028.

Worth knowing: The on-screen Keyboard tool already echoes every keystroke straight to the Terminal itself, independent of any program you write - it is a direct convenience wire built into the tool. That means once your own program is also polling $F011 and writing to $F028, each keystroke will appear TWICE (once from the Keyboard tool's own wire, once from your program). This is expected - it is not a bug in your assembly code. Watching this doubling happen is actually a good way to confirm your program really is reading and reacting to each keypress.

## Echo Typed Characters to the Terminal

This program also demonstrates translating one device's codes into another's: the Keyboard tool's ENTER key sends code $05, but the Terminal only recognises $0A as a newline, so the program converts one into the other before sending it on.

KEY:    .EQU    $F011
TERM:   .EQU    $F028
 
        .ORG    $4000
 
WAIT:   LDA     [KEY]      # read the keyboard port
        CMPA    $FF
        JZ      [WAIT]     # idle - keep waiting
 
        CMPA    $05        # is it the Keyboard tool's ENTER code?
        JNZ     [SEND]     # no - send the byte as typed
        LDA     $0A        # yes - translate to the Terminal's newline code
SEND:   STA     [TERM]
        JMP     [WAIT]
 
        .END    $4000

Assemble, Load -> CPU, click Run, then type on the on-screen Keyboard. Letters and digits appear in the Terminal (doubled, per the note above); pressing ENTER moves to a new line instead of printing the raw $05 byte.

Slow it down: If characters fly by too fast to follow, drop the System Clock to a few Hz (Tools -> System Clock...) so each poll of WAIT is easy to watch on the Memory Walker's PC marker.

# Exercise: Printing a Stored Message on the Terminal

This exercise does not need any input device - it prints a fixed message stored in the program itself, using the Index register to walk through a table of bytes. It is the best example in this tutorial for seeing .BYTE data, indexed addressing, and a loop-with-terminator pattern all at once.

TERM:   .EQU    $F028
 
        .ORG    $4000
 
        BLDX    $0000      # IX = 0 (index into MSG)
 
LOOP:   LDA     [MSG,X]    # load the next character
        JZ      [DONE]     # the $00 terminator ends the message
        STA     [TERM]     # send it to the Terminal
        INCX               # move to the next character
        JMP     [LOOP]
 
DONE:   HALT
 
MSG:    .BYTE   $48, $45, $4C, $4C, $4F, $2C, $20, $42
        .BYTE   $45, $42, $4F, $50, $55, $54, $45, $52
        .BYTE   $21, $0A, $00
 
        .END    $4000

MSG spells out "HELLO, BEBOPUTER!" followed by a newline ($0A) and a $00 terminator that ends the loop. Assemble, Load -> CPU, and click Run (or Step through it to watch IX count up and PC walk the loop on the CPU Registers panel).

Check your work: After assembling, look at the assembly listing in the output panel to find MSG's address, then type it into the Memory Walker's address field and click GO. The DATA column should show exactly the 19 bytes above, in order - a quick way to confirm the table assembled the way you expect before you even run the program.

Try this next: Change the .BYTE table to your own message. Remember every line must end with a $00 terminator for the JZ to find, and each .BYTE line can hold as many comma-separated values as you like (the Assembler / Editor's own Insert String helper wraps them at 8 per line, purely for readability).

# Challenge: Put It All Together

Once each exercise above is working on its own, try combining them into one program: poll the Keyboard at $F011, echo each character to the Terminal at $F028 (translating ENTER to newline as in Exercise 3), and also show the low nibble of the last key code on the Workbench's decoded 7-segment display at $F023 (as in Exercise 2). All three pieces reuse the same WAIT-and-poll structure - the only new work is writing to a second and third output port inside the same loop body before jumping back to WAIT. Set a breakpoint at WAIT in the Memory Walker and use RUN to BP to step through the combined loop one keypress at a time.

# Exercise: The Four-Function Calculator

## What You Will Build

This tutorial adds a real, working calculator program to the Beboputer virtual 8-bit computer — one that adds, subtracts, multiplies, and divides. Type a digit, an operator, another digit, then ’=’ on the Calculator, and the answer appears right on its display, exactly like a pocket calculator:

5 + 3 =    ->   shows   5+3=8

7 * 8 =    ->   shows   7*8=56

2 - 9 =    ->   shows   2-9=-7

9 / 0 =    ->   shows   9/0=Err

The source file is tutorial/10_calculator_four_function.asm. It builds directly on ideas from exercises 05 through 09 — reading the keypad, writing to the display — and adds one new idea: doing real arithmetic on a CPU that has no multiply or divide instruction at all.

## Before You Start

This tutorial assumes you have already worked through, or at least skimmed, these earlier exercises:

- 05_calculator_keypad_to_7seg.asm — how the keypad port ($F011) works as a read-clear latch.

- 06_calculator_display_text.asm — how to write characters to the Calculator’s own display ($F031).

- 09_terminal_print_message.asm — how .BYTE tables and reserved variables are laid out in a program.

Everything you need from those exercises is re-explained below, so you can also read this tutorial on its own.

## How the Calculator Talks to Your Program

Two memory-mapped ports do all the work:

| Port | Direction | What it does |
| --- | --- | --- |
| $F011 | Input | The keypad. Reading it returns the code of the last key pressed, or $FF if no key is waiting. The moment you read a non-$FF value, the port snaps back to $FF — a "read-clear latch" — so the same keypress is never processed twice. |
| $F031 | Output | The Calculator’s own display. Writing a printable ASCII byte (32–126) appends that character to the display, building up an expression left to right. Writing $0D, $10, or $1B clears the display back to empty. |

Because $F031 simply appends characters, the display behaves like a tiny one-line terminal rather than a fixed set of digit slots — which is exactly what lets this program build up "5+3=8" one character at a time as each key is pressed.

## Keypad Codes Used By This Program

The Calculator’s buttons send these exact byte values on $F011 (from tools/calculator.py):

| Key(s) | Code(s) | Meaning in this program |
| --- | --- | --- |
| 0–9 | $30–$39 | ASCII digit codes. AND $0F turns any of these into the plain binary value 0–9. |
| + − × ÷ | $2B $2D $2A $2F | The four operators, stored as-is and compared against later to choose which operation to run. |
| = | $3D | Triggers the calculation. Enter ($0D) does the same thing. |
| Clear / CE | $1B / $01 | Wipes the display and restarts the state machine from scratch. |

## Program Design: A Small State Machine

The whole program is one loop that reads a key, decides what it means based on what has happened so far, and reacts. "What has happened so far" is tracked in one variable, STAGE:

| STAGE | The program is… |
| --- | --- |
| 0 | Waiting for the first digit (operand 1). |
| 1 | Waiting for an operator (+, −, ×, or ÷). |
| 2 | Waiting for the second digit (operand 2). |
| 3 | Waiting for ’=’ (or Enter). |
| 4 | Showing an answer — the next digit pressed clears the display and starts a brand new sum. |

Every keypress runs through the same short dispatcher: read the key once, remember it, check for Clear/CE globally, then jump to the routine for the current STAGE. Each of those routines does three things — echoes the key to the display, updates its stored value, and advances STAGE — except the STAGE-3 routine, which triggers the actual calculation instead.

## Reading a Key Safely (Only Once!)

$F011 is a read-clear latch: the instant your program reads a non-$FF value from it, the port resets to $FF so the same keypress cannot be read twice. That is exactly the behavior that makes polling for a key safe — but it also means you must capture the value the first time you read it, because a second read a few instructions later will just see $FF.

This program reads $F011 exactly once per keypress, in the main WAIT loop, and immediately stores it in the KEYVAL variable:

WAIT:   LDA     [KEY]

CMPA    $FF             # $FF = idle, no key waiting

JZ      [WAIT]

STA     [KEYVAL]        # stash it - this is the ONLY read of KEY

From here on, every comparison for this keypress — "is it Clear?", "is it a digit?", "is it the operator I expect?" — reads KEYVAL instead of the port itself.

## Doing the Math Without a MUL or DIV Instruction

The Beboputer CPU’s instruction set has ADD, ADDC, SUB, SUBC, compare (CMPA), shifts, and rotates — but no hardware multiply or divide. Every multiply and divide routine in this codebase, including the full 16-bit versions in Data/int-mult-2-byte.asm and Data/int-div-2-byte.asm, builds those operations out of addition and subtraction in a loop. Because this calculator only ever works with single digits (0–9), the loops here are short (at most 9 passes) and simple enough to read start to finish.

### Addition

The simplest case — just ADD the two operands. The largest possible result is 9+9=18, which fits in a single byte with no overflow to worry about:

DO_ADD: LDA     [OP1]

ADD     [OP2]

STA     [RESULT]

### Subtraction, and the Meaning of the Carry Flag

Subtraction introduces the one flag this whole program leans on — and it is worth being precise about it, because getting it slightly wrong caused a real, hard-to-notice bug (see the callout below). After SUB or CMPA computes "A minus M", the Carry flag on this CPU is set to 1 only when A is STRICTLY GREATER THAN M — not the more common "no borrow was needed" convention used by most CPUs, which would also cover the case where A and M are equal. When A equals M, Carry comes back 0.

That means a single "jump if carry" branch only ever catches strictly-greater; it does not catch equal. Testing "is A at least M" correctly needs a three-way check: JC for strictly-greater, JZ for exactly-equal, and only fall through to "strictly less" if neither fired. This program has no way to display a negative number directly, so when the second operand is larger than the first, it instead computes the difference the other way around and remembers that the sign should be flipped with a NEG flag:

DO_SUB: LDA     [OP1]

CMPA    [OP2]

JC      [SUB_POS]       # C=1: OP1 > OP2, no borrow needed

JZ      [SUB_POS]       # Z=1: OP1 == OP2, result is 0, no borrow

LDA     [OP2]           # OP1 < OP2: compute OP2-OP1 instead

SUB     [OP1]

STA     [RESULT]

LDA     $01

STA     [NEG]

Later, when the result is displayed, a NEG value of 1 simply prints a − sign before the digits.

### A Bug This Caused, and the Fix

An earlier version of this program used only the single JC branch shown above, without the JZ line. It assembled cleanly and eight of the nine cases in the verification table below happened to still pass — but 5−5 displayed "−0" instead of "0", because CMPA saw the two equal operands, found Carry = 0 (not strictly greater), and incorrectly fell into the "OP1 < OP2" branch that computes 0 and sets the negative flag anyway. The fix is the JZ [SUB_POS] line above: exactly-equal now takes the same non-negative path as strictly-greater.

The same missing-equal-case bug also affected division and the decimal-digit conversion below, with more visible consequences: 4÷2 returned 1 instead of 2, and any answer that was an exact multiple of 10 (e.g. 2×5) printed a stray ":" character instead of "10". Both are fixed the same way — a JZ line added alongside the existing JC — and both are covered in the sections and verification table that follow.

### A Second Bug: Digit "1" and the CE Button Were the Same Byte

A separate, unrelated bug turned up after the flag fix above: pressing digit "1" was silently swallowed and treated as a clear instead of entering the digit. The cause has nothing to do with the CPU’s flags — it is about what byte a button actually sends.

Every DIY button writes a fixed byte to $F011 on click, but digit and hex-letter buttons are special-cased by the button framework itself (DIYButton._execute() in tools/diy_button.py): before transmitting, it converts their ASCII code ($30–$39 for ‘0’–‘9’) down to a raw nibble (0–15). That is deliberate — it lets CPU programs read keypad digits as plain binary values without needing to mask off the ASCII bits — but it means digit "1" does not arrive as $31; it arrives as plain byte $01.

The Calculator’s CE button used to be assigned that exact byte, $01, in its default configuration (_BUTTON_DEFAULTS in tools/calculator.py). That made CE and digit "1" bit-for-bit identical on the wire: this program had no way to tell which one had actually been pressed, so its explicit CE check (CMPA $01 / JZ [DO_CLEAR]) fired every time "1" was pressed too, wiping the display instead of accepting the digit.

This is not something the program’s own logic could fix — two different buttons were sending the identical byte, so no amount of cleverness in this .asm file could distinguish them. The real fix was moving CE to a free byte outside the 0–15 nibble range, $7F (ASCII DEL), in _BUTTON_DEFAULTS, so it can never collide with any digit or hex-letter button again. This program’s CE check was updated to match ($7F instead of $01); Clear ($1B, ESC) was never affected, since ESC falls well outside the 0–15 range on its own.

### Multiplication by Repeated Addition

With no multiply instruction, 5 × 3 is computed the way a student might do it by hand before learning the times table: add 5 to itself, 3 times. MCNT counts the remaining additions down to zero:

DO_MUL: LDA     $00

STA     [RESULT]

LDA     [OP2]

STA     [MCNT]

JMP     [MUL_TEST]

MUL_LOOP:

LDA     [RESULT]

ADD     [OP1]

STA     [RESULT]

LDA     [MCNT]

DECA

STA     [MCNT]

MUL_TEST:

LDA     [MCNT]

CMPA    $00

JNZ     [MUL_LOOP]

Checking the counter before the first addition (via the initial JMP to MUL_TEST) also means multiplying by zero correctly produces zero, with no additions performed at all.

### Division by Repeated Subtraction

Division works the same way in reverse: how many times does the divisor (OP2) subtract out of the dividend (OP1) before what is left is smaller than the divisor? Each successful subtraction increments a quotient counter (reusing RESULT). The loop needs to keep subtracting while the remainder is greater than OR equal to the divisor, so — per the flag note in 7.2 — that is the same three-way CMPA/JC/JZ test, not a single branch:

DO_DIV: LDA     [OP2]

CMPA    $00

JZ      [DIV_ERR]

LDA     $00

STA     [RESULT]        # quotient, counts up

LDA     [OP1]

STA     [DREM]          # remainder, counts down

DIV_LOOP:

LDA     [DREM]

CMPA    [OP2]

JC      [DIV_SUB]       # C=1: DREM > OP2, still fits

JZ      [DIV_SUB]       # Z=1: DREM == OP2, still fits exactly

JMP     [DIV_DONE]      # DREM < OP2, can't subtract again

DIV_SUB:

LDA     [DREM]

SUB     [OP2]

STA     [DREM]

LDA     [RESULT]

INCA

STA     [RESULT]

JMP     [DIV_LOOP]

Dividing by zero would never stop subtracting, so OP2 is checked for zero right at the start and diverted straight to a small routine that prints "Err" instead of looping forever. Without the JZ line, this loop stopped one subtraction too early whenever OP2 divided OP1 exactly — 4÷2 subtracted once (4→2), saw the remainder equal the divisor, wrongly treated that as "too small to fit," and stopped at a quotient of 1 instead of continuing to 2.

## Turning the Answer Back Into Decimal Digits

Every operation above leaves its answer, RESULT, as a plain binary byte — for example the byte value 56 after 7×8, not the two characters ’5’ and ’6’. Because RESULT never exceeds 81 (9×9, the largest possible answer here), converting it to decimal only ever needs at most a tens digit and a ones digit. The same repeated-subtraction idea from division does the conversion: subtract 10 from RESULT, counting how many times that succeeds (the tens digit), and whatever is left over afterwards is the ones digit — and, once again, "succeeds" means RESULT is greater than OR equal to 10, so the loop uses the same three-way CMPA/JC/JZ test rather than a single branch:

TENS_LOOP:

LDA     [RESULT]

CMPA    $0A

JC      [TENS_SUB]      # C=1: RESULT > 10, still fits

JZ      [TENS_SUB]      # Z=1: RESULT == 10, still fits exactly

JMP     [TENS_DONE]     # RESULT < 10, tens digit is done

TENS_SUB:

LDA     [RESULT]

SUB     $0A

STA     [RESULT]

LDA     [TENS]

INCA

STA     [TENS]

JMP     [TENS_LOOP]

Once each digit (0–9) is known, OR-ing it with $30 turns it back into the matching ASCII character — the same trick used in reverse by the AND $0F earlier, and the same one used in exercise 05. Without the JZ line, an exact multiple of 10 (e.g. 2×5=10) stopped this loop with RESULT still holding the raw value 10 (0x0A) instead of 0 — and 0x0A OR $30 produces 0x3A, the ":" character, printed in place of the ones digit.

## Verified Against the Real CPU Emulator

This program was actually run against the project’s own CPU class with simulated keypresses — not just assembled and read over — and every digit press was translated through the exact same ASCII-to-raw-nibble conversion the real button framework applies (DIYButton._execute()), not sent as raw ASCII, so these results reflect what the CPU genuinely receives. The table below includes the original scenarios (every operator, a subtraction that goes negative, a divide-by-zero, a multiply-by-zero, and a fresh digit right after an answer), the equal-operand and exact-division/exact-multiple-of-10 cases that exposed the Carry-flag bug (7.2.1), and the digit-"1"/CE cases that exposed the button-code collision (7.2.2): every one of these now produces exactly the expected display text.

| Keys Pressed | Expected Display |
| --- | --- |
| 5, +, 3, = | 5+3=8 |
| 7, ×, 8, = | 7*8=56 |
| 2, −, 9, = | 2-9=-7 |
| 9, −, 2, = | 9-2=7 |
| 9, ÷, 0, = | 9/0=Err |
| 8, ÷, 3, = | 8/3=2 |
| 0, ×, 9, = | 0*9=0 |
| (after a result) 4, +, 4, = | 4+4=8 (old expression cleared first) |
| 4, ÷, 2, = | 4/2=2 (exact division — was 1 before the fix) |
| 6, ÷, 3, = | 6/3=2 (exact division — was 1 before the fix) |
| 9, ÷, 3, = | 9/3=3 (exact division — was 2 before the fix) |
| 5, −, 5, = | 5-5=0 (equal operands — was "-0" before the fix) |
| 2, ×, 5, = | 2*5=10 (exact multiple of 10 — was ":" before the fix) |
| 5, +, 5, = | 5+5=10 (exact multiple of 10 — was ":" before the fix) |
| 1, +, 3, = | 1+3=4 (digit "1" — was swallowed as Clear before the fix) |
| 9, ÷, 1, = | 9/1=9 (digit "1" as divisor) |
| (fresh) 1 | 1 (just the digit — was misread as Clear before the fix) |
| 7, CE, 2, ... | 2 (CE now $7F, clears without colliding with "1") |

## Try It Yourself

Open the Workbench and Calculator, load tutorial/10_calculator_four_function.asm into the assembler, Assemble, Load → CPU, click Run, and on the Calculator press: a digit, an operator, a digit, then ’=’ (or Enter). Try all four operators, try a subtraction where the second number is bigger, and try dividing by zero. Open Display → Port Map Status alongside the exercise to watch each keypress land on $F011 and each character land on $F031 in real time.

## Ideas to Extend This Program

- Show the division remainder as well as the quotient.

- Accept two-digit operands — this needs a second state per operand and a way to combine tens and ones (tens × 10 + ones) back into one binary value.

- Replace the repeated-addition/subtraction loops with the faster shift-and-add / shift-and-subtract algorithms used in Data/fmult.asm and Data/int-div-2-byte.asm — same idea, far fewer loop iterations for large numbers.

## Appendix: Complete Source Listing

tutorial/10_calculator_four_function.asm

# ================================================================

# 10_calculator_four_function.asm

# Beboputer Hands-On Tutorial — Bonus Exercise (Calculator)

#

# A working 4-function (+ - * /) calculator, built entirely from

# the primitives shown in exercises 05-09: keypad polling, the

# Calculator's own display, and simple assembly-language loops.

#

# What this program does

# -----------------------

#   Lets you type a single-digit sum on the Calculator, e.g.

#       5  +  3  =        ->  shows  5+3=8

#       7  *  8  =        ->  shows  7*8=56

#       2  -  9  =        ->  shows  2-9=-7

#       9  /  0  =        ->  shows  9/0=Err

#   Every keypress is echoed to the display as you type it, so the

#   full expression builds up on screen exactly like a real

#   calculator, then the '=' key replaces nothing - it just appends

#   the computed answer.

#

# Limitations (kept simple on purpose - see "Try this next")

# ------------------------------------------------------------

#   - Operands are single digits only (0-9), no multi-digit numbers

#     and no decimal point.

#   - Division shows the integer quotient only (no remainder).

#   - Dividing by zero shows "Err" instead of a number.

#

# How it works

# -------------

#   The program is a small state machine driven by KEY ($F011):

#

#     STAGE 0   waiting for the first digit  (operand 1)

#     STAGE 1   waiting for an operator      (+ - * /)

#     STAGE 2   waiting for the second digit (operand 2)

#     STAGE 3   waiting for '='  (or Enter)

#     STAGE 4   answer is showing - the next digit starts a new sum

#

#   $F011 is a read-clear latch (see exercise 05): each keypress is

#   read exactly once into KEYVAL and every later comparison in that

#   pass re-uses KEYVAL, since a second LDA [KEY] would just see the

#   $FF idle code.

#

#   Multiply is repeated addition (add OP1 to itself OP2 times) and

#   divide is repeated subtraction (subtract OP2 from OP1 while it

#   still fits, counting how many times) - the CPU has no hardware

#   MUL or DIV instruction, so every calculator and multiply/divide

#   routine on this machine builds the operation out of ADD/SUB in

#   a loop. Because operands here are single digits (0-9 at most),

#   the loops are at most 9 iterations - simple enough to read in

#   one sitting, unlike the 16-bit shift-add/shift-subtract versions

#   in Data/int-mult-2-byte.asm and Data/int-div-2-byte.asm, which

#   solve the same problem for much bigger numbers.

#

#   A note on this CPU's flags (read this before you adapt the

#   pattern below for your own code): SUB/CMPA set the Carry flag

#   to mean "the accumulator is STRICTLY GREATER THAN the operand"

#   - not the more usual "no borrow was needed" (which on most CPUs

#   also covers the equal case). That means a single "jump if

#   carry" or "jump if no-carry" branch only ever catches strictly-

#   greater or strictly-less; it silently gets the equal case wrong.

#   Every "is the left-hand side >= the right-hand side?" test in

#   this program - can OP1 still absorb another subtraction of OP2,

#   is the result >= 10, is OP1 >= OP2 for a same-sign subtraction -

#   therefore uses a three-way test instead of one branch: CMPA,

#   then JC for strictly-greater, JZ for exactly-equal, and only

#   falls through to "strictly less" if neither fired. This was

#   discovered (and is explained at length) in exercise 11's

#   circumference-button program; every exact-division and exact-

#   multiple-of-10 case here needed the same fix.

#

#   A second gotcha, this time about the DIY buttons themselves:

#   pressing a digit button does NOT send its ASCII code ($30-$39)

#   to $F011 - the button framework (tools/diy_button.py) silently

#   converts digit/hex-letter buttons down to a raw nibble (0-15)

#   before transmitting, so CPU programs can read keypad digits as

#   plain binary values. That means digit "1" actually arrives as

#   byte $01. The Calculator's CE button used to be assigned that

#   same byte ($01), making CE and digit "1" bit-for-bit identical

#   on the wire - pressing "1" was silently swallowed and treated as

#   a clear instead of a digit, since this program had no way to

#   tell the two keypresses apart. The real fix was to move CE to a

#   free byte outside the 0-15 nibble range ($7F) in _BUTTON_DEFAULTS

#   (tools/calculator.py), so it can never again collide with any

#   digit or hex-letter button; this program watches for that new

#   value below, alongside Clear ($1B, ESC) as the two ways to reset.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, then on the Calculator press:

#   a digit, an operator, a digit, then '=' (the calculator's '='

#   key, or Enter). Try all four operators, try a subtraction where

#   the second number is bigger (negative result), try dividing by

#   0, and try a case where the operator divides evenly or the

#   result is an exact multiple of 10 (e.g. 4/2, 2*5) - these are

#   exactly the "equal" edge cases the flag note above covers.

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status alongside this exercise to see

#   each keypress arrive on $F011 and each character land on $F031

#   in real time.

#

# Try this next

# ---------------

#   - Show the division remainder as well as the quotient.

#   - Accept two-digit operands (you'll need a second state per

#     operand and a decimal-to-binary combine step: tens*10 + ones).

#   - Replace repeated addition/subtraction with the shift-and-add /

#     shift-and-subtract algorithms from Data/fmult.asm and

#     Data/int-div-2-byte.asm - same idea, fewer loop iterations.

# ================================================================

KEY:     .EQU    $F011

DISP:    .EQU    $F031

.ORG    $4000

START:  LDA     $00

STA     [STAGE]        # begin waiting for the first digit

# ---------------------------------------------------------------

# Main loop - poll the keypad, dispatch on STAGE

# ---------------------------------------------------------------

WAIT:   LDA     [KEY]

CMPA    $FF             # $FF = idle, no key waiting

JZ      [WAIT]

STA     [KEYVAL]        # stash it - this is the ONLY read of KEY

CMPA    $1B             # Clear key

JZ      [DO_CLEAR]

CMPA    $7F             # CE key

JZ      [DO_CLEAR]

LDA     [STAGE]

CMPA    $00

JZ      [ST_OP1]

CMPA    $01

JZ      [ST_OPCODE]

CMPA    $02

JZ      [ST_OP2]

CMPA    $04

JZ      [ST_NEWCALC]

JMP     [ST_EQUALS]     # only remaining value is STAGE 3

# ---------------------------------------------------------------

# STAGE 4 -> 0 : answer was showing, this digit starts a new sum

# ---------------------------------------------------------------

ST_NEWCALC:

LDA     $1B             # clear code - wipes the old expression

STA     [DISP]

JMP     [ST_OP1]

# ---------------------------------------------------------------

# STAGE 0 : first operand

# ---------------------------------------------------------------

ST_OP1: LDA     [KEYVAL]

STA     [DISP]          # echo the digit as typed

LDA     [KEYVAL]

AND     $0F             # ASCII '0'-'9' -> binary 0-9

STA     [OP1]

LDA     $01

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 1 : operator  (+ - * /)

# ---------------------------------------------------------------

ST_OPCODE:

LDA     [KEYVAL]

STA     [OPCODE]        # remember which operator was pressed

STA     [DISP]          # echo it

LDA     $02

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 2 : second operand

# ---------------------------------------------------------------

ST_OP2: LDA     [KEYVAL]

STA     [DISP]

LDA     [KEYVAL]

AND     $0F

STA     [OP2]

LDA     $03

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 3 : waiting for '=' (or Enter) - ignore anything else

# ---------------------------------------------------------------

ST_EQUALS:

LDA     [KEYVAL]

CMPA    $3D             # '='

JZ      [DO_EQUALS]

CMPA    $0D             # Enter

JZ      [DO_EQUALS]

JMP     [WAIT]

DO_EQUALS:

LDA     $3D

STA     [DISP]          # echo the '=' sign

LDA     [OPCODE]

CMPA    $2B             # '+'

JZ      [DO_ADD]

CMPA    $2D             # '-'

JZ      [DO_SUB]

CMPA    $2A             # '*'

JZ      [DO_MUL]

CMPA    $2F             # '/'

JZ      [DO_DIV]

JMP     [WAIT]          # (shouldn't happen)

# ---------------------------------------------------------------

# Addition:  RESULT = OP1 + OP2   (max 9+9=18, always positive)

# ---------------------------------------------------------------

DO_ADD: LDA     [OP1]

ADD     [OP2]

STA     [RESULT]

LDA     $00

STA     [NEG]

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Subtraction:  RESULT = OP1 - OP2  (negate + flag if OP1 < OP2)

#

# CMPA sets Carry only when OP1 is STRICTLY greater than OP2 (see

# the flag note up top), so both the greater-than case (C=1) and

# the equal case (Z=1, result is 0) need to reach SUB_POS - only

# OP1 strictly less than OP2 falls through to the negative branch.

# ---------------------------------------------------------------

DO_SUB: LDA     [OP1]

CMPA    [OP2]

JC      [SUB_POS]       # C=1: OP1 > OP2, no borrow needed

JZ      [SUB_POS]       # Z=1: OP1 == OP2, result is 0, no borrow

LDA     [OP2]           # OP1 < OP2: compute OP2-OP1 instead

SUB     [OP1]

STA     [RESULT]

LDA     $01

STA     [NEG]

JMP     [SHOW_RESULT]

SUB_POS:

LDA     [OP1]

SUB     [OP2]

STA     [RESULT]

LDA     $00

STA     [NEG]

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Multiply:  RESULT = OP1 * OP2, via OP2 repeated additions of OP1

# (max 9*9=81 - fits comfortably in one byte)

# ---------------------------------------------------------------

DO_MUL: LDA     $00

STA     [RESULT]

LDA     [OP2]

STA     [MCNT]

JMP     [MUL_TEST]

MUL_LOOP:

LDA     [RESULT]

ADD     [OP1]

STA     [RESULT]

LDA     [MCNT]

DECA

STA     [MCNT]

MUL_TEST:

LDA     [MCNT]

CMPA    $00

JNZ     [MUL_LOOP]

LDA     $00

STA     [NEG]

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Divide:  RESULT = OP1 / OP2 (integer quotient), via repeated

# subtraction of OP2 from a running remainder. OP2=0 -> "Err".

#

# Loop while DREM >= OP2 (checked as: DREM strictly greater than

# OP2, OR DREM exactly equal to OP2 - the three-way CMPA/JC/JZ

# combination from the flag note up top; a lone JNC would stop one

# subtraction too early whenever OP2 divides OP1 evenly, e.g. 4/2).

# ---------------------------------------------------------------

DO_DIV: LDA     [OP2]

CMPA    $00

JZ      [DIV_ERR]

LDA     $00

STA     [RESULT]        # quotient, counts up

LDA     [OP1]

STA     [DREM]          # remainder, counts down

DIV_LOOP:

LDA     [DREM]

CMPA    [OP2]

JC      [DIV_SUB]       # C=1: DREM > OP2, still fits

JZ      [DIV_SUB]       # Z=1: DREM == OP2, still fits exactly

JMP     [DIV_DONE]      # DREM < OP2, can't subtract again

DIV_SUB:

LDA     [DREM]

SUB     [OP2]

STA     [DREM]

LDA     [RESULT]

INCA

STA     [RESULT]

JMP     [DIV_LOOP]

DIV_DONE:

LDA     $00

STA     [NEG]

JMP     [SHOW_RESULT]

DIV_ERR:

LDA     $45             # 'E'

STA     [DISP]

LDA     $72             # 'r'

STA     [DISP]

LDA     $72             # 'r'

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Show RESULT (0-81, binary) as decimal digits, '-' first if NEG.

#

# Same three-way CMPA/JC/JZ test peels off the tens digit correctly

# even when RESULT is an exact multiple of 10 (e.g. 2*5=10) - a

# lone JNC would stop one subtraction too early and leave RESULT=10

# to be OR $30'd directly, producing ':' (0x3A) instead of "10".

# ---------------------------------------------------------------

SHOW_RESULT:

LDA     [NEG]

CMPA    $00

JZ      [SR_TENS]

LDA     $2D             # '-'

STA     [DISP]

SR_TENS:

LDA     $00

STA     [TENS]

TENS_LOOP:

LDA     [RESULT]

CMPA    $0A

JC      [TENS_SUB]      # C=1: RESULT > 10, still fits

JZ      [TENS_SUB]      # Z=1: RESULT == 10, still fits exactly

JMP     [TENS_DONE]     # RESULT < 10, tens digit is done

TENS_SUB:

LDA     [RESULT]

SUB     $0A

STA     [RESULT]

LDA     [TENS]

INCA

STA     [TENS]

JMP     [TENS_LOOP]

TENS_DONE:

LDA     [TENS]

CMPA    $00

JZ      [SR_ONES]       # no leading zero when tens digit is 0

LDA     [TENS]

OR      $30

STA     [DISP]

SR_ONES:

LDA     [RESULT]        # what's left (0-9) is the ones digit

OR      $30

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Explicit Clear / CE handling

# ---------------------------------------------------------------

DO_CLEAR:

LDA     $1B

STA     [DISP]

LDA     $00

STA     [STAGE]

JMP     [WAIT]

RESET_STATE:

LDA     $04             # STAGE 4: next digit clears & restarts

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# Variables (reserved bytes - see exercise 09 for this pattern)

# ---------------------------------------------------------------

STAGE:   .BYTE

OP1:     .BYTE

OP2:     .BYTE

OPCODE:  .BYTE

RESULT:  .BYTE

NEG:     .BYTE

KEYVAL:  .BYTE

MCNT:    .BYTE

DREM:    .BYTE

TENS:    .BYTE

.END    $4000

# Exercise: The Four-Function Calculator Using BCD

## What You Will Build

A 4-function calculator, just like exercise 10, except every number in this version is stored as packed BCD (binary-coded decimal) instead of plain binary. Type two-digit operands, an operator, and ‘=’, and read the answer off the display:

2  3  +  1  4  =        ->  shows  23+14=37

0  9  /  0  3  =        ->  shows  09/03=3

0  5  -  1  2  =        ->  shows  05-12=Err   (negative result)

6  0  *  6  0  =        ->  shows  60*60=Err   (3600 needs 3 digits)

9  9  /  0  0  =        ->  shows  99/00=Err   (divide by zero)

The source file is tutorial/13_bcd_math_routines.asm. It reuses the keypad state machine from exercise 10 almost unchanged, but replaces every ADD/SUB with this CPU’s dedicated BCD instructions — DADD, DADDC, DSUB, DSUBC — and, as a direct result, gets to delete the decimal-conversion loop every earlier tutorial needed just to put a number on the screen.

## Before You Start

This tutorial builds directly on:

- 10_calculator_four_function.asm — the keypad state-machine pattern this program reuses almost line for line, and the Carry-flag quirk of plain SUB/CMPA (see “Subtraction: DSUB’s Carry Means the Opposite of SUB/CMPA’s” for why BCD is different).

- 12_trig_functions_sincostan.asm — using SHL/SHR to move bits into position, reused here to pack/unpack nibbles instead of indexing a table.

## Packed BCD: Two Digits, One Byte, No Conversion

Exercises 10 through 12 all stored numbers in plain binary, and all three paid for it at display time: turning a binary total back into decimal digits took a repeated-subtraction loop — subtract 10, or 100, again and again, counting how many times — because a binary byte has no idea where one decimal digit ends and the next begins.

Packed BCD avoids the conversion by never losing the digit boundary in the first place. Each byte holds exactly two decimal digits, one per nibble: the high nibble is the tens digit, the low nibble is the ones digit, and both are always 0–9. The number 47 is stored as the byte $47 — not because 47 happens to equal 0x47 in binary (that is a coincidence of how hex digits are written down), but because the nibble 4 and the nibble 7 are exactly the digits ‘47’ is made of. Reading a BCD byte back out is just splitting it into its two nibbles and turning each into an ASCII character — no loop, no subtraction, no math at all.

The catch is that ordinary ADD/SUB do not know about this packing. Adding $47 and $08 in plain binary gives $4F — a nibble of 15, not a valid decimal digit — because binary addition has no concept of “carry at 10” the way decimal addition does. This CPU has four dedicated instructions for exactly this problem:

| Instruction | What it does |
| --- | --- |
| DADD | Adds two packed-BCD bytes, carrying 1 into the next nibble whenever a digit would overflow past 9. |
| DADDC | Same as DADD, but also adds in the Carry flag — for chaining multi-byte BCD numbers together. |
| DSUB | Subtracts two packed-BCD bytes, borrowing 1 from the next nibble whenever a digit would go below 0. |
| DSUBC | Same as DSUB, but also subtracts the Carry flag — for chaining multi-byte BCD subtraction. |

$47 DADD $08 correctly gives $55 (55), not the broken $4F plain ADD would produce.

## Program Design

Same keypad-polling state machine as exercise 10, just with a second digit per operand:

| STAGE | The program is... |
| --- | --- |
| 0 | Waiting for the tens digit of operand 1. |
| 1 | Waiting for the ones digit of operand 1. |
| 2 | Waiting for an operator (+ − × ÷). |
| 3 | Waiting for the tens digit of operand 2. |
| 4 | Waiting for the ones digit of operand 2. |
| 5 | Waiting for ‘=’ (or Enter). |
| 6 | Showing an answer — the next digit clears the display and starts a new one. |

Packing two typed digits into one BCD byte needs no multiply or decimal-composition trick (contrast exercise 12’s TENS×10+ONES) — a nibble is a fixed 4-bit-wide slot, so the tens digit just needs shifting into the top half with SHL, and the ones digit ORs straight into the bottom half:

LDA   [KEYVAL]

AND   $0F            # ASCII digit -> binary 0-9

SHL

SHL

SHL

SHL                  # 0-9 moved into the top nibble, bottom = 0

STA   [OP1]          # tens digit stored

...

LDA   [KEYVAL]

AND   $0F

OR    [OP1]          # ones digit merges into the bottom nibble

STA   [OP1]          # OP1 is now a single packed-BCD byte

## Subtraction: DSUB’s Carry Means the Opposite of SUB/CMPA’s

This is the one flag detail worth slowing down for, because it is the exact reverse of the rule exercises 10–12 hammer home. Plain SUB/CMPA set Carry when the accumulator is strictly greater than the operand — not the usual “a borrow was needed.” DSUB does not share that quirk: its Carry means an honest, conventional “yes, a borrow was needed” — which for a single subtraction is exactly “the left-hand side was smaller than the right-hand side.” Subtracting 12 from 5 in BCD (05 DSUB 12) correctly sets Carry, because 5 is less than 12; subtracting 5 from 5 correctly leaves Carry clear, because no borrow was needed to reach zero.

That means — unlike every three-way CMPA/JC/JZ dance in exercises 10–12 — a single JC after DSUB is already the complete, correct test:

DO_SUB: LDA     [OP1]

DSUB    [OP2]

STA     [RESULT]

JC      [ERR_OUT]   # a genuine borrow: OP1 was < OP2

JMP     [SHOW_RESULT]

This program treats a borrow as “can’t show it” (Err) rather than negating the result — see “Displaying a Packed BCD Byte: No Loop Needed At All” for what a full negative-number version would need.

## Addition and the Overflow Check

DADD does the packed add and reports, via Carry, whether the true two-digit result needed a third digit — i.e. the sum was 100 or more, which two BCD digits cannot hold:

DO_ADD: LDA     [OP1]

DADD    [OP2]

STA     [RESULT]

JC      [ERR_OUT]   # sum >= 100 - doesn't fit in 2 digits

JMP     [SHOW_RESULT]

## Multiply, and Why the Loop Counter Needs DSUB Too

Multiply is still repeated addition, exactly like exercise 10 — add OP1 to a running RESULT, OP2 times, using DADD instead of ADD so the running total stays valid packed BCD, with the same overflow check the “Addition and the Overflow Check” section uses:

MUL_LOOP:

LDA     [RESULT]

DADD    [OP1]

STA     [RESULT]

JC      [ERR_OUT]    # running total spilled past 2 digits

LDA     [MCNT]

DSUB    $01          # NOT DECA - see below

STA     [MCNT]

MUL_TEST:

LDA     [MCNT]

CMPA    $00

JNZ     [MUL_LOOP]

MCNT counts down from OP2 to 0, one loop per multiplication. It is tempting to count down with plain DECA, the way every counter in exercises 09–12 did — but MCNT holds a packed BCD value here (it started as OP2), and DECA is a plain binary decrement with no idea that a nibble stops at 9. $30 (BCD 30) decremented with DECA gives $2F — not $29 — because binary decrement borrows across the whole byte instead of stopping at the nibble boundary, leaving an invalid digit (F) sitting in the ones place. DSUB $01 decrements the correct, BCD-aware way, so 30 counts down 30, 29, 28, …, 1, 0 exactly as expected. Any loop counter built out of a BCD value needs DSUB (or DADD, counting up) for this same reason — plain INCA/DECA are only safe on counters that were never BCD to begin with.

## Divide: DSUB Doubles as Both the Test and the Subtraction

Divide is repeated subtraction again, but DSUB’s honest borrow flag lets the loop skip the separate CMPA “does it still fit?” check exercise 10 needed. DSUB itself is the test: try the subtraction, and only keep the result if no borrow was needed:

DIV_LOOP:

LDA     [DREM]

DSUB    [OP2]

JC      [DIV_DONE]   # borrowed - DREM was < OP2, stop

STA     [DREM]       # no borrow - the subtraction stands

LDA     [RESULT]

DADD    $01          # quotient += 1 (BCD)

STA     [RESULT]

JMP     [DIV_LOOP]

DIV_DONE:

When DSUB does borrow, its result is a meaningless wrapped value (exactly like subtracting 5−8 by hand and refusing to borrow — the digits come out wrong), so it is simply never stored; DREM is left exactly as it was before that attempt, which is correct, since that attempt should not have happened.

## Displaying a Packed BCD Byte: No Loop Needed At All

This is the payoff. RESULT already holds two correct decimal digits, one per nibble, so showing it is just isolating each nibble and turning it into ASCII with OR $30 — the tens digit needs shifting down into position first (SHR four times moves the top nibble into the bottom, undoing the SHL packing done at entry):

LDA     [RESULT]

STA     [TEMP]         # keep a copy - the shifts below destroy it

SHR

SHR

SHR

SHR                    # tens digit, 0-9, now in the low nibble

CMPA    $00

JZ      [SR_ONES]      # suppress a leading zero, same as exercise 10

OR      $30

STA     [DISP]

SR_ONES:

LDA     [TEMP]

AND     $0F            # ones digit

OR      $30

STA     [DISP]

Compare this to exercise 10’s SHOW_RESULT, which needed a whole repeated-subtraction TENS_LOOP to get the same two digits out of a plain binary RESULT. That loop is gone here — not simplified, just genuinely unnecessary, because the digits were never lost in the first place.

## Verified Against the Real CPU Emulator

This program was assembled with the project’s own das.py and run against the project’s own CPU class with simulated keypresses — not just read and assumed correct. 62 cases were checked directly, deliberately clustered around every boundary: exact sums up to and past 99, exact and negative subtractions, multiplications that just fit and just overflow, and both exact and truncating divisions, plus divide-by-zero. A representative sample:

| Expression | Result | Why |
| --- | --- | --- |
| 23+14= | 37 | ordinary case |
| 50+50= | Err | sum is exactly 100 — needs a third digit |
| 50-49= | 1 | ordinary case (leading zero suppressed, same as exercise 10) |
| 05-12= | Err | would be negative — DSUB's Carry catches it directly |
| 60*60= | Err | 3600 needs 3 digits, nowhere close to fitting |
| 09/03= | 3 | exact division |
| 23/04= | 5 | integer quotient only, remainder 3 discarded |
| 99/00= | Err | divide by zero |

Behavior around Clear, CE, Enter, and re-entry was also checked directly against the emulator: Clear mid-entry correctly wipes a partly-typed operand; CE after an answer correctly starts a new calculation; Enter works identically to ‘=’; typing digit “1” in an operand is read correctly and not confused with CE (CE's own code was moved to $7F in an earlier fix specifically so it can never collide with a raw digit — see the 4-Function Calculator tutorial); and pressing a fresh digit while an answer is showing correctly clears the display and starts the next entry.

## Try It Yourself

Assemble tutorial/13_bcd_math_routines.asm, Load → CPU, click Run. On the Calculator, type two digits, an operator, two more digits, then ‘=’ (or Enter). Try 09/03 to see exact division, 05-12 to see the borrow/Err path, 99/00 for divide-by-zero, and 60*60 to see the overflow Err path.

Open Display → Port Map Status alongside the exercise to watch each keypress arrive on $F011, and Registers to watch the Carry flag flip after each DADD/DSUB — notice it behaves like an ordinary “did this operation overflow/borrow?” flag here, the opposite of plain SUB/CMPA’s quirky “strictly greater than.”

## Ideas to Extend This Program

- Add a NEG flag and a ‘−’ sign, the way exercise 10 handles a negative result, so 05-12 shows -07 instead of Err. You will need to compute OP2 DSUB OP1 instead when the first attempt borrows, exactly mirroring exercise 10’s DO_SUB.

- Extend RESULT to two packed-BCD bytes (four digits, 0000–9999) using DADDC/DSUBC to chain the carry/borrow between them — the BCD equivalent of exercise 11’s 16-bit RESULT/RESULT+1.

- Show the division remainder (DREM already holds it when DIV_DONE is reached) alongside the quotient.

## Appendix: Complete Source Listing

tutorial/13_bcd_math_routines.asm

# ================================================================

# 13_bcd_math_routines.asm

# Beboputer Hands-On Tutorial — Bonus Exercise (Calculator)

#

# A 4-function (+ - * /) calculator built on packed BCD instead of

# plain binary - same idea as exercise 10, but every number in this

# program is stored the way this CPU's DADD/DSUB instructions expect:

# two decimal digits packed into one byte, one digit per nibble.

#

# What this program does

# -----------------------

#   Same look and feel as exercise 10, just with two-digit operands

#   (always type both digits - a leading zero for single-digit

#   values, e.g. 05):

#       2  3  +  1  4  =        ->  shows  23+14=37

#       0  9  /  0  3  =        ->  shows  09/03=03

#       0  5  -  1  2  =        ->  shows  05-12=Err       (negative - see "Limitations")

#       6  0  *  6  0  =        ->  shows  60*60=Err       (3600 needs 3 digits - see "Limitations")

#       9  9  /  0  0  =        ->  shows  99/00=Err

#

# Limitations (kept simple on purpose - see "Try this next")

# ------------------------------------------------------------

#   - Operands are two BCD digits only: 00-99.

#   - A result that would need a third digit (100 or more) shows

#     Err instead of wrapping or truncating - see "Packed BCD"

#     below for why this program has no easy way to show it.

#   - Subtraction shows Err instead of a negative number - there is

#     no packed-BCD "minus sign" convention used here (see "Try this

#     next" for how a real calculator would extend this).

#   - Division shows the integer quotient only (no remainder).

#

# Packed BCD: two decimal digits, one byte, no conversion needed

# -----------------------------------------------------------------

#   Every previous exercise stored numbers in plain binary and paid

#   for it at display time: exercises 10-12 all needed a repeated-

#   subtraction loop (subtract 10, or 100, again and again, counting

#   how many times) just to turn a binary total back into decimal

#   digits, because a binary byte has no idea where one decimal

#   digit ends and the next begins.

#

#   Packed BCD fixes that by never converting in the first place.

#   Each byte holds exactly two decimal digits, one per nibble - the

#   high nibble is the tens digit, the low nibble is the ones digit,

#   both always 0-9. The number 47 is stored as the single byte

#   $47 - not because 47 happens to equal 0x47 in binary (that is a

#   coincidence of how hex digits are written), but because the

#   nibble 4 and the nibble 7 are exactly the digits "47" is made of.

#   Reading a BCD byte back out is just splitting it into its two

#   nibbles and turning each into an ASCII character - no loop, no

#   subtraction, no math at all. SHOW_RESULT below is a third the

#   size of exercise 10's SHOW_RESULT for exactly this reason.

#

#   The catch is that ordinary ADD/SUB do not understand this

#   packing - ADD $47,$08 in plain binary gives $4F (a nibble of 15,

#   not a valid digit), because binary addition has no concept of

#   "carry at 10" the way decimal addition does. This CPU has four

#   extra instructions - DADD, DADDC, DSUB, DSUBC - that add and

#   subtract packed BCD bytes correctly: whenever a nibble would

#   overflow past 9, they carry (or borrow) 1 into the other nibble

#   automatically, the same way you carry a 1 when adding 47+8 by

#   hand. $47 DADD $08 correctly gives $55 (55), not $4F.

#

# How it works

# -------------

#   Same keypad-polling state machine as exercise 10, just with a

#   second digit per operand:

#

#     STAGE 0   waiting for the tens digit of operand 1

#     STAGE 1   waiting for the ones digit of operand 1

#     STAGE 2   waiting for an operator      (+ - * /)

#     STAGE 3   waiting for the tens digit of operand 2

#     STAGE 4   waiting for the ones digit of operand 2

#     STAGE 5   waiting for '='  (or Enter)

#     STAGE 6   answer is showing - the next digit starts a new sum

#

#   Packing two typed digits into one BCD byte needs no multiply or

#   decimal-composition trick (contrast exercise 12's TENS*10+ONES) -

#   a nibble is a fixed 4-bit-wide slot, so the tens digit just needs

#   shifting into the top half with SHL, and the ones digit ORs

#   straight into the bottom half:

#

#     LDA   [KEYVAL]

#     AND   $0F            # ASCII digit -> binary 0-9

#     SHL

#     SHL

#     SHL

#     SHL                  # 0-9 moved into the top nibble, bottom = 0

#     STA   [OP1]          # tens digit stored

#     ...

#     LDA   [KEYVAL]

#     AND   $0F

#     OR    [OP1]          # ones digit merges into the bottom nibble

#     STA   [OP1]          # OP1 is now a single packed-BCD byte

#

# Addition and the overflow check

# ---------------------------------

#   DADD does the packed add and reports, via Carry, whether the

#   true two-digit result needed a third digit (i.e. the sum was

#   100 or more - two BCD digits can only hold 00-99):

#

#     DO_ADD: LDA     [OP1]

#             DADD    [OP2]

#             STA     [RESULT]

#             JC      [ERR_OUT]   # sum >= 100 - doesn't fit in 2 digits

#             JMP     [SHOW_RESULT]

#

# Subtraction: DSUB's Carry means the OPPOSITE of SUB/CMPA's

# ------------------------------------------------------------------

#   This is the one flag detail worth slowing down for, because it

#   is the exact reverse of the rule exercises 10-12 hammer home.

#   Plain SUB/CMPA set Carry when the accumulator is STRICTLY

#   GREATER THAN the operand - not the usual "a borrow was needed."

#   DSUB does NOT share that quirk: its Carry means an honest,

#   conventional "yes, a borrow was needed" - which for a single

#   subtraction is exactly "the left-hand side was smaller than the

#   right-hand side." Subtracting 12 from 5 in BCD (05 DSUB 12)

#   correctly sets Carry, because 5 is less than 12; subtracting 5

#   from 5 (an equal case) correctly leaves Carry clear, because no

#   borrow was needed to reach zero. That means - unlike every

#   three-way CMPA/JC/JZ dance in exercises 10-12 - a single JC after

#   DSUB is already the complete, correct test:

#

#     DO_SUB: LDA     [OP1]

#             DSUB    [OP2]

#             STA     [RESULT]

#             JC      [ERR_OUT]   # a genuine borrow: OP1 was < OP2

#             JMP     [SHOW_RESULT]

#

#   (This program treats that as "can't show it" rather than

#   negating it - see "Try this next" for what a full negative-

#   number version would need.)

#

# Multiply, and why the loop counter needs DSUB too

# ----------------------------------------------------

#   Multiply is still repeated addition, exactly like exercise 10 -

#   add OP1 to a running RESULT, OP2 times, using DADD instead of

#   ADD so the running total stays valid packed BCD, with the same

#   overflow check DO_ADD uses:

#

#     MUL_LOOP:

#             LDA     [RESULT]

#             DADD    [OP1]

#             STA     [RESULT]

#             JC      [ERR_OUT]    # running total spilled past 2 digits

#             LDA     [MCNT]

#             DSUB    $01          # NOT DECA - see below

#             STA     [MCNT]

#     MUL_TEST:

#             LDA     [MCNT]

#             CMPA    $00

#             JNZ     [MUL_LOOP]

#

#   MCNT counts down from OP2 to 0, one loop per multiplication. It

#   is tempting to count down with plain DECA, the way every counter

#   in exercises 09-12 did - but MCNT holds a packed BCD value here

#   (it started as OP2), and DECA is a plain binary decrement with no

#   idea that a nibble stops at 9. $30 (BCD 30) decremented with DECA

#   gives $2F - not $29 - because binary decrement borrows across the

#   whole byte instead of stopping at the nibble boundary, leaving an

#   invalid digit (F) sitting in the ones place. DSUB $01 decrements

#   the correct, BCD-aware way, so 30 counts down 30, 29, 28, ...,

#   1, 0 exactly as expected. Any loop counter built out of a BCD

#   value needs DSUB (or DADD, counting up) for this same reason -

#   plain INCA/DECA are only safe on counters that were never BCD to

#   begin with (like exercise 10's MCNT, which counted a plain binary

#   digit 0-9 and never crossed a nibble boundary).

#

# Divide: DSUB doubles as both the test and the subtraction

# ---------------------------------------------------------------

#   Divide is repeated subtraction again, but DSUB's honest borrow

#   flag lets the loop skip the separate CMPA "does it still fit?"

#   check exercise 10 needed. DSUB itself IS the test: try the

#   subtraction, and only keep the result if no borrow was needed:

#

#     DIV_LOOP:

#             LDA     [DREM]

#             DSUB    [OP2]

#             JC      [DIV_DONE]   # borrowed - DREM was < OP2, stop

#             STA     [DREM]       # no borrow - the subtraction stands

#             LDA     [RESULT]

#             DADD    $01          # quotient += 1 (BCD)

#             STA     [RESULT]

#             JMP     [DIV_LOOP]

#     DIV_DONE:

#

#   When DSUB does borrow, its result is a meaningless wrapped value

#   (exactly like subtracting 5-8 by hand and refusing to borrow -

#   the digits come out wrong) - so it is simply never stored; DREM

#   is left exactly as it was before that attempt, which is correct,

#   since that attempt should not have happened.

#

# Displaying a packed BCD byte: no loop needed at all

# -------------------------------------------------------

#   This is the payoff mentioned up top. RESULT already holds two

#   correct decimal digits, one per nibble, so showing it is just

#   isolating each nibble and turning it into ASCII with OR $30 -

#   the tens digit needs shifting down into position first (SHR four

#   times moves the top nibble into the bottom, filling the vacated

#   top bits with 0, exactly undoing the SHL packing done at entry):

#

#     LDA     [RESULT]

#     STA     [TEMP]         # keep a copy - the shifts below destroy it

#     SHR

#     SHR

#     SHR

#     SHR                    # tens digit, 0-9, now in the low nibble

#     CMPA    $00

#     JZ      [SR_ONES]      # suppress a leading zero, same as exercise 10

#     OR      $30

#     STA     [DISP]

#     SR_ONES:

#     LDA     [TEMP]

#     AND     $0F            # ones digit

#     OR      $30

#     STA     [DISP]

#

#   Compare this to exercise 10's SHOW_RESULT, which needed a whole

#   repeated-subtraction TENS_LOOP to get the same two digits out of

#   a plain binary RESULT. That loop is gone here - not simplified,

#   just genuinely unnecessary, because the digits were never lost

#   in the first place.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, then on the Calculator press:

#   two digits, an operator, two digits, then '=' (or Enter). Try

#   09/03=03 to see exact division, 05-12 to see the borrow/Err path,

#   99/00 for divide-by-zero, and something like 60*60 to see the

#   overflow Err path (60*60=3600, needs four digits - nowhere close

#   to fitting in one packed-BCD byte).

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status alongside this exercise to watch

#   each keypress arrive on $F011, and Registers to watch the Carry

#   flag flip after each DADD/DSUB - notice it behaves like an

#   ordinary "did this operation overflow/borrow?" flag here, the

#   opposite of plain SUB/CMPA's quirky "strictly greater than".

#

# Try this next

# ---------------

#   - Add a NEG flag and a '-' sign, the way exercise 10 handles a

#     negative result, so 05-12 shows -07 instead of Err. You will

#     need to compute OP2 DSUB OP1 instead when the first attempt

#     borrows, exactly mirroring exercise 10's DO_SUB.

#   - Extend RESULT to two packed-BCD bytes (four digits, 0000-9999)

#     using DADDC/DSUBC to chain the carry/borrow between them, the

#     BCD equivalent of exercise 11's 16-bit RESULT/RESULT+1.

#   - Show the division remainder (DREM already holds it when

#     DIV_DONE is reached) alongside the quotient.

# ================================================================

KEY:     .EQU    $F011

DISP:    .EQU    $F031

.ORG    $4000

START:  LDA     $00

STA     [STAGE]        # begin waiting for operand 1's tens digit

# ---------------------------------------------------------------

# Main loop - poll the keypad, dispatch on STAGE

# ---------------------------------------------------------------

WAIT:   LDA     [KEY]

CMPA    $FF             # $FF = idle, no key waiting

JZ      [WAIT]

STA     [KEYVAL]        # stash it - this is the ONLY read of KEY

CMPA    $1B             # Clear key

JZ      [DO_CLEAR]

CMPA    $7F             # CE key

JZ      [DO_CLEAR]

LDA     [STAGE]

CMPA    $00

JZ      [ST_OP1_TENS]

CMPA    $01

JZ      [ST_OP1_ONES]

CMPA    $02

JZ      [ST_OPCODE]

CMPA    $03

JZ      [ST_OP2_TENS]

CMPA    $04

JZ      [ST_OP2_ONES]

CMPA    $06

JZ      [ST_NEWCALC]

JMP     [ST_EQUALS]      # only remaining value is STAGE 5

# ---------------------------------------------------------------

# STAGE 6 -> 0 : answer was showing, this digit starts a new sum

# ---------------------------------------------------------------

ST_NEWCALC:

LDA     $1B              # clear code - wipes the old expression

STA     [DISP]

JMP     [ST_OP1_TENS]

# ---------------------------------------------------------------

# STAGE 0 : operand 1, tens digit -> top nibble of OP1

# ---------------------------------------------------------------

ST_OP1_TENS:

LDA     [KEYVAL]

STA     [DISP]           # echo the digit as typed

LDA     [KEYVAL]

AND     $0F              # ASCII '0'-'9' -> binary 0-9

SHL

SHL

SHL

SHL                      # move it into the top nibble

STA     [OP1]

LDA     $01

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 1 : operand 1, ones digit -> bottom nibble of OP1

# ---------------------------------------------------------------

ST_OP1_ONES:

LDA     [KEYVAL]

STA     [DISP]

LDA     [KEYVAL]

AND     $0F

OR      [OP1]            # merge with the tens nibble already there

STA     [OP1]            # OP1 is now one packed-BCD byte, 00-99

LDA     $02

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 2 : operator  (+ - * /)

# ---------------------------------------------------------------

ST_OPCODE:

LDA     [KEYVAL]

STA     [OPCODE]         # remember which operator was pressed

STA     [DISP]           # echo it

LDA     $03

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 3 : operand 2, tens digit -> top nibble of OP2

# ---------------------------------------------------------------

ST_OP2_TENS:

LDA     [KEYVAL]

STA     [DISP]

LDA     [KEYVAL]

AND     $0F

SHL

SHL

SHL

SHL

STA     [OP2]

LDA     $04

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 4 : operand 2, ones digit -> bottom nibble of OP2

# ---------------------------------------------------------------

ST_OP2_ONES:

LDA     [KEYVAL]

STA     [DISP]

LDA     [KEYVAL]

AND     $0F

OR      [OP2]

STA     [OP2]

LDA     $05

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 5 : waiting for '=' (or Enter) - ignore anything else

# ---------------------------------------------------------------

ST_EQUALS:

LDA     [KEYVAL]

CMPA    $3D              # '='

JZ      [DO_EQUALS]

CMPA    $0D              # Enter

JZ      [DO_EQUALS]

JMP     [WAIT]

DO_EQUALS:

LDA     $3D

STA     [DISP]           # echo the '=' sign

LDA     [OPCODE]

CMPA    $2B              # '+'

JZ      [DO_ADD]

CMPA    $2D              # '-'

JZ      [DO_SUB]

CMPA    $2A              # '*'

JZ      [DO_MUL]

CMPA    $2F              # '/'

JZ      [DO_DIV]

JMP     [WAIT]           # (shouldn't happen)

# ---------------------------------------------------------------

# Addition:  RESULT = OP1 + OP2 (packed BCD). Carry after DADD means

# the true sum needed a third digit (>= 100) - too big to show here.

# ---------------------------------------------------------------

DO_ADD: LDA     [OP1]

DADD    [OP2]

STA     [RESULT]

JC      [ERR_OUT]

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Subtraction:  RESULT = OP1 - OP2 (packed BCD). Unlike plain

# SUB/CMPA, DSUB's Carry means an ordinary "a borrow was needed" -

# OP1 was strictly less than OP2 - so one JC is the whole test (see

# the header note for the full explanation of why this is the

# opposite convention from exercises 10-12).

# ---------------------------------------------------------------

DO_SUB: LDA     [OP1]

DSUB    [OP2]

STA     [RESULT]

JC      [ERR_OUT]

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Multiply:  RESULT = OP1 * OP2, via OP2 repeated BCD additions of

# OP1. MCNT (the loop countdown) is itself a packed-BCD value, so it

# must count down with DSUB $01, not DECA - see the header note.

# ---------------------------------------------------------------

DO_MUL: LDA     $00

STA     [RESULT]

LDA     [OP2]

STA     [MCNT]

JMP     [MUL_TEST]

MUL_LOOP:

LDA     [RESULT]

DADD    [OP1]

STA     [RESULT]

JC      [ERR_OUT]        # running total spilled past 2 digits

LDA     [MCNT]

DSUB    $01              # BCD decrement - NOT DECA

STA     [MCNT]

MUL_TEST:

LDA     [MCNT]

CMPA    $00

JNZ     [MUL_LOOP]

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Divide:  RESULT = OP1 / OP2 (integer quotient), via repeated BCD

# subtraction of OP2 from a running remainder. OP2=0 -> Err. DSUB's

# own Carry flag doubles as the "does it still fit?" test - see the

# header note for why no separate CMPA is needed here.

# ---------------------------------------------------------------

DO_DIV: LDA     [OP2]

CMPA    $00

JZ      [ERR_OUT]

LDA     $00

STA     [RESULT]         # quotient, BCD, counts up

LDA     [OP1]

STA     [DREM]           # remainder, BCD, counts down

DIV_LOOP:

LDA     [DREM]

DSUB    [OP2]

JC      [DIV_DONE]       # borrowed - DREM < OP2, stop here

STA     [DREM]           # no borrow - the subtraction stands

LDA     [RESULT]

DADD    $01              # quotient += 1 (BCD)

STA     [RESULT]

JMP     [DIV_LOOP]

DIV_DONE:

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Out-of-range result for whichever operator was pressed

# ---------------------------------------------------------------

ERR_OUT:

LDA     $45              # 'E'

STA     [DISP]

LDA     $72              # 'r'

STA     [DISP]

LDA     $72              # 'r'

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Show RESULT: a packed-BCD byte needs no conversion loop, just a

# nibble split - see the header note for the full explanation.

# ---------------------------------------------------------------

SHOW_RESULT:

LDA     [RESULT]

STA     [TEMP]           # keep a copy - SHR below destroys ACC

SHR

SHR

SHR

SHR                      # tens digit, 0-9, now in the low nibble

CMPA    $00

JZ      [SR_ONES]        # suppress a leading zero on the tens digit

OR      $30

STA     [DISP]

SR_ONES:

LDA     [TEMP]

AND     $0F              # ones digit

OR      $30

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Explicit Clear / CE handling

# ---------------------------------------------------------------

DO_CLEAR:

LDA     $1B

STA     [DISP]

LDA     $00

STA     [STAGE]

JMP     [WAIT]

RESET_STATE:

LDA     $06              # STAGE 6: next digit clears & restarts

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# Variables (reserved bytes - see exercise 09 for this pattern)

# ---------------------------------------------------------------

STAGE:   .BYTE

OP1:     .BYTE

OP2:     .BYTE

OPCODE:  .BYTE

RESULT:  .BYTE

TEMP:    .BYTE

KEYVAL:  .BYTE

MCNT:    .BYTE

DREM:    .BYTE

.END    $4000

# Exercise: Adding a Circumference Button

## What You Will Build

This tutorial gives the Calculator a brand new button. Type a radius, press the button, and read the circle’s circumference straight off the display:

9  Circ    ->   shows   9=56.5

1  Circ    ->   shows   1=6.2

0  Circ    ->   shows   0=0.0

The source file is tutorial/11_circle_circumference_button.asm. It reuses the state-machine and display techniques from the 4-function calculator tutorial, and adds two new ideas: how "adding a button" actually works on this machine, and how to do fixed-point decimal math (and 16-bit arithmetic) on a CPU that only has 8-bit integers.

## Before You Start

This tutorial builds directly on:

- 10_calculator_four_function.asm — the keypad state-machine pattern, and reading/writing the Calculator’s display port.

- The 4-Function Calculator tutorial document — port map, keypad codes, and the meaning of the Carry flag after a subtraction.

## A Bank of Buttons Reserved for This

Every key on the Calculator — every digit, every operator, Sin, Cos, all of them — is what the project calls a "DIY Button": a small widget that writes one fixed byte to port $F011 the moment it is clicked. That is the entire mechanism. A button does not run any code of its own, and there is no way to attach custom logic to a click — it always boils down to one byte landing on $F011, exactly like pressing "5" or "+" does.

The left-hand side of the keyboard has a 5×4 grid of buttons that ship blank on purpose: label empty, Code $00, Description "Unassigned." That is the project’s built-in way of adding a feature — claim one of those slots rather than repurposing a button (like Sin) that already has a role of its own.

### Configuring the Button

- With the Calculator switched OFF, right-click any blank button in the left-hand grid. This opens Configure Button Attributes. (The top-left slot is a good default choice.)

- Change Annotation to Circ.

- Change Description to Circumference from radius.

- Change Code to $80 — the .asm program below watches for that exact byte, so the button and the program have to agree on it. $80 is free: nothing in the built-in button table uses it, and it falls outside the ASCII digit/hex-letter ranges the button code special-cases, so it passes straight through unchanged.

- Click OK, then switch the Calculator back on.

With that done, clicking the newly-labelled Circ button sends the byte $80 to port $F011 — nothing more, nothing less. Everything that byte causes to happen next lives entirely inside the .asm program, exactly like every other exercise so far.

One thing worth knowing: clicking OK auto-saves this button into Config/defbuttons.ini, but that file is shared across every exercise — reconfiguring a different button later, or using File → Restore Defaults, overwrites it and Circ reverts to "Unassigned." Once the layout works the way you want, use File → Save Button File.... The dialog defaults to %APPDATA%\PY-DIYCALCULATOR\buttons, but save it into %APPDATA%\PY-DIYCALCULATOR\Config instead — the same folder defbuttons.ini lives in — so the backup (e.g. circ_button.ini) sits right next to it and can be restored either with File → Load Button File..., or just by copying it back over defbuttons.ini directly.

## Program Design

The state machine has one fewer stage than the 4-function calculator, since there is only one operand (the radius) and one trigger (Circ) instead of two operands and four operators:

Table 10 outlines the circumference program's design stages.

Table 10: Circumference Program Design Stages

| STAGE | The program is… |
| --- | --- |
| 0 | Waiting for the radius digit (0–9). |
| 1 | Waiting for the Circ button. |
| 2 | Showing an answer — the next digit clears the display and starts a new one. |

## Fixed-Point Math: Faking Decimals With Whole Numbers

Circumference = 2 × π × radius. 2×π is about 6.28 — not a whole number, and this CPU only has integer arithmetic. The standard trick (used everywhere fixed-point math is done on simple hardware) is to scale everything up by 100 first: approximate π as 3.14, so 2×π×100 becomes the whole number 628.

Multiplying that constant by the radius gives the circumference already multiplied by 100 — for radius 9, 628×9=5652, standing in for 56.52. Dividing back down by 100 later peels the decimal point back off. Since 3.14 is only accurate to two decimal places, this tutorial only ever displays one decimal digit, which is as much precision as the approximation actually supports.

## Working in 16 Bits

628 × 9 = 5652, which no longer fits in a single byte (max 255). Unlike the 4-function calculator, the running total here — RESULT — is a 16-bit value stored as two bytes, RESULT (high byte) and RESULT+1 (low byte), the same big-endian layout the codebase already uses for .2BYTE elsewhere (see Data/int-mult-2-byte.asm). The constant 628 is declared the same way:

K:        .2BYTE  $0274         # 628 decimal = 2 x pi(3.14) x 100

Multiplying by repeated addition works the same way as the 4-function calculator’s multiply routine, just across two bytes: ADD on the low bytes, ADDC (add-with-carry) on the high bytes, so a carry out of the low byte correctly ripples into the high one — the 16-bit equivalent of carrying a 1 when adding by hand.

MUL_LOOP:

LDA     [RESULT+1]       # low byte

ADD     [K+1]            # + K's low byte

STA     [RESULT+1]

LDA     [RESULT]         # high byte

ADDC    [K]              # + K's high byte + carry

STA     [RESULT]

## A Flag Quirk Worth Knowing About

This is the part of the exercise that took the most care to get right, and it is worth understanding even if you never write another line of assembly for this machine: this CPU’s Carry flag after SUB or CMPA means "the accumulator is strictly greater than the operand" — not the more common "no borrow was needed," which on most CPUs also covers the case where the two values are equal.

That has two consequences:

- Testing "is A ≥ K?" needs three branches, not one. A lone "jump if no-carry" only catches strictly-less-than; you also need to check for the equal case separately (CMPA K, then JC for greater-than, JZ for equal, and only fall through to "less-than" if neither fired).

- Chaining SUBC after SUB to do a 16-bit subtraction does not work on this CPU. SUBC treats the Carry flag as if it directly means "a borrow is needed" — but Carry from the preceding SUB usually means the opposite (greater-than), so SUBC ends up subtracting one too many from the high byte when the low byte subtraction succeeded cleanly, and one too few when it didn’t.

This was confirmed by direct testing against the CPU: subtracting 100 from 628 using SUB then SUBC produced 272 instead of the correct 528 — the high byte lost an extra 1 it should never have lost. Every 16-bit or repeated subtraction in this program (DIV100_LOOP, TENTHS_LOOP, WTENS_LOOP) instead subtracts the low byte with a plain SUB, then decides by hand — from that SUB’s own flags — whether to knock 1 off the high byte, rather than trusting SUBC to do it automatically:

DIV100_LOOP:

LDA     [RESULT]         # high byte

CMPA    $00

JNZ     [DIV100_SUB]     # nonzero -> RESULT >= 256 > 100, subtract

LDA     [RESULT+1]       # high byte is 0 - compare low byte to 100

CMPA    $64

JC      [DIV100_SUB]     # low byte > 100, still fits, subtract

JZ      [DIV100_SUB]     # low byte == 100, still fits exactly, subtract

JMP     [DIV100_DONE]    # low byte < 100 and high byte is 0, stop

DIV100_SUB:

LDA     [RESULT+1]

SUB     $64              # 100 decimal

STA     [RESULT+1]

LDA     [RESULT]

JC      [DIV100_NB]      # SUB was strictly greater -> no borrow, skip

JZ      [DIV100_NB]      # SUB was exactly equal -> no borrow, skip

SUB     $01              # otherwise the low byte borrowed - take it from the high byte

DIV100_NB:

STA     [RESULT]

LDA     [WHOLE]

INCA

STA     [WHOLE]

JMP     [DIV100_LOOP]

If you take one thing away from this exercise, let it be this: never assume a flag convention from habit on unfamiliar hardware — check it. A one-line test (subtract two known numbers and look at the flag) would have caught this in seconds; instead it showed up as circumferences that were off by hundreds for no visible reason. This program’s answers were confirmed correct for every radius 0–9 by running it against the real CPU emulator, not just by reading the code and assuming it was right.

## Converting Back to Decimal Digits

Once RESULT holds circumference×100 as a plain 16-bit number, getting it back to something displayable takes two rounds of the same repeated-subtraction idea used throughout these tutorials:

- Subtract 100 from RESULT (16-bit, using the manual-borrow technique above) until it won’t fit any more. The count is the whole-number part of the circumference (always ≤ 56, so it fits in a single byte from here on); what is left over (0–99) is the fractional part, still multiplied by 100.

- Subtract 10 from that leftover (now a plain 8-bit value) until it won’t fit any more. That count (0–9) is the one decimal digit that gets displayed.

Each whole-number digit is then turned into its ASCII character with OR $30, exactly as in the 4-function calculator tutorial.

## Verified Against the Real CPU Emulator

This program was run against the project’s own CPU class with simulated keypresses for every radius from 0 to 9, plus a check that pressing a new digit after a result clears the display and starts fresh, and that Clear works mid-entry. Every case matched the expected value exactly (circumference computed as 628×radius÷100, truncated to one decimal place, matching the program’s own fixed-point design).

Table 11 verifies the circumference program's output against true 2πr.

Table 11: Circumference Program Verification Results

| Radius | True 2πr | Program’s Answer |
| --- | --- | --- |
| 0 | 0.00 | 0=0.0 |
| 1 | 6.28 | 1=6.2 |
| 2 | 12.57 | 2=12.5 |
| 3 | 18.85 | 3=18.8 |
| 4 | 25.13 | 4=25.1 |
| 5 | 31.42 | 5=31.4 |
| 6 | 37.70 | 6=37.6 |
| 7 | 43.98 | 7=43.9 |
| 8 | 50.27 | 8=50.2 |
| 9 | 56.55 | 9=56.5 |

The small gap between the "true" value and the program’s answer in every row is entirely explained by the 3.14 approximation of π, not by any remaining arithmetic error — e.g. for radius 9, true circumference is 56.549 while the program’s constant gives 56.52, both of which correctly truncate to display "56.5".

## Try It Yourself

Configure a blank button as described in “A Bank of Buttons Reserved for This”, assemble tutorial/11_circle_circumference_button.asm, Load → CPU, click Run, then on the Calculator type a digit 0–9 and press Circ. Compare the answer against 2×π×radius on any calculator — it should match to one decimal place. Open Display → Port Map Status alongside the exercise to see $F011 receive $80 the moment you click Circ, and each character land on $F031 as the answer is written out.

## Ideas to Extend This Program

- Add an "Area" button too (area = π × r²) — you’ll need to multiply the radius by itself first, then by 314, then divide by 100 twice, since both the squaring and the π-scaling each add a factor of 100.

- Use a closer approximation of π (e.g. 3142, scaled by 1000 instead of 100) for one more decimal digit of accuracy.

- Accept a two-digit radius, the same extension suggested at the end of the 4-function calculator tutorial.

## Complete Source Listing

tutorial/11_circle_circumference_button.asm

# ================================================================

# 11_circle_circumference_button.asm

# Beboputer Hands-On Tutorial — Bonus Exercise (Calculator)

#

# Give the Calculator a new button: type a radius, press it, and

# read off the circle's circumference.

#

# Before you assemble this: create the button

# ----------------------------------------------

#   Every key on the Calculator - digits, operators, Sin, Cos, all

#   of them - is a "DIY Button" that just writes one fixed byte to

#   port $F011 when clicked (see tools/diy_button.py). The left-hand

#   bank has a 5x4 grid of blank buttons kept in reserve for exactly

#   this - each one defaults to Code $00, Description "Unassigned",

#   meaning it isn't wired to anything yet and is free to use. This

#   exercise claims one of those instead of repurposing a button

#   (like Sin) that already has a role of its own.

#

#     1. With the Calculator switched OFF, right-click any blank

#        button in that left-hand grid to open Configure Button

#        Attributes (top-left slot is a good default choice).

#     2. Change Annotation to  Circ

#     3. Change Description to  Circumference from radius

#     4. Change Code to  $80  - this program watches for that same

#        value, so the button and the program agree on what byte

#        means "compute the circumference". ($80 is free: nothing

#        in _BUTTON_DEFAULTS uses it, and it falls outside the

#        ASCII digit/hex-letter ranges diy_button.py special-cases,

#        so it passes straight through unchanged.)

#     5. Click OK, then switch the Calculator back ON.

#

#   That's the whole trick: a "custom button" is just a label and a

#   byte value. All the actual behaviour - reading the radius,

#   doing the math, showing the answer - lives entirely in this

#   .asm program, exactly like every other exercise so far.

#

#   Keep this layout: clicking OK auto-saves the button to

#   Config/defbuttons.ini, but that file is shared by every exercise

#   - reconfiguring a different button later, or clicking File ->

#   Restore Defaults, overwrites it and this button reverts to

#   "Unassigned". To keep a copy you can always get back, use File ->

#   Save Button File... once you're happy with the setup. The dialog

#   defaults to %APPDATA%\PY-DIYCALCULATOR\buttons, but save it into

#   %APPDATA%\PY-DIYCALCULATOR\Config instead - the same folder

#   defbuttons.ini lives in - so the backup (e.g. circ_button.ini)

#   sits right next to it and you can restore by copying it back over

#   defbuttons.ini directly, as well as via File -> Load Button File....

#

# What this program does

# -----------------------

#   Type a single digit (0-9) for the radius, then press Circ. The

#   display fills in "=" followed by the circumference, e.g.

#       9  Circ        ->  shows  9=56.5

#       1  Circ        ->  shows  1=6.2

#       0  Circ        ->  shows  0=0.0

#   Circumference = 2 x pi x radius. Since this only ever works

#   with a single-digit radius (0-9), the answer never needs more

#   than two whole-number digits plus one decimal place.

#

# How it works

# -------------

#   Same keypad-polling state machine as exercise 10:

#

#     STAGE 0   waiting for the radius digit

#     STAGE 1   waiting for the Circ button

#     STAGE 2   answer is showing - next digit clears and restarts

#

#   The new part is the arithmetic. 2 x pi is approximately 6.28,

#   which isn't a whole number - and this CPU only does integer

#   arithmetic. The fix is the same trick real fixed-point code

#   uses everywhere: scale pi up by 100 first (pi -> 314), so

#   2 x pi x 100 becomes the whole number 628. Multiplying that by

#   the radius gives the circumference already multiplied by 100

#   (so 56.52 arrives as the whole number 5652) - then dividing

#   back down by 100 at display time peels the decimal point back

#   off. 314/100 is only accurate to about 2 decimal places, which

#   is why the exercise only ever shows one decimal digit.

#

#   628 x 9 = 5652 no longer fits in one byte (max 255), so unlike

#   exercise 10 the running total here is a 16-bit value, RESULT /

#   RESULT+1 (high byte / low byte, the same big-endian layout used

#   for .2BYTE elsewhere in this codebase - see Data/int-mult-2-

#   byte.asm). Adding 628 to it 9 times uses ADD on the low bytes

#   and ADDC (add-with-carry) on the high bytes, so a carry out of

#   the low byte correctly ripples into the high byte - the 16-bit

#   equivalent of carrying a 1 when you add by hand. (ADD/ADDC's

#   carry chains the ordinary way on this CPU - it's only the

#   subtract side that needs the workaround explained below.)

#

#   Turning that 16-bit total back into digits happens in two

#   passes, both built from the same repeated-subtraction idea as

#   exercise 10's decimal conversion:

#     1. Subtract 100 from RESULT until it won't fit any more. The

#        count is the whole-number part of the circumference

#        (always <= 56, so it fits back in one byte from here on);

#        whatever is left (0-99) is the fractional part times 100.

#     2. Subtract 10 from that leftover until it won't fit any

#        more. That count (0-9) is the one decimal digit shown.

#

#   A note on this CPU's flags (read this if you're adapting the

#   pattern below for your own code):

#     SUB/CMPA set the Carry flag to mean "accumulator is STRICTLY

#     GREATER THAN the operand" - not the more usual "no borrow was

#     needed" (which on most CPUs also covers the equal case). One

#     consequence: after CMPA K, the pair "JC greater / JZ equal /

#     falls through to less-than" is what you need to test

#     "is ACC >= K" - a lone JNC does NOT cover the equal case, it

#     only catches strictly-less-than.

#       The second consequence is bigger: chaining SUBC after SUB to

#     do a 16-bit subtract does NOT work on this CPU, because SUBC

#     consumes the Carry flag as if it directly were "borrow needed"

#     - but Carry from the preceding SUB means the opposite thing

#     (greater-than) in the common case, so SUBC ends up subtracting

#     one too many from the high byte whenever the low-byte

#     subtraction did NOT need to borrow, and one too few when it

#     did. Both DIV100_LOOP and TENTHS_LOOP/WTENS_LOOP below use a

#     manual borrow test instead: subtract the low byte, then look

#     at the flags from THAT subtraction to decide by hand whether

#     to knock 1 off the high byte, rather than trusting SUBC to do

#     it. This was verified empirically against the real CPU - see

#     the project notes for the test cases that pin this down

#     (e.g. 628-100 with SUBC alone silently loses 256 from the

#     high byte; the manual-borrow version gets 528 exactly right).

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run. On the Calculator, type a

#   digit 0-9, then press Circ (the blank button you just

#   configured). Compare the answer against 2 x pi x radius on any

#   calculator - it should match to one decimal place.

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status alongside this exercise to see

#   $F011 receive $80 (Circ) the moment you click it, and each

#   character land on $F031 as the answer is written out.

#

# Try this next

# ---------------

#   - Add an "Area" button too (area = pi x r x r - you'll need to

#     multiply the radius by itself first, then by 314, then divide

#     by 100 twice since both the squaring and the pi-scaling add a

#     factor of 100).

#   - Use a closer approximation of pi (e.g. 3142, scaled by 1000

#     instead of 100) for one more decimal digit of accuracy.

#   - Accept a two-digit radius, the same extension suggested at

#     the end of exercise 10.

# ================================================================

KEY:      .EQU    $F011

DISP:     .EQU    $F031

CIRC_KEY: .EQU    $80        # code assigned to the blank "Circ" button

.ORG    $4000

START:  LDA     $00

STA     [STAGE]        # begin waiting for the radius digit

# ---------------------------------------------------------------

# Main loop - poll the keypad, dispatch on STAGE

# ---------------------------------------------------------------

WAIT:   LDA     [KEY]

CMPA    $FF             # $FF = idle, no key waiting

JZ      [WAIT]

STA     [KEYVAL]        # stash it - this is the ONLY read of KEY

CMPA    $1B             # Clear key

JZ      [DO_CLEAR]

CMPA    $01             # CE key

JZ      [DO_CLEAR]

LDA     [STAGE]

CMPA    $00

JZ      [ST_RADIUS]

CMPA    $02

JZ      [ST_NEWCALC]

JMP     [ST_CIRC]        # only remaining value is STAGE 1

# ---------------------------------------------------------------

# STAGE 2 -> 0 : answer was showing, this digit starts a new one

# ---------------------------------------------------------------

ST_NEWCALC:

LDA     $1B              # clear code - wipes the old expression

STA     [DISP]

JMP     [ST_RADIUS]

# ---------------------------------------------------------------

# STAGE 0 : radius digit

# ---------------------------------------------------------------

ST_RADIUS:

LDA     [KEYVAL]

STA     [DISP]           # echo the digit as typed

LDA     [KEYVAL]

AND     $0F              # ASCII '0'-'9' -> binary 0-9

STA     [RADIUS]

LDA     $01

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 1 : waiting for Circ - ignore anything else

# ---------------------------------------------------------------

ST_CIRC:

LDA     [KEYVAL]

CMPA    CIRC_KEY

JZ      [DO_CIRC]

JMP     [WAIT]

# ---------------------------------------------------------------

# RESULT (16-bit, big-endian: RESULT=hi, RESULT+1=lo)

#       = K * RADIUS = 628 * RADIUS

#       = circumference, x100  (2 x pi x radius x 100, pi = 3.14)

# ---------------------------------------------------------------

DO_CIRC:

LDA     $3D              # '=' sign

STA     [DISP]

LDA     $00

STA     [RESULT]

STA     [RESULT+1]

LDA     [RADIUS]

STA     [CNT]

JMP     [MUL_TEST]

MUL_LOOP:

LDA     [RESULT+1]       # low byte

ADD     [K+1]            # + K's low byte

STA     [RESULT+1]

LDA     [RESULT]         # high byte

ADDC    [K]              # + K's high byte + carry

STA     [RESULT]

LDA     [CNT]

DECA

STA     [CNT]

MUL_TEST:

LDA     [CNT]

CMPA    $00

JNZ     [MUL_LOOP]

# ---------------------------------------------------------------

# Divide RESULT by 100 -> WHOLE (integer circumference, fits in a

# byte: max 56) and a 0-99 remainder left behind in RESULT+1.

#

# Loop while RESULT >= 100 (checked as: high byte nonzero, OR low

# byte strictly greater than 100, OR low byte equal to 100 - that

# three-way CMPA/JNZ + JC + JZ combination is what "greater-than-

# or-equal" actually takes on this CPU, per the flag note above).

#

# Each subtraction is a manual-borrow 16-bit subtract: SUB the low

# byte by 100 first, then look at ITS flags (not SUBC) to decide

# whether the high byte needs to lose 1 - JC or JZ after that SUB

# means the low byte did NOT need to borrow (it was >= 100), so the

# high byte is left alone; otherwise it borrowed, so the high byte

# is decremented by 1 with a plain SUB $01.

# ---------------------------------------------------------------

LDA     $00

STA     [WHOLE]

DIV100_LOOP:

LDA     [RESULT]         # high byte

CMPA    $00

JNZ     [DIV100_SUB]     # nonzero -> RESULT >= 256 > 100, subtract

LDA     [RESULT+1]       # high byte is 0 - compare low byte to 100

CMPA    $64

JC      [DIV100_SUB]     # C=1: low byte > 100, subtract

JZ      [DIV100_SUB]     # Z=1: low byte == 100, subtract

JMP     [DIV100_DONE]    # otherwise low byte < 100, stop

DIV100_SUB:

LDA     [RESULT+1]

SUB     $64              # 100 decimal

STA     [RESULT+1]

JC      [DIV100_NOBORROW]  # C=1: old low byte > 100, no borrow

JZ      [DIV100_NOBORROW]  # Z=1: old low byte == 100, no borrow

LDA     [RESULT]           # borrow: high byte loses 1

SUB     $01

STA     [RESULT]

JMP     [DIV100_CNT]

DIV100_NOBORROW:

LDA     [RESULT]           # no borrow: high byte unchanged

STA     [RESULT]

DIV100_CNT:

LDA     [WHOLE]

INCA

STA     [WHOLE]

JMP     [DIV100_LOOP]

DIV100_DONE:

# ---------------------------------------------------------------

# Divide the 0-99 remainder by 10 -> TENTHS (the one decimal digit

# shown). What's left after that (hundredths) is simply dropped.

#

# This is only ever an 8-bit value, so no borrow chaining is

# needed - just the same CMPA/JC/JZ "greater-than-or-equal" test

# used above, in place of the single JNC this loop used to use

# (JNC alone missed the case where REM was exactly 10).

# ---------------------------------------------------------------

LDA     [RESULT+1]

STA     [REM]

LDA     $00

STA     [TENTHS]

TENTHS_LOOP:

LDA     [REM]

CMPA    $0A

JC      [TENTHS_SUB]     # C=1: REM > 10

JZ      [TENTHS_SUB]     # Z=1: REM == 10

JMP     [TENTHS_DONE]    # REM < 10, done

TENTHS_SUB:

LDA     [REM]

SUB     $0A

STA     [REM]

LDA     [TENTHS]

INCA

STA     [TENTHS]

JMP     [TENTHS_LOOP]

TENTHS_DONE:

# ---------------------------------------------------------------

# Display WHOLE (tens + ones, no leading zero) . TENTHS

#

# Same fix here as TENTHS_LOOP above: CMPA/JC/JZ instead of a lone

# JNC, so a WHOLE of exactly 10, 20, ... 50 peels off a tens digit

# correctly instead of stopping one iteration short.

# ---------------------------------------------------------------

LDA     $00

STA     [WTENS]

WTENS_LOOP:

LDA     [WHOLE]

CMPA    $0A

JC      [WTENS_SUB]      # C=1: WHOLE > 10

JZ      [WTENS_SUB]      # Z=1: WHOLE == 10

JMP     [WTENS_DONE]     # WHOLE < 10, done

WTENS_SUB:

LDA     [WHOLE]

SUB     $0A

STA     [WHOLE]

LDA     [WTENS]

INCA

STA     [WTENS]

JMP     [WTENS_LOOP]

WTENS_DONE:

LDA     [WTENS]

CMPA    $00

JZ      [W_ONES]         # skip leading zero on the tens digit

LDA     [WTENS]

OR      $30

STA     [DISP]

W_ONES: LDA     [WHOLE]          # what's left (0-9) is the ones digit

OR      $30

STA     [DISP]

LDA     $2E              # '.'

STA     [DISP]

LDA     [TENTHS]

OR      $30

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Explicit Clear / CE handling

# ---------------------------------------------------------------

DO_CLEAR:

LDA     $1B

STA     [DISP]

LDA     $00

STA     [STAGE]

JMP     [WAIT]

RESET_STATE:

LDA     $02              # STAGE 2: next digit clears & restarts

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# Variables (reserved bytes - see exercise 09 for this pattern)

# ---------------------------------------------------------------

STAGE:    .BYTE

RADIUS:   .BYTE

KEYVAL:   .BYTE

CNT:      .BYTE

RESULT:   .2BYTE

WHOLE:    .BYTE

WTENS:    .BYTE

REM:      .BYTE

TENTHS:   .BYTE

K:        .2BYTE  $0274         # 628 decimal = 2 x pi(3.14) x 100

.END    $4000

# Appendix A: Full Port and Instruction Reference

Every memory-mapped port used across the four exercises in this tutorial, in one place.

Table 12 is the full port reference for the emulator.

Table 12: Full Port Reference

| Port | Device | Direction / Notes |
| --- | --- | --- |
| $F000 | Workbench Switch Bank 1 | Input - 8 toggle switches, bit 7 = leftmost |
| $F001 | Workbench Switch Bank 2 | Input - 8 toggle switches, bit 7 = leftmost |
| $F011 | Keypad (Calculator keys AND on-screen Keyboard) | Input - last key code; idle value is $FF; auto-clears to $FF the instant your program reads a non-$FF value |
| $F021 | Workbench 7-Segment (undecoded) | Output - bits 0-6 drive segments a-g directly |
| $F022 | Workbench 8-Bit LED Bar | Output - one bit per LED |
| $F023 | Workbench 7-Segment (decoded) | Output - low nibble (0-F) selects the digit shape |
| $F024 | Workbench Dual 7-Segment (decoded) | Output - high nibble = left digit, low nibble = right digit |
| $F028 | Terminal | Output - ASCII 32-126 prints the character; $0A prints a newline; everything else is ignored |
| $F031 | Calculator Display | Output - ASCII 32-126 appends to the display; $0D, $10, $1B clear it |
| $F032 | Calculator LEDs | Output - bit 5 = leftmost LED ... bit 0 = rightmost LED |

## The $F011 Keypad Port Has Two Personalities

Both the Calculator and the on-screen Keyboard write to the same port, but they write different kinds of values:

Table 13 explains the two personalities of the $F011 keypad port.

Table 13: $F011 Keypad Port Behavior

| Source | What lands in $F011 |
| --- | --- |
| Calculator digit keys (0-9) | Raw nibble 0-9 (translated from the button's ASCII code before writing) |
| Calculator hex keys (A-F) | Raw nibble 10-15 (translated from the button's ASCII code before writing) |
| Calculator operator/function keys (+, -, =, Clear, ...) | The button's own configured code, unmodified |
| On-screen Keyboard letters, digits, punctuation | Standard ASCII value, unmodified |
| On-screen Keyboard ENTER / BSpace / arrows | Fixed control codes ($05, $04, $07-$0A) - not standard ASCII |

Because of this, a single polling loop cannot reliably tell which physical control sent a value just by looking at the byte. Each exercise in this tutorial is written for one input device at a time to keep the examples unambiguous - Exercise 2 assumes the Calculator's digit/hex keys, Exercise 3 assumes the on-screen Keyboard.

## Full Instruction Set Reference

Every instruction implemented by the CPU, grouped by function. Opcodes match Appendix A, Tables A-2a/A-2b of The Official DIY Calculator Data Book. Addressing-mode abbreviations (imm, abs, abs-x, ind, x-ind, ind-x) are the same ones used in the Assembler Syntax Quick Reference; — marks instructions that take no operand.

### Data Transfer

Table 14 lists the data transfer instructions.

Table 14: Data Transfer Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| LDA | imm $90 / abs $91 / abs-x $92 / ind $93 / x-ind $94 / ind-x $95 | Load the Accumulator from an immediate value or from memory, using any of the five addressing modes. Sets the Negative and Zero flags from the loaded value. |
| STA | abs $99 / abs-x $9A / ind $9B / x-ind $9C / ind-x $9D | Store the Accumulator to memory. No immediate form (there is nowhere to store to). |
| BLDX | imm $A0 / abs $A1 | Load the 16-bit Index register (IX), either from a 16-bit immediate value or from a RAM address. |
| BSTX | abs $A9 | Store the 16-bit Index register (IX) to a RAM address. |
| BLDSP | imm $50 / abs $51 | Load the 16-bit Stack Pointer (SP), either from a 16-bit immediate value or from a RAM address. |
| BSTSP | abs $59 | Store the 16-bit Stack Pointer (SP) to a RAM address. |
| BLDIV | imm $F0 / abs $F1 | Load the 16-bit Interrupt Vector, either from a 16-bit immediate value or from a RAM address. |

### Arithmetic

Table 15 lists the arithmetic instructions.

Table 15: Arithmetic Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| ADD | imm $10 / abs $11 / abs-x $12 | Add a value to the Accumulator. |
| ADDC | imm $18 / abs $19 / abs-x $1A | Add a value to the Accumulator together with the Carry flag, for chaining multi-byte addition. |
| SUB | imm $20 / abs $21 / abs-x $22 | Subtract a value from the Accumulator. |
| SUBC | imm $28 / abs $29 / abs-x $2A | Subtract a value from the Accumulator together with the Carry flag (as a borrow), for chaining multi-byte subtraction. |
| CMPA | imm $60 / abs $61 / abs-x $62 | Compare the Accumulator with a value; sets flags only, the Accumulator is unchanged. Carry is set only when the Accumulator is strictly greater than the operand. |
| INCA | — $80 | Increment the Accumulator by 1. |
| DECA | — $81 | Decrement the Accumulator by 1. |
| INCX | — $82 | Increment the Index register (IX) by 1. |
| DECX | — $83 | Decrement the Index register (IX) by 1. |

### BCD (Decimal) Arithmetic

Table 16 lists the BCD (decimal) arithmetic instructions.

Table 16: BCD (Decimal) Arithmetic Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| DADD | imm $48 / abs $49 / abs-x $4A | Add two BCD (binary-coded decimal) values, producing a correctly-adjusted decimal result. |
| DADDC | imm $68 / abs $69 / abs-x $6A | Add two BCD values together with the Carry flag, for chaining multi-byte decimal addition. |
| DSUB | imm $88 / abs $89 / abs-x $8A | Subtract two BCD values, producing a correctly-adjusted decimal result. |
| DSUBC | imm $B8 / abs $B9 / abs-x $BA | Subtract two BCD values together with the Carry flag (as a borrow), for chaining multi-byte decimal subtraction. |

### Logical (Bitwise)

Table 17 lists the logical (bitwise) instructions.

Table 17: Logical (Bitwise) Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| AND | imm $30 / abs $31 / abs-x $32 | Bitwise AND a value with the Accumulator. |
| OR | imm $38 / abs $39 / abs-x $3A | Bitwise OR a value with the Accumulator. |
| XOR | imm $40 / abs $41 / abs-x $42 | Bitwise XOR a value with the Accumulator. |

### Shift and Rotate

Table 18 lists the shift and rotate instructions.

Table 18: Shift and Rotate Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| SHL | — $70 | Shift the Accumulator left one bit. Bit 7 moves into Carry; bit 0 fills with 0. |
| SHR | — $71 | Shift the Accumulator right one bit. Bit 0 moves into Carry; bit 7 fills with 0. |
| ROLC | — $78 | Rotate the Accumulator left through Carry (a 9-bit rotate including the Carry flag). |
| RORC | — $79 | Rotate the Accumulator right through Carry (a 9-bit rotate including the Carry flag). |

### Stack Operations

Table 19 lists the stack operation instructions.

Table 19: Stack Operation Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| PSHA / PUSHA | — $B2 | Push the Accumulator onto the stack. PUSHA is an alias for PSHA. |
| POPA | — $B0 | Pop the top of the stack into the Accumulator. |
| PUSHSR | — $B3 | Push the Status Register (flags) onto the stack. |
| POPSR | — $B1 | Pop the top of the stack into the Status Register (flags). |

### Program Control — Jumps and Subroutines

Table 20 lists the jump and subroutine instructions.

Table 20: Jump and Subroutine Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| JMP | abs $C1 / abs-x $C2 / ind $C3 / x-ind $C4 / ind-x $C5 | Unconditional jump to a new address. |
| JSR | abs $C9 / abs-x $CA / ind $CB / x-ind $CC / ind-x $CD | Jump to a subroutine; pushes the return address onto the stack first. |
| RTS | — $CF | Return from a subroutine; pops the return address off the stack. |
| RTI | — $C7 | Return from an interrupt; restores the flags and Program Counter from the stack. |

### Program Control — Conditional Jumps

Table 21 lists the conditional jump instructions.

Table 21: Conditional Jump Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| JZ | abs $D1 | Jump if the Zero flag is set. |
| JNZ | abs $D6 | Jump if the Zero flag is clear. |
| JN | abs $D9 | Jump if the Negative flag is set. |
| JNN | abs $DE | Jump if the Negative flag is clear. |
| JC | abs $E1 | Jump if the Carry flag is set. |
| JNC | abs $E6 | Jump if the Carry flag is clear. |
| JO | abs $E9 | Jump if the Overflow flag is set. |
| JNO | abs $EE | Jump if the Overflow flag is clear. |

### Interrupt Control

Table 22 lists the interrupt control instructions.

Table 22: Interrupt Control Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| SETIM | — $08 | Set the Interrupt Mask bit, disabling maskable interrupts. |
| CLRIM | — $09 | Clear the Interrupt Mask bit, enabling maskable interrupts. |

### System

Table 23 lists the system instructions.

Table 23: System Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| NOP | — $00 | No operation; advances the Program Counter only. |
| HALT | — $01 | Stop the CPU. A Reset is required to resume execution. |

# Appendix B: Troubleshooting

- Program does not seem to run at all: check that the Calculator is powered on (its On/Off button) - Run, Step, and Reset all silently do nothing while it is off.

- My program vanished after I turned the Calculator on: powering on purges RAM. Always power on first, then Assemble and Load -> CPU.

- Assemble succeeded but nothing happens on Run: remember Assemble does not load the program by itself - click Load -> CPU afterward. The button is disabled until assembly succeeds.

- Reloading a second program does not clear the display/LEDs from the first one: Load -> CPU clears the program area in RAM (including the I/O port bytes) before writing your new bytes, but the on-screen displays only repaint when your program writes to them again - so a stale LED or 7-segment value can stay on screen until your new program overwrites that port.

- A key doesn't seem to register: check WAIT loops correctly test for the $FF idle value with CMPA $FF and JZ - if the loop condition is inverted (JNZ instead of JZ, or vice-versa) it will either spin forever or fall straight through without waiting. The I/O Ports Display's $F011 box (current + previous value) is the fastest way to confirm whether a keypress actually reached the port.

- The Terminal or Workbench displays stay blank even though my program is clearly running: confirm the panel is actually open (Display -> Terminal / Tools -> Workbench 1) and that the Calculator is powered on - both devices silently discard writes while powered off.

- Everything runs too fast to follow: lower the clock speed with Tools -> System Clock... (1 Hz is easiest for tracing a loop one pass at a time; the default is 100 Hz).

- Not sure what a program actually assembled to: use the Memory Walker's GO field to jump to the address in question and read the DATA column directly, or double-click a DATA cell to poke a test value in by hand.

# Appendix C: ASCII Character Set

Standard 7-bit ASCII, codes $00-$7F (0-127). Codes 0-31 and 127 are non-printing control codes shown by their standard mnemonic (e.g. LF, ESC) rather than a glyph; code 32 is the printable space character, shown here as SPACE.

The emulator's I/O ports only recognise a handful of these codes directly: the Terminal (port $F028, Appendix A) prints ASCII 32-126 as-is and treats $0A (LF) as a newline, ignoring everything else; the Calculator's display (port $F031) appends ASCII 32-126 and clears on $0D (CR), $10, or $1B (ESC). All other control codes are accepted by ports that expect a raw byte (e.g. the Workbench's 7-segment outputs) but have no special meaning to the emulator itself.

Table 24 is the full ASCII character set reference.

Table 24: ASCII Character Set

| Dec | Hex | Char | Dec | Hex | Char | Dec | Hex | Char | Dec | Hex | Char |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | $00 | NUL | 32 | $20 | SPACE | 64 | $40 | @ | 96 | $60 | ` |
| 1 | $01 | SOH | 33 | $21 | ! | 65 | $41 | A | 97 | $61 | a |
| 2 | $02 | STX | 34 | $22 | " | 66 | $42 | B | 98 | $62 | b |
| 3 | $03 | ETX | 35 | $23 | # | 67 | $43 | C | 99 | $63 | c |
| 4 | $04 | EOT | 36 | $24 | $ | 68 | $44 | D | 100 | $64 | d |
| 5 | $05 | ENQ | 37 | $25 | % | 69 | $45 | E | 101 | $65 | e |
| 6 | $06 | ACK | 38 | $26 | & | 70 | $46 | F | 102 | $66 | f |
| 7 | $07 | BEL | 39 | $27 | ' | 71 | $47 | G | 103 | $67 | g |
| 8 | $08 | BS | 40 | $28 | ( | 72 | $48 | H | 104 | $68 | h |
| 9 | $09 | TAB | 41 | $29 | ) | 73 | $49 | I | 105 | $69 | i |
| 10 | $0A | LF | 42 | $2A | * | 74 | $4A | J | 106 | $6A | j |
| 11 | $0B | VT | 43 | $2B | + | 75 | $4B | K | 107 | $6B | k |
| 12 | $0C | FF | 44 | $2C | , | 76 | $4C | L | 108 | $6C | l |
| 13 | $0D | CR | 45 | $2D | - | 77 | $4D | M | 109 | $6D | m |
| 14 | $0E | SO | 46 | $2E | . | 78 | $4E | N | 110 | $6E | n |
| 15 | $0F | SI | 47 | $2F | / | 79 | $4F | O | 111 | $6F | o |
| 16 | $10 | DLE | 48 | $30 | 0 | 80 | $50 | P | 112 | $70 | p |
| 17 | $11 | DC1 | 49 | $31 | 1 | 81 | $51 | Q | 113 | $71 | q |
| 18 | $12 | DC2 | 50 | $32 | 2 | 82 | $52 | R | 114 | $72 | r |
| 19 | $13 | DC3 | 51 | $33 | 3 | 83 | $53 | S | 115 | $73 | s |
| 20 | $14 | DC4 | 52 | $34 | 4 | 84 | $54 | T | 116 | $74 | t |
| 21 | $15 | NAK | 53 | $35 | 5 | 85 | $55 | U | 117 | $75 | u |
| 22 | $16 | SYN | 54 | $36 | 6 | 86 | $56 | V | 118 | $76 | v |
| 23 | $17 | ETB | 55 | $37 | 7 | 87 | $57 | W | 119 | $77 | w |
| 24 | $18 | CAN | 56 | $38 | 8 | 88 | $58 | X | 120 | $78 | x |
| 25 | $19 | EM | 57 | $39 | 9 | 89 | $59 | Y | 121 | $79 | y |
| 26 | $1A | SUB | 58 | $3A | : | 90 | $5A | Z | 122 | $7A | z |
| 27 | $1B | ESC | 59 | $3B | ; | 91 | $5B | [ | 123 | $7B | { |
| 28 | $1C | FS | 60 | $3C | < | 92 | $5C | \ | 124 | $7C | ¦ |
| 29 | $1D | GS | 61 | $3D | = | 93 | $5D | ] | 125 | $7D | } |
| 30 | $1E | RS | 62 | $3E | > | 94 | $5E | ^ | 126 | $7E | ~ |
| 31 | $1F | US | 63 | $3F | ? | 95 | $5F | _ | 127 | $7F | DEL |