# A Virtual 8-Bit Computer on a Raspberry Pi

## How PY-DIYCALCULATOR brings a classic computing education project to life on the world’s favourite single-board computer

By Alvin Brown & Clive “Max” Maxfield

There is something deeply satisfying about understanding how a computer works from the ground up — not in the abstract sense of software layers and operating system calls, but right down to the transistors, logic gates, registers, and raw binary arithmetic that make every digital device tick. For decades, the book How Computers Do Math by Clive “Max” Maxfield and Alvin Brown has guided curious minds through the architecture of an imaginary 8-bit computer called the Beboputer, teaching assembly language, memory-mapped input and output, binary arithmetic, and the fundamentals of CPU design. Now, with PY-DIYCALCULATOR — a faithful Python and tkinter reimagining of the original companion software — the Beboputer has arrived on the Raspberry Pi, and the combination is nothing short of wonderful.

This article walks you through everything you need to know: what PY-DIYCALCULATOR is, what it can do, how to get it running on your Pi, and how to start writing your first assembly language programs.

## What Is PY-DIYCALCULATOR?

PY-DIYCALCULATOR is a fully functional emulator of the fictional 8-bit Beboputer CPU, implemented in Python 3 and tkinter. It faithfully recreates the instruction set, memory architecture, and peripheral devices described in How Computers Do Math, giving readers — and anyone curious about computing history and CPU design — a hands-on environment in which to experiment without needing any physical hardware beyond the computer in front of them.

Don’t Panic if any of the terms on this page are unfamiliar to you, because everything is explained in our book How Computers Do Math, and by the time you’ve read this little scamp you’ll be an expert.

Figure 1 shows the PY-DIYCALCULATOR main window.

The DIY Calculator is based on a very simple microcomputer system comprising a central processing unit (CPU), some memory, and some input and output (I/O) ports (these ports allow the system to “talk” with the outside world). The CPU itself is very straightforward and easy to understand: it has an 8-bit data bus and a 16-bit address bus; it contains a small number of simple registers like an accumulator, index register, and stack pointer; and it supports a very simple instruction set along the lines of ADD, SUBTRACT, SHIFT, ROTATE, AND, and OR.

This virtual system comes equipped with an assembler and a variety of diagnostic tools, such as a CPU Register Display, a Memory Display, and more — the list goes on.

The DIY Calculator also features a virtual calculator front panel with buttons, lights, and a pseudo-liquid crystal display (LCD). This front panel is “connected” to the virtual computer via its input and output ports.

The application presents a rich graphical user interface with multiple floating panels, a built-in assembler and text editor, a virtual workbench with switches and display devices, a calculator widget, a terminal, a memory walker, a disassembler, and a simulated clock. Every instruction the virtual CPU executes is visible in real time. Every memory location can be inspected. Every port can be watched. It is, in effect, a complete learning laboratory for 8-bit computing.

During the course of How Computers Do Math, we learn how to create software routines that detect when the various buttons are pressed, input decimal numbers and convert them into the binary representations used by the computer, perform math operations on these numbers, and then convert them back into decimal values to be presented on the main display.

First we learn about the binary and hexadecimal number systems, and then we learn a little about computers and calculators and perform some simple experiments with the DIY Calculator. Next we are introduced to some fundamental computing concepts such as the use of the index register, the stack, and subroutines. Along the way we gain familiarity with the assembly language used to create programs for the DIY Calculator.

As we proceed through the book, we create a simple four-function (add, subtract, multiply, and divide) calculator program that treats all values as 16-bit integers. Although it is fun to see this calculator perform its magic, we also discover the limitations associated with our 16-bit number representations. Thus, in the More Cool Stuff area on the website, we introduce more sophisticated representations such as fixed-point and floating-point.

And in addition to all of the above, we’ll discover lots of interesting snippets of information, such as the origin of the math symbols we use like +, −, ×, ÷, and =.

What makes the Raspberry Pi such an ideal host for this software is a combination of factors: the Pi runs a Debian-based Linux distribution, Python 3 is a first-class citizen of the platform, tkinter ships with the standard Python installer, so there is no separate GUI toolkit to install, and the Pi is inexpensive enough that a student or hobbyist can dedicate one entirely to this kind of project. Running PY-DIYCALCULATOR on a Raspberry Pi 4 or Pi 5 with a connected display feels completely natural — the GUI is responsive, the assembler is fast, and the whole experience has the right flavour of sitting in front of a dedicated learning machine.

## Before You Begin: What You Will Need

To follow this guide you will need:

- A Raspberry Pi 3B+, Pi 4 (any RAM variant), or Pi 5

- Raspberry Pi OS (Bookworm or Bullseye, 32-bit or 64-bit)

- A keyboard, mouse, and monitor (or VNC access)

- An internet connection for the initial package installation

- The PY-DIYCALCULATOR project archive (a zip file of the Bebop_python folder)

Older Pi models such as the Pi 2 or Pi Zero are not recommended — they lack the processing power to keep the GUI responsive, and the tkinter interface can be slow to redraw on lower-specification hardware. On a Pi 4 or Pi 5, however, the application launches in under five seconds and runs perfectly throughout a long session.

## Installing PY-DIYCALCULATOR on Raspberry Pi OS

There are two ways to get PY-DIYCALCULATOR onto a Pi: install the prebuilt .deb package straight from the project’s GitHub releases (one dpkg command and you’re done), or clone the source and run it directly with python -m beboputer_tk — no separate installer script is needed, since tkinter ships with Python itself. Both take well under two minutes on a Pi 4 with a decent internet connection, and both leave you with the same beboputer launcher command and application-menu entry.

### Method 1: Install the Prebuilt .deb Package (Recommended)

The project publishes a Debian package, beboputer_<version>_all.deb, built with the build_deb.sh script that ships in the repo. Because PY-DIYCALCULATOR is pure Python, the package is architecture-independent (all) — the same file installs on a Pi 3, 4, or 5, 32-bit or 64-bit.

Open a terminal and fetch the latest release’s .deb asset directly from GitHub with curl and wget:

DEB_URL=$(curl -s https://api.github.com/repos/alvinmb/DIYCalculator/releases/latest \
  | grep “browser_download_url.*\.deb” \
  | cut -d ’”’ -f 4)
wget “$DEB_URL”

Then install it with dpkg, and let apt-get install -f pull in the one dependency the package declares (python3-tk) if it isn’t already on your system:

sudo dpkg -i beboputer_*.deb
sudo apt-get install -f

That’s the whole installation. dpkg unpacks the application to /usr/share/beboputer, installs a beboputer launcher to /usr/bin, and registers a .desktop entry so PY-DIYCALCULATOR appears in the Education category of your Raspberry Pi OS application menu, icon included.

### Method 2: Run Directly from Source

If you want the full project tree on your Pi — to read or tweak the Python source, add your own tutorial exercises, or track development between releases — clone the repository instead and run it straight from the source tree. Because tkinter ships with the standard Python installer, there is no separate installer script or GUI toolkit to install.

cd ~
git clone https://github.com/alvinmb/DIYCalculator.git Bebop_python

No git installed? sudo apt install git, or download the zip archive from the green Code button on the GitHub repository page and unzip it:

cd ~
unzip DIYCalculator-main.zip
mv DIYCalculator-main Bebop_python

Check that tkinter is available — it ships with Python on Raspberry Pi OS, but if python3 -c "import tkinter" reports an error, install it with sudo apt install python3-tk. Then launch the application directly:

cd ~/Bebop_python/bin
python3 -m beboputer_tk

That’s it — no installer script, no files copied elsewhere, no desktop entry. Running this way is ideal for development (edit the source in place and re-run to see changes immediately), but it does not give you a beboputer launcher command or an application-menu icon — use Method 1 if you want the fully integrated desktop experience.

### Launching the Application

If you installed via Method 1, click the newly created menu entry, or launch it from a terminal:

beboputer

If you ran it from source (Method 2), use the same python3 -m beboputer_tk command shown above instead. Either way, the application opens maximised, filling your screen with the main window — a normal, resizable window with the usual title bar, but the intent is still a dedicated, full-screen learning environment, so the cleanest way to exit is File → Exit, which shuts down the CPU timer and closes every floating panel before quitting.

## A Tour of the Interface

When PY-DIYCALCULATOR first opens, you are greeted by the main window with a menu bar across the top. All of the application’s tools and panels are accessed through this menu bar. Let us walk through each section.

Figure 3: The CPU Registers Panel

Accessible from the Display menu, the CPU Registers panel is the heartbeat of the emulator. It shows the current state of every register in the Beboputer CPU:

- Accumulator (ACC) — the primary 8-bit working register through which arithmetic and logic operations flow

- Program Counter (PC) — the 16-bit address of the next instruction to be fetched

- Stack Pointer (SP) — points to the current top of the hardware stack in RAM

- Index Register (IX) — an 8-bit register used for indexed addressing modes

- Flags register — individual bits showing the current state of the Carry (C), Zero (Z), Negative (N), and Overflow (V) flags after each instruction

Watching these registers during program execution is one of the most educational aspects of the tool. You can see exactly how a CMPA instruction sets the Carry flag, how an ADD operation causes the Overflow flag to trigger, or how a branch instruction reads the Zero flag to decide whether to jump.

### The Message Display

The Message Display is the application’s diagnostic log — a scrolling text panel that records system events. When you load a program, reset the CPU, step through instructions, halt execution, or purge RAM, the message display reports what happened and when. It is always visible and serves as a running commentary on the state of the emulator.

### The Terminal

The Terminal panel is a CRT-style output device driven by the Beboputer itself via the memory-mapped port at address $F028. When a program writes a byte to this address, the corresponding ASCII character appears in the terminal window. This allows Beboputer programs to produce scrolling text output, creating a genuine interaction between the virtual CPU and a simulated output device.

### The Memory Walker

The Memory Walker is a powerful inspection tool that shows the contents of RAM in real time. You can navigate to any address in the 64KB memory space and see the raw byte values. As programs are loaded and executed, the Memory Walker reflects changes immediately. It is particularly useful for understanding how the stack grows and shrinks during subroutine calls, or for verifying that a program has written the expected values to output ports.

Figure 4 shows the Disassembler panel.

### The Disassembler

The Disassembler panel reads from the Program Counter and shows the human-readable assembly mnemonics corresponding to the machine code currently in memory. This is invaluable when debugging: rather than reading raw hex values, you can see the actual instructions the CPU is about to execute.

### The Port Map Display

Figure 5: The Port Map Status

The Port Map panel shows the current values of all memory-mapped input and output ports. This is where you can see in real time what data has been written to the workbench displays or what value a switch bank is currently presenting.

## The Tools Menu

Beyond the display panels, PY-DIYCALCULATOR provides a set of interactive tool windows.

### System Clock

Reached via Setup → System Clock... (the one tool in this section that lives under Setup rather than Tools). The System Clock tool controls the speed at which the emulated CPU runs. The default is 100 Hz — 100 instruction cycles per second — which is slow enough to watch the registers change in real time. You can increase this for programs that need to run quickly, or slow it down further for careful step-by-step study.

### EPROM Burner

The EPROM Burner simulates the process of programming read-only memory. In a real 8-bit system from the 1970s or 1980s, programs were often stored on EPROM chips that had to be erased with ultraviolet light and reprogrammed with a special device. The Beboputer’s EPROM Burner lets you load assembled programs into specific areas of the memory map, mimicking this workflow without the UV lamp.

### The Calculator

The Calculator is a full-featured RPN (Reverse Polish Notation) calculator built into the application, reflecting the educational context of the original How Computers Do Math book. It supports hexadecimal, decimal, octal, and binary arithmetic — exactly the kind of number-base conversions you need constantly when writing assembly language.

Figure 6 shows the Calculator tool.

Figure 6: The Calculator Tool

### The Keyboard

The Keyboard tool provides a virtual hexadecimal keypad that simulates the physical keyboard described in How Computers Do Math. It presents sixteen hex digit keys (0–9 and A–F) plus additional function keys, and is connected to the Beboputer via two memory-mapped ports:

Table 1 summarizes the Keyboard’s I/O ports.

Table 1: Keyboard I/O Ports

| Port | Address | Direction | Description |
| --- | --- | --- | --- |
| Keyboard data | $F020 | Input | ASCII value of the last key pressed |
| Keyboard status | $F010 | Input | Bit 0 = 1 when a new keypress is waiting |

Figure 7 shows the Keyboard tool.

Figure 7: The Keyboard Tool

This two-port design mirrors the way real hardware keyboards work: a program first polls the status port to check whether a key has been pressed, and only reads the data port once it knows a value is waiting. This prevents the program from accidentally reading a stale value from a previous keypress.

A typical keyboard-polling loop in Beboputer assembly looks like this:

KBSTAT:  .EQU    $F010       # keyboard status port
KBDATA:  .EQU    $F020       # keyboard data port

         .ORG    $4000

WAIT:    LDA     [KBSTAT]    # read keyboard status
         AND     $01         # isolate bit 0 (key-ready flag)
         JZ      [WAIT]      # loop until a key is available
         LDA     [KBDATA]    # read the key value
         ...                 # process it

The AND $01 masks off everything except the key-ready bit. When no key has been pressed, bit 0 is 0 and the JZ (Jump if Zero) instruction sends the program back to the top of the loop. The moment a key is pressed in the Keyboard window, bit 0 goes high, the loop exits, and the program reads the waiting ASCII value from the data port.

This pattern — polling a status port before reading a data port — is fundamental to real-world hardware interfacing and is one of the most important concepts the Beboputer teaches. Once you have understood it in this visual, interactive environment, you will recognise it immediately when you encounter it in device driver code, embedded firmware, or any system that communicates with external peripherals.

Figure 8 shows the Workbench 1 tool.

Figure 8: Workbench 1

The Workbench is perhaps the most visually engaging part of the entire application. It simulates a physical electronics workbench populated with switches and display devices, all wired to the Beboputer’s memory-mapped I/O ports. The devices and their addresses are:

Table 2 lists Workbench 1’s device port addresses.

Table 2: Workbench 1 Device Ports

| Device | Port Address | Description |
| --- | --- | --- |
| Switch Bank 1 | $F000 | 8 toggle switches — input |
| Switch Bank 2 | $F001 | 8 toggle switches — input |
| Keyboard status | $F010 | Bit 0 = 1 when a keypress is waiting — input |
| Keyboard data | $F020 | ASCII value of last key pressed — input |
| 7-seg undecoded | $F021 | Bits 6–0 drive segments a–g directly |
| 8-bit LED bar | $F022 | 8 individual LEDs |
| Single 7-seg decoded | $F023 | Low nibble shown as hex digit 0–F |
| Dual 7-seg decoded | $F024 | High nibble = left digit, low nibble = right |
| Calculator display | $F031 | Scrolling ASCII character display |
| Calculator LEDs | $F032 | 6 individual LEDs |

Writing to these addresses from an assembly language program causes the corresponding display to update instantly. Flipping the switches changes the values readable from the input addresses. The combination creates a genuinely tactile simulation of hardware I/O.

### The Assembler / Editor

The Assembler and Editor is the integrated development environment where you write, assemble, and load Beboputer programs. It is divided into several functional areas:

The source editor occupies the main portion of the window — a plain-text editing area where you type your assembly language source code. It supports full keyboard editing including cut, copy, paste, undo, and redo. Programs are written one instruction per line, with labels in the left column, mnemonics in the centre, and operands to the right. Comments are introduced with the # character and run to the end of the line.

Figure 9 shows the Assembler / Editor tool.

Figure 9: The Assembler / Editor Tool

The toolbar across the top of the window provides the key workflow controls:

- New — clears the editor and starts a fresh program

- Open — loads a previously saved .asm source file from disk

- Save / Save As — writes the current source to a .asm file so your work is preserved between sessions

- Assemble — translates the source code into machine code. If the assembly succeeds, the resulting bytes are loaded directly into the Beboputer’s RAM at the address specified by the .ORG directive, ready to run immediately. If errors are found, they are reported in the output panel below the editor.

- Clear — clears the output panel

The output panel at the bottom of the window shows the results of each assembly attempt. A successful assembly prints a listing of each source line alongside its assembled address and hex machine-code bytes — for example:

4000  A9        LDA     [SW1]
4003  8D        STA     [LED8]
4006  4C        JMP     [LOOP]

This listing is invaluable for understanding the relationship between assembly language and machine code. You can see exactly how many bytes each instruction occupies and what its opcode looks like in hexadecimal. If an error occurs — a typo in a mnemonic, an undefined label, or a missing .ORG directive — the output panel highlights the offending line and describes the problem clearly.

The Insert menu saves you from typing out boilerplate syntax or looking up addressing-mode bracket forms from memory — every command drops ready-made text in at the current cursor position:

- Insert → Directive offers a template line for each assembler directive — .ORG, .BYTE, .2BYTE, .4BYTE, .EQU, and .END — complete with a placeholder label and value that you then edit in place.

- Insert → Instruction does the same for addressing modes: choosing Implied, Immediate, Big Immediate, Absolute, Indexed, Indirect, or PreIndexed inserts a correctly-bracketed example instruction in that mode (PreIndexed, for instance, drops in LDA [[$4000,X]]), which is a fast way to get the punctuation right for the trickier indirect forms without hunting back through the syntax table.

- Insert → Insert String... prompts for a line of text and inserts it as a null-terminated .BYTE data table, automatically wrapped at eight bytes per line — the same layout used for the message strings in this tutorial’s Terminal exercises. Typing “HELLO” and pressing OK, for example, produces a ready-to-use .BYTE block with the ASCII codes for each letter followed by a $00 terminator, instead of you having to hand-convert each character to hex.

- Insert → Insert File... splices the contents of another text file in at the cursor, which is handy for pulling a previously-written subroutine into the current program without retyping it.

Assembler syntax at a glance:

Table 3 is a quick reference for assembler syntax.

Table 3: Assembler Syntax Quick Reference

| Element | Syntax | Example |
| --- | --- | --- |
| Comment | # to end of line | # read switch bank |
| Label definition | NAME: in column 1 | LOOP: |
| Constant definition | .EQU value | SW1: .EQU $F000 |
| Load address | .ORG address | .ORG $4000 |
| Immediate operand | bare value | LDA $FF or ADD 10 |
| Memory reference | [address or label] | LDA [SW1] |
| End of source | .END | .END |

Once assembled, the program is live in RAM and can be run, stepped through, or inspected in the Memory Walker immediately — no separate load step is required.

## Writing Your First Assembly Language Program

The Beboputer assembly language is clean and readable. Instructions use familiar mnemonics: LDA to load the accumulator, STA to store it, ADD and SUB for arithmetic, JMP to jump unconditionally, JZ to jump if the Zero flag is set, JSR to call a subroutine, RTS to return. The assembler supports labels, constants defined with .EQU, and the .ORG directive to set the load address.

Comments begin with #. Immediate values are written as plain numbers (e.g. $3F for hex, 63 for decimal). Memory references are written in square brackets (e.g. [SW1] or [$F000]).

Here is a simple program that reads Switch Bank 1 and mirrors its value continuously to the LED bar:

SW1:     .EQU    $F000
LED8:    .EQU    $F022

         .ORG    $4000

LOOP:    LDA     [SW1]
         STA     [LED8]
         JMP     [LOOP]

         .END

Type this into the Assembler / Editor, click Assemble, then click Run in the Control toolbar. Flip the switches in the Workbench 1 window and watch the LED bar respond immediately.

The CMPA instruction deserves special mention because it behaves differently from many other 8-bit CPUs: in the Beboputer, the Carry flag is set when the Accumulator is greater than the operand, rather than the more common convention where Carry indicates a borrow. Keep this in mind when writing comparison and branching logic — the branch instruction JC (Jump if Carry) will fire when ACC > operand.

## Stepping Through a Program

One of the most powerful learning features is single-step execution. Instead of clicking Run, click Step. The CPU executes exactly one instruction and then halts, updating all register displays, the Memory Walker, and the Port Map. You can see precisely what each instruction did: which flags changed, what value arrived in the accumulator, what address the Program Counter moved to. Stepping through a subroutine call and watching the Stack Pointer decrement as the return address is pushed onto the stack is the kind of insight that no textbook diagram can fully convey.

## Managing Memory

The Beboputer has a full 64KB address space ($0000 to $FFFF). User programs are typically loaded at $4000, leaving the lower addresses free for data and the stack, and the upper range ($F000–$F0FF) reserved for memory-mapped I/O. RAM can be cleared at any time using File → Purge RAM, which zeroes all 64KB and restores the I/O sentinel values, then refreshes the Memory Walker and all display panels automatically.

The EPROM Burner can load pre-assembled binary images into any region of the address space, simulating the presence of ROM chips in a physical system.

## Exiting the Application

The main window has a normal title bar, but exiting via the operating system close button skips the emulator’s own cleanup, so the intended way to quit is File → Exit. This cleanly shuts down the CPU timer, closes all floating panels and tool windows, and terminates the application.

## Why the Raspberry Pi Is the Perfect Host

The Raspberry Pi’s combination of affordable hardware, a mature Debian-based operating system, and strong Python support makes it an ideal dedicated machine for PY-DIYCALCULATOR. A Pi 4 or Pi 5 with a seven-inch or ten-inch touchscreen display, housed in a small case on a desk, becomes a permanent learning station — always ready to assemble and run Beboputer programs, always presenting the virtual workbench, always available for experimentation.

There is also something philosophically appropriate about running an 8-bit computer emulator on a modern ARM processor. The Beboputer’s fictional CPU processes instructions one at a time at 100 cycles per second. The Pi 4’s ARM Cortex-A72 is executing hundreds of millions of instructions per second underneath it — all in service of simulating something so gloriously, instructively simple.

For educators, the combination is compelling. A classroom of Raspberry Pis, each running PY-DIYCALCULATOR, gives every student their own complete assembly language development environment for the cost of a single textbook. The visual immediacy of the workbench displays, the register panel, and the step-by-step execution mode makes abstract concepts — flags, stacks, memory-mapped I/O, subroutine calls — concrete and observable.

## Next Steps

PY-DIYCALCULATOR on the Raspberry Pi is more than just a piece of software. It is a bridge between the golden age of 8-bit computing and modern hardware, between theory and practice, between a book and a blinking LED. Whether you are a student encountering assembly language for the first time, an educator looking for an engaging hands-on tool, or a seasoned engineer who learned to program on a Z80 or 6502 and wants to revisit those fundamentals, the Beboputer has something to offer.

Install it. Flip the switches. Watch the LEDs. Write a few dozen lines of assembly. Step through them instruction by instruction and watch the Program Counter tick forward, the flags flip, the accumulator fill with values. Then write something more ambitious — a hex display routine, a sorting algorithm, a looping pattern on the LED bar — and feel the satisfaction of understanding exactly what the machine is doing at every moment.

That understanding is rarer than it should be. PY-DIYCALCULATOR, on a Raspberry Pi, puts it within reach of everyone.

Figure 10 shows How Computers Do Math, the companion book to PY-DIYCALCULATOR.

Festooned with nuggets of information and tidbits of trivia, our book How Computers Do Math (ISBN: 0471732788) provides an incredibly fun and interesting introduction to the way in which computers perform their magic in general and how they do math in particular. PY-DIYCALCULATOR is the companion software to the book, published by Wiley. The Python software can be downloaded from the DIY Calculator website: DIY Calculator :: Home of How Computers Do Math

Figure 10: How Computers Do Math — the Companion Book

# Part 2 — Additional Learning Material

The remainder of this document is the Beboputer Hands-On Tutorial: a set of worked, tested assembly-language exercises for PY-DIYCALCULATOR’s four interactive peripherals (Workbench 1, the Calculator, the on-screen Keyboard, the Terminal), plus reference appendices covering every memory-mapped port, the full instruction set, and the ASCII character set. Where the article above introduces PY-DIYCALCULATOR and its interface, this tutorial is the hands-on companion — start here once you have the application installed and want to write your own Beboputer programs.

# Beboputer Hands-On Tutorial

Exercising the Workbench, Calculator, Keyboard, and Terminal with real assembly programs

# Before You Start

This tutorial gives you working, tested assembly-language programs for each of the four interactive peripherals in PY-DIYCALCULATOR: Workbench 1 (switches, LEDs, 7-segment displays), the Calculator (hex keypad, LCD display, indicator LEDs), the on-screen Keyboard (QWERTY typing), and the Terminal (text output). It also covers the three tools you will lean on constantly while working through the exercises: the Memory Walker, the I/O Ports Display, and the System Clock. Every example uses the real memory-mapped ports and instruction set implemented in the emulator, and every program has been traced instruction-by-instruction against the CPU’s actual opcode behaviour.

## The One Rule That Trips Everyone Up: Power On First

Important: Turning the Calculator’s On/Off button ON purges all 64KB of RAM and resets the CPU. If you assemble and load a program before powering on, switching the Calculator on afterward will wipe it out. Always power on the Calculator BEFORE you assemble and load your program.

Here is why: the Calculator’s On/Off button is wired to more than just the calculator screen. Powering on also enables the Terminal, enables the Workbench’s switches and displays, and (the surprising part) automatically purges RAM, resets the CPU, and starts it running. So the safe sequence for every exercise in this tutorial is:

- Launch PY-DIYCALCULATOR. The Calculator window opens automatically (powered off).

- Click the Calculator’s On/Off button to power everything on.

- Open whichever panels the exercise needs (Workbench 1, Keyboard, Terminal) from the Tools and Display menus.

- Open Tools -> Assembler / Editor, type in the example program, and click Assemble.

- Click Load -> CPU (this button becomes active once assembly succeeds).

- Use the Calculator’s Step or Run buttons, or the Memory Walker’s own STEP column, to execute your program.

Also note: Run, Step, and Reset only exist as buttons on the Calculator’s bottom bar - there is no separate menu item for them. Even a Workbench-only or Terminal-only exercise still needs the Calculator open and powered on, because that is the only place to start your program running (the Memory Walker can single-step independently, but Run still lives on the Calculator).

## Menu Reference

Table 4 maps each panel and tool to its menu path.

Table 4: Menu Reference

| Panel / Tool | Menu Path |
| --- | --- |
| Calculator | Tools -> Calculator... |
| Workbench 1 | Tools -> Workbench 1... |
| Keyboard | Tools -> Keyboard... |
| Terminal | Display -> Terminal |
| Memory Walker | Memory -> Memory Walker (already open by default at startup) |
| I/O Ports Display | Display -> Port Map Status |
| System Clock | Setup -> System Clock... |
| Assembler / Editor | Tools -> Assembler / Editor... |
| CPU Registers | Display -> CPU Registers |
| Purge RAM | File -> Purge RAM... |

## Assembler Syntax Quick Reference

Table 5 gives the full assembler syntax reference used in this tutorial.

Table 5: Assembler Syntax Quick Reference

| Element | Syntax | Example |
| --- | --- | --- |
| Comment | # to end of line | # read the switches |
| Label | NAME: in column 1 | LOOP: |
| Constant | NAME: .EQU value | SW1: .EQU $F000 |
| Load address | .ORG address | .ORG $4000 |
| Reserve bytes | .BYTE v1, v2, ... | MSG: .BYTE $48, $49, $00 |
| Immediate operand | bare hex value | LDA $FF |
| Direct (memory) operand | [address or label] | LDA [SW1] |
| Indexed operand | [address or label,X] | LDA [MSG,X] |
| Indirect operand | [[address or label]] | JMP [[$4000]] |
| Pre-indexed indirect | [[address or label,X]] | LDA [[$4000,X]] |
| End of source | .END address | .END $4000 |

All the examples in this tutorial write hex values with a $ prefix ($4000, $0A, and so on), which is the form used consistently throughout the built-in Assembler / Editor’s own Insert menu snippets.

## Instruction Set Used in This Tutorial

Table 6 lists the instruction set used in this tutorial.

Table 6: Instruction Set Used in This Tutorial

| Mnemonic | What it does |
| --- | --- |
| LDA | Load the Accumulator (immediate, direct, indexed, indirect, or pre-indexed indirect) |
| STA | Store the Accumulator to memory (direct, indexed, indirect, pre-indexed indirect - no immediate form) |
| ADD / SUB | Add / subtract into the Accumulator |
| CMPA | Compare the Accumulator with a value. Sets the Zero flag when equal. Unusually, Carry is set only when the Accumulator is strictly greater than the operand. |
| AND / OR / XOR | Bitwise logic against the Accumulator (immediate, direct, or indexed) |
| SHL | Shift the Accumulator left one bit (carry out captures the vacated bit) |
| BLDX | Load the 16-bit Index register (immediate or direct) |
| INCX / DECX | Increment / decrement the Index register |
| JMP | Unconditional jump (direct, indirect, or pre-indexed indirect) |
| JZ / JNZ | Jump if the Zero flag is / is not set |
| JC / JNC | Jump if Carry is / is not set |
| JSR / RTS | Call a subroutine / return from one |
| HALT | Stop the CPU |

Tip: LDA already sets the Zero and Negative flags from the value it loads. In the message-printing exercise later in this tutorial, a loop tests for a zero byte with a plain JZ right after the LDA - no separate CMPA is needed.

# Tools You Will Use While Debugging

These three tools are not tied to any one exercise - you will reach for all of them constantly while working through this tutorial. Get comfortable with them now.

## The Memory Walker

The Memory Walker is already open when the application starts (it is one of the three panels shown by default, alongside the Message Display and the Calculator), and it can also be reopened from Memory -> Memory Walker. It shows a scrollable, live 256-row window of RAM with four columns: BP, STEP, ADDRESS, and DATA.

Table 7 describes the Memory Walker’s controls.

Table 7: Memory Walker Controls

| Control | What it does |
| --- | --- |
| Address field + GO | Type a 4-digit hex address (e.g. 4000) and click GO to jump the view there. This locks the view in place - it will not auto-follow the Program Counter until you step from inside the Memory Walker again. |
| Click a BP cell | Toggles a breakpoint (shown as a red dot) at that row's address. |
| Click a STEP cell | Executes exactly one CPU instruction, independently of the Calculator's Step button, and updates the status line with the mnemonic that ran and the resulting Accumulator value. |
| RUN to BP | Runs the CPU until it hits any breakpoint, executes HALT, or reaches the safety limit of 500,000 steps. |
| Clear BPs | Removes every breakpoint. |
| Double-click a DATA cell | Lets you type a new hex byte directly into that RAM address - a live memory editor. |

While your program is running or single-stepping, an amber triangle marks the row at the current Program Counter, and the view automatically scrolls to keep it in sight (unless you have manually navigated there with GO).

Try this: Load the switches-to-LEDs program from the Workbench exercise below, then click the BP cell next to the LOOP label’s address before pressing Run. Execution will stop the instant it reaches that address, and you can then use the STEP column to advance one instruction at a time and watch the DATA column change as STA writes to $F022.

Also useful: After assembling a program that includes a .BYTE table (see the message-printing exercise later in this tutorial), type the table’s address into the Memory Walker and click GO to confirm the assembler laid out exactly the bytes you expect.

## The I/O Ports Display

Open with Display -> Port Map Status. Unlike the Memory Walker, this panel does not show general RAM - it is a dedicated live monitor for exactly three fixed addresses: the ones the Calculator itself uses.

Table 8 shows what the I/O Ports Display shows at each address.

Table 8: I/O Ports Display Address Map

| Address | What is shown |
| --- | --- |
| $F031 | The last byte written to the Calculator's display, in hex and as a printable character/annotation. |
| $F032 | The last byte written to the Calculator's LEDs, in hex and as an 8-bit binary pattern. |
| $F011 | The current key code (hex + annotation) and the previous key code (hex + annotation), so you can see a keypress transition even though the port auto-clears back to $FF. |

Important: The I/O Ports Display does NOT show the Workbench’s ports ($F000, $F001, $F021-$F024) or the Terminal’s port ($F028) - only the three addresses above. For the Workbench exercises, watch the Workbench panel itself; the I/O Ports Display is most useful for the Calculator and Keyboard exercises.

The “I/P from Buttons” field is editable: type a hex value (e.g. 07) into it and press Enter, and it writes that value directly into RAM at $F011 - simulating a keypress without touching the Calculator or the on-screen Keyboard at all. This is a convenient way to feed a specific test value into a WAIT/poll loop while you are debugging it.

## The System Clock

Open with Setup -> System Clock... This dialog sets how fast the emulated CPU runs, from 1 Hz up to 10,000 Hz (the default is 100 Hz). Type a new value and click OK; if Run is currently active, the change takes effect immediately.

Try this: Slow the clock down to around 2-5 Hz before running one of the WAIT-and-poll loops in this tutorial, so you can watch the Memory Walker’s PC marker and the I/O Ports Display update on every single pass of the loop. Speed it back up to the default 100 Hz (or higher) once you just want the program to run.

# Exercise: Workbench 1

Open Tools -> Workbench 1... Workbench 1 gives you two 8-switch input banks and four different output displays, all wired to fixed ports:

Table 9 lists Workbench 1’s ports and devices.

Table 9: Workbench 1 Port Reference

| Port | Device |
| --- | --- |
| $F000 | Switch Bank 1 (input) |
| $F001 | Switch Bank 2 (input) |
| $F021 | 7-Segment, undecoded (bits 0-6 = segments a-g) |
| $F022 | 8-bit LED bar |
| $F023 | 7-Segment, decoded (low nibble = hex digit 0-F) |
| $F024 | Dual 7-Segment, decoded (high nibble = left digit, low nibble = right digit) |

## Mirror the Switches on the LED Bar

The simplest possible I/O program: whatever Switch Bank 1 reads, the LED bar shows.

SW1:    .EQU    $F000
LED8:   .EQU    $F022
 
        .ORG    $4000
 
LOOP:   LDA     [SW1]
        STA     [LED8]
        JMP     [LOOP]
 
        .END    $4000

Assemble, Load -> CPU, then click Run on the Calculator. Flip switches in Switch Bank 1 and watch the LED bar respond immediately.

Debug it: Set a breakpoint on the LOOP address in the Memory Walker, click RUN to BP, then use the STEP column to advance one instruction at a time - you’ll see the DATA byte at $F022 change the moment STA executes.

## Show a Switch Value as a Hex Digit

This masks Switch Bank 1 down to its low nibble and shows it as a single hex digit (0-F) on the decoded 7-segment display.

SW1:    .EQU    $F000
SEG2:   .EQU    $F023
 
        .ORG    $4000
 
LOOP:   LDA     [SW1]
        AND     $0F        # keep only the low nibble
        STA     [SEG2]
        JMP     [LOOP]
 
        .END    $4000

## Drive the Undecoded 7-Segment Directly

$F021 is not decoded - each of its low 7 bits drives one segment (a through g) directly. Any 7-bit pattern from the switches becomes a (probably nonsensical, and genuinely fun to explore) segment pattern.

SW1:    .EQU    $F000
SEG1:   .EQU    $F021
 
        .ORG    $4000
 
LOOP:   LDA     [SW1]
        AND     $7F        # keep bits 0-6 (segments a-g)
        STA     [SEG1]
        JMP     [LOOP]
 
        .END    $4000

## Combine Both Switch Banks on the Dual 7-Segment Display

This one is a bit more advanced: it builds a single byte where the low nibble of Switch Bank 1 becomes the left digit and the low nibble of Switch Bank 2 becomes the right digit, using four SHL instructions to move a nibble into the high position.

SW1:    .EQU    $F000
SW2:    .EQU    $F001
SEG3:   .EQU    $F024
 
        .ORG    $4000
 
LOOP:   LDA     [SW1]
        AND     $0F        # low nibble of bank 1 -> left digit
        SHL
        SHL
        SHL
        SHL                # shift it into the high nibble
        STA     [TEMP]
        LDA     [SW2]
        AND     $0F        # low nibble of bank 2 -> right digit
        OR      [TEMP]     # combine hi(bank1) | lo(bank2)
        STA     [SEG3]
        JMP     [LOOP]
 
TEMP:   .BYTE   $00         # scratch byte (never executed - JMP always skips it)
 
        .END    $4000

Debug it: Navigate the Memory Walker to the TEMP label’s address (use GO) to watch the intermediate shifted value land there on every pass.

# Exercise: The Calculator

Open Tools -> Calculator... Every key on the Calculator writes a code to port $F011 (the same port the on-screen Keyboard uses - see the note in the next exercise). Digit keys 0-9 and hex keys A-F are translated to their raw nibble value (0-15) before being written, which lines up perfectly with the decoded 7-segment display’s own low-nibble convention.

## Show the Last Key Pressed on a 7-Segment Display

Open Workbench 1 as well for this one, so you have somewhere to see the result. Press any digit or hex key (0-9, A-F) on the Calculator and watch the decoded 7-segment display echo it.

KEY:    .EQU    $F011
SEG2:   .EQU    $F023
 
        .ORG    $4000
 
WAIT:   LDA     [KEY]      # read the keypad port
        CMPA    $FF        # $FF means idle - no key waiting
        JZ      [WAIT]     # keep waiting
        AND     $0F        # keep the low nibble (0-9, A-F)
        STA     [SEG2]     # show it
        JMP     [WAIT]
 
        .END    $4000

How this works: Port $F011 is a read-clear latch: the moment your program reads a non-$FF value, the underlying byte snaps back to $FF so the same keypress is never processed twice. Your LDA still gets the real key value in the Accumulator before that happens - only the copy in RAM is cleared.

Watch it happen: Open the I/O Ports Display (Display -> Port Map Status) alongside this exercise. Its $F011 box shows both the current and previous key code with annotations, which makes the read-clear behaviour easy to see in real time. You can also type a hex value straight into its “I/P from Buttons” field to feed the loop a test key without touching the Calculator at all.

## Write Text to the Calculator’s Own Display

The Calculator’s LCD is itself a memory-mapped output device at $F031. Printable ASCII (32-126) appends a character; codes $0D, $10, or $1B clear the display.

DISP:   .EQU    $F031
 
        .ORG    $4000
 
        LDA     $1B        # clear code
        STA     [DISP]
 
        LDA     $48        # 'H'
        STA     [DISP]
        LDA     $49        # 'I'
        STA     [DISP]
 
        HALT
 
        .END    $4000

Assemble, Load -> CPU, then Step through it a few instructions at a time (rather than Run) so you can watch each character land on the display before moving to the next. The I/O Ports Display’s $F031 box will show each character the instant STA executes.

## Light Up the Calculator’s LEDs

$F032 drives the Calculator’s six indicator LEDs - bit 5 is the leftmost, bit 0 the rightmost.

LEDS:   .EQU    $F032
 
        .ORG    $4000
 
        LDA     $2A        # 00101010 -> alternating LEDs on
        STA     [LEDS]
 
        HALT
 
        .END    $4000

# Exercise: The Keyboard and Terminal

Open Tools -> Keyboard... and Display -> Terminal. The on-screen Keyboard writes raw ASCII codes to the same port the Calculator uses, $F011 (letters and digits as their normal ASCII values - unlike the Calculator’s hex keys, the Keyboard tool does not translate digits to raw nibbles). This exercise polls that port and echoes each keystroke to the Terminal at $F028.

Worth knowing: The on-screen Keyboard tool already echoes every keystroke straight to the Terminal itself, independent of any program you write - it is a direct convenience wire built into the tool. That means once your own program is also polling $F011 and writing to $F028, each keystroke will appear TWICE (once from the Keyboard tool’s own wire, once from your program). This is expected - it is not a bug in your assembly code. Watching this doubling happen is actually a good way to confirm your program really is reading and reacting to each keypress.

## Echo Typed Characters to the Terminal

This program also demonstrates translating one device’s codes into another’s: the Keyboard tool’s ENTER key sends code $05, but the Terminal only recognises $0A as a newline, so the program converts one into the other before sending it on.

KEY:    .EQU    $F011
TERM:   .EQU    $F028
 
        .ORG    $4000
 
WAIT:   LDA     [KEY]      # read the keyboard port
        CMPA    $FF
        JZ      [WAIT]     # idle - keep waiting
 
        CMPA    $05        # is it the Keyboard tool's ENTER code?
        JNZ     [SEND]     # no - send the byte as typed
        LDA     $0A        # yes - translate to the Terminal's newline code
SEND:   STA     [TERM]
        JMP     [WAIT]
 
        .END    $4000

Assemble, Load -> CPU, click Run, then type on the on-screen Keyboard. Letters and digits appear in the Terminal (doubled, per the note above); pressing ENTER moves to a new line instead of printing the raw $05 byte.

Slow it down: If characters fly by too fast to follow, drop the System Clock to a few Hz (Setup -> System Clock...) so each poll of WAIT is easy to watch on the Memory Walker’s PC marker.

# Exercise: Printing a Stored Message on the Terminal

This exercise does not need any input device - it prints a fixed message stored in the program itself, using the Index register to walk through a table of bytes. It is the best example in this tutorial for seeing .BYTE data, indexed addressing, and a loop-with-terminator pattern all at once.

TERM:   .EQU    $F028
 
        .ORG    $4000
 
        BLDX    $0000      # IX = 0 (index into MSG)
 
LOOP:   LDA     [MSG,X]    # load the next character
        JZ      [DONE]     # the $00 terminator ends the message
        STA     [TERM]     # send it to the Terminal
        INCX               # move to the next character
        JMP     [LOOP]
 
DONE:   HALT
 
MSG:    .BYTE   $48, $45, $4C, $4C, $4F, $2C, $20, $42
        .BYTE   $45, $42, $4F, $50, $55, $54, $45, $52
        .BYTE   $21, $0A, $00
 
        .END    $4000

MSG spells out “HELLO, BEBOPUTER!” followed by a newline ($0A) and a $00 terminator that ends the loop. Assemble, Load -> CPU, and click Run (or Step through it to watch IX count up and PC walk the loop on the CPU Registers panel).

Check your work: After assembling, look at the assembly listing in the output panel to find MSG’s address, then type it into the Memory Walker’s address field and click GO. The DATA column should show exactly the 19 bytes above, in order - a quick way to confirm the table assembled the way you expect before you even run the program.

Try this next: Change the .BYTE table to your own message. Remember every line must end with a $00 terminator for the JZ to find, and each .BYTE line can hold as many comma-separated values as you like (the Assembler / Editor’s own Insert String helper wraps them at 8 per line, purely for readability).

# Challenge: Put It All Together

Once each exercise above is working on its own, try combining them into one program: poll the Keyboard at $F011, echo each character to the Terminal at $F028 (translating ENTER to newline as in Exercise 3), and also show the low nibble of the last key code on the Workbench’s decoded 7-segment display at $F023 (as in Exercise 2). All three pieces reuse the same WAIT-and-poll structure - the only new work is writing to a second and third output port inside the same loop body before jumping back to WAIT. Set a breakpoint at WAIT in the Memory Walker and use RUN to BP to step through the combined loop one keypress at a time.

# Exercise: The Four-Function Calculator

## What You Will Build

This tutorial adds a real, working calculator program to the Beboputer virtual 8-bit computer — one that adds, subtracts, multiplies, and divides. Type a digit, an operator, another digit, then ‘=’ on the Calculator, and the answer appears right on its display, exactly like a pocket calculator:

5 + 3 =    ->   shows   5+3=8

7 * 8 =    ->   shows   7*8=56

2 - 9 =    ->   shows   2-9=-7

9 / 0 =    ->   shows   9/0=Err

The source file is tutorial/10_calculator_four_function.asm. It builds directly on ideas from exercises 05 through 09 — reading the keypad, writing to the display — and adds one new idea: doing real arithmetic on a CPU that has no multiply or divide instruction at all.

## Before You Start

This tutorial assumes you have already worked through, or at least skimmed, these earlier exercises:

- 05_calculator_keypad_to_7seg.asm — how the keypad port ($F011) works as a read-clear latch.

- 06_calculator_display_text.asm — how to write characters to the Calculator’s own display ($F031).

- 09_terminal_print_message.asm — how .BYTE tables and reserved variables are laid out in a program.

Everything you need from those exercises is re-explained below, so you can also read this tutorial on its own.

## How the Calculator Talks to Your Program

Two memory-mapped ports do all the work:

| Port | Direction | What it does |
| --- | --- | --- |
| $F011 | Input | The keypad. Reading it returns the code of the last key pressed, or $FF if no key is waiting. The moment you read a non-$FF value, the port snaps back to $FF — a "read-clear latch" — so the same keypress is never processed twice. |
| $F031 | Output | The Calculator’s own display. Writing a printable ASCII byte (32–126) appends that character to the display, building up an expression left to right. Writing $0D, $10, or $1B clears the display back to empty. |

Because $F031 simply appends characters, the display behaves like a tiny one-line terminal rather than a fixed set of digit slots — which is exactly what lets this program build up “5+3=8” one character at a time as each key is pressed.

## Keypad Codes Used By This Program

The Calculator’s buttons send these exact byte values on $F011 (from tools/calculator.py):

| Key(s) | Code(s) | Meaning in this program |
| --- | --- | --- |
| 0–9 | $30–$39 | ASCII digit codes. AND $0F turns any of these into the plain binary value 0–9. |
| + − × ÷ | $2B $2D $2A $2F | The four operators, stored as-is and compared against later to choose which operation to run. |
| = | $3D | Triggers the calculation. Enter ($0D) does the same thing. |
| Clear / CE | $1B / $01 | Wipes the display and restarts the state machine from scratch. |

## Program Design: A Small State Machine

The whole program is one loop that reads a key, decides what it means based on what has happened so far, and reacts. “What has happened so far” is tracked in one variable, STAGE:

| STAGE | The program is… |
| --- | --- |
| 0 | Waiting for the first digit (operand 1). |
| 1 | Waiting for an operator (+, −, ×, or ÷). |
| 2 | Waiting for the second digit (operand 2). |
| 3 | Waiting for ’=’ (or Enter). |
| 4 | Showing an answer — the next digit pressed clears the display and starts a brand new sum. |

Every keypress runs through the same short dispatcher: read the key once, remember it, check for Clear/CE globally, then jump to the routine for the current STAGE. Each of those routines does three things — echoes the key to the display, updates its stored value, and advances STAGE — except the STAGE-3 routine, which triggers the actual calculation instead.

## Reading a Key Safely (Only Once!)

$F011 is a read-clear latch: the instant your program reads a non-$FF value from it, the port resets to $FF so the same keypress cannot be read twice. That is exactly the behavior that makes polling for a key safe — but it also means you must capture the value the first time you read it, because a second read a few instructions later will just see $FF.

This program reads $F011 exactly once per keypress, in the main WAIT loop, and immediately stores it in the KEYVAL variable:

WAIT:   LDA     [KEY]

CMPA    $FF             # $FF = idle, no key waiting

JZ      [WAIT]

STA     [KEYVAL]        # stash it - this is the ONLY read of KEY

From here on, every comparison for this keypress — “is it Clear?”, “is it a digit?”, “is it the operator I expect?” — reads KEYVAL instead of the port itself.

## Doing the Math Without a MUL or DIV Instruction

The Beboputer CPU’s instruction set has ADD, ADDC, SUB, SUBC, compare (CMPA), shifts, and rotates — but no hardware multiply or divide. Every multiply and divide routine in this codebase, including the full 16-bit versions in Data/int-mult-2-byte.asm and Data/int-div-2-byte.asm, builds those operations out of addition and subtraction in a loop. Because this calculator only ever works with single digits (0–9), the loops here are short (at most 9 passes) and simple enough to read start to finish.

### Addition

The simplest case — just ADD the two operands. The largest possible result is 9+9=18, which fits in a single byte with no overflow to worry about:

DO_ADD: LDA     [OP1]

ADD     [OP2]

STA     [RESULT]

### Subtraction, and the Meaning of the Carry Flag

Subtraction introduces the one flag this whole program leans on — and it is worth being precise about it, because getting it slightly wrong caused a real, hard-to-notice bug (see the callout below). After SUB or CMPA computes “A minus M”, the Carry flag on this CPU is set to 1 only when A is STRICTLY GREATER THAN M — not the more common “no borrow was needed” convention used by most CPUs, which would also cover the case where A and M are equal. When A equals M, Carry comes back 0.

That means a single “jump if carry” branch only ever catches strictly-greater; it does not catch equal. Testing “is A at least M” correctly needs a three-way check: JC for strictly-greater, JZ for exactly-equal, and only fall through to “strictly less” if neither fired. This program has no way to display a negative number directly, so when the second operand is larger than the first, it instead computes the difference the other way around and remembers that the sign should be flipped with a NEG flag:

DO_SUB: LDA     [OP1]

CMPA    [OP2]

JC      [SUB_POS]       # C=1: OP1 > OP2, no borrow needed

JZ      [SUB_POS]       # Z=1: OP1 == OP2, result is 0, no borrow

LDA     [OP2]           # OP1 < OP2: compute OP2-OP1 instead

SUB     [OP1]

STA     [RESULT]

LDA     $01

STA     [NEG]

Later, when the result is displayed, a NEG value of 1 simply prints a − sign before the digits.

### A Bug This Caused, and the Fix

An earlier version of this program used only the single JC branch shown above, without the JZ line. It assembled cleanly and eight of the nine cases in the verification table below happened to still pass — but 5−5 displayed “−0” instead of “0”, because CMPA saw the two equal operands, found Carry = 0 (not strictly greater), and incorrectly fell into the “OP1 < OP2” branch that computes 0 and sets the negative flag anyway. The fix is the JZ [SUB_POS] line above: exactly-equal now takes the same non-negative path as strictly-greater.

The same missing-equal-case bug also affected division and the decimal-digit conversion below, with more visible consequences: 4÷2 returned 1 instead of 2, and any answer that was an exact multiple of 10 (e.g. 2×5) printed a stray “:” character instead of “10”. Both are fixed the same way — a JZ line added alongside the existing JC — and both are covered in the sections and verification table that follow.

### A Second Bug: Digit “1” and the CE Button Were the Same Byte

A separate, unrelated bug turned up after the flag fix above: pressing digit “1” was silently swallowed and treated as a clear instead of entering the digit. The cause has nothing to do with the CPU’s flags — it is about what byte a button actually sends.

Every DIY button writes a fixed byte to $F011 on click, but digit and hex-letter buttons are special-cased by the button framework itself (DIYButton._execute() in tools/diy_button.py): before transmitting, it converts their ASCII code ($30–$39 for ‘0’–‘9’) down to a raw nibble (0–15). That is deliberate — it lets CPU programs read keypad digits as plain binary values without needing to mask off the ASCII bits — but it means digit “1” does not arrive as $31; it arrives as plain byte $01.

The Calculator’s CE button used to be assigned that exact byte, $01, in its default configuration (_BUTTON_DEFAULTS in tools/calculator.py). That made CE and digit “1” bit-for-bit identical on the wire: this program had no way to tell which one had actually been pressed, so its explicit CE check (CMPA $01 / JZ [DO_CLEAR]) fired every time “1” was pressed too, wiping the display instead of accepting the digit.

This is not something the program’s own logic could fix — two different buttons were sending the identical byte, so no amount of cleverness in this .asm file could distinguish them. The real fix was moving CE to a free byte outside the 0–15 nibble range, $7F (ASCII DEL), in _BUTTON_DEFAULTS, so it can never collide with any digit or hex-letter button again. This program’s CE check was updated to match ($7F instead of $01); Clear ($1B, ESC) was never affected, since ESC falls well outside the 0–15 range on its own.

### Multiplication by Repeated Addition

With no multiply instruction, 5 × 3 is computed the way a student might do it by hand before learning the times table: add 5 to itself, 3 times. MCNT counts the remaining additions down to zero:

DO_MUL: LDA     $00

STA     [RESULT]

LDA     [OP2]

STA     [MCNT]

JMP     [MUL_TEST]

MUL_LOOP:

LDA     [RESULT]

ADD     [OP1]

STA     [RESULT]

LDA     [MCNT]

DECA

STA     [MCNT]

MUL_TEST:

LDA     [MCNT]

CMPA    $00

JNZ     [MUL_LOOP]

Checking the counter before the first addition (via the initial JMP to MUL_TEST) also means multiplying by zero correctly produces zero, with no additions performed at all.

### Division by Repeated Subtraction

Division works the same way in reverse: how many times does the divisor (OP2) subtract out of the dividend (OP1) before what is left is smaller than the divisor? Each successful subtraction increments a quotient counter (reusing RESULT). The loop needs to keep subtracting while the remainder is greater than OR equal to the divisor, so — per the flag note in 7.2 — that is the same three-way CMPA/JC/JZ test, not a single branch:

DO_DIV: LDA     [OP2]

CMPA    $00

JZ      [DIV_ERR]

LDA     $00

STA     [RESULT]        # quotient, counts up

LDA     [OP1]

STA     [DREM]          # remainder, counts down

DIV_LOOP:

LDA     [DREM]

CMPA    [OP2]

JC      [DIV_SUB]       # C=1: DREM > OP2, still fits

JZ      [DIV_SUB]       # Z=1: DREM == OP2, still fits exactly

JMP     [DIV_DONE]      # DREM < OP2, can't subtract again

DIV_SUB:

LDA     [DREM]

SUB     [OP2]

STA     [DREM]

LDA     [RESULT]

INCA

STA     [RESULT]

JMP     [DIV_LOOP]

Dividing by zero would never stop subtracting, so OP2 is checked for zero right at the start and diverted straight to a small routine that prints “Err” instead of looping forever. Without the JZ line, this loop stopped one subtraction too early whenever OP2 divided OP1 exactly — 4÷2 subtracted once (4→2), saw the remainder equal the divisor, wrongly treated that as “too small to fit,” and stopped at a quotient of 1 instead of continuing to 2.

## Turning the Answer Back Into Decimal Digits

Every operation above leaves its answer, RESULT, as a plain binary byte — for example the byte value 56 after 7×8, not the two characters ‘5’ and ‘6’. Because RESULT never exceeds 81 (9×9, the largest possible answer here), converting it to decimal only ever needs at most a tens digit and a ones digit. The same repeated-subtraction idea from division does the conversion: subtract 10 from RESULT, counting how many times that succeeds (the tens digit), and whatever is left over afterwards is the ones digit — and, once again, “succeeds” means RESULT is greater than OR equal to 10, so the loop uses the same three-way CMPA/JC/JZ test rather than a single branch:

TENS_LOOP:

LDA     [RESULT]

CMPA    $0A

JC      [TENS_SUB]      # C=1: RESULT > 10, still fits

JZ      [TENS_SUB]      # Z=1: RESULT == 10, still fits exactly

JMP     [TENS_DONE]     # RESULT < 10, tens digit is done

TENS_SUB:

LDA     [RESULT]

SUB     $0A

STA     [RESULT]

LDA     [TENS]

INCA

STA     [TENS]

JMP     [TENS_LOOP]

Once each digit (0–9) is known, OR-ing it with $30 turns it back into the matching ASCII character — the same trick used in reverse by the AND $0F earlier, and the same one used in exercise 05. Without the JZ line, an exact multiple of 10 (e.g. 2×5=10) stopped this loop with RESULT still holding the raw value 10 (0x0A) instead of 0 — and 0x0A OR $30 produces 0x3A, the “:” character, printed in place of the ones digit.

## Verified Against the Real CPU Emulator

This program was actually run against the project’s own CPU class with simulated keypresses — not just assembled and read over — and every digit press was translated through the exact same ASCII-to-raw-nibble conversion the real button framework applies (DIYButton._execute()), not sent as raw ASCII, so these results reflect what the CPU genuinely receives. The table below includes the original scenarios (every operator, a subtraction that goes negative, a divide-by-zero, a multiply-by-zero, and a fresh digit right after an answer), the equal-operand and exact-division/exact-multiple-of-10 cases that exposed the Carry-flag bug (7.2.1), and the digit-”1”/CE cases that exposed the button-code collision (7.2.2): every one of these now produces exactly the expected display text.

| Keys Pressed | Expected Display |
| --- | --- |
| 5, +, 3, = | 5+3=8 |
| 7, ×, 8, = | 7*8=56 |
| 2, −, 9, = | 2-9=-7 |
| 9, −, 2, = | 9-2=7 |
| 9, ÷, 0, = | 9/0=Err |
| 8, ÷, 3, = | 8/3=2 |
| 0, ×, 9, = | 0*9=0 |
| (after a result) 4, +, 4, = | 4+4=8 (old expression cleared first) |
| 4, ÷, 2, = | 4/2=2 (exact division — was 1 before the fix) |
| 6, ÷, 3, = | 6/3=2 (exact division — was 1 before the fix) |
| 9, ÷, 3, = | 9/3=3 (exact division — was 2 before the fix) |
| 5, −, 5, = | 5-5=0 (equal operands — was "-0" before the fix) |
| 2, ×, 5, = | 2*5=10 (exact multiple of 10 — was ":" before the fix) |
| 5, +, 5, = | 5+5=10 (exact multiple of 10 — was ":" before the fix) |
| 1, +, 3, = | 1+3=4 (digit "1" — was swallowed as Clear before the fix) |
| 9, ÷, 1, = | 9/1=9 (digit "1" as divisor) |
| (fresh) 1 | 1 (just the digit — was misread as Clear before the fix) |
| 7, CE, 2, ... | 2 (CE now $7F, clears without colliding with "1") |

## Try It Yourself

Open the Workbench and Calculator, load tutorial/10_calculator_four_function.asm into the assembler, Assemble, Load → CPU, click Run, and on the Calculator press: a digit, an operator, a digit, then ‘=’ (or Enter). Try all four operators, try a subtraction where the second number is bigger, and try dividing by zero. Open Display → Port Map Status alongside the exercise to watch each keypress land on $F011 and each character land on $F031 in real time.

## Ideas to Extend This Program

- Show the division remainder as well as the quotient.

- Accept two-digit operands — this needs a second state per operand and a way to combine tens and ones (tens × 10 + ones) back into one binary value.

- Replace the repeated-addition/subtraction loops with the faster shift-and-add / shift-and-subtract algorithms used in Data/fmult.asm and Data/int-div-2-byte.asm — same idea, far fewer loop iterations for large numbers.

## Appendix: Complete Source Listing

tutorial/10_calculator_four_function.asm

# ================================================================

# 10_calculator_four_function.asm

# Beboputer Hands-On Tutorial — Bonus Exercise (Calculator)

#

# A working 4-function (+ - * /) calculator, built entirely from

# the primitives shown in exercises 05-09: keypad polling, the

# Calculator's own display, and simple assembly-language loops.

#

# What this program does

# -----------------------

#   Lets you type a single-digit sum on the Calculator, e.g.

#       5  +  3  =        ->  shows  5+3=8

#       7  *  8  =        ->  shows  7*8=56

#       2  -  9  =        ->  shows  2-9=-7

#       9  /  0  =        ->  shows  9/0=Err

#   Every keypress is echoed to the display as you type it, so the

#   full expression builds up on screen exactly like a real

#   calculator, then the '=' key replaces nothing - it just appends

#   the computed answer.

#

# Limitations (kept simple on purpose - see "Try this next")

# ------------------------------------------------------------

#   - Operands are single digits only (0-9), no multi-digit numbers

#     and no decimal point.

#   - Division shows the integer quotient only (no remainder).

#   - Dividing by zero shows "Err" instead of a number.

#

# How it works

# -------------

#   The program is a small state machine driven by KEY ($F011):

#

#     STAGE 0   waiting for the first digit  (operand 1)

#     STAGE 1   waiting for an operator      (+ - * /)

#     STAGE 2   waiting for the second digit (operand 2)

#     STAGE 3   waiting for '='  (or Enter)

#     STAGE 4   answer is showing - the next digit starts a new sum

#

#   $F011 is a read-clear latch (see exercise 05): each keypress is

#   read exactly once into KEYVAL and every later comparison in that

#   pass re-uses KEYVAL, since a second LDA [KEY] would just see the

#   $FF idle code.

#

#   Multiply is repeated addition (add OP1 to itself OP2 times) and

#   divide is repeated subtraction (subtract OP2 from OP1 while it

#   still fits, counting how many times) - the CPU has no hardware

#   MUL or DIV instruction, so every calculator and multiply/divide

#   routine on this machine builds the operation out of ADD/SUB in

#   a loop. Because operands here are single digits (0-9 at most),

#   the loops are at most 9 iterations - simple enough to read in

#   one sitting, unlike the 16-bit shift-add/shift-subtract versions

#   in Data/int-mult-2-byte.asm and Data/int-div-2-byte.asm, which

#   solve the same problem for much bigger numbers.

#

#   A note on this CPU's flags (read this before you adapt the

#   pattern below for your own code): SUB/CMPA set the Carry flag

#   to mean "the accumulator is STRICTLY GREATER THAN the operand"

#   - not the more usual "no borrow was needed" (which on most CPUs

#   also covers the equal case). That means a single "jump if

#   carry" or "jump if no-carry" branch only ever catches strictly-

#   greater or strictly-less; it silently gets the equal case wrong.

#   Every "is the left-hand side >= the right-hand side?" test in

#   this program - can OP1 still absorb another subtraction of OP2,

#   is the result >= 10, is OP1 >= OP2 for a same-sign subtraction -

#   therefore uses a three-way test instead of one branch: CMPA,

#   then JC for strictly-greater, JZ for exactly-equal, and only

#   falls through to "strictly less" if neither fired. This was

#   discovered (and is explained at length) in exercise 11's

#   circumference-button program; every exact-division and exact-

#   multiple-of-10 case here needed the same fix.

#

#   A second gotcha, this time about the DIY buttons themselves:

#   pressing a digit button does NOT send its ASCII code ($30-$39)

#   to $F011 - the button framework (tools/diy_button.py) silently

#   converts digit/hex-letter buttons down to a raw nibble (0-15)

#   before transmitting, so CPU programs can read keypad digits as

#   plain binary values. That means digit "1" actually arrives as

#   byte $01. Every button's actual code lives in one place -

#   Config/defbuttons.ini, the file the Calculator loads on startup

#   (see Configure Button Attributes to inspect any button) - and

#   Clear/CE/the four operators all sit outside the 0-15 digit/hex

#   range there ($10-$1A), so none of them can ever collide with a

#   digit or hex-letter keypress.

#

#   A third gotcha: the display (port $F031) does NOT understand

#   every byte as a character. It only recognizes printable ASCII

#   (32-126), raw digit codes $00-$09, raw hex codes $0A-$0F, and

#   $0D/$10/$1B as "clear" - see tools/calculator.py's

#   write_display() for the full list. The keypad codes for the

#   operators ($16-$19) and '=' ($1A) fall outside all of those

#   ranges, so this program can't just echo a keypress straight from

#   KEY to DISP the way it does for digits (which happen to already

#   be valid display codes) - ST_OPCODE below translates each

#   operator's keypad code to its ASCII glyph ('+' is $2B, etc.)

#   before writing it to DISP.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, then on the Calculator press:

#   a digit, an operator, a digit, then '=' (the calculator's '='

#   key, or Enter). Try all four operators, try a subtraction where

#   the second number is bigger (negative result), try dividing by

#   0, and try a case where the operator divides evenly or the

#   result is an exact multiple of 10 (e.g. 4/2, 2*5) - these are

#   exactly the "equal" edge cases the flag note above covers.

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status alongside this exercise to see

#   each keypress arrive on $F011 and each character land on $F031

#   in real time.

#

# Try this next

# ---------------

#   - Show the division remainder as well as the quotient.

#   - Accept two-digit operands (you'll need a second state per

#     operand and a decimal-to-binary combine step: tens*10 + ones).

#   - Replace repeated addition/subtraction with the shift-and-add /

#     shift-and-subtract algorithms from Data/fmult.asm and

#     Data/int-div-2-byte.asm - same idea, fewer loop iterations.

# ================================================================

KEY:     .EQU    $F011

DISP:    .EQU    $F031

.ORG    $4000

START:  LDA     $00

STA     [STAGE]        # begin waiting for the first digit

# ---------------------------------------------------------------

# Main loop - poll the keypad, dispatch on STAGE

# ---------------------------------------------------------------

WAIT:   LDA     [KEY]

CMPA    $FF             # $FF = idle, no key waiting

JZ      [WAIT]

STA     [KEYVAL]        # stash it - this is the ONLY read of KEY

CMPA    $10             # Clear key

JZ      [DO_CLEAR]

CMPA    $11             # CE key

JZ      [DO_CLEAR]

LDA     [STAGE]

CMPA    $00

JZ      [ST_OP1]

CMPA    $01

JZ      [ST_OPCODE]

CMPA    $02

JZ      [ST_OP2]

CMPA    $04

JZ      [ST_NEWCALC]

JMP     [ST_EQUALS]     # only remaining value is STAGE 3

# ---------------------------------------------------------------

# STAGE 4 -> 0 : answer was showing, this digit starts a new sum

# ---------------------------------------------------------------

ST_NEWCALC:

LDA     $1B             # clear code - wipes the old expression

STA     [DISP]

JMP     [ST_OP1]

# ---------------------------------------------------------------

# STAGE 0 : first operand

# ---------------------------------------------------------------

ST_OP1: LDA     [KEYVAL]

STA     [DISP]          # echo the digit as typed

LDA     [KEYVAL]

AND     $0F             # ASCII '0'-'9' -> binary 0-9

STA     [OP1]

LDA     $01

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 1 : operator  (+ - * /)

#

# OPCODE keeps the raw keypad code ($16/$17/$18/$19) for DO_EQUALS'

# dispatch below, but that raw code isn't a display character (see

# the header note), so it has to be translated to its ASCII glyph

# ('+' $2B, '-' $2D, '*' $2A, '/' $2F) before echoing to DISP.

# ---------------------------------------------------------------

ST_OPCODE:

LDA     [KEYVAL]

STA     [OPCODE]        # remember which operator was pressed

CMPA    $16             # '+'

JZ      [OP_ECHO_ADD]

CMPA    $17             # '-'

JZ      [OP_ECHO_SUB]

CMPA    $18             # '*'

JZ      [OP_ECHO_MUL]

JMP     [OP_ECHO_DIV]   # only remaining value is '/'

OP_ECHO_ADD:

LDA     $2B

JMP     [OP_ECHO_DONE]

OP_ECHO_SUB:

LDA     $2D

JMP     [OP_ECHO_DONE]

OP_ECHO_MUL:

LDA     $2A

JMP     [OP_ECHO_DONE]

OP_ECHO_DIV:

LDA     $2F

OP_ECHO_DONE:

STA     [DISP]          # echo the operator's ASCII glyph

LDA     $02

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 2 : second operand

# ---------------------------------------------------------------

ST_OP2: LDA     [KEYVAL]

STA     [DISP]

LDA     [KEYVAL]

AND     $0F

STA     [OP2]

LDA     $03

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 3 : waiting for '=' (or Enter) - ignore anything else

# ---------------------------------------------------------------

ST_EQUALS:

LDA     [KEYVAL]

CMPA    $1A             # '=' key

JZ      [DO_EQUALS]

CMPA    $13             # Enter key

JZ      [DO_EQUALS]

JMP     [WAIT]

DO_EQUALS:

LDA     $3D             # ASCII '=' -- the display glyph, not the

STA     [DISP]          # keypad code (see header note)

LDA     [OPCODE]

CMPA    $16             # '+'

JZ      [DO_ADD]

CMPA    $17             # '-'

JZ      [DO_SUB]

CMPA    $18             # '*'

JZ      [DO_MUL]

CMPA    $19             # '/'

JZ      [DO_DIV]

JMP     [WAIT]          # (shouldn't happen)

# ---------------------------------------------------------------

# Addition:  RESULT = OP1 + OP2   (max 9+9=18, always positive)

# ---------------------------------------------------------------

DO_ADD: LDA     [OP1]

ADD     [OP2]

STA     [RESULT]

LDA     $00

STA     [NEG]

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Subtraction:  RESULT = OP1 - OP2  (negate + flag if OP1 < OP2)

#

# CMPA sets Carry only when OP1 is STRICTLY greater than OP2 (see

# the flag note up top), so both the greater-than case (C=1) and

# the equal case (Z=1, result is 0) need to reach SUB_POS - only

# OP1 strictly less than OP2 falls through to the negative branch.

# ---------------------------------------------------------------

DO_SUB: LDA     [OP1]

CMPA    [OP2]

JC      [SUB_POS]       # C=1: OP1 > OP2, no borrow needed

JZ      [SUB_POS]       # Z=1: OP1 == OP2, result is 0, no borrow

LDA     [OP2]           # OP1 < OP2: compute OP2-OP1 instead

SUB     [OP1]

STA     [RESULT]

LDA     $01

STA     [NEG]

JMP     [SHOW_RESULT]

SUB_POS:

LDA     [OP1]

SUB     [OP2]

STA     [RESULT]

LDA     $00

STA     [NEG]

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Multiply:  RESULT = OP1 * OP2, via OP2 repeated additions of OP1

# (max 9*9=81 - fits comfortably in one byte)

# ---------------------------------------------------------------

DO_MUL: LDA     $00

STA     [RESULT]

LDA     [OP2]

STA     [MCNT]

JMP     [MUL_TEST]

MUL_LOOP:

LDA     [RESULT]

ADD     [OP1]

STA     [RESULT]

LDA     [MCNT]

DECA

STA     [MCNT]

MUL_TEST:

LDA     [MCNT]

CMPA    $00

JNZ     [MUL_LOOP]

LDA     $00

STA     [NEG]

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Divide:  RESULT = OP1 / OP2 (integer quotient), via repeated

# subtraction of OP2 from a running remainder. OP2=0 -> "Err".

#

# Loop while DREM >= OP2 (checked as: DREM strictly greater than

# OP2, OR DREM exactly equal to OP2 - the three-way CMPA/JC/JZ

# combination from the flag note up top; a lone JNC would stop one

# subtraction too early whenever OP2 divides OP1 evenly, e.g. 4/2).

# ---------------------------------------------------------------

DO_DIV: LDA     [OP2]

CMPA    $00

JZ      [DIV_ERR]

LDA     $00

STA     [RESULT]        # quotient, counts up

LDA     [OP1]

STA     [DREM]          # remainder, counts down

DIV_LOOP:

LDA     [DREM]

CMPA    [OP2]

JC      [DIV_SUB]       # C=1: DREM > OP2, still fits

JZ      [DIV_SUB]       # Z=1: DREM == OP2, still fits exactly

JMP     [DIV_DONE]      # DREM < OP2, can't subtract again

DIV_SUB:

LDA     [DREM]

SUB     [OP2]

STA     [DREM]

LDA     [RESULT]

INCA

STA     [RESULT]

JMP     [DIV_LOOP]

DIV_DONE:

LDA     $00

STA     [NEG]

JMP     [SHOW_RESULT]

DIV_ERR:

LDA     $45             # 'E'

STA     [DISP]

LDA     $72             # 'r'

STA     [DISP]

LDA     $72             # 'r'

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Show RESULT (0-81, binary) as decimal digits, '-' first if NEG.

#

# Same three-way CMPA/JC/JZ test peels off the tens digit correctly

# even when RESULT is an exact multiple of 10 (e.g. 2*5=10) - a

# lone JNC would stop one subtraction too early and leave RESULT=10

# to be OR $30'd directly, producing ':' (0x3A) instead of "10".

# ---------------------------------------------------------------

SHOW_RESULT:

LDA     [NEG]

CMPA    $00

JZ      [SR_TENS]

LDA     $2D             # '-'

STA     [DISP]

SR_TENS:

LDA     $00

STA     [TENS]

TENS_LOOP:

LDA     [RESULT]

CMPA    $0A

JC      [TENS_SUB]      # C=1: RESULT > 10, still fits

JZ      [TENS_SUB]      # Z=1: RESULT == 10, still fits exactly

JMP     [TENS_DONE]     # RESULT < 10, tens digit is done

TENS_SUB:

LDA     [RESULT]

SUB     $0A

STA     [RESULT]

LDA     [TENS]

INCA

STA     [TENS]

JMP     [TENS_LOOP]

TENS_DONE:

LDA     [TENS]

CMPA    $00

JZ      [SR_ONES]       # no leading zero when tens digit is 0

LDA     [TENS]

OR      $30

STA     [DISP]

SR_ONES:

LDA     [RESULT]        # what's left (0-9) is the ones digit

OR      $30

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Explicit Clear / CE handling

# ---------------------------------------------------------------

DO_CLEAR:

LDA     $1B

STA     [DISP]

LDA     $00

STA     [STAGE]

JMP     [WAIT]

RESET_STATE:

LDA     $04             # STAGE 4: next digit clears & restarts

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# Variables (reserved bytes - see exercise 09 for this pattern)

# ---------------------------------------------------------------

STAGE:   .BYTE

OP1:     .BYTE

OP2:     .BYTE

OPCODE:  .BYTE

RESULT:  .BYTE

NEG:     .BYTE

KEYVAL:  .BYTE

MCNT:    .BYTE

DREM:    .BYTE

TENS:    .BYTE

.END    $4000

# Exercise: The Four-Function Calculator Using BCD

## What You Will Build

A 4-function calculator, just like exercise 10, except every number in this version is stored as packed BCD (binary-coded decimal) instead of plain binary. Type two-digit operands, an operator, and ‘=’, and read the answer off the display:

2  3  +  1  4  =        ->  shows  23+14=37

0  9  /  0  3  =        ->  shows  09/03=3

0  5  -  1  2  =        ->  shows  05-12=Err   (negative result)

6  0  *  6  0  =        ->  shows  60*60=Err   (3600 needs 3 digits)

9  9  /  0  0  =        ->  shows  99/00=Err   (divide by zero)

The source file is tutorial/13_bcd_math_routines.asm. It reuses the keypad state machine from exercise 10 almost unchanged, but replaces every ADD/SUB with this CPU’s dedicated BCD instructions — DADD, DADDC, DSUB, DSUBC — and, as a direct result, gets to delete the decimal-conversion loop every earlier tutorial needed just to put a number on the screen.

## Before You Start

This tutorial builds directly on:

- 10_calculator_four_function.asm — the keypad state-machine pattern this program reuses almost line for line, and the Carry-flag quirk of plain SUB/CMPA (see “Subtraction: DSUB’s Carry Means the Opposite of SUB/CMPA’s” for why BCD is different).

- 12_trig_functions_sincostan.asm — using SHL/SHR to move bits into position, reused here to pack/unpack nibbles instead of indexing a table.

## Packed BCD: Two Digits, One Byte, No Conversion

Exercises 10 through 12 all stored numbers in plain binary, and all three paid for it at display time: turning a binary total back into decimal digits took a repeated-subtraction loop — subtract 10, or 100, again and again, counting how many times — because a binary byte has no idea where one decimal digit ends and the next begins.

Packed BCD avoids the conversion by never losing the digit boundary in the first place. Each byte holds exactly two decimal digits, one per nibble: the high nibble is the tens digit, the low nibble is the ones digit, and both are always 0–9. The number 47 is stored as the byte $47 — not because 47 happens to equal 0x47 in binary (that is a coincidence of how hex digits are written down), but because the nibble 4 and the nibble 7 are exactly the digits ‘47’ is made of. Reading a BCD byte back out is just splitting it into its two nibbles and turning each into an ASCII character — no loop, no subtraction, no math at all.

The catch is that ordinary ADD/SUB do not know about this packing. Adding $47 and $08 in plain binary gives $4F — a nibble of 15, not a valid decimal digit — because binary addition has no concept of “carry at 10” the way decimal addition does. This CPU has four dedicated instructions for exactly this problem:

| Instruction | What it does |
| --- | --- |
| DADD | Adds two packed-BCD bytes, carrying 1 into the next nibble whenever a digit would overflow past 9. |
| DADDC | Same as DADD, but also adds in the Carry flag — for chaining multi-byte BCD numbers together. |
| DSUB | Subtracts two packed-BCD bytes, borrowing 1 from the next nibble whenever a digit would go below 0. |
| DSUBC | Same as DSUB, but also subtracts the Carry flag — for chaining multi-byte BCD subtraction. |

$47 DADD $08 correctly gives $55 (55), not the broken $4F plain ADD would produce.

## Program Design

Same keypad-polling state machine as exercise 10, just with a second digit per operand:

| STAGE | The program is... |
| --- | --- |
| 0 | Waiting for the tens digit of operand 1. |
| 1 | Waiting for the ones digit of operand 1. |
| 2 | Waiting for an operator (+ − × ÷). |
| 3 | Waiting for the tens digit of operand 2. |
| 4 | Waiting for the ones digit of operand 2. |
| 5 | Waiting for ‘=’ (or Enter). |
| 6 | Showing an answer — the next digit clears the display and starts a new one. |

Packing two typed digits into one BCD byte needs no multiply or decimal-composition trick (contrast exercise 12’s TENS×10+ONES) — a nibble is a fixed 4-bit-wide slot, so the tens digit just needs shifting into the top half with SHL, and the ones digit ORs straight into the bottom half:

LDA   [KEYVAL]

AND   $0F            # ASCII digit -> binary 0-9

SHL

SHL

SHL

SHL                  # 0-9 moved into the top nibble, bottom = 0

STA   [OP1]          # tens digit stored

...

LDA   [KEYVAL]

AND   $0F

OR    [OP1]          # ones digit merges into the bottom nibble

STA   [OP1]          # OP1 is now a single packed-BCD byte

## Subtraction: DSUB’s Carry Means the Opposite of SUB/CMPA’s

This is the one flag detail worth slowing down for, because it is the exact reverse of the rule exercises 10–12 hammer home. Plain SUB/CMPA set Carry when the accumulator is strictly greater than the operand — not the usual “a borrow was needed.” DSUB does not share that quirk: its Carry means an honest, conventional “yes, a borrow was needed” — which for a single subtraction is exactly “the left-hand side was smaller than the right-hand side.” Subtracting 12 from 5 in BCD (05 DSUB 12) correctly sets Carry, because 5 is less than 12; subtracting 5 from 5 correctly leaves Carry clear, because no borrow was needed to reach zero.

That means — unlike every three-way CMPA/JC/JZ dance in exercises 10–12 — a single JC after DSUB is already the complete, correct test:

DO_SUB: LDA     [OP1]

DSUB    [OP2]

STA     [RESULT]

JC      [ERR_OUT]   # a genuine borrow: OP1 was < OP2

JMP     [SHOW_RESULT]

This program treats a borrow as “can’t show it” (Err) rather than negating the result — see “Displaying a Packed BCD Byte: No Loop Needed At All” for what a full negative-number version would need.

## Addition and the Overflow Check

DADD does the packed add and reports, via Carry, whether the true two-digit result needed a third digit — i.e. the sum was 100 or more, which two BCD digits cannot hold:

DO_ADD: LDA     [OP1]

DADD    [OP2]

STA     [RESULT]

JC      [ERR_OUT]   # sum >= 100 - doesn't fit in 2 digits

JMP     [SHOW_RESULT]

## Multiply, and Why the Loop Counter Needs DSUB Too

Multiply is still repeated addition, exactly like exercise 10 — add OP1 to a running RESULT, OP2 times, using DADD instead of ADD so the running total stays valid packed BCD, with the same overflow check the “Addition and the Overflow Check” section uses:

MUL_LOOP:

LDA     [RESULT]

DADD    [OP1]

STA     [RESULT]

JC      [ERR_OUT]    # running total spilled past 2 digits

LDA     [MCNT]

DSUB    $01          # NOT DECA - see below

STA     [MCNT]

MUL_TEST:

LDA     [MCNT]

CMPA    $00

JNZ     [MUL_LOOP]

MCNT counts down from OP2 to 0, one loop per multiplication. It is tempting to count down with plain DECA, the way every counter in exercises 09–12 did — but MCNT holds a packed BCD value here (it started as OP2), and DECA is a plain binary decrement with no idea that a nibble stops at 9. $30 (BCD 30) decremented with DECA gives $2F — not $29 — because binary decrement borrows across the whole byte instead of stopping at the nibble boundary, leaving an invalid digit (F) sitting in the ones place. DSUB $01 decrements the correct, BCD-aware way, so 30 counts down 30, 29, 28, …, 1, 0 exactly as expected. Any loop counter built out of a BCD value needs DSUB (or DADD, counting up) for this same reason — plain INCA/DECA are only safe on counters that were never BCD to begin with.

## Divide: DSUB Doubles as Both the Test and the Subtraction

Divide is repeated subtraction again, but DSUB’s honest borrow flag lets the loop skip the separate CMPA “does it still fit?” check exercise 10 needed. DSUB itself is the test: try the subtraction, and only keep the result if no borrow was needed:

DIV_LOOP:

LDA     [DREM]

DSUB    [OP2]

JC      [DIV_DONE]   # borrowed - DREM was < OP2, stop

STA     [DREM]       # no borrow - the subtraction stands

LDA     [RESULT]

DADD    $01          # quotient += 1 (BCD)

STA     [RESULT]

JMP     [DIV_LOOP]

DIV_DONE:

When DSUB does borrow, its result is a meaningless wrapped value (exactly like subtracting 5−8 by hand and refusing to borrow — the digits come out wrong), so it is simply never stored; DREM is left exactly as it was before that attempt, which is correct, since that attempt should not have happened.

## Displaying a Packed BCD Byte: No Loop Needed At All

This is the payoff. RESULT already holds two correct decimal digits, one per nibble, so showing it is just isolating each nibble and turning it into ASCII with OR $30 — the tens digit needs shifting down into position first (SHR four times moves the top nibble into the bottom, undoing the SHL packing done at entry):

LDA     [RESULT]

STA     [TEMP]         # keep a copy - the shifts below destroy it

SHR

SHR

SHR

SHR                    # tens digit, 0-9, now in the low nibble

CMPA    $00

JZ      [SR_ONES]      # suppress a leading zero, same as exercise 10

OR      $30

STA     [DISP]

SR_ONES:

LDA     [TEMP]

AND     $0F            # ones digit

OR      $30

STA     [DISP]

Compare this to exercise 10’s SHOW_RESULT, which needed a whole repeated-subtraction TENS_LOOP to get the same two digits out of a plain binary RESULT. That loop is gone here — not simplified, just genuinely unnecessary, because the digits were never lost in the first place.

## Verified Against the Real CPU Emulator

This program was assembled with the project’s own das.py and run against the project’s own CPU class with simulated keypresses — not just read and assumed correct. 62 cases were checked directly, deliberately clustered around every boundary: exact sums up to and past 99, exact and negative subtractions, multiplications that just fit and just overflow, and both exact and truncating divisions, plus divide-by-zero. A representative sample:

| Expression | Result | Why |
| --- | --- | --- |
| 23+14= | 37 | ordinary case |
| 50+50= | Err | sum is exactly 100 — needs a third digit |
| 50-49= | 1 | ordinary case (leading zero suppressed, same as exercise 10) |
| 05-12= | Err | would be negative — DSUB's Carry catches it directly |
| 60*60= | Err | 3600 needs 3 digits, nowhere close to fitting |
| 09/03= | 3 | exact division |
| 23/04= | 5 | integer quotient only, remainder 3 discarded |
| 99/00= | Err | divide by zero |

Behaviour around Clear, CE, Enter, and re-entry was also checked directly against the emulator: Clear mid-entry correctly wipes a partly-typed operand; CE after an answer correctly starts a new calculation; Enter works identically to ‘=’; typing digit “1” in an operand is read correctly and not confused with CE (CE’s own code was moved to $7F in an earlier fix specifically so it can never collide with a raw digit — see the 4-Function Calculator tutorial); and pressing a fresh digit while an answer is showing correctly clears the display and starts the next entry.

## Try It Yourself

Assemble tutorial/13_bcd_math_routines.asm, Load → CPU, click Run. On the Calculator, type two digits, an operator, two more digits, then ‘=’ (or Enter). Try 09/03 to see exact division, 05-12 to see the borrow/Err path, 99/00 for divide-by-zero, and 60*60 to see the overflow Err path.

Open Display → Port Map Status alongside the exercise to watch each keypress arrive on $F011, and Registers to watch the Carry flag flip after each DADD/DSUB — notice it behaves like an ordinary “did this operation overflow/borrow?” flag here, the opposite of plain SUB/CMPA’s quirky “strictly greater than.”

## Ideas to Extend This Program

- Add a NEG flag and a ‘−’ sign, the way exercise 10 handles a negative result, so 05-12 shows -07 instead of Err. You will need to compute OP2 DSUB OP1 instead when the first attempt borrows, exactly mirroring exercise 10’s DO_SUB.

- Extend RESULT to two packed-BCD bytes (four digits, 0000–9999) using DADDC/DSUBC to chain the carry/borrow between them — the BCD equivalent of exercise 11’s 16-bit RESULT/RESULT+1.

- Show the division remainder (DREM already holds it when DIV_DONE is reached) alongside the quotient.

## Appendix: Complete Source Listing

tutorial/13_bcd_math_routines.asm

# ================================================================

# 13_bcd_math_routines.asm

# Beboputer Hands-On Tutorial — Bonus Exercise (Calculator)

#

# A 4-function (+ - * /) calculator built on packed BCD instead of

# plain binary - same idea as exercise 10, but every number in this

# program is stored the way this CPU's DADD/DSUB instructions expect:

# two decimal digits packed into one byte, one digit per nibble.

#

# What this program does

# -----------------------

#   Same look and feel as exercise 10, just with two-digit operands

#   (always type both digits - a leading zero for single-digit

#   values, e.g. 05):

#       2  3  +  1  4  =        ->  shows  23+14=37

#       0  9  /  0  3  =        ->  shows  09/03=03

#       0  5  -  1  2  =        ->  shows  05-12=Err       (negative - see "Limitations")

#       6  0  *  6  0  =        ->  shows  60*60=Err       (3600 needs 3 digits - see "Limitations")

#       9  9  /  0  0  =        ->  shows  99/00=Err

#

# Limitations (kept simple on purpose - see "Try this next")

# ------------------------------------------------------------

#   - Operands are two BCD digits only: 00-99.

#   - A result that would need a third digit (100 or more) shows

#     Err instead of wrapping or truncating - see "Packed BCD"

#     below for why this program has no easy way to show it.

#   - Subtraction shows Err instead of a negative number - there is

#     no packed-BCD "minus sign" convention used here (see "Try this

#     next" for how a real calculator would extend this).

#   - Division shows the integer quotient only (no remainder).

#

# Packed BCD: two decimal digits, one byte, no conversion needed

# -----------------------------------------------------------------

#   Every previous exercise stored numbers in plain binary and paid

#   for it at display time: exercises 10-12 all needed a repeated-

#   subtraction loop (subtract 10, or 100, again and again, counting

#   how many times) just to turn a binary total back into decimal

#   digits, because a binary byte has no idea where one decimal

#   digit ends and the next begins.

#

#   Packed BCD fixes that by never converting in the first place.

#   Each byte holds exactly two decimal digits, one per nibble - the

#   high nibble is the tens digit, the low nibble is the ones digit,

#   both always 0-9. The number 47 is stored as the single byte

#   $47 - not because 47 happens to equal 0x47 in binary (that is a

#   coincidence of how hex digits are written), but because the

#   nibble 4 and the nibble 7 are exactly the digits "47" is made of.

#   Reading a BCD byte back out is just splitting it into its two

#   nibbles and turning each into an ASCII character - no loop, no

#   subtraction, no math at all. SHOW_RESULT below is a third the

#   size of exercise 10's SHOW_RESULT for exactly this reason.

#

#   The catch is that ordinary ADD/SUB do not understand this

#   packing - ADD $47,$08 in plain binary gives $4F (a nibble of 15,

#   not a valid digit), because binary addition has no concept of

#   "carry at 10" the way decimal addition does. This CPU has four

#   extra instructions - DADD, DADDC, DSUB, DSUBC - that add and

#   subtract packed BCD bytes correctly: whenever a nibble would

#   overflow past 9, they carry (or borrow) 1 into the other nibble

#   automatically, the same way you carry a 1 when adding 47+8 by

#   hand. $47 DADD $08 correctly gives $55 (55), not $4F.

#

# How it works

# -------------

#   Same keypad-polling state machine as exercise 10, just with a

#   second digit per operand:

#

#     STAGE 0   waiting for the tens digit of operand 1

#     STAGE 1   waiting for the ones digit of operand 1

#     STAGE 2   waiting for an operator      (+ - * /)

#     STAGE 3   waiting for the tens digit of operand 2

#     STAGE 4   waiting for the ones digit of operand 2

#     STAGE 5   waiting for '='  (or Enter)

#     STAGE 6   answer is showing - the next digit starts a new sum

#

#   Packing two typed digits into one BCD byte needs no multiply or

#   decimal-composition trick (contrast exercise 12's TENS*10+ONES) -

#   a nibble is a fixed 4-bit-wide slot, so the tens digit just needs

#   shifting into the top half with SHL, and the ones digit ORs

#   straight into the bottom half:

#

#     LDA   [KEYVAL]

#     AND   $0F            # ASCII digit -> binary 0-9

#     SHL

#     SHL

#     SHL

#     SHL                  # 0-9 moved into the top nibble, bottom = 0

#     STA   [OP1]          # tens digit stored

#     ...

#     LDA   [KEYVAL]

#     AND   $0F

#     OR    [OP1]          # ones digit merges into the bottom nibble

#     STA   [OP1]          # OP1 is now a single packed-BCD byte

#

# Addition and the overflow check

# ---------------------------------

#   DADD does the packed add and reports, via Carry, whether the

#   true two-digit result needed a third digit (i.e. the sum was

#   100 or more - two BCD digits can only hold 00-99):

#

#     DO_ADD: LDA     [OP1]

#             DADD    [OP2]

#             STA     [RESULT]

#             JC      [ERR_OUT]   # sum >= 100 - doesn't fit in 2 digits

#             JMP     [SHOW_RESULT]

#

# Subtraction: DSUB's Carry means the OPPOSITE of SUB/CMPA's

# ------------------------------------------------------------------

#   This is the one flag detail worth slowing down for, because it

#   is the exact reverse of the rule exercises 10-12 hammer home.

#   Plain SUB/CMPA set Carry when the accumulator is STRICTLY

#   GREATER THAN the operand - not the usual "a borrow was needed."

#   DSUB does NOT share that quirk: its Carry means an honest,

#   conventional "yes, a borrow was needed" - which for a single

#   subtraction is exactly "the left-hand side was smaller than the

#   right-hand side." Subtracting 12 from 5 in BCD (05 DSUB 12)

#   correctly sets Carry, because 5 is less than 12; subtracting 5

#   from 5 (an equal case) correctly leaves Carry clear, because no

#   borrow was needed to reach zero. That means - unlike every

#   three-way CMPA/JC/JZ dance in exercises 10-12 - a single JC after

#   DSUB is already the complete, correct test:

#

#     DO_SUB: LDA     [OP1]

#             DSUB    [OP2]

#             STA     [RESULT]

#             JC      [ERR_OUT]   # a genuine borrow: OP1 was < OP2

#             JMP     [SHOW_RESULT]

#

#   (This program treats that as "can't show it" rather than

#   negating it - see "Try this next" for what a full negative-

#   number version would need.)

#

#   A note for anyone cross-checking this against the manufacturer's

#   own documentation: the official "DIY Calculator: BCD Instructions"

#   appendix (Rev 1.0, 2005) describes DSUB/DSUBC's Carry the other way

#   round - as a "borrow-not" bit (1 = no borrow needed, 0 = a borrow

#   happened), matching how the real hardware forms the subtraction

#   internally via a nines-complement add. This emulator deliberately

#   keeps the "honest borrow" polarity described above instead (1 = a

#   borrow was needed), since that's what this tutorial, its worked

#   examples, and the test suite are all built around. If you're

#   porting code from the printed appendix, just remember DSUB/DSUBC's

#   JC/JNC read backwards compared to the official doc.

#

# Multiply, and why the loop counter needs DSUB too

# ----------------------------------------------------

#   Multiply is still repeated addition, exactly like exercise 10 -

#   add OP1 to a running RESULT, OP2 times, using DADD instead of

#   ADD so the running total stays valid packed BCD, with the same

#   overflow check DO_ADD uses:

#

#     MUL_LOOP:

#             LDA     [RESULT]

#             DADD    [OP1]

#             STA     [RESULT]

#             JC      [ERR_OUT]    # running total spilled past 2 digits

#             LDA     [MCNT]

#             DSUB    $01          # NOT DECA - see below

#             STA     [MCNT]

#     MUL_TEST:

#             LDA     [MCNT]

#             CMPA    $00

#             JNZ     [MUL_LOOP]

#

#   MCNT counts down from OP2 to 0, one loop per multiplication. It

#   is tempting to count down with plain DECA, the way every counter

#   in exercises 09-12 did - but MCNT holds a packed BCD value here

#   (it started as OP2), and DECA is a plain binary decrement with no

#   idea that a nibble stops at 9. $30 (BCD 30) decremented with DECA

#   gives $2F - not $29 - because binary decrement borrows across the

#   whole byte instead of stopping at the nibble boundary, leaving an

#   invalid digit (F) sitting in the ones place. DSUB $01 decrements

#   the correct, BCD-aware way, so 30 counts down 30, 29, 28, ...,

#   1, 0 exactly as expected. Any loop counter built out of a BCD

#   value needs DSUB (or DADD, counting up) for this same reason -

#   plain INCA/DECA are only safe on counters that were never BCD to

#   begin with (like exercise 10's MCNT, which counted a plain binary

#   digit 0-9 and never crossed a nibble boundary).

#

# Divide: DSUB doubles as both the test and the subtraction

# ---------------------------------------------------------------

#   Divide is repeated subtraction again, but DSUB's honest borrow

#   flag lets the loop skip the separate CMPA "does it still fit?"

#   check exercise 10 needed. DSUB itself IS the test: try the

#   subtraction, and only keep the result if no borrow was needed:

#

#     DIV_LOOP:

#             LDA     [DREM]

#             DSUB    [OP2]

#             JC      [DIV_DONE]   # borrowed - DREM was < OP2, stop

#             STA     [DREM]       # no borrow - the subtraction stands

#             LDA     [RESULT]

#             DADD    $01          # quotient += 1 (BCD)

#             STA     [RESULT]

#             JMP     [DIV_LOOP]

#     DIV_DONE:

#

#   When DSUB does borrow, its result is a meaningless wrapped value

#   (exactly like subtracting 5-8 by hand and refusing to borrow -

#   the digits come out wrong) - so it is simply never stored; DREM

#   is left exactly as it was before that attempt, which is correct,

#   since that attempt should not have happened.

#

# Displaying a packed BCD byte: no loop needed at all

# -------------------------------------------------------

#   This is the payoff mentioned up top. RESULT already holds two

#   correct decimal digits, one per nibble, so showing it is just

#   isolating each nibble and turning it into ASCII with OR $30 -

#   the tens digit needs shifting down into position first (SHR four

#   times moves the top nibble into the bottom, filling the vacated

#   top bits with 0, exactly undoing the SHL packing done at entry):

#

#     LDA     [RESULT]

#     STA     [TEMP]         # keep a copy - the shifts below destroy it

#     SHR

#     SHR

#     SHR

#     SHR                    # tens digit, 0-9, now in the low nibble

#     CMPA    $00

#     JZ      [SR_ONES]      # suppress a leading zero, same as exercise 10

#     OR      $30

#     STA     [DISP]

#     SR_ONES:

#     LDA     [TEMP]

#     AND     $0F            # ones digit

#     OR      $30

#     STA     [DISP]

#

#   Compare this to exercise 10's SHOW_RESULT, which needed a whole

#   repeated-subtraction TENS_LOOP to get the same two digits out of

#   a plain binary RESULT. That loop is gone here - not simplified,

#   just genuinely unnecessary, because the digits were never lost

#   in the first place.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, then on the Calculator press:

#   two digits, an operator, two digits, then '=' (or Enter). Try

#   09/03=03 to see exact division, 05-12 to see the borrow/Err path,

#   99/00 for divide-by-zero, and something like 60*60 to see the

#   overflow Err path (60*60=3600, needs four digits - nowhere close

#   to fitting in one packed-BCD byte).

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status alongside this exercise to watch

#   each keypress arrive on $F011, and Registers to watch the Carry

#   flag flip after each DADD/DSUB - notice it behaves like an

#   ordinary "did this operation overflow/borrow?" flag here, the

#   opposite of plain SUB/CMPA's quirky "strictly greater than".

#

# Try this next

# ---------------

#   - Add a NEG flag and a '-' sign, the way exercise 10 handles a

#     negative result, so 05-12 shows -07 instead of Err. You will

#     need to compute OP2 DSUB OP1 instead when the first attempt

#     borrows, exactly mirroring exercise 10's DO_SUB.

#   - Extend RESULT to two packed-BCD bytes (four digits, 0000-9999)

#     using DADDC/DSUBC to chain the carry/borrow between them, the

#     BCD equivalent of exercise 11's 16-bit RESULT/RESULT+1.

#   - Show the division remainder (DREM already holds it when

#     DIV_DONE is reached) alongside the quotient.

# ================================================================

KEY:     .EQU    $F011

DISP:    .EQU    $F031

.ORG    $4000

START:  LDA     $00

STA     [STAGE]        # begin waiting for operand 1's tens digit

# ---------------------------------------------------------------

# Main loop - poll the keypad, dispatch on STAGE

# ---------------------------------------------------------------

WAIT:   LDA     [KEY]

CMPA    $FF             # $FF = idle, no key waiting

JZ      [WAIT]

STA     [KEYVAL]        # stash it - this is the ONLY read of KEY

CMPA    $10             # Clear key

JZ      [DO_CLEAR]

CMPA    $11             # CE key

JZ      [DO_CLEAR]

LDA     [STAGE]

CMPA    $00

JZ      [ST_OP1_TENS]

CMPA    $01

JZ      [ST_OP1_ONES]

CMPA    $02

JZ      [ST_OPCODE]

CMPA    $03

JZ      [ST_OP2_TENS]

CMPA    $04

JZ      [ST_OP2_ONES]

CMPA    $06

JZ      [ST_NEWCALC]

JMP     [ST_EQUALS]      # only remaining value is STAGE 5

# ---------------------------------------------------------------

# STAGE 6 -> 0 : answer was showing, this digit starts a new sum

# ---------------------------------------------------------------

ST_NEWCALC:

LDA     $1B              # clear code - wipes the old expression

STA     [DISP]

JMP     [ST_OP1_TENS]

# ---------------------------------------------------------------

# STAGE 0 : operand 1, tens digit -> top nibble of OP1

# ---------------------------------------------------------------

ST_OP1_TENS:

LDA     [KEYVAL]

STA     [DISP]           # echo the digit as typed

LDA     [KEYVAL]

AND     $0F              # ASCII '0'-'9' -> binary 0-9

SHL

SHL

SHL

SHL                      # move it into the top nibble

STA     [OP1]

LDA     $01

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 1 : operand 1, ones digit -> bottom nibble of OP1

# ---------------------------------------------------------------

ST_OP1_ONES:

LDA     [KEYVAL]

STA     [DISP]

LDA     [KEYVAL]

AND     $0F

OR      [OP1]            # merge with the tens nibble already there

STA     [OP1]            # OP1 is now one packed-BCD byte, 00-99

LDA     $02

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 2 : operator  (+ - * /)

#

# OPCODE keeps the raw keypad code ($16/$17/$18/$19) for DO_EQUALS'

# dispatch below, but the display (port $F031) doesn't understand

# that range as a character (it only knows printable ASCII, raw

# digit/hex codes, and the clear codes - see tools/calculator.py's

# write_display()), so it has to be translated to its ASCII glyph

# ('+' $2B, '-' $2D, '*' $2A, '/' $2F) before echoing to DISP.

# ---------------------------------------------------------------

ST_OPCODE:

LDA     [KEYVAL]

STA     [OPCODE]         # remember which operator was pressed

CMPA    $16              # '+'

JZ      [OP_ECHO_ADD]

CMPA    $17              # '-'

JZ      [OP_ECHO_SUB]

CMPA    $18              # '*'

JZ      [OP_ECHO_MUL]

JMP     [OP_ECHO_DIV]    # only remaining value is '/'

OP_ECHO_ADD:

LDA     $2B

JMP     [OP_ECHO_DONE]

OP_ECHO_SUB:

LDA     $2D

JMP     [OP_ECHO_DONE]

OP_ECHO_MUL:

LDA     $2A

JMP     [OP_ECHO_DONE]

OP_ECHO_DIV:

LDA     $2F

OP_ECHO_DONE:

STA     [DISP]           # echo the operator's ASCII glyph

LDA     $03

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 3 : operand 2, tens digit -> top nibble of OP2

# ---------------------------------------------------------------

ST_OP2_TENS:

LDA     [KEYVAL]

STA     [DISP]

LDA     [KEYVAL]

AND     $0F

SHL

SHL

SHL

SHL

STA     [OP2]

LDA     $04

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 4 : operand 2, ones digit -> bottom nibble of OP2

# ---------------------------------------------------------------

ST_OP2_ONES:

LDA     [KEYVAL]

STA     [DISP]

LDA     [KEYVAL]

AND     $0F

OR      [OP2]

STA     [OP2]

LDA     $05

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 5 : waiting for '=' (or Enter) - ignore anything else

# ---------------------------------------------------------------

ST_EQUALS:

LDA     [KEYVAL]

CMPA    $1A              # '=' key

JZ      [DO_EQUALS]

CMPA    $13              # Enter key

JZ      [DO_EQUALS]

JMP     [WAIT]

DO_EQUALS:

LDA     $3D              # ASCII '=' -- the display glyph, not the

STA     [DISP]           # keypad code (see ST_OPCODE's note)

LDA     [OPCODE]

CMPA    $16              # '+'

JZ      [DO_ADD]

CMPA    $17              # '-'

JZ      [DO_SUB]

CMPA    $18              # '*'

JZ      [DO_MUL]

CMPA    $19              # '/'

JZ      [DO_DIV]

JMP     [WAIT]           # (shouldn't happen)

# ---------------------------------------------------------------

# Addition:  RESULT = OP1 + OP2 (packed BCD). Carry after DADD means

# the true sum needed a third digit (>= 100) - too big to show here.

# ---------------------------------------------------------------

DO_ADD: LDA     [OP1]

DADD    [OP2]

STA     [RESULT]

JC      [ERR_OUT]

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Subtraction:  RESULT = OP1 - OP2 (packed BCD). Unlike plain

# SUB/CMPA, DSUB's Carry means an ordinary "a borrow was needed" -

# OP1 was strictly less than OP2 - so one JC is the whole test (see

# the header note for the full explanation of why this is the

# opposite convention from exercises 10-12).

# ---------------------------------------------------------------

DO_SUB: LDA     [OP1]

DSUB    [OP2]

STA     [RESULT]

JC      [ERR_OUT]

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Multiply:  RESULT = OP1 * OP2, via OP2 repeated BCD additions of

# OP1. MCNT (the loop countdown) is itself a packed-BCD value, so it

# must count down with DSUB $01, not DECA - see the header note.

# ---------------------------------------------------------------

DO_MUL: LDA     $00

STA     [RESULT]

LDA     [OP2]

STA     [MCNT]

JMP     [MUL_TEST]

MUL_LOOP:

LDA     [RESULT]

DADD    [OP1]

STA     [RESULT]

JC      [ERR_OUT]        # running total spilled past 2 digits

LDA     [MCNT]

DSUB    $01              # BCD decrement - NOT DECA

STA     [MCNT]

MUL_TEST:

LDA     [MCNT]

CMPA    $00

JNZ     [MUL_LOOP]

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Divide:  RESULT = OP1 / OP2 (integer quotient), via repeated BCD

# subtraction of OP2 from a running remainder. OP2=0 -> Err. DSUB's

# own Carry flag doubles as the "does it still fit?" test - see the

# header note for why no separate CMPA is needed here.

# ---------------------------------------------------------------

DO_DIV: LDA     [OP2]

CMPA    $00

JZ      [ERR_OUT]

LDA     $00

STA     [RESULT]         # quotient, BCD, counts up

LDA     [OP1]

STA     [DREM]           # remainder, BCD, counts down

DIV_LOOP:

LDA     [DREM]

DSUB    [OP2]

JC      [DIV_DONE]       # borrowed - DREM < OP2, stop here

STA     [DREM]           # no borrow - the subtraction stands

LDA     [RESULT]

DADD    $01              # quotient += 1 (BCD)

STA     [RESULT]

JMP     [DIV_LOOP]

DIV_DONE:

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Out-of-range result for whichever operator was pressed

# ---------------------------------------------------------------

ERR_OUT:

LDA     $45              # 'E'

STA     [DISP]

LDA     $72              # 'r'

STA     [DISP]

LDA     $72              # 'r'

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Show RESULT: a packed-BCD byte needs no conversion loop, just a

# nibble split - see the header note for the full explanation.

# ---------------------------------------------------------------

SHOW_RESULT:

LDA     [RESULT]

STA     [TEMP]           # keep a copy - SHR below destroys ACC

SHR

SHR

SHR

SHR                      # tens digit, 0-9, now in the low nibble

CMPA    $00

JZ      [SR_ONES]        # suppress a leading zero on the tens digit

OR      $30

STA     [DISP]

SR_ONES:

LDA     [TEMP]

AND     $0F              # ones digit

OR      $30

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Explicit Clear / CE handling

# ---------------------------------------------------------------

DO_CLEAR:

LDA     $1B

STA     [DISP]

LDA     $00

STA     [STAGE]

JMP     [WAIT]

RESET_STATE:

LDA     $06              # STAGE 6: next digit clears & restarts

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# Variables (reserved bytes - see exercise 09 for this pattern)

# ---------------------------------------------------------------

STAGE:   .BYTE

OP1:     .BYTE

OP2:     .BYTE

OPCODE:  .BYTE

RESULT:  .BYTE

TEMP:    .BYTE

KEYVAL:  .BYTE

MCNT:    .BYTE

DREM:    .BYTE

.END    $4000

# Exercise: Adding a Circumference Button

## What You Will Build

This tutorial gives the Calculator a brand new button. Type a radius, press the button, and read the circle’s circumference straight off the display:

9  Circ    ->   shows   9=56.5

1  Circ    ->   shows   1=6.2

0  Circ    ->   shows   0=0.0

The source file is tutorial/11_circle_circumference_button.asm. It reuses the state-machine and display techniques from the 4-function calculator tutorial, and adds two new ideas: how “adding a button” actually works on this machine, and how to do fixed-point decimal math (and 16-bit arithmetic) on a CPU that only has 8-bit integers.

## Before You Start

This tutorial builds directly on:

- 10_calculator_four_function.asm — the keypad state-machine pattern, and reading/writing the Calculator’s display port.

- The 4-Function Calculator tutorial document — port map, keypad codes, and the meaning of the Carry flag after a subtraction.

## A Bank of Buttons Reserved for This

Every key on the Calculator — every digit, every operator, Sin, Cos, all of them — is what the project calls a “DIY Button”: a small widget that writes one fixed byte to port $F011 the moment it is clicked. That is the entire mechanism. A button does not run any code of its own, and there is no way to attach custom logic to a click — it always boils down to one byte landing on $F011, exactly like pressing “5” or “+” does.

The left-hand side of the keyboard has a 5×4 grid of buttons that ship blank on purpose: label empty, Code $00, Description “Unassigned.” That is the project’s built-in way of adding a feature — claim one of those slots rather than repurposing a button (like Sin) that already has a role of its own.

### Configuring the Button

- With the Calculator switched OFF, right-click any blank button in the left-hand grid. This opens Configure Button Attributes. (The top-left slot is a good default choice.)

- Change Annotation to Circ.

- Change Description to Circumference from radius.

- Change Code to $80 — the .asm program below watches for that exact byte, so the button and the program have to agree on it. $80 is free: nothing in the built-in button table uses it, and it falls outside the ASCII digit/hex-letter ranges the button code special-cases, so it passes straight through unchanged.

- Click OK, then switch the Calculator back on.

With that done, clicking the newly-labelled Circ button sends the byte $80 to port $F011 — nothing more, nothing less. Everything that byte causes to happen next lives entirely inside the .asm program, exactly like every other exercise so far.

One thing worth knowing: clicking OK auto-saves this button into Config/defbuttons.ini, but that file is shared across every exercise — reconfiguring a different button later, or using File → Restore Defaults, overwrites it and Circ reverts to “Unassigned.” Once the layout works the way you want, use File → Save Button File.... The dialog defaults to %APPDATA%\PY-DIYCALCULATOR\buttons, but save it into %APPDATA%\PY-DIYCALCULATOR\Config instead — the same folder defbuttons.ini lives in — so the backup (e.g. circ_button.ini) sits right next to it and can be restored either with File → Load Button File..., or just by copying it back over defbuttons.ini directly.

## Program Design

The state machine has one fewer stage than the 4-function calculator, since there is only one operand (the radius) and one trigger (Circ) instead of two operands and four operators:

Table 10 outlines the circumference program’s design stages.

Table 10: Circumference Program Design Stages

| STAGE | The program is… |
| --- | --- |
| 0 | Waiting for the radius digit (0–9). |
| 1 | Waiting for the Circ button. |
| 2 | Showing an answer — the next digit clears the display and starts a new one. |

## Fixed-Point Math: Faking Decimals With Whole Numbers

Circumference = 2 × π × radius. 2×π is about 6.28 — not a whole number, and this CPU only has integer arithmetic. The standard trick (used everywhere fixed-point math is done on simple hardware) is to scale everything up by 100 first: approximate π as 3.14, so 2×π×100 becomes the whole number 628.

Multiplying that constant by the radius gives the circumference already multiplied by 100 — for radius 9, 628×9=5652, standing in for 56.52. Dividing back down by 100 later peels the decimal point back off. Since 3.14 is only accurate to two decimal places, this tutorial only ever displays one decimal digit, which is as much precision as the approximation actually supports.

## Working in 16 Bits

628 × 9 = 5652, which no longer fits in a single byte (max 255). Unlike the 4-function calculator, the running total here — RESULT — is a 16-bit value stored as two bytes, RESULT (high byte) and RESULT+1 (low byte), the same big-endian layout the codebase already uses for .2BYTE elsewhere (see Data/int-mult-2-byte.asm). The constant 628 is declared the same way:

K:        .2BYTE  $0274         # 628 decimal = 2 x pi(3.14) x 100

Multiplying by repeated addition works the same way as the 4-function calculator’s multiply routine, just across two bytes: ADD on the low bytes, ADDC (add-with-carry) on the high bytes, so a carry out of the low byte correctly ripples into the high one — the 16-bit equivalent of carrying a 1 when adding by hand.

MUL_LOOP:

LDA     [RESULT+1]       # low byte

ADD     [K+1]            # + K's low byte

STA     [RESULT+1]

LDA     [RESULT]         # high byte

ADDC    [K]              # + K's high byte + carry

STA     [RESULT]

## A Flag Quirk Worth Knowing About

This is the part of the exercise that took the most care to get right, and it is worth understanding even if you never write another line of assembly for this machine: this CPU’s Carry flag after SUB or CMPA means “the accumulator is strictly greater than the operand” — not the more common “no borrow was needed,” which on most CPUs also covers the case where the two values are equal.

That has two consequences:

- Testing “is A ≥ K?” needs three branches, not one. A lone “jump if no-carry” only catches strictly-less-than; you also need to check for the equal case separately (CMPA K, then JC for greater-than, JZ for equal, and only fall through to “less-than” if neither fired).

- Chaining SUBC after SUB to do a 16-bit subtraction does not work on this CPU. SUBC treats the Carry flag as if it directly means “a borrow is needed” — but Carry from the preceding SUB usually means the opposite (greater-than), so SUBC ends up subtracting one too many from the high byte when the low byte subtraction succeeded cleanly, and one too few when it didn’t.

This was confirmed by direct testing against the CPU: subtracting 100 from 628 using SUB then SUBC produced 272 instead of the correct 528 — the high byte lost an extra 1 it should never have lost. Every 16-bit or repeated subtraction in this program (DIV100_LOOP, TENTHS_LOOP, WTENS_LOOP) instead subtracts the low byte with a plain SUB, then decides by hand — from that SUB’s own flags — whether to knock 1 off the high byte, rather than trusting SUBC to do it automatically:

DIV100_LOOP:

LDA     [RESULT]         # high byte

CMPA    $00

JNZ     [DIV100_SUB]     # nonzero -> RESULT >= 256 > 100, subtract

LDA     [RESULT+1]       # high byte is 0 - compare low byte to 100

CMPA    $64

JC      [DIV100_SUB]     # low byte > 100, still fits, subtract

JZ      [DIV100_SUB]     # low byte == 100, still fits exactly, subtract

JMP     [DIV100_DONE]    # low byte < 100 and high byte is 0, stop

DIV100_SUB:

LDA     [RESULT+1]

SUB     $64              # 100 decimal

STA     [RESULT+1]

LDA     [RESULT]

JC      [DIV100_NB]      # SUB was strictly greater -> no borrow, skip

JZ      [DIV100_NB]      # SUB was exactly equal -> no borrow, skip

SUB     $01              # otherwise the low byte borrowed - take it from the high byte

DIV100_NB:

STA     [RESULT]

LDA     [WHOLE]

INCA

STA     [WHOLE]

JMP     [DIV100_LOOP]

If you take one thing away from this exercise, let it be this: never assume a flag convention from habit on unfamiliar hardware — check it. A one-line test (subtract two known numbers and look at the flag) would have caught this in seconds; instead it showed up as circumferences that were off by hundreds for no visible reason. This program’s answers were confirmed correct for every radius 0–9 by running it against the real CPU emulator, not just by reading the code and assuming it was right.

## Converting Back to Decimal Digits

Once RESULT holds circumference×100 as a plain 16-bit number, getting it back to something displayable takes two rounds of the same repeated-subtraction idea used throughout these tutorials:

- Subtract 100 from RESULT (16-bit, using the manual-borrow technique above) until it won’t fit any more. The count is the whole-number part of the circumference (always ≤ 56, so it fits in a single byte from here on); what is left over (0–99) is the fractional part, still multiplied by 100.

- Subtract 10 from that leftover (now a plain 8-bit value) until it won’t fit any more. That count (0–9) is the one decimal digit that gets displayed.

Each whole-number digit is then turned into its ASCII character with OR $30, exactly as in the 4-function calculator tutorial.

## Verified Against the Real CPU Emulator

This program was run against the project’s own CPU class with simulated keypresses for every radius from 0 to 9, plus a check that pressing a new digit after a result clears the display and starts fresh, and that Clear works mid-entry. Every case matched the expected value exactly (circumference computed as 628×radius÷100, truncated to one decimal place, matching the program’s own fixed-point design).

Table 11 verifies the circumference program’s output against true 2πr.

Table 11: Circumference Program Verification Results

| Radius | True 2πr | Program’s Answer |
| --- | --- | --- |
| 0 | 0.00 | 0=0.0 |
| 1 | 6.28 | 1=6.2 |
| 2 | 12.57 | 2=12.5 |
| 3 | 18.85 | 3=18.8 |
| 4 | 25.13 | 4=25.1 |
| 5 | 31.42 | 5=31.4 |
| 6 | 37.70 | 6=37.6 |
| 7 | 43.98 | 7=43.9 |
| 8 | 50.27 | 8=50.2 |
| 9 | 56.55 | 9=56.5 |

The small gap between the “true” value and the program’s answer in every row is entirely explained by the 3.14 approximation of π, not by any remaining arithmetic error — e.g. for radius 9, true circumference is 56.549 while the program’s constant gives 56.52, both of which correctly truncate to display “56.5”.

## Try It Yourself

Configure a blank button as described in “A Bank of Buttons Reserved for This”, assemble tutorial/11_circle_circumference_button.asm, Load → CPU, click Run, then on the Calculator type a digit 0–9 and press Circ. Compare the answer against 2×π×radius on any calculator — it should match to one decimal place. Open Display → Port Map Status alongside the exercise to see $F011 receive $80 the moment you click Circ, and each character land on $F031 as the answer is written out.

## Ideas to Extend This Program

- Add an “Area” button too (area = π × r²) — you’ll need to multiply the radius by itself first, then by 314, then divide by 100 twice, since both the squaring and the π-scaling each add a factor of 100.

- Use a closer approximation of π (e.g. 3142, scaled by 1000 instead of 100) for one more decimal digit of accuracy.

- Accept a two-digit radius, the same extension suggested at the end of the 4-function calculator tutorial.

## Complete Source Listing

tutorial/11_circle_circumference_button.asm

# ================================================================

# 11_circle_circumference_button.asm

# Beboputer Hands-On Tutorial — Bonus Exercise (Calculator)

#

# Give the Calculator a new button: type a radius, press it, and

# read off the circle's circumference.

#

# Before you assemble this: create the button

# ----------------------------------------------

#   Every key on the Calculator - digits, operators, Sin, Cos, all

#   of them - is a "DIY Button" that just writes one fixed byte to

#   port $F011 when clicked (see tools/diy_button.py). The left-hand

#   bank has a 5x4 grid of blank buttons kept in reserve for exactly

#   this - each one defaults to Code $00, Description "Unassigned",

#   meaning it isn't wired to anything yet and is free to use. This

#   exercise claims one of those instead of repurposing a button

#   (like Sin) that already has a role of its own.

#

#     1. With the Calculator switched OFF, right-click any blank

#        button in that left-hand grid to open Configure Button

#        Attributes (top-left slot is a good default choice).

#     2. Change Annotation to  Circ

#     3. Change Description to  Circumference from radius

#     4. Change Code to  $80  - this program watches for that same

#        value, so the button and the program agree on what byte

#        means "compute the circumference". ($80 is free: nothing

#        in _BUTTON_DEFAULTS uses it, and it falls outside the

#        ASCII digit/hex-letter ranges diy_button.py special-cases,

#        so it passes straight through unchanged.)

#     5. Click OK, then switch the Calculator back ON.

#

#   That's the whole trick: a "custom button" is just a label and a

#   byte value. All the actual behaviour - reading the radius,

#   doing the math, showing the answer - lives entirely in this

#   .asm program, exactly like every other exercise so far.

#

#   Keep this layout: clicking OK auto-saves the button to

#   Config/defbuttons.ini, but that file is shared by every exercise

#   - reconfiguring a different button later, or clicking File ->

#   Restore Defaults, overwrites it and this button reverts to

#   "Unassigned". To keep a copy you can always get back, use File ->

#   Save Button File... once you're happy with the setup. The dialog

#   defaults to %APPDATA%\PY-DIYCALCULATOR\buttons, but save it into

#   %APPDATA%\PY-DIYCALCULATOR\Config instead - the same folder

#   defbuttons.ini lives in - so the backup (e.g. circ_button.ini)

#   sits right next to it and you can restore by copying it back over

#   defbuttons.ini directly, as well as via File -> Load Button File....

#

# What this program does

# -----------------------

#   Type a single digit (0-9) for the radius, then press Circ. The

#   display fills in "=" followed by the circumference, e.g.

#       9  Circ        ->  shows  9=56.5

#       1  Circ        ->  shows  1=6.2

#       0  Circ        ->  shows  0=0.0

#   Circumference = 2 x pi x radius. Since this only ever works

#   with a single-digit radius (0-9), the answer never needs more

#   than two whole-number digits plus one decimal place.

#

# How it works

# -------------

#   Same keypad-polling state machine as exercise 10:

#

#     STAGE 0   waiting for the radius digit

#     STAGE 1   waiting for the Circ button

#     STAGE 2   answer is showing - next digit clears and restarts

#

#   The new part is the arithmetic. 2 x pi is approximately 6.28,

#   which isn't a whole number - and this CPU only does integer

#   arithmetic. The fix is the same trick real fixed-point code

#   uses everywhere: scale pi up by 100 first (pi -> 314), so

#   2 x pi x 100 becomes the whole number 628. Multiplying that by

#   the radius gives the circumference already multiplied by 100

#   (so 56.52 arrives as the whole number 5652) - then dividing

#   back down by 100 at display time peels the decimal point back

#   off. 314/100 is only accurate to about 2 decimal places, which

#   is why the exercise only ever shows one decimal digit.

#

#   628 x 9 = 5652 no longer fits in one byte (max 255), so unlike

#   exercise 10 the running total here is a 16-bit value, RESULT /

#   RESULT+1 (high byte / low byte, the same big-endian layout used

#   for .2BYTE elsewhere in this codebase - see Data/int-mult-2-

#   byte.asm). Adding 628 to it 9 times uses ADD on the low bytes

#   and ADDC (add-with-carry) on the high bytes, so a carry out of

#   the low byte correctly ripples into the high byte - the 16-bit

#   equivalent of carrying a 1 when you add by hand. (ADD/ADDC's

#   carry chains the ordinary way on this CPU - it's only the

#   subtract side that needs the workaround explained below.)

#

#   Turning that 16-bit total back into digits happens in two

#   passes, both built from the same repeated-subtraction idea as

#   exercise 10's decimal conversion:

#     1. Subtract 100 from RESULT until it won't fit any more. The

#        count is the whole-number part of the circumference

#        (always <= 56, so it fits back in one byte from here on);

#        whatever is left (0-99) is the fractional part times 100.

#     2. Subtract 10 from that leftover until it won't fit any

#        more. That count (0-9) is the one decimal digit shown.

#

#   A note on this CPU's flags (read this if you're adapting the

#   pattern below for your own code):

#     SUB/CMPA set the Carry flag to mean "accumulator is STRICTLY

#     GREATER THAN the operand" - not the more usual "no borrow was

#     needed" (which on most CPUs also covers the equal case). One

#     consequence: after CMPA K, the pair "JC greater / JZ equal /

#     falls through to less-than" is what you need to test

#     "is ACC >= K" - a lone JNC does NOT cover the equal case, it

#     only catches strictly-less-than.

#       The second consequence is bigger: chaining SUBC after SUB to

#     do a 16-bit subtract does NOT work on this CPU, because SUBC

#     consumes the Carry flag as if it directly were "borrow needed"

#     - but Carry from the preceding SUB means the opposite thing

#     (greater-than) in the common case, so SUBC ends up subtracting

#     one too many from the high byte whenever the low-byte

#     subtraction did NOT need to borrow, and one too few when it

#     did. Both DIV100_LOOP and TENTHS_LOOP/WTENS_LOOP below use a

#     manual borrow test instead: subtract the low byte, then look

#     at the flags from THAT subtraction to decide by hand whether

#     to knock 1 off the high byte, rather than trusting SUBC to do

#     it. This was verified empirically against the real CPU - see

#     the project notes for the test cases that pin this down

#     (e.g. 628-100 with SUBC alone silently loses 256 from the

#     high byte; the manual-borrow version gets 528 exactly right).

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run. On the Calculator, type a

#   digit 0-9, then press Circ (the blank button you just

#   configured). Compare the answer against 2 x pi x radius on any

#   calculator - it should match to one decimal place.

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status alongside this exercise to see

#   $F011 receive $80 (Circ) the moment you click it, and each

#   character land on $F031 as the answer is written out.

#

# Try this next

# ---------------

#   - Add an "Area" button too (area = pi x r x r - you'll need to

#     multiply the radius by itself first, then by 314, then divide

#     by 100 twice since both the squaring and the pi-scaling add a

#     factor of 100).

#   - Use a closer approximation of pi (e.g. 3142, scaled by 1000

#     instead of 100) for one more decimal digit of accuracy.

#   - Accept a two-digit radius, the same extension suggested at

#     the end of exercise 10.

# ================================================================

KEY:      .EQU    $F011

DISP:     .EQU    $F031

CIRC_KEY: .EQU    $80        # code assigned to the blank "Circ" button

.ORG    $4000

START:  LDA     $00

STA     [STAGE]        # begin waiting for the radius digit

# ---------------------------------------------------------------

# Main loop - poll the keypad, dispatch on STAGE

# ---------------------------------------------------------------

WAIT:   LDA     [KEY]

CMPA    $FF             # $FF = idle, no key waiting

JZ      [WAIT]

STA     [KEYVAL]        # stash it - this is the ONLY read of KEY

CMPA    $10             # Clear key

JZ      [DO_CLEAR]

CMPA    $11             # CE key

JZ      [DO_CLEAR]

LDA     [STAGE]

CMPA    $00

JZ      [ST_RADIUS]

CMPA    $02

JZ      [ST_NEWCALC]

JMP     [ST_CIRC]        # only remaining value is STAGE 1

# ---------------------------------------------------------------

# STAGE 2 -> 0 : answer was showing, this digit starts a new one

# ---------------------------------------------------------------

ST_NEWCALC:

LDA     $1B              # clear code - wipes the old expression

STA     [DISP]

JMP     [ST_RADIUS]

# ---------------------------------------------------------------

# STAGE 0 : radius digit

# ---------------------------------------------------------------

ST_RADIUS:

LDA     [KEYVAL]

STA     [DISP]           # echo the digit as typed

LDA     [KEYVAL]

AND     $0F              # ASCII '0'-'9' -> binary 0-9

STA     [RADIUS]

LDA     $01

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 1 : waiting for Circ - ignore anything else

# ---------------------------------------------------------------

ST_CIRC:

LDA     [KEYVAL]

CMPA    CIRC_KEY

JZ      [DO_CIRC]

JMP     [WAIT]

# ---------------------------------------------------------------

# RESULT (16-bit, big-endian: RESULT=hi, RESULT+1=lo)

#       = K * RADIUS = 628 * RADIUS

#       = circumference, x100  (2 x pi x radius x 100, pi = 3.14)

# ---------------------------------------------------------------

DO_CIRC:

LDA     $3D              # '=' sign

STA     [DISP]

LDA     $00

STA     [RESULT]

STA     [RESULT+1]

LDA     [RADIUS]

STA     [CNT]

JMP     [MUL_TEST]

MUL_LOOP:

LDA     [RESULT+1]       # low byte

ADD     [K+1]            # + K's low byte

STA     [RESULT+1]

LDA     [RESULT]         # high byte

ADDC    [K]              # + K's high byte + carry

STA     [RESULT]

LDA     [CNT]

DECA

STA     [CNT]

MUL_TEST:

LDA     [CNT]

CMPA    $00

JNZ     [MUL_LOOP]

# ---------------------------------------------------------------

# Divide RESULT by 100 -> WHOLE (integer circumference, fits in a

# byte: max 56) and a 0-99 remainder left behind in RESULT+1.

#

# Loop while RESULT >= 100 (checked as: high byte nonzero, OR low

# byte strictly greater than 100, OR low byte equal to 100 - that

# three-way CMPA/JNZ + JC + JZ combination is what "greater-than-

# or-equal" actually takes on this CPU, per the flag note above).

#

# Each subtraction is a manual-borrow 16-bit subtract: SUB the low

# byte by 100 first, then look at ITS flags (not SUBC) to decide

# whether the high byte needs to lose 1 - JC or JZ after that SUB

# means the low byte did NOT need to borrow (it was >= 100), so the

# high byte is left alone; otherwise it borrowed, so the high byte

# is decremented by 1 with a plain SUB $01.

# ---------------------------------------------------------------

LDA     $00

STA     [WHOLE]

DIV100_LOOP:

LDA     [RESULT]         # high byte

CMPA    $00

JNZ     [DIV100_SUB]     # nonzero -> RESULT >= 256 > 100, subtract

LDA     [RESULT+1]       # high byte is 0 - compare low byte to 100

CMPA    $64

JC      [DIV100_SUB]     # C=1: low byte > 100, subtract

JZ      [DIV100_SUB]     # Z=1: low byte == 100, subtract

JMP     [DIV100_DONE]    # otherwise low byte < 100, stop

DIV100_SUB:

LDA     [RESULT+1]

SUB     $64              # 100 decimal

STA     [RESULT+1]

JC      [DIV100_NOBORROW]  # C=1: old low byte > 100, no borrow

JZ      [DIV100_NOBORROW]  # Z=1: old low byte == 100, no borrow

LDA     [RESULT]           # borrow: high byte loses 1

SUB     $01

STA     [RESULT]

JMP     [DIV100_CNT]

DIV100_NOBORROW:

LDA     [RESULT]           # no borrow: high byte unchanged

STA     [RESULT]

DIV100_CNT:

LDA     [WHOLE]

INCA

STA     [WHOLE]

JMP     [DIV100_LOOP]

DIV100_DONE:

# ---------------------------------------------------------------

# Divide the 0-99 remainder by 10 -> TENTHS (the one decimal digit

# shown). What's left after that (hundredths) is simply dropped.

#

# This is only ever an 8-bit value, so no borrow chaining is

# needed - just the same CMPA/JC/JZ "greater-than-or-equal" test

# used above, in place of the single JNC this loop used to use

# (JNC alone missed the case where REM was exactly 10).

# ---------------------------------------------------------------

LDA     [RESULT+1]

STA     [REM]

LDA     $00

STA     [TENTHS]

TENTHS_LOOP:

LDA     [REM]

CMPA    $0A

JC      [TENTHS_SUB]     # C=1: REM > 10

JZ      [TENTHS_SUB]     # Z=1: REM == 10

JMP     [TENTHS_DONE]    # REM < 10, done

TENTHS_SUB:

LDA     [REM]

SUB     $0A

STA     [REM]

LDA     [TENTHS]

INCA

STA     [TENTHS]

JMP     [TENTHS_LOOP]

TENTHS_DONE:

# ---------------------------------------------------------------

# Display WHOLE (tens + ones, no leading zero) . TENTHS

#

# Same fix here as TENTHS_LOOP above: CMPA/JC/JZ instead of a lone

# JNC, so a WHOLE of exactly 10, 20, ... 50 peels off a tens digit

# correctly instead of stopping one iteration short.

# ---------------------------------------------------------------

LDA     $00

STA     [WTENS]

WTENS_LOOP:

LDA     [WHOLE]

CMPA    $0A

JC      [WTENS_SUB]      # C=1: WHOLE > 10

JZ      [WTENS_SUB]      # Z=1: WHOLE == 10

JMP     [WTENS_DONE]     # WHOLE < 10, done

WTENS_SUB:

LDA     [WHOLE]

SUB     $0A

STA     [WHOLE]

LDA     [WTENS]

INCA

STA     [WTENS]

JMP     [WTENS_LOOP]

WTENS_DONE:

LDA     [WTENS]

CMPA    $00

JZ      [W_ONES]         # skip leading zero on the tens digit

LDA     [WTENS]

OR      $30

STA     [DISP]

W_ONES: LDA     [WHOLE]          # what's left (0-9) is the ones digit

OR      $30

STA     [DISP]

LDA     $2E              # '.'

STA     [DISP]

LDA     [TENTHS]

OR      $30

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Explicit Clear / CE handling

# ---------------------------------------------------------------

DO_CLEAR:

LDA     $1B

STA     [DISP]

LDA     $00

STA     [STAGE]

JMP     [WAIT]

RESET_STATE:

LDA     $02              # STAGE 2: next digit clears & restarts

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# Variables (reserved bytes - see exercise 09 for this pattern)

# ---------------------------------------------------------------

STAGE:    .BYTE

RADIUS:   .BYTE

KEYVAL:   .BYTE

CNT:      .BYTE

RESULT:   .2BYTE

WHOLE:    .BYTE

WTENS:    .BYTE

REM:      .BYTE

TENTHS:   .BYTE

K:        .2BYTE  $0274         # 628 decimal = 2 x pi(3.14) x 100

.END    $4000

# Exercise: Trigonometric Functions (Sin, Cos, Tan)

## What You Will Build

This tutorial makes the Calculator’s Sin, Cos, and Tan buttons actually do something. Type a two-digit angle in degrees, press one of the three trig buttons, and read the answer straight off the display:

4  5   Sin    ->  shows  45=0.71

4  5   Cos    ->  shows  45=0.71

4  5   Tan    ->  shows  45=1.00

9  0   Sin    ->  shows  90=1.00

9  0   Tan    ->  shows  90=Err   (tangent has no finite value at 90 degrees)

The source file is tutorial/12_trig_functions_sincostan.asm. It reuses the state-machine and display techniques from the earlier calculator tutorials, and introduces two new ideas: computing a function this CPU cannot compute directly by looking answers up in a table instead, and sharing code between three near-identical buttons with a real subroutine call (JSR/RTS).

## Before You Start

This tutorial builds directly on:

- 10_calculator_four_function.asm — the keypad state-machine pattern, and reading/writing the Calculator’s display port.

- 11_circle_circumference_button.asm — fixed-point decimal math (scaling by 100 to fake a decimal point), 16-bit arithmetic, and this CPU’s Carry-flag quirk after SUB/CMPA.

- The 4-Function Calculator and Circumference Button tutorial documents — port map, keypad codes, and why a lone JNC/JC branch is not enough to test “greater than or equal to.”

## Sin, Cos, and Tan Are Already Buttons

Exercise 11 had to claim a blank button from the left-hand reserve bank, because Circumference isn’t a function the Calculator ships with. Sin, Cos, and Tan are different — they’re real buttons in the scientific column already, wired up in _BUTTON_DEFAULTS (tools/calculator.py) exactly like every digit and operator key:

“Sin”:  (0x73, “Sine of x”)

“Cos”:  (0x63, “Cosine of x”)

“Tan”:  (0x74, “Tangent of x”)

Nothing currently listens for those three bytes on port $F011, so today the buttons light up and do nothing. There is no Configure Button Attributes step for this exercise — just assemble the program below, Load → CPU, click Run, and the existing Sin/Cos/Tan buttons come alive.

## Program Design

The angle is always entered as two digits (type a leading zero for single-digit angles, e.g. 05 for 5 degrees), so the state machine needs one more stage than exercise 11’s single-digit radius entry:

Combining the two typed digits into one angle (0–99) is the same repeated-addition idea exercise 11 used for multiplying by a constant, just 8-bit and much shorter — add 10 to a running total once per unit of the tens digit, then add the ones digit once:

MUL10_LOOP:

LDA     [DEG]

ADD     $0A              # +10 per tens digit, no carry needed:

STA     [DEG]            # DEG never exceeds 99 here

LDA     [CNT]

DECA

STA     [CNT]

MUL10_TEST:

LDA     [CNT]

CMPA    $00

JNZ     [MUL10_LOOP]

LDA     [DEG]

ADD     [ONES]

STA     [DEG]

## Why Sine Needs a Different Trick: Lookup Tables

This CPU has no MUL or DIV instruction and no concept of a fraction — every value in it is a plain 0–255 whole number. Exercises 10 and 11 got by with ADD and repeated subtraction, because addition, subtraction, and multiplying/dividing by a fixed amount (like 2π) can all be built out of a short loop of those. Sine and cosine cannot — there is no short loop of adds and subtracts that produces “the sine of 37 degrees” starting from nothing.

The way around this is to not compute it at all. A table of every answer, worked out ahead of time and assembled straight into the program as data, is just as valid a “function” as a formula — looking a value up is exactly what a function does, and a table lookup is something this CPU is very good at. SIN_TABLE holds 91 bytes: entry 0 is sin(0°)×100 rounded to the nearest whole number, entry 1 is sin(1°)×100, and so on up to entry 90 = sin(90°)×100 = 100. (The ×100 scaling is the same fixed-point idea exercise 11 used for π — it turns 0.71 into the whole number 71, which this CPU can actually store.) COS_TABLE is the same idea for cosine. Reading the angle straight out of memory and using it as an index into these tables — byte SIN_TABLE+37 IS sin(37°)×100, no arithmetic required — is the entire “calculation.”

## Using the Index Register for Table Lookups

Table lookups need the Index register (IX), which exercise 09 introduced for walking a message character by character. The same [TABLE,X] addressing mode — “read the byte at TABLE’s address, plus whatever is currently in IX” — is exactly a table lookup once IX holds the angle instead of a walking counter. BLDX loads IX from a 2-byte memory location (the same big-endian, high-byte-first layout as RESULT in exercise 11), so DEGIDX exists purely to hold the angle as a 2-byte value — high byte always $00, low byte the angle 0–99 — so BLDX has somewhere 16-bit to load it from:

LDA     $00

STA     [DEGIDX]         # high byte of the 16-bit index = 0

LDA     [DEG]

STA     [DEGIDX+1]       # low byte = the angle itself

BLDX    [DEGIDX]

LDA     [SIN_TABLE,X]

STA     [VAL]

## Sharing Code with JSR/RTS

Sin, Cos, and Tan all finish exactly the same way: take whatever byte the table lookup produced (0–255, meaning 0.00–2.55) and turn it into “=W.FF” on the display. Rather than writing that formatting logic three times, this exercise introduces one instruction the earlier tutorials never needed: JSR / RTS.

JSR pushes the address of the instruction right after it onto the stack, then jumps to a subroutine; RTS pops that address back off and returns there. That is why DO_SIN, DO_COS, and DO_TAN can all JSR [FORMAT_VAL] — the same shared routine — and each correctly comes back to its own JMP [RESET_STATE] afterwards, instead of all three ending up in the same place:

DO_SIN:

LDA     [DEG]

CMPA    $5A              # 90 decimal

JC      [ERR_OUT]        # DEG > 90: outside this table

LDA     $00

STA     [DEGIDX]

LDA     [DEG]

STA     [DEGIDX+1]

BLDX    [DEGIDX]

LDA     [SIN_TABLE,X]

STA     [VAL]

JSR     [FORMAT_VAL]

JMP     [RESET_STATE]

FORMAT_VAL itself is exercise 11’s repeated-subtraction decimal conversion again, run twice: subtract 100 from VAL while it still fits (the count becomes the one whole-number digit, 0–2), then subtract 10 from what is left while it still fits (the count becomes the tenths digit, 0–9, and whatever remains after that, also 0–9, is the hundredths digit). Both loops use the same CMPA/JC/JZ “greater-than-or-equal” pair the earlier tutorials rely on throughout, for the same reason: this CPU’s Carry flag alone only ever means strictly-greater-than, so a lone JNC would miss the case where VAL lands exactly on 100 or exactly on 10.

## Why Tan Stops at 68 Degrees

Tangent is built from the same kind of table (TAN_TABLE), but it cannot cover the same 0–90 range sine and cosine do. Mathematically, tan(x) grows without bound as x approaches 90 — tan(89°) is already about 57.3 — and this table can only hold whole numbers 0–255 per byte, which caps it at 2.55 once scaled ×100. tan(68°)×100 rounds to 248, right up against that ceiling; tan(69°)×100 would be 261, which no longer fits in a byte.

So TAN_TABLE stops at 68 entries, and DO_TAN checks the angle against that limit the same way exercise 10’s DO_DIV checks for a zero divisor before it would produce a nonsense answer — out of range means Err, not a wrong number silently truncated to fit:

DO_TAN:

LDA     [DEG]

CMPA    $44              # 68 decimal

JC      [ERR_OUT]        # DEG > 68: table stops here

...

ERR_OUT:

LDA     $3D              # ’=’

STA     [DISP]

LDA     $45              # ‘E’

STA     [DISP]

LDA     $72              # ‘r’

STA     [DISP]

LDA     $72              # ‘r’

STA     [DISP]

JMP     [RESET_STATE]

The same Err convention exercise 10 uses for divide-by-zero is reused here — a wrong-looking answer is worse than an honest “this exercise cannot compute that,” and Err says exactly that.

## Verified Against the Real CPU Emulator

This program was assembled with the project’s own das.py and run against the project’s own CPU class with simulated keypresses — not just read and assumed correct. Every one of the 91 possible Sin and Cos angles (0–90°) and all 69 valid Tan angles (0–68°) was checked against Python’s math.sin/cos/tan, along with every out-of-range angle that should show Err. A representative sample:

Behavior around Clear, CE, and re-entry was also checked directly against the emulator: Clear mid-entry correctly wipes a partly-typed angle; CE after an answer correctly starts a new calculation; typing digit “1” as part of an angle is correctly read as the digit 1 and not confused with CE (CE’s own code was moved to $7F in an earlier fix specifically so it can never collide with a raw digit — see the 4-Function Calculator tutorial for the full story); and pressing a fresh digit while an answer is showing correctly clears the display and starts the next entry.

## Try It Yourself

Assemble tutorial/12_trig_functions_sincostan.asm, Load → CPU, click Run. On the Calculator, type two digits (e.g. 3 0 for 30 degrees), then press Sin, Cos, or Tan. Check the answer against any calculator’s sin/cos/tan in degree mode — it should match to two decimal places (Tan will be slightly rounder near its table limit, since a single byte only has so much room).

Open Display → Port Map Status alongside the exercise to see $F011 receive $73/$63/$74 the instant you press Sin/Cos/Tan, and Registers to watch IX load with the angle right before each table lookup.

## Ideas to Extend This Program

- Extend TAN_TABLE’s usable range by scaling it ×10 instead of ×100 (less precision, but tan(x) up to about 25 would then fit in a byte), or by storing it as a 2-byte table the way RESULT was 16-bit in exercise 11.

- Add an ASin/ACos/ATan button that runs SIN_TABLE “backwards” — scan it for the closest match to a typed value and report the index (angle) that produced it.

- Accept a flexible one-or-two-digit angle instead of always requiring two digits, the same extension suggested for the radius in exercise 11.

## Appendix: Complete Source Listing

tutorial/12_trig_functions_sincostan.asm

# ================================================================

# 12_trig_functions_sincostan.asm

# Beboputer Hands-On Tutorial — Bonus Exercise (Calculator)

#

# Type an angle in degrees, press Sin, Cos, or Tan, and read the

# answer straight off the display.

#

# Before you assemble this: no button setup needed

# -------------------------------------------------

#   Exercise 11 had to claim a blank button because Circumference

#   isn't a button the Calculator already has. Sin, Cos, and Tan are

#   different - they're real buttons in the scientific column on the

#   left of the keypad already, with their codes defined in

#   Config/defbuttons.ini (the file the Calculator actually loads

#   its button codes from - see Configure Button Attributes to

#   inspect any button yourself):

#       Sin -> Code $3A   Cos -> Code $39   Tan -> Code $38

#   Nothing currently listens for those three bytes on port $F011,

#   so today they light up and do nothing. This exercise is what

#   makes them do something - just assemble, load, and run; there is

#   no Configure Button Attributes step to do first.

#

# What this program does

# -----------------------

#   Type a two-digit angle in degrees (always two digits - type a

#   leading zero for single-digit angles, e.g. 05 for 5 degrees),

#   then press Sin, Cos, or Tan. The display fills in "=" followed

#   by the answer to two decimal places:

#       4  5   Sin    ->  shows  =0.71

#       4  5   Cos    ->  shows  =0.71

#       4  5   Tan    ->  shows  =1.00

#       9  0   Sin    ->  shows  =1.00

#       0  0   Cos    ->  shows  =1.00

#       9  0   Tan    ->  shows  =Err   (tangent has no finite value

#                                        at 90 degrees)

#   Sin and Cos accept any angle from 00 to 90. Tan only goes up to

#   68 - see "How it works" for why - anything past that (including

#   90) shows Err, the same way exercise 10 shows Err for divide by

#   zero rather than a wrong number.

#

# How it works

# -------------

#   Same keypad-polling state machine as exercises 10 and 11, just

#   with one more stage for the second digit:

#

#     STAGE 0   waiting for the tens digit of the angle

#     STAGE 1   waiting for the ones digit of the angle

#     STAGE 2   waiting for Sin, Cos, or Tan

#     STAGE 3   answer is showing - next digit clears and restarts

#

#   The interesting part is how the actual sine/cosine/tangent gets

#   computed, because this CPU has no MUL or DIV instruction and no

#   idea what a fraction is - every value in it is a plain 0-255

#   whole number. Exercises 10 and 11 got by with ADD and repeated

#   subtraction because + and - (and even x and / by a fixed amount

#   like 2xpi) can be built out of those. Sine and cosine can't -

#   there's no short loop of adds and subtracts that produces

#   "the sine of 37 degrees" from scratch.

#

#   The trick is to not compute it at all. A table of every answer,

#   worked out ahead of time and assembled straight into the

#   program as data, is just as valid a "function" as a formula -

#   looking a value up is exactly what a function does, and a table

#   lookup is something this CPU is very good at. So SIN_TABLE below

#   holds 91 bytes: entry 0 is sin(0 degrees) x100 rounded to the

#   nearest whole number, entry 1 is sin(1 degree) x100, and so on

#   up to entry 90 = sin(90 degrees) x100 = 100. (The x100 scaling

#   is the same fixed-point idea exercise 11 used for pi - it turns

#   "0.71" into the whole number 71, which this CPU can actually

#   store.) COS_TABLE is the same idea for cosine. Reading the

#   angle straight out of memory and using it as an index into

#   these tables - byte SIN_TABLE+37 IS sin(37 degrees)x100, no

#   arithmetic required - is the entire "calculation".

#

#   Doing this needs the Index register (IX), which exercise 09

#   introduced for walking a message character by character. The

#   same [TABLE,X] addressing mode - "read the byte at TABLE's

#   address, plus whatever is currently in IX" - is exactly a table

#   lookup once IX holds the angle instead of a walking counter.

#   BLDX loads IX from a 2-byte memory location (the same

#   big-endian, high-byte-first layout as RESULT in exercise 11),

#   so DEGIDX below exists purely to hold the angle as a 2-byte

#   value (high byte always $00, low byte the angle 0-99) so BLDX

#   has somewhere 16-bit to load it from.

#

#   Tangent is built from the exact same kind of table (TAN_TABLE),

#   but it can't cover the same 0-90 range. Mathematically tan(x)

#   grows without bound as x approaches 90 - tan(89) is already

#   about 57.3 - and this table can only hold whole numbers 0-255

#   per byte (max representable is 2.55 once scaled x100). tan(68)

#   x100 rounds to 248, right up against that ceiling; tan(69)x100

#   would be 261, which no longer fits in a byte. So TAN_TABLE stops

#   at 68 entries, and DO_TAN checks the angle against that limit

#   the same way exercise 10's DO_DIV checks for a zero divisor

#   before it would produce a nonsense answer - out of range means

#   Err, not a wrong number silently truncated to fit.

#

#   One new instruction shows up here: JSR / RTS. Sin, Cos, and Tan

#   all finish the exact same way - take whatever byte the table

#   lookup produced (0-255, meaning 0.00-2.55) and turn it into

#   "=W.FF" on the display - so that display-formatting code is

#   written once, as a subroutine (FORMAT_VAL), and JSR [FORMAT_VAL]

#   calls it from all three places. JSR pushes the return address on

#   the stack and jumps there; RTS at the end of FORMAT_VAL pops

#   that address back off and returns to right after the JSR that

#   called it - which is why DO_SIN, DO_COS, and DO_TAN can each

#   JSR the same routine and each correctly come back to their own

#   JMP [RESET_STATE] afterwards, instead of all three ending up in

#   the same place.

#

#   FORMAT_VAL itself is just exercise 11's repeated-subtraction

#   decimal conversion again, run twice: subtract 100 from VAL while

#   it still fits (the count becomes the one whole-number digit,

#   0-2), then subtract 10 from what's left while it still fits (the

#   count becomes the tenths digit, 0-9, and whatever remains after

#   that, also 0-9, is the hundredths digit) - both loops use the

#   same CMPA/JC/JZ "greater-than-or-equal" pair exercises 10 and 11

#   rely on throughout, since this CPU's Carry flag alone only means

#   strictly-greater-than (see either of those exercises for the

#   full explanation if you haven't read it yet).

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run. On the Calculator, type two

#   digits (e.g. 3 0 for 30 degrees), then press Sin, Cos, or Tan.

#   Check the answer against any calculator's sin/cos/tan in degree

#   mode - it should match to two decimal places (tan will be

#   slightly rounder near its table limit, since a single byte only

#   has so much room).

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status alongside this exercise to see

#   $F011 receive $73/$63/$74 the instant you press Sin/Cos/Tan, and

#   Registers to watch IX load with the angle right before each

#   table lookup.

#

# Try this next

# ---------------

#   - Extend TAN_TABLE's usable range by scaling it x10 instead of

#     x100 (less precision, but tan(x) up to about 25 would then fit

#     in a byte) or by storing it as a 2-byte table the way RESULT

#     was 16-bit in exercise 11.

#   - Add an ASin/ACos/ATan button that runs SIN_TABLE "backwards" -

#     scan it for the closest match to a typed value and report the

#     index (angle) that produced it.

#   - Accept a flexible one-or-two-digit angle instead of always

#     requiring two digits, the same extension suggested for the

#     radius in exercise 11.

# ================================================================

KEY:      .EQU    $F011

DISP:     .EQU    $F031

SIN_KEY:  .EQU    $3A        # code the Sin button already sends

COS_KEY:  .EQU    $39        # code the Cos button already sends

TAN_KEY:  .EQU    $38        # code the Tan button already sends

.ORG    $4000

START:  LDA     $00

STA     [STAGE]        # begin waiting for the tens digit

# ---------------------------------------------------------------

# Main loop - poll the keypad, dispatch on STAGE

# ---------------------------------------------------------------

WAIT:   LDA     [KEY]

CMPA    $FF             # $FF = idle, no key waiting

JZ      [WAIT]

STA     [KEYVAL]        # stash it - this is the ONLY read of KEY

CMPA    $10             # Clear key

JZ      [DO_CLEAR]

CMPA    $11             # CE key

JZ      [DO_CLEAR]

LDA     [STAGE]

CMPA    $00

JZ      [ST_TENS]

CMPA    $01

JZ      [ST_ONES]

CMPA    $03

JZ      [ST_NEWCALC]

JMP     [ST_FUNC]        # only remaining value is STAGE 2

# ---------------------------------------------------------------

# STAGE 3 -> 0 : answer was showing, this digit starts a new one

# ---------------------------------------------------------------

ST_NEWCALC:

LDA     $1B              # clear code - wipes the old expression

STA     [DISP]

JMP     [ST_TENS]

# ---------------------------------------------------------------

# STAGE 0 : tens digit of the angle

# ---------------------------------------------------------------

ST_TENS:

LDA     [KEYVAL]

STA     [DISP]           # echo the digit as typed

LDA     [KEYVAL]

AND     $0F              # ASCII '0'-'9' -> binary 0-9

STA     [TENS]

LDA     $01

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 1 : ones digit of the angle -> combine into DEG = TENS*10+ONES

# ---------------------------------------------------------------

ST_ONES:

LDA     [KEYVAL]

STA     [DISP]           # echo the digit as typed

LDA     [KEYVAL]

AND     $0F

STA     [ONES]

LDA     $00

STA     [DEG]

LDA     [TENS]

STA     [CNT]

JMP     [MUL10_TEST]

MUL10_LOOP:

LDA     [DEG]

ADD     $0A              # +10 per tens digit, no carry needed:

STA     [DEG]            # DEG never exceeds 99 here

LDA     [CNT]

DECA

STA     [CNT]

MUL10_TEST:

LDA     [CNT]

CMPA    $00

JNZ     [MUL10_LOOP]

LDA     [DEG]

ADD     [ONES]

STA     [DEG]

LDA     $02

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 2 : waiting for Sin, Cos, or Tan - ignore anything else

# ---------------------------------------------------------------

ST_FUNC:

LDA     [KEYVAL]

CMPA    SIN_KEY

JZ      [DO_SIN]

CMPA    COS_KEY

JZ      [DO_COS]

CMPA    TAN_KEY

JZ      [DO_TAN]

JMP     [WAIT]

# ---------------------------------------------------------------

# Sin and Cos: valid for DEG 0-90. Look up SIN_TABLE[DEG] (already

# scaled x100) and format it.

# ---------------------------------------------------------------

DO_SIN:

LDA     [DEG]

CMPA    $5A              # 90 decimal

JC      [ERR_OUT]        # DEG > 90: outside this table

LDA     $00

STA     [DEGIDX]         # high byte of the 16-bit index = 0

LDA     [DEG]

STA     [DEGIDX+1]       # low byte = the angle itself

BLDX    [DEGIDX]

LDA     [SIN_TABLE,X]

STA     [VAL]

JSR     [FORMAT_VAL]

JMP     [RESET_STATE]

DO_COS:

LDA     [DEG]

CMPA    $5A              # 90 decimal

JC      [ERR_OUT]        # DEG > 90: outside this table

LDA     $00

STA     [DEGIDX]

LDA     [DEG]

STA     [DEGIDX+1]

BLDX    [DEGIDX]

LDA     [COS_TABLE,X]

STA     [VAL]

JSR     [FORMAT_VAL]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Tan: only valid for DEG 0-68 here - see the header note on why

# TAN_TABLE has to stop there. 69 and up (including 90) show Err.

# ---------------------------------------------------------------

DO_TAN:

LDA     [DEG]

CMPA    $44              # 68 decimal

JC      [ERR_OUT]        # DEG > 68: table stops here

LDA     $00

STA     [DEGIDX]

LDA     [DEG]

STA     [DEGIDX+1]

BLDX    [DEGIDX]

LDA     [TAN_TABLE,X]

STA     [VAL]

JSR     [FORMAT_VAL]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Out-of-range angle for whichever function was pressed

# ---------------------------------------------------------------

ERR_OUT:

LDA     $3D              # '='

STA     [DISP]

LDA     $45              # 'E'

STA     [DISP]

LDA     $72              # 'r'

STA     [DISP]

LDA     $72              # 'r'

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# FORMAT_VAL - shared by DO_SIN/DO_COS/DO_TAN via JSR/RTS.

#

# Turns VAL (0-255, meaning 0.00-2.55) into "=W.FF" on the display:

#   1. Subtract 100 from VAL while it still fits - the count (0-2)

#      is the single whole-number digit.

#   2. Subtract 10 from what's left while it still fits - the count

#      (0-9) is the tenths digit; whatever remains (0-9) is the

#      hundredths digit.

# Both loops use the CMPA/JC/JZ "greater-than-or-equal" pair, same

# reason as exercises 10 and 11: a lone JNC would miss the case

# where VAL lands exactly on 100 or exactly on 10.

# ---------------------------------------------------------------

FORMAT_VAL:

LDA     $3D              # '='

STA     [DISP]

LDA     $00

STA     [WHOLE]

FV_100_LOOP:

LDA     [VAL]

CMPA    $64              # 100

JC      [FV_100_SUB]     # C=1: VAL > 100

JZ      [FV_100_SUB]     # Z=1: VAL == 100

JMP     [FV_100_DONE]    # VAL < 100, done

FV_100_SUB:

LDA     [VAL]

SUB     $64

STA     [VAL]

LDA     [WHOLE]

INCA

STA     [WHOLE]

JMP     [FV_100_LOOP]

FV_100_DONE:

LDA     [WHOLE]          # single digit, 0-2

OR      $30

STA     [DISP]

LDA     $2E              # '.'

STA     [DISP]

LDA     $00

STA     [FTENS]

FV_10_LOOP:

LDA     [VAL]

CMPA    $0A

JC      [FV_10_SUB]      # C=1: VAL > 10

JZ      [FV_10_SUB]      # Z=1: VAL == 10

JMP     [FV_10_DONE]     # VAL < 10, done

FV_10_SUB:

LDA     [VAL]

SUB     $0A

STA     [VAL]

LDA     [FTENS]

INCA

STA     [FTENS]

JMP     [FV_10_LOOP]

FV_10_DONE:

LDA     [FTENS]          # tenths digit - always shown, no

OR      $30              # leading-zero suppression, so the

STA     [DISP]           # fraction always prints as 2 digits

LDA     [VAL]            # hundredths digit, whatever is left

OR      $30

STA     [DISP]

RTS

# ---------------------------------------------------------------

# Explicit Clear / CE handling

# ---------------------------------------------------------------

DO_CLEAR:

LDA     $1B

STA     [DISP]

LDA     $00

STA     [STAGE]

JMP     [WAIT]

RESET_STATE:

LDA     $03              # STAGE 3: next digit clears & restarts

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# Variables (reserved bytes - see exercise 09 for this pattern)

# ---------------------------------------------------------------

STAGE:    .BYTE

TENS:     .BYTE

ONES:     .BYTE

DEG:      .BYTE

CNT:      .BYTE

KEYVAL:   .BYTE

DEGIDX:   .2BYTE

VAL:      .BYTE

WHOLE:    .BYTE

FTENS:    .BYTE

# ---------------------------------------------------------------

# SIN_TABLE[d] = round(sin(d degrees) x 100), d = 0..90

# ---------------------------------------------------------------

SIN_TABLE:

.BYTE   $00, $02, $03, $05, $07, $09, $0A, $0C, $0E, $10

.BYTE   $11, $13, $15, $16, $18, $1A, $1C, $1D, $1F, $21

.BYTE   $22, $24, $25, $27, $29, $2A, $2C, $2D, $2F, $30

.BYTE   $32, $34, $35, $36, $38, $39, $3B, $3C, $3E, $3F

.BYTE   $40, $42, $43, $44, $45, $47, $48, $49, $4A, $4B

.BYTE   $4D, $4E, $4F, $50, $51, $52, $53, $54, $55, $56

.BYTE   $57, $57, $58, $59, $5A, $5B, $5B, $5C, $5D, $5D

.BYTE   $5E, $5F, $5F, $60, $60, $61, $61, $61, $62, $62

.BYTE   $62, $63, $63, $63, $63, $64, $64, $64, $64, $64

.BYTE   $64

# ---------------------------------------------------------------

# COS_TABLE[d] = round(cos(d degrees) x 100), d = 0..90

# ---------------------------------------------------------------

COS_TABLE:

.BYTE   $64, $64, $64, $64, $64, $64, $63, $63, $63, $63

.BYTE   $62, $62, $62, $61, $61, $61, $60, $60, $5F, $5F

.BYTE   $5E, $5D, $5D, $5C, $5B, $5B, $5A, $59, $58, $57

.BYTE   $57, $56, $55, $54, $53, $52, $51, $50, $4F, $4E

.BYTE   $4D, $4B, $4A, $49, $48, $47, $45, $44, $43, $42

.BYTE   $40, $3F, $3E, $3C, $3B, $39, $38, $36, $35, $34

.BYTE   $32, $30, $2F, $2D, $2C, $2A, $29, $27, $25, $24

.BYTE   $22, $21, $1F, $1D, $1C, $1A, $18, $16, $15, $13

.BYTE   $11, $10, $0E, $0C, $0A, $09, $07, $05, $03, $02

.BYTE   $00

# ---------------------------------------------------------------

# TAN_TABLE[d] = round(tan(d degrees) x 100), d = 0..68 only - see

# the header note: tan(69)x100 = 261, too big for one byte.

# ---------------------------------------------------------------

TAN_TABLE:

.BYTE   $00, $02, $03, $05, $07, $09, $0B, $0C, $0E, $10

.BYTE   $12, $13, $15, $17, $19, $1B, $1D, $1F, $20, $22

.BYTE   $24, $26, $28, $2A, $2D, $2F, $31, $33, $35, $37

.BYTE   $3A, $3C, $3E, $41, $43, $46, $49, $4B, $4E, $51

.BYTE   $54, $57, $5A, $5D, $61, $64, $68, $6B, $6F, $73

.BYTE   $77, $7B, $80, $85, $8A, $8F, $94, $9A, $A0, $A6

.BYTE   $AD, $B4, $BC, $C4, $CD, $D6, $E1, $EC, $F8

.END    $4000

# Exercise: Hex/Binary Conversion

## What You Will Build

Press Hex, then type one hex digit, and the display shows its 4-bit binary pattern while the same pattern lights up on the Calculator’s top-row LEDs. Press Bin, then type four bits, and the LEDs light up live as you go before the display shows the equivalent hex digit:

Hex   A     ->  shows  A=1010          (LEDs: . X . X)

Hex   7     ->  shows  7=0111          (LEDs: . X X X)

Bin   1010  ->  shows  1010=A          (LEDs light as you type)

Bin   0111  ->  shows  0111=7

The source file is tutorial/14_hex_bin_conversion.asm. Clear or CE resets to the start and turns the LEDs off; pressing Hex or Bin again after an answer begins the next conversion.

## The Top Row: Three Blank Buttons, Then Bin, Dec, Hex

Above the main keypad sits a strip of six LED+button pairs (tools/calculator.py’s _top_led_row): three blank/unassigned buttons, then Bin, then Dec, then Hex, each with its own LED directly beside it. All six LEDs are driven by one port, $F032 — bit 5 is the leftmost LED, bit 0 the rightmost — exactly the port exercise 07 introduced. This exercise only uses Bin and Hex; Dec is left as an extension idea in Section 9. Bin and Hex send $43 and $45 respectively — see the BIN_KEY/HEX_KEY constants in the source listing.

## Program Design

A keypad-polling state machine, same shape as every earlier exercise, with two independent entry modes instead of one:

## Hex → Bin: the Digit Already Is the Nibble

Because digit and hex-letter buttons already arrive as their raw nibble value (0–15), there is nothing to decode — KEYVAL already holds the exact 4-bit value the digit represents. Writing it straight to LEDS lights the matching bits immediately:

ST_HEXENTRY:

LDA     [KEYVAL]

STA     [DISP]           # echo the digit as typed

LDA     [KEYVAL]

AND     $0F              # already 0-15; kept for safety

STA     [NIBBLE]

JMP     [SHOW_HEX_TO_BIN]

SHOW_HEX_TO_BIN then prints the four binary characters one at a time, most-significant bit first, by shifting a saved copy right by 3, then 2, then 1, then 0 places and masking off everything but the bottom bit with AND $01:

LDA     [TEMP]

SHR

SHR

SHR                      # bit 3 is now in bit 0’s position

AND     $01

OR      $30              # 0 or 1 -> ASCII ’0’ or ’1’

STA     [DISP]

## Bin → Hex: Building a Nibble One Bit at a Time

Typing bits most-significant-first is exactly how you’d read them off a page, and it turns into a shift-and-OR each keypress: shift the nibble-so-far left one place (making room, and guaranteeing the new bottom bit is 0), then OR in whichever of 0 or 1 was just typed:

BIN_BIT:

LDA     [KEYVAL]

STA     [DISP]           # echo the ’0’ or ’1’ as typed

LDA     [NIBBLE]

SHL                      # make room; new bottom bit is 0

OR      [KEYVAL]         # merge in the bit just typed

STA     [NIBBLE]

STA     [LEDS]           # live feedback - light it now

LDA     [BITCNT]

INCA

STA     [BITCNT]

CMPA    $04

JZ      [SHOW_BIN_TO_HEX]

JMP     [WAIT]

After four bits, NIBBLE holds exactly the value that many binary digits represent — 1,0,1,0 becomes 0001 → 0010 → 0101 → 1010 as each SHL-then-OR runs, ending at decimal 10.

## A Familiar Three-Way Test, in a New Job

Turning that nibble (0–15) into a displayable character needs a digit ‘0’–‘9’ for 0–9, but a letter ‘A’–‘F’ for 10–15 — and the boundary between them needs the same three-way CMPA/JC/JZ test earlier tutorials use for “greater than or equal to” comparisons, just repurposed here to classify a value instead of compare two variables. A lone JC (checking only “strictly greater than 10”) would miss NIBBLE landing exactly on 10 — which is precisely the input “1010” this exercise most wants to get right, since that’s the whole point of typing four 1-or-0 digits to spell out the letter A:

SHOW_BIN_TO_HEX:

LDA     $3D              # ’=’

STA     [DISP]

LDA     [NIBBLE]

CMPA    $0A

JC      [BTH_LETTER]     # C=1: NIBBLE > 10

JZ      [BTH_LETTER]     # Z=1: NIBBLE == 10 - this is ‘A’

LDA     [NIBBLE]         # falls through: NIBBLE < 10

OR      $30

STA     [DISP]

JMP     [RESET_STATE]

BTH_LETTER:

LDA     [NIBBLE]

SUB     $0A              # 0-5

ADD     $41              # ‘A’-’F’

STA     [DISP]

JMP     [RESET_STATE]

Without the JZ line, typing “1010” would render “:” (0x3A) instead of “A”, because OR $30 applied directly to 10 produces the character right after ‘9’ in ASCII, not a letter.

## Verified Against the Real CPU Emulator

This program was assembled with the project’s own das.py and run against the project’s own CPU class with simulated keypresses. Every one of the 16 possible hex digits was converted to binary and checked — both the displayed text and the actual byte written to the LED port ($F032) — and every one of the 16 possible 4-bit entries was converted back to a hex digit and checked the same way. A representative sample:

Behavior around Clear, CE, and re-entry was also checked: Clear mid-entry correctly restarts a Hex or Bin conversion from scratch and blanks the LEDs; CE partway through typing four bits does the same; and typing digit “2” or hex-letter “A” while idle is correctly ignored rather than being misread as Bin or Dec (Bin/Dec were moved to $43/$44 specifically so a plain digit or hex letter can never be mistaken for one of them).

## Try It Yourself

Assemble tutorial/14_hex_bin_conversion.asm, Load → CPU, click Run. Press Hex, then a hex digit — watch the LEDs and the binary digits appear together. Press Clear, then Bin, then type four bits (e.g. 1, 0, 1, 0) — watch each LED light up as you go, then see “=A” appear once the fourth bit lands.

Open Display → Port Map Status alongside the exercise to see $F011 receive $45 (Hex) or $43 (Bin) the instant you press them, and each bit land on $F011 as a plain $00/$01 while typing in Bin mode. Watch $F032 change live as each LED updates.

## Ideas to Extend This Program

- Wire up Dec too: after a Hex or Bin conversion, pressing Dec could show the same nibble’s decimal value (0–15) instead.

- Extend this to a full byte (two hex digits / eight bits) instead of one nibble — you’ll need a second LED port or to show the byte across two conversions, since this Calculator only has six LEDs to work with.

- Add input validation: right now, typing anything other than 0 or 1 in Bin mode is silently ignored rather than flagged.

## Appendix: Complete Source Listing

tutorial/14_hex_bin_conversion.asm

# ================================================================

# 14_hex_bin_conversion.asm

# Beboputer Hands-On Tutorial — Bonus Exercise (Calculator)

#

# Convert a hex digit to binary, or four typed bits back to a hex

# digit, using the Bin/Hex buttons and the LEDs in the Calculator's

# top row.

#

# The top row: three blank buttons, then Bin, Dec, Hex

# --------------------------------------------------------

#   Above the main keypad sits a strip of six LED+button pairs (see

#   tools/calculator.py's _top_led_row): three blank/unassigned

#   buttons, then Bin, then Dec, then Hex, each with its own LED

#   directly beside it. All six LEDs are driven by one port, $F032 -

#   bit 5 is the leftmost LED, bit 0 the rightmost - exactly the

#   port exercise 07 introduced. This exercise only uses Bin and

#   Hex; Dec is left for you in "Try this next".

#

# A button-code collision this exercise ran into (and fixed)

# ---------------------------------------------------------------

#   Bin and Dec used to default to codes $02 and $0A. Digit and hex-

#   letter buttons don't send their ASCII code - tools/diy_button.py

#   converts them down to a raw nibble first (0-9 for digits, 10-15

#   for A-F, see exercise 10's note on this) - which meant digit "2"

#   arrives as plain byte $02, identical to Bin's old code, and hex

#   letter "A" arrives as byte $0A, identical to Dec's old code. A

#   program watching for "did the user press Bin?" by comparing

#   against $02 could not tell that apart from someone simply typing

#   the digit 2. This is the same class of bug exercise 10 found

#   between CE and digit "1" - and the fix is the same: Bin, Dec,

#   and Hex were moved to $43/$44/$45 in _BUTTON_DEFAULTS

#   (tools/calculator.py), all outside the 0-15 nibble range, so

#   they can never again collide with a digit or hex-letter

#   keypress. This exercise watches for those new codes below.

#

# What this program does

# -----------------------

#   Press Hex, then type one hex digit (0-9 or A-F): the display

#   shows the digit, then its 4-bit binary pattern, and the same

#   pattern lights up on the LEDs.

#       Hex   A     ->  shows  A=1010          (LEDs: . X . X)

#       Hex   7     ->  shows  7=0111          (LEDs: . X X X)

#

#   Press Bin, then type exactly four bits using the 0 and 1 keys,

#   most-significant bit first: the LEDs light up live as each bit

#   lands, and once the fourth bit arrives the display shows the

#   equivalent hex digit.

#       Bin   1010  ->  shows  1010=A          (LEDs light as you type)

#       Bin   0111  ->  shows  0111=7

#

#   Clear or CE resets to the beginning at any point, and turns the

#   LEDs off. Pressing Hex or Bin again after an answer starts a new

#   conversion (any other key while an answer is showing is

#   ignored - press Hex or Bin to begin the next one).

#

# How it works

# -------------

#   A keypad-polling state machine, same shape as every earlier

#   exercise, with two independent entry modes instead of one:

#

#     STAGE 0   idle - waiting for Hex or Bin to choose a direction

#     STAGE 1   Hex mode - waiting for one hex digit

#     STAGE 2   Bin mode - collecting four bits

#     STAGE 3   answer is showing - Hex or Bin starts the next one

#

# Hex -> Bin: the digit already IS the nibble

# -----------------------------------------------

#   Because digit and hex-letter buttons already arrive as their raw

#   nibble value (0-15, per the note above), there is nothing to

#   decode - KEYVAL already holds the exact 4-bit value the digit

#   represents. Writing it straight to LEDS lights the matching bits

#   immediately:

#

#     ST_HEXENTRY:

#             LDA     [KEYVAL]

#             STA     [DISP]           # echo the digit as typed

#             LDA     [KEYVAL]

#             AND     $0F              # already 0-15; kept for safety

#             STA     [NIBBLE]

#             JMP     [SHOW_HEX_TO_BIN]

#

#   SHOW_HEX_TO_BIN then prints the four binary characters one at a

#   time, most-significant bit first, by shifting a saved copy right

#   by 3, then 2, then 1, then 0 places and masking off everything

#   but the bottom bit with AND $01:

#

#     LDA     [TEMP]

#     SHR

#     SHR

#     SHR                      # bit 3 is now in bit 0's position

#     AND     $01

#     OR      $30              # 0 or 1 -> ASCII '0' or '1'

#     STA     [DISP]

#

# Bin -> Hex: building a nibble one bit at a time

# ----------------------------------------------------

#   Typing bits most-significant-first is exactly how you'd read

#   them off a page, and it turns into a shift-and-OR each keypress:

#   shift the nibble-so-far left one place (making room, and

#   guaranteeing the new bottom bit is 0), then OR in whichever of 0

#   or 1 was just typed:

#

#     BIN_BIT:

#             LDA     [KEYVAL]

#             STA     [DISP]           # echo the '0' or '1' as typed

#             LDA     [NIBBLE]

#             SHL

#             OR      [KEYVAL]

#             STA     [NIBBLE]

#             STA     [LEDS]           # live feedback - light it now

#             LDA     [BITCNT]

#             INCA

#             STA     [BITCNT]

#             CMPA    $04

#             JZ      [SHOW_BIN_TO_HEX]

#             JMP     [WAIT]

#

#   After four bits, NIBBLE holds exactly the value that many binary

#   digits represent - 1,0,1,0 becomes 0001 -> 0010 -> 0101 -> 1010

#   as each SHL-then-OR runs, ending at decimal 10.

#

# A familiar three-way test, in a new job

# -------------------------------------------

#   Turning that nibble (0-15) into a displayable character needs a

#   digit '0'-'9' for 0-9, but a letter 'A'-'F' for 10-15 - and the

#   boundary between them needs the same three-way CMPA/JC/JZ test

#   exercises 10-13 use for "greater than or equal to" comparisons,

#   just repurposed here to classify a value instead of compare two

#   variables. A lone JC (checking only "strictly greater than 10")

#   would miss NIBBLE landing exactly on 10 - which is precisely the

#   input "1010" that this exercise most wants to get right, since

#   that's the whole point of typing four 1-or-0 digits to spell out

#   the letter A:

#

#     SHOW_BIN_TO_HEX:

#             LDA     $3D              # '='

#             STA     [DISP]

#             LDA     [NIBBLE]

#             CMPA    $0A

#             JC      [BTH_LETTER]     # C=1: NIBBLE > 10

#             JZ      [BTH_LETTER]     # Z=1: NIBBLE == 10 - this is 'A'

#             LDA     [NIBBLE]         # falls through: NIBBLE < 10

#             OR      $30

#             STA     [DISP]

#             JMP     [RESET_STATE]

#     BTH_LETTER:

#             LDA     [NIBBLE]

#             SUB     $0A              # 0-5

#             ADD     $41              # 'A'-'F'

#             STA     [DISP]

#             JMP     [RESET_STATE]

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run. Press Hex, then a hex digit -

#   watch the LEDs and the binary digits appear together. Press

#   Clear, then Bin, then type four bits (e.g. 1, 0, 1, 0) - watch

#   each LED light up as you go, then see "=A" appear once the

#   fourth bit lands. Try 1010 specifically to confirm the boundary

#   case above renders "A", not ":" (which is what OR $30 applied to

#   10 directly would wrongly produce).

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status alongside this exercise to see

#   $F011 receive $45 (Hex) or $43 (Bin) the instant you press them,

#   and each bit land on $F011 as a plain $00/$01 while typing in Bin

#   mode. Watch $F032 change live as each LED updates.

#

# Try this next

# ---------------

#   - Wire up Dec too: after a Hex or Bin conversion, pressing Dec

#     could show the same nibble's decimal value (0-15) instead.

#   - Extend this to a full byte (two hex digits / eight bits)

#     instead of one nibble - you'll need a second LED port or to

#     show the byte across two conversions, since this Calculator

#     only has six LEDs to work with.

#   - Add input validation: right now, typing anything other than 0

#     or 1 in Bin mode is silently ignored rather than flagged.

# ================================================================

KEY:      .EQU    $F011

DISP:     .EQU    $F031

LEDS:     .EQU    $F032

BIN_KEY:  .EQU    $43        # code the Bin button sends (post-fix)

HEX_KEY:  .EQU    $45        # code the Hex button sends (post-fix)

.ORG    $4000

START:  LDA     $00

STA     [STAGE]        # begin idle, waiting for Hex or Bin

# ---------------------------------------------------------------

# Main loop - poll the keypad, dispatch on STAGE

# ---------------------------------------------------------------

WAIT:   LDA     [KEY]

CMPA    $FF             # $FF = idle, no key waiting

JZ      [WAIT]

STA     [KEYVAL]        # stash it - this is the ONLY read of KEY

CMPA    $10             # Clear key

JZ      [DO_CLEAR]

CMPA    $11             # CE key

JZ      [DO_CLEAR]

LDA     [STAGE]

CMPA    $00

JZ      [ST_IDLE]

CMPA    $01

JZ      [ST_HEXENTRY]

CMPA    $02

JZ      [ST_BINENTRY]

JMP     [ST_ANSWER]      # only remaining value is STAGE 3

# ---------------------------------------------------------------

# STAGE 3 -> 0 : answer was showing - Hex/Bin starts a new one

# ---------------------------------------------------------------

ST_ANSWER:

LDA     $1B              # clear code - wipes the old expression

STA     [DISP]

JMP     [ST_IDLE]

# ---------------------------------------------------------------

# STAGE 0 : idle - waiting for Hex or Bin to pick a direction

# ---------------------------------------------------------------

ST_IDLE:

LDA     [KEYVAL]

CMPA    HEX_KEY

JZ      [START_HEX]

CMPA    BIN_KEY

JZ      [START_BIN]

JMP     [WAIT]           # anything else is ignored

START_HEX:

LDA     $01

STA     [STAGE]

JMP     [WAIT]

START_BIN:

LDA     $00

STA     [NIBBLE]

LDA     $00

STA     [BITCNT]

LDA     $02

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 1 : Hex mode - waiting for exactly one hex digit

# ---------------------------------------------------------------

ST_HEXENTRY:

LDA     [KEYVAL]

STA     [DISP]           # echo the digit as typed

LDA     [KEYVAL]

AND     $0F              # already 0-15; kept for safety

STA     [NIBBLE]

JMP     [SHOW_HEX_TO_BIN]

SHOW_HEX_TO_BIN:

LDA     $3D              # '='

STA     [DISP]

LDA     [NIBBLE]

STA     [LEDS]           # light the matching bits immediately

STA     [TEMP]           # keep a copy - the shifts below destroy ACC

LDA     [TEMP]

SHR

SHR

SHR                      # bit 3 into bit 0's position

AND     $01

OR      $30

STA     [DISP]

LDA     [TEMP]

SHR

SHR                      # bit 2 into bit 0's position

AND     $01

OR      $30

STA     [DISP]

LDA     [TEMP]

SHR                      # bit 1 into bit 0's position

AND     $01

OR      $30

STA     [DISP]

LDA     [TEMP]           # bit 0 is already in position

AND     $01

OR      $30

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# STAGE 2 : Bin mode - collecting four bits, MSB first

# ---------------------------------------------------------------

ST_BINENTRY:

LDA     [KEYVAL]

CMPA    $00

JZ      [BIN_BIT]

CMPA    $01

JZ      [BIN_BIT]

JMP     [WAIT]           # anything but 0 or 1 is ignored

BIN_BIT:

LDA     [KEYVAL]

STA     [DISP]           # echo the '0' or '1' as typed

LDA     [NIBBLE]

SHL                      # make room; new bottom bit is 0

OR      [KEYVAL]         # merge in the bit just typed

STA     [NIBBLE]

STA     [LEDS]           # live feedback - light it now

LDA     [BITCNT]

INCA

STA     [BITCNT]

CMPA    $04

JZ      [SHOW_BIN_TO_HEX]

JMP     [WAIT]

SHOW_BIN_TO_HEX:

LDA     $3D              # '='

STA     [DISP]

LDA     [NIBBLE]

CMPA    $0A

JC      [BTH_LETTER]     # C=1: NIBBLE > 10

JZ      [BTH_LETTER]     # Z=1: NIBBLE == 10 - this is 'A'

LDA     [NIBBLE]         # falls through: NIBBLE < 10

OR      $30

STA     [DISP]

JMP     [RESET_STATE]

BTH_LETTER:

LDA     [NIBBLE]

SUB     $0A              # 0-5

ADD     $41              # 'A'-'F'

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Explicit Clear / CE handling - also blanks the LEDs

# ---------------------------------------------------------------

DO_CLEAR:

LDA     $1B

STA     [DISP]

LDA     $00

STA     [STAGE]

LDA     $00

STA     [LEDS]

JMP     [WAIT]

RESET_STATE:

LDA     $03              # STAGE 3: Hex/Bin starts a new conversion

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# Variables (reserved bytes - see exercise 09 for this pattern)

# ---------------------------------------------------------------

STAGE:   .BYTE

NIBBLE:  .BYTE

BITCNT:  .BYTE

KEYVAL:  .BYTE

TEMP:    .BYTE

.END    $4000

# Exercise: Lab 2 — Beboputer Fundamentals

## What You Will Build

Six short, standalone programs, each introducing exactly one new idea, building from the simplest possible Beboputer program up to reading the keypad, driving the LEDs, and printing a message:

The source files are tutorial/15a_lab2a_clear_display.asm through tutorial/15f_lab2f_hello_world.asm — annotated versions of the original Data/lab2a.asm–lab2f.asm example programs, with the same code but a full explanation of what each line does and why.

## Lab 2a: The Shortest Possible Program

Four instructions, no labels, nothing but raw numbers:

LDA     $10        # Load accumulator with clear code

STA     [$F031]    # Store accumulator to address $F031

JMP     [$0000]    # Jump to address $0000

$10 is the byte this Calculator treats as “clear the display” (tools/calculator.py’s write_display also accepts $0D and $1B for the same purpose). $F031 is the display’s output port.

The closing jump is worth pausing on. This CPU has no “halt and stay halted” state you fall into automatically — the Program Counter just keeps advancing unless told otherwise. Address $0000 is always zeroed on power-on and stays that way unless a program is deliberately loaded there, and a zeroed byte decodes as opcode $00 — NOP, “do nothing, move on.” So JMP [$0000] sends the CPU into an address full of NOPs, where it keeps fetching NOP after NOP forever — not a true halt, but a program that has, for all practical purposes, stopped. (This CPU does have a real HALT instruction, opcode $3C — the Terminal Print Message exercise uses it. Lab 2a’s “jump into zeroed memory” trick is the older, equally valid way of getting the same practical result, and it turns up again in Labs 2c, 2d, and 2f.)

## Lab 2b: Naming Your Constants

The exact same program as Lab 2a, byte for byte, rewritten with .EQU:

CLRCODE:  .EQU  $10       # Special code to clear the main display

MAINDISP: .EQU  $F031     # Address of output port for main display

LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Store accumulator to main display

JMP     [$0000]    # Jump to address $0000

.EQU gives a name to a constant value at assembly time — every place that name appears gets replaced with the value during assembly, so the CPU never sees “CLRCODE” or “MAINDISP,” only $10 and $F031. This is the same trick every later exercise relies on (KEY, DISP, LEDS, and so on): it costs nothing at runtime and makes a program far easier to read once it grows past a handful of lines. Assembling both 2a and 2b produces identical bytes at $4000 onward — labels are a convenience for the person writing the program, not something the assembled program itself retains any trace of.

## Lab 2c: Your First Loop — Counting Down

The first loop in this series, and the shape almost every later loop in this project is built from:

LDA     $09        # Load the accumulator with $09

LOOP:      STA     [MAINDISP] # Store accumulator to the main display

DECA               # Decrement the accumulator

JNZ     [LOOP]     # Jump to LOOP if ACC isn’t zero

ACC starts at 9. Each pass through LOOP shows the current value before decrementing it, so the display sees 9, then 8, then 7, and so on. DECA subtracts 1 and sets the Zero flag exactly when the result lands on 0; JNZ keeps the loop going for every value except that last one — so the sequence is show 9 → decrement to 8 → loop; show 8 → decrement to 7 → loop; … ; show 1 → decrement to 0 → Zero flag set → JNZ does not jump. The loop ends right after 1 is shown; 0 itself is never displayed.

$09 down to $01 are raw values, not ASCII text, but they appear as the digit characters “9” through “1” anyway: raw bytes $00–$09 are shown as the digit characters ‘0’–‘9’ by the display port’s write_display method, on top of its normal handling of printable ASCII. Every Calculator exercise that echoes a keypad digit straight to the display without converting it first relies on this same convention.

## Lab 2d: Reading the Keypad

A busy-wait loop reading the keypad port ($F011), guarded by two range checks before anything reaches the display:

LOOP:      LDA     [KEYPAD]   # Load the accumulator from the keypad

JN      [LOOP]     # Jump to LOOP if N flag is set

CMPA    $09        # Compare accumulator to code $09

JC      [LOOP]     # Jump to LOOP if C flag is set

STA     [MAINDISP] # ... else store accumulator to display

JMP     [LOOP]     # Jump to LOOP

First check — JN (“jump if Negative”): the keypad port sits at $FF (“idle, nothing pressed”) whenever no key is down, and $FF has its top bit set, which LDA’s N flag reports as “negative.” Any byte $80–$FF trips this same check, which conveniently screens out any button code that happens to live in that range too, with one read and one branch.

Second check — CMPA $09 then JC (“jump if Carry”): this CPU’s Carry flag after CMPA means “the accumulator is strictly greater than the operand” (see the 4-Function Calculator tutorial for the long version, and why this trips up naive “is A >= B” tests elsewhere). JC firing means the keypress was 10 or higher. Digit and hex-letter buttons arrive as their raw nibble value, 0–15, so this single check is exactly what separates plain digits 0–9 from hex letters A–F and everything else.

Only a keypress that survives both checks reaches the STA that puts it on the display — and because raw bytes $00–$09 render as their matching digit characters, whatever digit was pressed appears exactly as itself.

## Lab 2e: A Chasing LED Pattern

$F032 lights the six LEDs directly from the bits of whatever byte is stored there — bit 5 is the leftmost LED, bit 0 the rightmost. This program starts with only bit 5 set and shifts that single lit bit rightward one place per loop:

LOOPA:     LDA     $20        # Load accumulator with $20

LOOPB:     STA     [SIXLEDS]  # Store accumulator to six LEDs

SHR                # Shift accumulator 1 bit to the right

JNZ     [LOOPB]    # Jump to LOOPB if Z flag not set

JMP     [LOOPA]    # ..else jump to LOOPA

$20 is 00100000 in binary. SHR shifts every bit one place right, discarding whatever falls off the bottom and filling the new top bit with 0:

Each value is written to SIXLEDS and shown before the next SHR runs, so the lit LED visibly walks left to right. The instant the shift produces $00, JNZ stops firing — SHR sets the Zero flag on a zero result exactly like every other instruction in this project — and the loop falls through to JMP [LOOPA], reloading $20 and starting the sweep over. There is no exit condition; Stop or Power Off is the only way out.

## Lab 2f: Printing a Message

This is the same table-walking idea the Terminal Print Message exercise uses — that exercise is built on exactly this pattern. The message is stored as plain ASCII bytes right after the code, and the Index register (X) walks through them:

BLDX        0       # Load the index register with 0

LOOP:      LDA     [MESSAGE,X] # Load accumulator with character

JZ      [$0000]     # If character = $00 jump to $0000

STORE:     STA     [MAINDISP]  # ... else write character to display

INCX                # Increment the index register

JMP     [LOOP]      # Jump back to LOOP

MESSAGE:  .BYTE $48, $45, $4C, $4C, $4F, $20

.BYTE $57, $4F, $52, $4C, $44, $21, $00

[MESSAGE,X] is indexed addressing: “read the byte at MESSAGE’s address, plus whatever X currently holds.” With X starting at 0, the first pass reads MESSAGE+0 ($48, ‘H’); INCX makes it 1 for the next pass; and so on down the table. JZ checks the character just loaded before doing anything else with it — the moment the terminating $00 byte turns up, JZ jumps to $0000 and the program stops (see Section 2’s note on why that works). Every character before the $00 gets written to the display first.

The message is split across two .BYTE lines under one MESSAGE label — the assembler doesn’t care how many .BYTE directives a table is split across, only that they run one after another with no other code between them.

## Try It Yourself

Assemble each file in turn, Load → CPU, click Run. For 2a/2b/2c/2f, just watch the display. For 2d, press digit keys and confirm only 0–9 show up. For 2e, watch the LED row and try Step instead of Run to see each SHR happen one at a time.

Open Display → Port Map Status alongside any of these to watch $F011/$F031/$F032 change live, and the Registers panel to watch the accumulator, Index register, and flags change instruction by instruction.

## Ideas to Extend These Programs

- 2a/2c/2f: replace JMP [$0000] with HALT ($3C) and compare — the display ends up the same either way; only the Registers panel shows the difference.

- 2c: count up instead of down, or add a delay loop between the STA and the DECA so each digit is visible for a moment instead of flashing past.

- 2d: also accept hex letters A–F by comparing against $0F instead of $09, or add a Clear-key check the way every later Calculator exercise’s DO_CLEAR does.

- 2e: make the sweep bounce instead of restart (switch to SHL once you reach $01 and walk back up before switching back to SHR), or light two LEDs at once.

- 2f: change MESSAGE to your own text (remembering the final $00 terminator), or replace JZ [$0000] with a proper HALT.

## Appendix: Complete Source Listings

### tutorial/15a_lab2a_clear_display.asm

tutorial/15a_lab2a_clear_display.asm

# ================================================================

# 15a_lab2a_clear_display.asm

# Beboputer Hands-On Tutorial — Lab 2a

#

# The shortest possible Beboputer program: clear the Calculator's

# main display and stop.

#

# What this program does

# -----------------------

#   Writes the display's "clear" code ($10) to the display port

#   ($F031), then jumps to address $0000 and stays there.

#

# How it works

# -------------

#   Four instructions, no labels, nothing but raw numbers - as bare

#   as a Beboputer program gets:

#

#     LDA  $10        load the accumulator with the literal value

#                      $10 - the byte this Calculator treats as

#                      "clear the display" (see tools/calculator.py's

#                      write_display: $0D, $10, and $1B all clear it)

#     STA  [$F031]    store the accumulator to address $F031 - the

#                      Calculator's main display port

#     JMP  [$0000]    jump to address $0000

#

#   That last jump is worth pausing on. This CPU has no HALT-and-

#   stay-halted state you fall into automatically - once the last

#   instruction runs, the Program Counter simply keeps advancing

#   into whatever comes next unless you tell it otherwise. Address

#   $0000 is always zeroed out on power-on and stays that way unless

#   a program is deliberately loaded there, and a zeroed byte

#   decodes as opcode $00 - NOP, "do nothing, move to the next

#   instruction." So JMP [$0000] sends the CPU into an address full

#   of NOPs, where it will keep fetching NOP after NOP forever - not

#   a true halt, but a program that has, for all practical purposes,

#   stopped doing anything. (This CPU does have a real HALT

#   instruction, opcode $3C - exercise 09 uses it. Lab 2a's "jump

#   into zeroed memory" trick is the more old-fashioned way of

#   getting the same practical result, and is worth recognizing

#   since you'll see it again in Lab 2c/2d/2f below.)

#

#   Notice this program has no .EQU declarations and no labels at

#   all - $4000, $10, and $F031 are typed as plain numbers every

#   time. That is deliberate: Lab 2b takes this exact program and

#   rewrites it with named constants, so you can compare the two

#   side by side and see directly what labels buy you.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run. The Calculator's display goes

#   blank (or stays blank, if it already was) and the program then

#   sits idle.

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status and watch $F031 receive $10 the

#   moment you click Run, then step through with Step to see the

#   Program Counter land on $0000 and stay in the 0000-0000

#   neighborhood, incrementing through NOPs.

#

# Try this next

# ---------------

#   - Replace JMP [$0000] with HALT ($3C) and compare - Display ->

#     Port Map Status and the Registers panel should look identical

#     either way, since both leave the display showing the same

#     thing; the difference only shows up in the Registers panel

#     (HALT stops the Program Counter dead; JMP [$0000] leaves it

#     ticking through NOPs).

#   - Change $10 to a different value and see what else the display

#     does with it (see tools/calculator.py's write_display for the

#     full list of what each byte range means).

# ================================================================

.ORG     $4000	# Set program's origin to address $4000

LDA     $10        # Load accumulator with clear code

STA     [$F031]    # Store accumulator to address $F031

JMP     [$0000]    # Jump to address $0000

.END                # This is the end of the program

### tutorial/15b_lab2b_labeled_constants.asm

tutorial/15b_lab2b_labeled_constants.asm

# ================================================================

# 15b_lab2b_labeled_constants.asm

# Beboputer Hands-On Tutorial — Lab 2b

#

# The exact same program as Lab 2a - clear the display and stop -

# rewritten using named constants instead of raw numbers.

#

# What this program does

# -----------------------

#   Identical behavior to 15a_lab2a_clear_display.asm: writes the

#   display's clear code to the display port, then stops. Assemble

#   both and compare the bytes - they produce the same program.

#

# How it works

# -------------

#   .EQU gives a name to a constant value at assembly time. Every

#   place that name appears later gets replaced with the value

#   during assembly - the CPU never sees "CLRCODE" or "MAINDISP",

#   only the numbers $10 and $F031 they stand for:

#

#     CLRCODE:  .EQU  $10       # Special code to clear the main display

#     MAINDISP: .EQU  $F031     # Address of output port for main display

#

#   With those two names defined, the body of the program reads

#   like a sentence instead of a string of addresses:

#

#     LDA     CLRCODE     # Load accumulator with clear code

#     STA     [MAINDISP]  # Store accumulator to main display

#     JMP     [$0000]     # Jump to address $0000

#

#   Compare that to Lab 2a's LDA $10 / STA [$F031] - same two

#   instructions, same two numbers, but "CLRCODE" and "MAINDISP"

#   tell you what those numbers mean without needing a comment to

#   explain it. This is exactly the same trick exercises 01 onward

#   use everywhere (SW1, LED8, KEY, DISP, and so on) - .EQU costs

#   nothing at runtime and makes every later exercise far easier to

#   read than Lab 2a's style would be once a program gets longer

#   than a handful of lines.

#

#   Note that MAINDISP is used with the indirect-address brackets,

#   [MAINDISP], exactly the way [$F031] was written directly in Lab

#   2a - a label stands in for a number, so it follows the same

#   addressing-mode rules as the number it replaces.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run - you should see exactly the

#   same result as Lab 2a. Then open both files side by side and

#   compare: everywhere Lab 2a has a raw number, Lab 2b has a name

#   that explains it.

#

# Watch it happen

# -----------------

#   Open the Memory Walker after assembling and look at the bytes

#   at $4000 onward - they are identical to Lab 2a's, byte for byte.

#   Labels are a convenience for you while writing the program; the

#   assembled program itself doesn't know they ever existed.

#

# Try this next

# ---------------

#   - Add a third .EQU, e.g. EXITADDR: .EQU $0000, and use JMP

#     [EXITADDR] instead of JMP [$0000] - now every address in the

#     program has a name.

# ================================================================

CLRCODE:  .EQU     $10        # Special code to clear the main display

MAINDISP: .EQU     $F031      # Address of output port for main display

.ORG     $4000	# Set program's origin to address $4000

LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Store accumulator to main display

JMP     [$0000]    # Jump to address $0000

.END                # This is the end of the program

### tutorial/15c_lab2c_countdown_loop.asm

tutorial/15c_lab2c_countdown_loop.asm

# ================================================================

# 15c_lab2c_countdown_loop.asm

# Beboputer Hands-On Tutorial — Lab 2c

#

# Clear the display, then count down from 9 to 1 on it.

#

# What this program does

# -----------------------

#   Clears the display (as in Labs 2a/2b), then shows 9, 8, 7, 6,

#   5, 4, 3, 2, 1 in quick succession before stopping - too fast to

#   read each digit individually at full speed, but easy to follow

#   with Step.

#

# How it works

# -------------

#   The first loop this series introduces - decrement-and-test,

#   the same shape almost every later exercise's loops are built

#   from:

#

#     LDA     $09        # Load the accumulator with $09

#     LOOP:      STA     [MAINDISP] # Store accumulator to the main display

#                DECA               # Decrement the accumulator

#                JNZ     [LOOP]     # Jump to LOOP if ACC isn't zero

#

#   ACC starts at 9. Each pass through LOOP shows the CURRENT value

#   before decrementing it, so the display sees 9 first, then 8,

#   then 7, and so on. DECA subtracts 1 and sets the Zero flag when

#   the result lands on exactly 0; JNZ ("jump if not zero") keeps

#   the loop going for every value except that last one. So the

#   sequence is: show 9, decrement to 8, still nonzero, loop; show

#   8, decrement to 7, loop; ...; show 1, decrement to 0, Zero flag

#   set, JNZ does NOT jump - the loop ends right after 1 was shown,

#   which is exactly the "9 downto 1" the header promises (0 is

#   never displayed, since the loop stops as soon as ACC reaches it,

#   before another STA can run).

#

#   $09 down to $01 are raw values, not ASCII text - but they show

#   up as the digit characters "9" through "1" anyway, because the

#   display port has a special case for exactly this range: raw

#   bytes $00-$09 are shown as the digit characters '0'-'9' (see

#   tools/calculator.py's write_display), on top of its normal

#   handling of printable ASCII. This is the same convention every

#   Calculator exercise relies on when it echoes a keypad digit

#   straight to the display without converting it to ASCII first.

#

#   Like Lab 2a, this program ends with JMP [$0000] rather than

#   HALT - see 15a's note for why that works.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run - the display flashes through

#   9 down to 1 almost instantly. Switch to Step instead of Run to

#   watch each digit land one at a time, and watch the accumulator

#   count down in the Registers panel in lockstep.

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status and Step through the program -

#   you'll see $F031 receive 9, then 8, then 7, ... down to 1, one

#   value per trip through LOOP.

#

# Try this next

# ---------------

#   - Count up instead of down: start ACC at $01, INCA instead of

#     DECA, and stop the loop once ACC passes 9 (CMPA $09 then a

#     conditional jump, the way later exercises test ranges).

#   - Insert a delay loop between the STA and the DECA so each digit

#     is visible for a moment instead of flashing past - Lab 2e's

#     inner SHR loop is a good model for a simple busy-wait.

# ================================================================

CLRCODE:  .EQU     $10        # Special code to clear the main display

MAINDISP: .EQU     $F031      # Address of output port for main display

.ORG     $4000	# Set program's origin to address $4000

LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Store accumulator to main display

LDA     $09        # Load the accumulator with $09

LOOP:      STA     [MAINDISP] # Store accumulator to the main display

DECA               # Decrement the accumulator

JNZ     [LOOP]     # Jump to LOOP if ACC isn't zero

JMP     [$0000]    # Jump to address $0000

.END                # This is the end of the program

### tutorial/15d_lab2d_keypad_echo_digits.asm

tutorial/15d_lab2d_keypad_echo_digits.asm

# ================================================================

# 15d_lab2d_keypad_echo_digits.asm

# Beboputer Hands-On Tutorial — Lab 2d

#

# Clear the display, then echo digit keys 0-9 to it - and quietly

# ignore everything else.

#

# What this program does

# -----------------------

#   Clears the display, then sits in a loop reading the keypad

#   port. Press a digit key (0-9) and it appears on the display.

#   Press anything else - an operator, Clear, a hex letter, Sin,

#   whatever - and nothing happens; the program just keeps waiting.

#

# How it works

# -------------

#   A busy-wait loop reading one port ($F011, the keypad), guarded

#   by two range checks before anything reaches the display:

#

#     LOOP:      LDA     [KEYPAD]   # Load the accumulator from the keypad

#                JN      [LOOP]     # Jump to LOOP if N flag is set

#                CMPA    $09        # Compare accumulator to code $09

#                JC      [LOOP]     # Jump to LOOP if C flag is set

#                STA     [MAINDISP] # ... else store accumulator to display

#                JMP     [LOOP]     # Jump to LOOP

#

#   First check - JN ("jump if Negative"): the keypad port sits at

#   $FF ("idle, nothing pressed") whenever no key is down, and $FF

#   has its top bit set, which LDA's N flag reports as "negative."

#   Any byte $80-$FF - not just $FF itself - trips this same check,

#   which conveniently also screens out any button whose code

#   happens to live up in that range, with one read and one branch.

#   As long as nothing is pressed, JN keeps firing and the loop just

#   spins on this line.

#

#   Second check - CMPA $09 then JC ("jump if Carry"): once a key

#   with its top bit clear gets through the first check, this one

#   asks "is it bigger than 9?" This CPU's Carry flag after CMPA

#   means "the accumulator is STRICTLY GREATER THAN the operand"

#   (see exercise 10 for the long version of this, and why it trips

#   up naive "is A >= B" tests elsewhere) - so JC firing means the

#   keypress was 10 or higher, and gets looped past just like an

#   idle read did. Digit and hex-letter buttons arrive as their raw

#   nibble value, 0-15 (see exercise 10's note on tools/diy_button.py)

#   - so this check is exactly what separates plain digits 0-9 from

#   hex letters A-F (10-15) and everything else, without needing to

#   list out ten separate values.

#

#   Only a keypress that survives both checks - not negative, and

#   not greater than 9 - reaches the STA that puts it on the

#   display. Because raw bytes $00-$09 show up as the digit

#   characters '0'-'9' on this display (the same convention Lab 2c

#   and every Calculator exercise relies on), whatever digit you

#   pressed appears exactly as itself.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, switch the Calculator on if it

#   isn't already, and press digit keys - each one appears on the

#   display. Press Clear, an operator, or a letter key and watch

#   nothing happen.

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status and watch $F011 as you press

#   different keys - notice it always briefly shows the byte you

#   pressed and then reverts to $FF (see exercise 05's note on this

#   read-clear-latch behavior), and that only digit keys make it to

#   $F031.

#

# Try this next

# ---------------

#   - Also accept hex letters A-F: change the second check to

#     compare against $0F instead of $09.

#   - Add a Clear-key check ($1B) that resets the display, the way

#     every later Calculator exercise's DO_CLEAR does.

# ================================================================

CLRCODE:  .EQU     $10        # Special code to clear the main display

MAINDISP: .EQU     $F031      # Address of output port for main display

KEYPAD:   .EQU     $F011      # Address of input port for keypad

.ORG     $4000	# Set program's origin to address $4000

LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Store accumulator to main display

LOOP:      LDA     [KEYPAD]   # Load the accumulator from the keypad

JN      [LOOP]     # Jump to LOOP if N flag is set

CMPA    $09        # Compare accumulator to code $09

JC      [LOOP]     # Jump to LOOP if C flag is set

STA     [MAINDISP] # ... else store accumulator to display

JMP     [LOOP]     # Jump to LOOP

.END                # This is the end of the program

### tutorial/15e_lab2e_led_chase_pattern.asm

tutorial/15e_lab2e_led_chase_pattern.asm

# ================================================================

# 15e_lab2e_led_chase_pattern.asm

# Beboputer Hands-On Tutorial — Lab 2e

#

# Walk a single lit LED across the Calculator's six-LED row, one

# position at a time, forever.

#

# What this program does

# -----------------------

#   Clears the display, then lights the leftmost LED alone, then

#   the next one, then the next, all the way to the rightmost -

#   then starts over from the left again. A classic "chase" or

#   "scanner" pattern, built from one instruction most exercises

#   before this one haven't needed yet: SHR.

#

# How it works

# -------------

#   Exercise 07 showed that $F032 lights the six LEDs directly from

#   the bits of whatever byte you store there - bit 5 is the

#   leftmost LED, bit 0 the rightmost. This program starts with only

#   bit 5 set and shifts that single lit bit rightward one place

#   per trip through the inner loop:

#

#     LOOPA:     LDA     $20        # Load accumulator with $20

#     LOOPB:     STA     [SIXLEDS]  # Store accumulator to six LEDs

#                SHR                # Shift accumulator 1 bit to the right

#                JNZ     [LOOPB]    # Jump to LOOPB if Z flag not set

#                JMP     [LOOPA]    # ..else jump to LOOPA

#

#   $20 is 00100000 in binary - bit 5 set, every other bit clear.

#   SHR shifts every bit one place to the right, discarding whatever

#   falls off the bottom and filling the new top bit with 0. Watch

#   what that does across six passes through LOOPB:

#

#       $20 = 00100000   (leftmost LED)

#       $10 = 00010000

#       $08 = 00001000

#       $04 = 00000100

#       $02 = 00000010

#       $01 = 00000001   (rightmost LED)

#       $00 = 00000000   <- SHR of $01 - the bit finally shifts off

#                            the end entirely, and nothing replaces it

#

#   Each of those six values gets written to SIXLEDS and shown

#   before the next SHR runs, so the lit LED visibly walks from left

#   to right. The moment the shift produces $00, JNZ ("jump if Z

#   flag not set") stops firing - SHR sets the Zero flag exactly

#   when its result is zero, the same flag every other instruction

#   in this project sets on a zero result - so the loop falls

#   through to JMP [LOOPA], which reloads $20 and starts the whole

#   sweep over from the left again. The result is a lit LED that

#   walks all the way across and then jumps back to the start,

#   repeating forever (there is no exit condition in this program -

#   Stop or Power Off on the Calculator is the only way out).

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, and watch the LED row - one

#   LED lit at a time, sweeping left to right, restarting at the

#   left each time it reaches the far right.

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status and watch $F032's value cycle

#   through $20, $10, $08, $04, $02, $01, then jump back to $20 -

#   Step through slowly to see SHR's effect on the accumulator in

#   the Registers panel one bit at a time.

#

# Try this next

# ---------------

#   - Make it bounce instead of restart: once you reach $01, switch

#     to SHL and walk back up to $20 before switching back to SHR,

#     instead of jumping straight back to LOOPA.

#   - Light two LEDs at once (e.g. start at $22 = 00100010) and

#     watch both march across in step.

#   - Slow it down: exercise 15c's note on adding a delay loop

#     applies here too - without one, the sweep runs faster than the

#     eye can separate each individual step at full emulator speed.

# ================================================================

CLRCODE:  .EQU     $10        # Special code to clear the main display

MAINDISP: .EQU     $F031      # Address of output port for main display

SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs

.ORG     $4000	# Set program's origin to address $4000

LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Store accumulator to main display

LOOPA:     LDA     $20        # Load accumulator with $20

LOOPB:     STA     [SIXLEDS]  # Store accumulator to six LEDs

SHR                # Shift accumulator 1 bit to the right

JNZ     [LOOPB]    # Jump to LOOPB if Z flag not set

JMP     [LOOPA]    # ..else jump to LOOPA

.END                # This is the end of the program

### tutorial/15f_lab2f_hello_world.asm

tutorial/15f_lab2f_hello_world.asm

# ================================================================

# 15f_lab2f_hello_world.asm

# Beboputer Hands-On Tutorial — Lab 2f

#

# Print "HELLO WORLD!" on the Calculator's main display.

#

# What this program does

# -----------------------

#   Clears the display, then writes out the message "HELLO WORLD!"

#   one character at a time and stops.

#

# How it works

# -------------

#   This is the same table-walking idea exercise 09 uses for its own

#   message - in fact, if this looks familiar, it's because exercise

#   09 is built on exactly this pattern. The message is stored as

#   plain ASCII bytes right after the code, and the Index register

#   (X) walks through them one at a time:

#

#     BLDX        0       # Load the index register with 0

#     LOOP:      LDA     [MESSAGE,X] # Load accumulator with character

#                JZ      [$0000]     # If character = $00 jump to $0000

#     STORE:     STA     [MAINDISP]  # ... else write character to display

#                INCX                # Increment the index register

#                JMP     [LOOP]      # Jump back to LOOP

#

#     MESSAGE:  .BYTE $48, $45, $4C, $4C, $4F, $20

#                #      H    E    L    L    O  SPACE

#                .BYTE $57, $4F, $52, $4C, $44, $21, $00

#                #      W    0    R    L    D    !  NUL

#

#   [MESSAGE,X] is indexed addressing: "read the byte at MESSAGE's

#   address, plus whatever X currently holds." With X starting at 0,

#   the first pass reads MESSAGE+0 ($48, 'H'); INCX makes it 1 for

#   the next pass (MESSAGE+1, $45 'E'); and so on down the table.

#   JZ checks the character that was just loaded before doing

#   anything else with it - the moment a $00 byte turns up (the

#   very last entry in the table, deliberately placed there as a

#   terminator), JZ jumps to $0000 and the program stops (see Lab

#   2a's note on why "jump to zeroed memory" works as a stopping

#   point). Every character before that $00 gets written to the

#   display and the loop continues.

#

#   Notice the message is written as two separate .BYTE lines under

#   one MESSAGE label - the assembler doesn't care how many .BYTE

#   directives you split a table across, only that they all come one

#   after another with no other code between them; the label just

#   marks where the very first byte lands, and INCX walks past the

#   line break exactly the same as it would if it were all one line

#   (see exercise 09 for the same technique used with a shorter

#   message).

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run - "HELLO WORLD!" appears on the

#   display, one character at a time (fast enough that it looks

#   instantaneous at full run speed - switch to Step to watch each

#   character land individually).

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status and Step through the program to

#   watch $F031 receive each character in turn, and watch the Index

#   register count up 0, 1, 2, ... in the Registers panel as X walks

#   the table.

#

# Try this next

# ---------------

#   - Change MESSAGE to your own text - remember the final $00

#     terminator, or the loop will walk straight past the end of the

#     table into whatever bytes happen to follow it.

#   - Replace JZ [$0000] with a proper HALT ($3C), the way exercise

#     09 does, and compare the two approaches directly.

# ================================================================

CLRCODE:  .EQU     $10         # Special code to clear the main display

MAINDISP: .EQU     $F031       # Address of main display output port

.ORG     $4000	 # Set program's origin to address $4000

LDA     CLRCODE     # Load accumulator with clear code

STA     [MAINDISP]  # Store accumulator to main display

BLDX        0       # Load the index register with 0

LOOP:      LDA     [MESSAGE,X] # Load accumulator with character

JZ      [$0000]     # If character = $00 jump to $0000

STORE:     STA     [MAINDISP]  # ... else write character to display

INCX                # Increment the index register

JMP     [LOOP]      # Jump back to LOOP

MESSAGE:  .BYTE $48, $45, $4C, $4C, $4F, $20

#      H    E    L    L    O  SPACE

.BYTE $57, $4F, $52, $4C, $44, $21, $00

#      W    0    R    L    D    !  NUL

.END

# Exercise: Lab 3 — Assembly Language Fundamentals

This tutorial walks through all seventeen Lab 3 assembly-language exercises included with the DIY Calculator (Beboputer) project, found as tutorial/16a through tutorial/16q. Each exercise is an annotated copy of one of the Data/lab3*.asm files that ship with the project - the underlying code is unchanged from the original; only explanatory header comments have been added. Every file in this set has been verified to assemble to byte-identical machine code as its original, and to behave identically when run in the CPU emulator.

Lab 3 is the biggest lab in this tutorial series so far - seventeen small programs instead of five or six - because it covers more ground than any lab before it: looping versus unrolling, four different ways to extract a hex digit from a byte, 8-bit overflow, two different techniques for reversing a sequence (an array versus the stack), nested versus flattened subroutines, and finally genuine recursion. The seventeen files are grouped below by theme, in the order it makes sense to read them, rather than strictly alphabetically.

## Unrolled vs. Looped: Binary Display (16a, 16b)

These two files show the exact same behavior - read a key, show its raw code as eight binary digits - built two different ways.

### 16a - Fully Unrolled

tutorial/16a_lab3a_bin_display_unrolled.asm writes out eight nearly-identical blocks of code, one per bit (TEST7 through TEST0), each doing the same three-step dance: SHL the working byte, test the bit that fell into Carry via JC, and write the matching character. The source file's own title calls this “one painful bit at a time” - and it is: the same three-instruction pattern, copy-pasted eight times, with only the target label changing between copies.

TEST7:     LDA     [TEMP8]    # Reload ACC with copy of key code
           SHL                # Shift left 1 bit
           STA     [TEMP8]    # Store new value in temp location
           JC      [DISP7_1]  # If carry = 1, jump to display a 1
DISP7_0:   LDA     0          # ... otherwise load acc with 0
           STA     [MAINDISP] # ... and store it to main display
           JMP     [TEST6]    # ... then go and deal with bit 6
DISP7_1:   LDA     1          # Load acc with 1
           STA     [MAINDISP] # ... and store it to main display

One small quirk worth noting: the DISP1_1 block's comment reads “Load acc with 0” even though the instruction is LDA 1 - a stale copy-paste comment. The code itself is correct; only the comment is wrong. Preserved exactly as found.

### 16b - Loop-ified

tutorial/16b_lab3a_bin_display_loop.asm replaces all eight TEST blocks with one block that runs eight times, counted down by the Index register:

BLDX    8          # Load X reg with number of bits
LOOP:      LDA     [TEMP8]    # Reload ACC with copy of key code
           SHL                # Shift left 1 bit
           STA     [TEMP8]    # Store new value in temp location
           JC      [DISP_1]   # If carry = 1, jump to display a 1
DISP_0:    LDA     0          # ... otherwise load acc with 0
           STA     [MAINDISP] # ... and store it to main display
           JMP     [DISPDECX] # ... then go and decrement the X reg
DISP_1:    LDA     1          # Load acc with 1
           STA     [MAINDISP] # ... and store it to main display
DISPDECX:  DECX               # Decrement the X reg
           JNZ     [LOOP]     # If not zero jump back for next bit

BLDX 8 loads the loop counter, and DECX/JNZ repeats the body until it reaches zero - the same counting-loop idea introduced for countdown and LED-chase exercises earlier in this series. The two files produce identical output for every key; only the assembled size differs (16a is considerably larger, since its logic is eight full copies instead of one).

## Four Ways to Extract a Hex Nybble (16c-16f)

All four of these files do the exact same job - split a byte’s top nybble out and show it as a hex digit - using a different shift/rotate instruction each time. Comparing them side by side is the point of this group.

16c (SHR) is the simplest: shifting right four times naturally slides the top nybble down to the bottom, and the trailing mask is just insurance.

DISPMSN:   LDA     [TEMP8]    # Reload ACC with copy of key code
           SHR                # Shift right 1 bit (= 1 bit shift)
           SHR                # Shift right 1 bit (= 2 bit shift)
           SHR                # Shift right 1 bit (= 3 bit shift)
           SHR                # Shift right 1 bit (= 4 bit shift)
           AND     %00001111  # Clear MS 4 bits (not really necessary)
           STA     [MAINDISP] # Copy result to main display

16d (SHL) has to work harder, because shifting left moves bits the wrong way for this job. Its trick: catch the bit that SHL discards into Carry, and immediately fold it back in at the bottom via JNC/OR. Four rounds of “shift left, then recycle the lost bit” amounts to rotating the whole byte by four places using the Carry flag as scratch space.

DISPMSN:   BLDX     4         # Load X reg with number of shifts
           LDA     [TEMP8]    # Reload ACC with copy of key code
DISPMSNL:  SHL                # Shift left 1 bit
           JNC     [DISPMSN0] # Jump if carry flag = 0
           OR      %00000001  #   otherwise set LS bit to 1
DISPMSN0:  DECX               # Decrement index register
           JNZ     [DISPMSNL] # If X !=0 jump back and shift again
           AND     %00001111  # Clear MS 4 bits (not really necessary)
           STA     [MAINDISP] # Copy result to main display

16e (ROLC) looks like it should need less manual work than 16d, since ROLC is built to automatically feed the Carry flag into bit 0. But this file doesn’t trust that automatic fill - the incoming Carry value on the very first ROLC is whatever was left over from earlier arithmetic, not necessarily 0 - so it explicitly overrides bit 0 every single pass with an OR or an AND, exactly mirroring 16d’s approach.

16f (RORC) is the cleanest of the rotate-based versions: it doesn't control bit 0 (or bit 7) at all during the four rotates. Whatever garbage ends up in the top nybble from the unpredictable carry-in is simply thrown away by the final AND $0F. Where 16c avoids the problem by shifting the correct direction in the first place, 16f embraces the “wrong” direction but cleans up only once, at the very end, instead of once per pass.

## An Overflow Detour (16g)

tutorial/16g_lab3b_accumulator_wraparound.asm is the shortest program in the whole set - four lines - built purely to demonstrate 8-bit overflow with nothing else going on:

LDA         0       # Load accumulator with zero
LOOP:      INCA                # Increment the accumulator
           JNZ     [LOOP]      # Jump to LOOP if ACC !=0
           JMP     [$0000]     # Jump to address $0000

INCA counts the accumulator up by 1 each pass; JNZ keeps looping as long as the result isn’t zero. Since an 8-bit register can only hold 0-255, the 256th increment (255 + 1) has nowhere to go but back around to 0 - and that’s the one and only time JNZ stops firing. The loop runs exactly 255 times, not 256, since it started counting up from 0 rather than counting through it again on the way to the wrap.

## A Refined Binary-Display Pair (16h, 16i)

tutorial/16h_lab3c_bin_display_restructured.asm is a relabeled, reorganized copy of 16b's loop - same logic, different label names and a slightly different layout, a useful exercise in reading for behavior rather than appearance. tutorial/16i_lab3c_bin_display_nibble_dash.asm builds on it by splitting the single 8-bit loop into two 4-bit loops with a dash written in between, so the display reads “XXXX-XXXX” instead of an unbroken run of eight digits:

LOADXA:    BLDX    4           # Load X reg with 4
LOOPA:     ...                 # (display top 4 bits, same SHL/JC pattern)
DISPDASH:  LDA     $2D          # Load ACC with ASCII code for ’-’
           STA     [MAINDISP]  # ... and store it to main display
LOADXB:    BLDX    4           # Load X reg with 4
LOOPB:     ...                 # (display bottom 4 bits, same pattern again)

LOOPA and LOOPB are two separate copies of the same short loop body, rather than one shared loop with a counter - a small, deliberate trade so that the dash can be written exactly once, cleanly, between the two halves.

## Hex-Digit Echo, Forward and Reversed (16j, 16k, 16m)

16j is the baseline: a keypad loop that accepts hex digits (raw codes $00-$0F) and displays each one immediately, in the order typed - the same shape as the digit-only echo from Lab 2, just widened from $09 to $0F so hex letters A-F get through too.

GETKEY:    LDA     [KEYPAD]   # Load ACC from the keypad
           JN      [GETKEY]   # Jump back if no key pressed
           CMPA    $0F        # Compare ACC to $0F
           JC      [GETKEY]   # Jump back if ACC is bigger
           STA     [MAINDISP] # ... else store ACC to display
           JMP     [GETKEY]   # Go and wait for another key

16k and 16m both reverse a typed sequence - last key typed appears first - using two very different techniques.

### 16k - Reversal via an Array and the Index Register

Every accepted key is written into a small array (STORE) using indexed addressing, with the Index register both counting how many keys were typed and tracking where the next one goes:

GETKEY:    LDA     [KEYPAD]    # Load ACC from the keypad
           ...
           STA     [STORE,X]   # ... else store ACC to memory
           INCX                # ... and increment the index reg
           JMP     [GETKEY]    # Go and wait for another key
DISPSTUF:  LDA     [STORE-1,X] # Load ACC with a key code
           STA     [MAINDISP]  # Store it to the main display
           DECX                # Decrement the index register
           JNZ     [DISPSTUF]  # If index reg not 0 get next code

[STORE-1,X] is indexed addressing with label arithmetic folded into the operand: the effective address is (STORE - 1) + X. Since X still holds the count of keys typed when playback begins, the very first read lands on the last slot filled - the last key typed - and DECX walks backward from there. One edge case is worth knowing: if the very first key pressed is a non-hex key (X still 0), this loop reads one byte before STORE and then DECX takes X from 0 to $FFFF, looping indefinitely rather than stopping - always type at least one hex digit first to avoid it.

### 16m - Reversal via the Stack

This version reaches for PUSHA/POPA instead of an array. Every accepted key gets pushed; because a stack is naturally Last-In-First-Out, the most recently pushed byte is always the first one POPA retrieves - no counting, no index arithmetic at all:

BLDSP   $EFFF       # Load stack pointer with $EFFF
GETKEY:    NOP                 # a harmless one-cycle delay
LOOP:      LDA     [KEYPAD]    # Load ACC from the keypad
           ...
           PUSHA               # ... else push ACC onto the stack
           JMP     [GETKEY]    # Go and wait for another key
DISPSTUF:  POPA                # Pop ACC off the stack
           STA     [MAINDISP]  # Store it to the main display
           JMP     [GETKEY]    # Go and wait for another key

Unlike 16k, this version shows only one digit per non-hex keypress rather than the whole sequence at once, and it never terminates - it’s always ready to accept and reverse another run. Comparing 16k and 16m directly is the real point of this pair: one builds LIFO ordering by hand with an array and index math, the other gets it for free from the CPU’s own stack.

## Displaying the Index Register Itself (16l)

tutorial/16l_lab3c_display_x_register.asm reuses 16k’s exact fill loop (hex digits go into STORE, counted by X) but never reads STORE back out. Instead, once a non-hex key stops the count, it displays X’s own final value - as a 4-digit hex number - using BSTX, the store counterpart to BLDX:

DISPSTUF:  BSTX    [TEMP16]    # Store X reg into 2-byte temp location
           LDA     $24         # Load ACC with ASCII code for ’$’
           STA     [MAINDISP]  # Write ’$’ to main display
           ... (four nybble extractions from TEMP16 and TEMP16+1,
                the same SHR + AND technique as 16c, one per hex digit)

Because the Index register is 16 bits wide, this count can run past 255 - something no single 8-bit byte could hold - which is exactly why this exercise needs four hex digits of display instead of two.

## Nested vs. Flattened Subroutines (16n, 16o)

Both files display four fixed bytes ($23, $5A, $06, $4C) as hex pairs with identical output; they differ only in how the subroutine that does the work is structured internally.

### 16n - Nested

DISPBYTE calls two separate subroutines, DISPMSN and DISPLSN, each with its own JSR and RTS:

DISPBYTE:  PUSHA              # Push a copy of ACC onto stack
           JSR     [DISPMSN]  # Call sub for MS nybble
           POPA               # Pop copy of original byte off stack
           JSR     [DISPLSN]  # Call sub for LS nybble
           RTS                # Return to calling location

The PUSHA/POPA pair around the DISPMSN call exists because DISPMSN’s four SHRs are destructive - they consume the byte’s bits while pulling the high nybble down - so the original value has to be stashed on the stack and restored before DISPLSN can extract the low nybble from it.

### 16o - Flattened

The same two nybble-extraction blocks are folded directly into DISPBYTE as fall-through labels - no JSR, no RTS until the very end:

DISPBYTE:  PUSHA              # Push a copy of ACC onto stack
DISPMSN:   SHR / SHR / SHR / SHR
           AND     %00001111
           STA     [MAINDISP]
DISPLSN:   POPA               # Pop copy of original byte off stack
           AND     %00001111
           STA     [MAINDISP]
           RTS                # Return from subroutine

DISPMSN and DISPLSN are still labels - useful for reading - but nothing outside DISPBYTE ever jumps to them. Execution simply falls from one block into the next. Same data flow (the same PUSHA/POPA protecting the same destructive shift), same output, purely different internal structure: two callable subroutines versus one subroutine with internal signposts.

## Recursion: Factorial and String Reversal (16p, 16q)

The last two files introduce genuine recursion - a subroutine calling itself - the natural extension of everything the stack-based exercises above built toward.

### 16p - Recursive Factorial

Press a key from 1 to 5 and FACTOR computes its factorial by calling itself:

FACTOR:    PUSHA              # Push copy of ACC onto stack
           CMPA     $01       # Compare ACC to 1
           JNC     [COMEOUT]  # If ACC not > 1 start to come out
           DECA               #   else decrement ACC
           JSR     [FACTOR]   #   then this routine calls itself
COMEOUT:   PUSHA              # Push copy of ACC onto stack
           JSR     [FMULT]    # Multiply two bytes on top of stack
           POPA               # Retrieve 8-bit result
           RTS                # End this instantiation of the routine

Each call pushes its own copy of n before recursing on n-1, so computing 5! leaves five values on the stack (5, 4, 3, 2, 1) - one per call. When the base case (n=1) is reached, COMEOUT multiplies the two topmost stack values via a separate, non-recursive FMULT subroutine (a classic 8x8-bit shift-and-add multiply) and the product propagates back up one level at a time: 1x1=1, 2x1=2, 3x2=6, 4x6=24, 5x24=120. The keypad range is capped at 1-5 for a concrete reason: 6! is 720, which doesn’t fit in an 8-bit accumulator - every factorial this program can actually compute (1, 2, 6, 24, 120) fits in one byte by design.

### 16q - Recursive String Reversal

REVERSE prints a stored message backward by calling itself once per character and letting the call stack remember the order to print them back in:

REVERSE:   LDA     [PHRASE,X] # Load ACC with a character
           JNZ     [GOIN]     # If it’s not NUL jump to GO_IN
RETURN1:   RTS                # Otherwise return from subroutine
GOIN:      PUSHA              # Push the character onto the stack
           INCX               # Increment the index register
INSIDE:    JSR     [REVERSE]  # The subroutine calls itself recursively
COMEOUT:   POPA               # Pop a character off the stack
           STA     [MAINDISP] # Copy to main display
RETURN2:   RTS                # Return from subroutine

Nothing is displayed until the recursion bottoms out at the table's NUL terminator; from there, each returning call pops and displays the one character it pushed on the way in, then returns to whichever call invoked it. The stored phrase, “SWAP PAWS”, was chosen because it's a palindrome (S-W-A-P-space-P-A-W-S reversed is the identical sequence) - which means the display alone can't prove the reversal actually happened. Watching the stack pointer and the Index register in the Registers panel while stepping through is the real evidence: it visibly descends through all nine characters before anything is displayed, then unwinds one character at a time on the way back out.

## Appendix: Full Source Listings

### 16a - Binary Display, Unrolled

tutorial/16a_lab3a_bin_display_unrolled.asm

# ================================================================

# 16a_lab3a_bin_display_unrolled.asm

# Beboputer Hands-On Tutorial — Lab 3a (version 1)

#

# Display a key's raw code as 8 binary digits - the "painful",

# fully-unrolled way.

#

# What this program does

# -----------------------

#   Waits for any key, clears the display, writes a '%' character

#   (this project's convention for "what follows is binary" - see

#   the six-LED BINMODE/DECMODE/HEXMODE codes used since exercise 07),

#   then writes out that key's raw code as eight separate '0'/'1'

#   characters, most-significant bit first. Then it waits for the

#   next key and does it all again, forever.

#

# How it works

# -------------

#   The whole bit-by-bit display is eight nearly-identical blocks of

#   code, one per bit, each doing the same three things: shift the

#   working byte left one place, check whether a 1 or a 0 just fell

#   out the top, and write the matching character - then jump to the

#   next block down for the next bit:

#

#     TEST7:     LDA     [TEMP8]    # Reload ACC with copy of key code

#                SHL                # Shift left 1 bit

#                STA     [TEMP8]    # Store new value in temp location

#                JC      [DISP7_1]  # If carry = 1, jump to display a 1

#     DISP7_0:   LDA     0          # ... otherwise load acc with 0

#                STA     [MAINDISP] # ... and store it to main display

#                JMP     [TEST6]    # ... then go and deal with bit 6

#     DISP7_1:   LDA     1          # Load acc with 1

#                STA     [MAINDISP] # ... and store it to main display

#

#   SHL shifts every bit in the accumulator one place left, and the

#   bit that falls off the top edge lands in the Carry flag - that's

#   exactly the bit this block needs to test. JC ("jump if Carry")

#   picks the DISP*_1 path when that bit was a 1, and falls through

#   to the DISP*_0 path otherwise. Either way the digit character

#   goes to the display, and the block ends by jumping to the next

#   bit's block (TEST6, then TEST5, and so on down to TEST0).

#

#   This is deliberately the "hard way" - the source file's own title

#   calls it "one painful bit at a time." Every one of the eight

#   blocks does the exact same three-step dance, differing only in

#   which bit they're working on and which block comes next. That

#   repetition is the whole point: it sets up exercise 16b, which

#   notices the pattern and replaces all eight copies with one loop.

#

#   One quirk worth a second look: TEST1's DISP1_1 block is commented

#   "Load acc with 0" even though the instruction is LDA 1 - a leftover

#   copy-paste comment from the DISP*_0 block above it. The code

#   itself is correct (it does load 1 and display a '1'); only the

#   comment is stale. Left exactly as found, since this tutorial

#   preserves the original source unchanged.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, switch the Calculator on, and

#   press any key - its raw code appears as eight 1s and 0s after a

#   '%'. Try Hex mode's 'A'-'F' keys too (raw codes 10-15) and watch

#   the top nibble come out as all zeros.

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status and Step through one key-press -

#   watch $F011 (KEYPAD) get read once, then TEMP8 shift left eight

#   times as the eight TEST blocks run in turn, each writing one

#   character to $F031 (MAINDISP).

#

# Try this next

# ---------------

#   - Count the bytes this version assembles to, then compare against

#     16b's loop-ified version of the exact same behavior.

#   - Fix the stale "Load acc with 0" comment on DISP1_1 and see that

#     nothing about the program's behavior changes - comments don't

#     affect what the CPU does.

# ================================================================

## Lab 3a binary display one painful bit at a time

#######################################################################

## Start of constant declarations                                    ##

#######################################################################

MAINDISP: .EQU     $F031      # Address of output port for main display

SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs

KEYPAD:   .EQU     $F011      # Address of input port for keypad

CLRCODE:  .EQU     $10        # Special code to clear the main display

BINMODE:  .EQU     %00000100  # LED code to indicate binary mode

DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode

HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode

#######################################################################

## End of constant declarations                                      ##

#######################################################################

.ORG     $4000      # Set program origin

#######################################################################

## Start of initialization                                           ##

#######################################################################

INIT:      LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Write clear code to main display

LDA     BINMODE    # Load accumulator with bin mode code

STA     [SIXLEDS]  # Write to port driving six LEDs

#######################################################################

## End of initialization                                             ##

#######################################################################

#######################################################################

## Start of main program body                                        ##

#######################################################################

########## Wait for key to be pressed

GETKEY:    LDA     [KEYPAD]   # Load ACC with code from keypad

JN      [GETKEY]   # Jump back if no key pressed

STA     [TEMP8]    # Store key code in temp location

########## Prepare the main display

CLRDISP:   LDA     CLRCODE    # Load ACC with clear code

STA     [MAINDISP] # Clear main display

DISPPERC:  LDA     $25        # Load ACC with ASCII code for '%'

STA     [MAINDISP] # Write '%' to main display

########## Display the binary value

## Process bit 7

TEST7:     LDA     [TEMP8]    # Reload ACC with copy of key code

SHL                # Shift left 1 bit

STA     [TEMP8]    # Store new value in temp location

JC      [DISP7_1]  # If carry = 1, jump to display a 1

DISP7_0:   LDA     0          # ... otherwise load acc with 0

STA     [MAINDISP] # ... and store it to main display

JMP     [TEST6]    # ... then go and deal with bit 6

DISP7_1:   LDA     1          # Load acc with 1

STA     [MAINDISP] # ... and store it to main display

## Process bit 6

TEST6:     LDA     [TEMP8]    # Reload ACC from temp location

SHL                # Shift left 1 bit

STA     [TEMP8]    # Store new value in temp location

JC      [DISP6_1]  # If carry = 1, jump to display a 1

DISP6_0:   LDA     0          # ... otherwise load acc with 0

STA     [MAINDISP] # ... and store it to main display

JMP     [TEST5]    # ... then go and deal with bit 5

DISP6_1:   LDA     1          # Load acc with 1

STA     [MAINDISP] # ... and store it to main display

## Process bit 5

TEST5:     LDA     [TEMP8]    # Reload ACC from temp location

SHL                # Shift left 1 bit

STA     [TEMP8]    # Store new value in temp location

JC      [DISP5_1]  # If carry = 1, jump to display a 1

DISP5_0:   LDA     0          # ... otherwise load acc with 0

STA     [MAINDISP] # ... and store it to main display

JMP     [TEST4]    # ... then go and deal with bit 4

DISP5_1:   LDA     1          # Load acc with 1

STA     [MAINDISP] # ... and store it to main display

## Process bit 4

TEST4:     LDA     [TEMP8]    # Reload ACC from temp location

SHL                # Shift left 1 bit

STA     [TEMP8]    # Store new value in temp location

JC      [DISP4_1]  # If carry = 1, jump to display a 1

DISP4_0:   LDA     0          # ... otherwise load acc with 0

STA     [MAINDISP] # ... and store it to main display

JMP     [TEST3]    # ... then go and deal with bit 3

DISP4_1:   LDA     1          # Load acc with 1

STA     [MAINDISP] # ... and store it to main display

## Process bit 3

TEST3:     LDA     [TEMP8]    # Reload ACC from temp location

SHL                # Shift left 1 bit

STA     [TEMP8]    # Store new value in temp location

JC      [DISP3_1]  # If carry = 1, jump to display a 1

DISP3_0:   LDA     0          # ... otherwise load acc with 0

STA     [MAINDISP] # ... and store it to main display

JMP     [TEST2]    # ... then go and deal with bit 2

DISP3_1:   LDA     1          # Load acc with 1

STA     [MAINDISP] # ... and store it to main display

## Process bit 2

TEST2:     LDA     [TEMP8]    # Reload ACC from temp location

SHL                # Shift left 1 bit

STA     [TEMP8]    # Store new value in temp location

JC      [DISP2_1]  # If carry = 1, jump to display a 1

DISP2_0:   LDA     0          # ... otherwise load acc with 0

STA     [MAINDISP] # ... and store it to main display

JMP     [TEST1]    # ... then go and deal with bit 1

DISP2_1:   LDA     1          # Load acc with 1

STA     [MAINDISP] # ... and store it to main display

## Process bit 1

TEST1:     LDA     [TEMP8]    # Reload ACC from temp location

SHL                # Shift left 1 bit

STA     [TEMP8]    # Store new value in temp location

JC      [DISP1_1]  # If carry = 1, jump to display a 1

DISP1_0:   LDA     0          # ... otherwise load acc with 0

STA     [MAINDISP] # ... and store it to main display

JMP     [TEST0]    # ... then go and deal with bit 6

DISP1_1:   LDA     1          # Load acc with 0

STA     [MAINDISP] # ... and store it to main display

## Process bit 0

TEST0:     LDA     [TEMP8]    # Reload ACC from temp location

SHL                # Shift left 1 bit

STA     [TEMP8]    # Store new value in temp location

JC      [DISP0_1]  # If carry = 1, jump to display a 1

DISP0_0:   LDA     0          # ... otherwise load acc with 0

STA     [MAINDISP] # ... and store it to main display

JMP     [DONE]     # ... then go and deal with bit 6

DISP0_1:   LDA     1          # Load acc with 1

STA     [MAINDISP] # ... and store it to main display

########## Do it all again

DONE:      JMP     [GETKEY]   # Jump back and wait for new key

#######################################################################

## End of main program body                                          ##

#######################################################################

#######################################################################

## Start of subroutines                                              ##

#######################################################################

#######################################################################

## End of subroutines                                                ##

#######################################################################

#######################################################################

## Start of global data                                              ##

#######################################################################

TEMP8:    .BYTE               # 8-bit temp location to store data

#######################################################################

## End of global data                                                ##

#######################################################################

.END

# That's all folks

### 16b - Binary Display, Loop

tutorial/16b_lab3a_bin_display_loop.asm

# ================================================================

# 16b_lab3a_bin_display_loop.asm

# Beboputer Hands-On Tutorial — Lab 3a (version 2)

#

# The same binary display as 16a - as one loop instead of eight

# copy-pasted blocks.

#

# What this program does

# -----------------------

#   Behaves identically to 16a: wait for a key, clear the display,

#   show '%', then show that key's raw code as eight binary digits.

#   The visible result on the display is exactly the same. What's

#   different is entirely inside the code.

#

# How it works

# -------------

#   16a's eight nearly-identical TEST blocks collapse into one block

#   that runs eight times, counted down by the Index register:

#

#     BLDX    8          # Load X reg with number of bits

#     LOOP:      LDA     [TEMP8]    # Reload ACC with copy of key code

#                SHL                # Shift left 1 bit

#                STA     [TEMP8]    # Store new value in temp location

#                JC      [DISP_1]   # If carry = 1, jump to display a 1

#     DISP_0:    LDA     0          # ... otherwise load acc with 0

#                STA     [MAINDISP] # ... and store it to main display

#                JMP     [DISPDECX] # ... then go and decrement the X reg

#     DISP_1:    LDA     1          # Load acc with 1

#                STA     [MAINDISP] # ... and store it to main display

#     DISPDECX:  DECX               # Decrement the X reg

#                JNZ     [LOOP]     # If not zero jump back for next bit

#

#   BLDX 8 loads the Index register with the loop count (8 bits to

#   show). Each pass through LOOP does the exact same SHL/JC dance

#   16a did per bit, then DECX counts the register down by one and

#   JNZ ("jump if Z flag not set") repeats LOOP as long as X hasn't

#   reached zero yet. The eighth pass leaves X at 0, JNZ stops

#   firing, and the code falls through to DONE.

#

#   This is the same "loop instead of unrolling" idea exercise 15c's

#   countdown loop and 15e's LED chase both use, just applied to a

#   fixed bit-count instead of a changing value or a shifting LED

#   pattern - one small block of code, plus a counter, replaces eight

#   copies of nearly the same thing.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, press keys - the display should

#   look identical to 16a's for every key you try.

#

# Watch it happen

# -----------------

#   Open Registers and watch the Index register count down 8, 7, 6,

#   ... 1, 0 as LOOP repeats - compare this to 16a, which has no

#   Index register activity at all (it used the source code itself

#   as its "loop counter," unrolled eight times).

#

# Try this next

# ---------------

#   - Compare the total byte count of 16a vs 16b (Load -> CPU reports

#     size, or check the assembler's listing) - the loop version is

#     dramatically smaller for identical behavior, at the cost of a

#     few extra instructions executed per pass (DECX and JNZ, which

#     16a's unrolled version didn't need).

#   - See exercise 16h and 16i for a further-refined version of this

#     same loop idea, applied with a nybble separator added in.

# ================================================================

## Lab 3a binary display using the index (X) register

#######################################################################

## Start of constant declarations                                    ##

#######################################################################

MAINDISP: .EQU     $F031      # Address of output port for main display

SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs

KEYPAD:   .EQU     $F011      # Address of input port for keypad

CLRCODE:  .EQU     $10        # Special code to clear the main display

BINMODE:  .EQU     %00000100  # LED code to indicate binary mode

DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode

HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode

#######################################################################

## End of constant declarations                                      ##

#######################################################################

.ORG     $4000      # Set program origin

#######################################################################

## Start of initialization                                           ##

#######################################################################

INIT:      LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Write clear code to main display

LDA     BINMODE    # Load accumulator with bin mode code

STA     [SIXLEDS]  # Write to port driving six LEDs

#######################################################################

## End of initialization                                             ##

#######################################################################

#######################################################################

## Start of main program body                                        ##

#######################################################################

########## Wait for key to be pressed

GETKEY:    LDA     [KEYPAD]   # Load ACC with code from keypad

JN      [GETKEY]   # Jump back if no key pressed

STA     [TEMP8]    # Store key code in temp location

########## Prepare the main display

CLRDISP:   LDA     CLRCODE    # Load ACC with clear code

STA     [MAINDISP] # Clear main display

DISPPERC:  LDA     $25        # Load ACC with ASCII code for '%'

STA     [MAINDISP] # Write '%' to main display

########## Display the binary value

BLDX    8          # Load X reg with number of bits

LOOP:      LDA     [TEMP8]    # Reload ACC with copy of key code

SHL                # Shift left 1 bit

STA     [TEMP8]    # Store new value in temp location

JC      [DISP_1]   # If carry = 1, jump to display a 1

DISP_0:    LDA     0          # ... otherwise load acc with 0

STA     [MAINDISP] # ... and store it to main display

JMP     [DISPDECX] # ... then go and decrement the X reg

DISP_1:    LDA     1          # Load acc with 1

STA     [MAINDISP] # ... and store it to main display

DISPDECX:  DECX               # Decrement the X reg

JNZ     [LOOP]     # If not zero jump back for next bit

########## Do it all again

DONE:      JMP     [GETKEY]   # Jump back and wait for new key

#######################################################################

## End of main program body                                          ##

#######################################################################

#######################################################################

## Start of subroutines                                              ##

#######################################################################

#######################################################################

## End of subroutines                                                ##

#######################################################################

#######################################################################

## Start of global data                                              ##

#######################################################################

TEMP8:    .BYTE               # 8-bit temp location to store data

#######################################################################

## End of global data                                                ##

#######################################################################

.END

# That's all folks

### 16c - Hex Nybble via SHR

tutorial/16c_lab3a_hex_nibbles_shr.asm

# ================================================================

# 16c_lab3a_hex_nibbles_shr.asm

# Beboputer Hands-On Tutorial — Lab 3a (hex display, SHR version)

#

# Display a key's raw code as two hex digits, extracting the

# most-significant nybble with four SHR instructions.

#

# What this program does

# -----------------------

#   Waits for any key, clears the display, writes a '$' (this

#   project's convention for "what follows is hex," the same one

#   tutorial 14 uses), then shows that key's raw code as two hex

#   digit characters. Then it waits for the next key and repeats.

#

# How it works

# -------------

#   Two nybbles, two ways of getting them - and this version's most

#   significant nybble extraction is about as simple as it gets:

#

#     DISPMSN:   LDA     [TEMP8]    # Reload ACC with copy of key code

#                SHR                # Shift right 1 bit (= 1 bit shift)

#                SHR                # Shift right 1 bit (= 2 bit shift)

#                SHR                # Shift right 1 bit (= 3 bit shift)

#                SHR                # Shift right 1 bit (= 4 bit shift)

#                AND     %00001111  # Clear MS 4 bits (not really necessary)

#                STA     [MAINDISP] # Copy result to main display

#

#   SHR shifts every bit right one place and fills the vacated top

#   bit with 0 - do that four times and the byte's top nybble has

#   slid down into the bottom nybble, with zeros above it. The

#   trailing AND $0F is (as the comment admits) not strictly needed

#   here, since four SHRs on an 8-bit byte already leave the top

#   nybble as zero - but it's cheap insurance, and it matches the

#   pattern used for the least-significant nybble right below it:

#

#     DISPLSN:   LDA     [TEMP8]    # Reload ACC with copy of key code

#                AND     %00001111  # Mask out (clear) MS nybble

#                STA     [MAINDISP] # Copy result to main display

#

#   No shifting needed for the low nybble - it's already sitting in

#   the bottom 4 bits, so a single AND is enough to blank out the top

#   nybble before displaying it. This exact SHR + AND nybble-split is

#   the same technique tutorial 14's hex/bin conversion tutorial uses

#   to turn a 4-bit binary value back into a hex digit character.

#

#   This tutorial and the next three (16d, 16e, 16f) all extract the

#   exact same most-significant nybble, from the exact same byte,

#   and display the exact same result - only the instruction used to

#   do the shifting changes. Compare them side by side to see how

#   much (or how little) extra bookkeeping each approach costs.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, press any key - its raw code

#   appears as two hex digits after a '$'.

#

# Watch it happen

# -----------------

#   Step through DISPMSN and watch the accumulator in the Registers

#   panel shift right one nybble's worth over the four SHRs.

#

# Try this next

# ---------------

#   - Compare against 16d (SHL + manual carry reinsertion), 16e

#     (ROLC + explicit bit0 override), and 16f (RORC, no manual

#     bit-forcing needed at all) - four different roads to the same

#     nybble.

# ================================================================

## Lab 3a hex display using SHR instructions

#######################################################################

## Start of constant declarations                                    ##

#######################################################################

MAINDISP: .EQU     $F031      # Address of output port for main display

SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs

KEYPAD:   .EQU     $F011      # Address of input port for keypad

CLRCODE:  .EQU     $10        # Special code to clear the main display

BINMODE:  .EQU     %00000100  # LED code to indicate binary mode

DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode

HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode

#######################################################################

## End of constant declarations                                      ##

#######################################################################

.ORG     $4000      # Set program origin

#######################################################################

## Start of initialization                                           ##

#######################################################################

INIT:      LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Write clear code to main display

LDA     HEXMODE    # Load accumulator with hex mode code

STA     [SIXLEDS]  # Write to port driving six LEDs

#######################################################################

## End of initialization                                             ##

#######################################################################

#######################################################################

## Start of main program body                                        ##

#######################################################################

########## Wait for key to be pressed

GETKEY:    LDA     [KEYPAD]   # Load ACC with code from keypad

JN      [GETKEY]   # Jump back if no key pressed

STA     [TEMP8]    # Store key code in temp location

########## Prepare the main display

CLRDISP:   LDA     CLRCODE    # Load ACC with clear code

STA     [MAINDISP] # Clear main display

DISPDOLL:  LDA     $24        # Load ACC with ASCII code for '$'

STA     [MAINDISP] # Write '$' to main display

########## Extract and display the most-significant nybble

DISPMSN:   LDA     [TEMP8]    # Reload ACC with copy of key code

SHR                # Shift right 1 bit (= 1 bit shift)

SHR                # Shift right 1 bit (= 2 bit shift)

SHR                # Shift right 1 bit (= 3 bit shift)

SHR                # Shift right 1 bit (= 4 bit shift)

AND     %00001111  # Clear MS 4 bits (not really necessary)

STA     [MAINDISP] # Copy result to main display

########## Extract and display the least-significant nybble

DISPLSN:   LDA     [TEMP8]    # Reload ACC with copy of key code

AND     %00001111  # Mask out (clear) MS nybble

STA     [MAINDISP] # Copy result to main display

########## Do it all again

DONE:      JMP     [GETKEY]   # Jump back and wait for new key

#######################################################################

## End of main program body                                          ##

#######################################################################

#######################################################################

## Start of subroutines                                              ##

#######################################################################

#######################################################################

## End of subroutines                                                ##

#######################################################################

#######################################################################

## Start of global data                                              ##

#######################################################################

TEMP8:    .BYTE               # 8-bit temp location to store data

#######################################################################

## End of global data                                                ##

#######################################################################

.END                # That's all folks

### 16d - Hex Nybble via SHL

tutorial/16d_lab3a_hex_nibbles_shl.asm

# ================================================================

# 16d_lab3a_hex_nibbles_shl.asm

# Beboputer Hands-On Tutorial — Lab 3a (hex display, SHL version)

#

# The same most-significant nybble as 16c - extracted with SHL and a

# manual carry reinsertion trick instead of plain SHR.

#

# What this program does

# -----------------------

#   Identical behavior to 16c: wait for a key, show '$', then show

#   that key's raw code as two hex digits.

#

# How it works

# -------------

#   The least-significant nybble (DISPLSN) is unchanged from 16c -

#   a single AND does the job. The most-significant nybble is where

#   this version does something less obvious:

#

#     DISPMSN:   BLDX     4         # Load X reg with number of shifts

#                LDA     [TEMP8]    # Reload ACC with copy of key code

#     DISPMSNL:  SHL                # Shift left 1 bit

#                JNC     [DISPMSN0] # Jump if carry flag = 0

#                OR      %00000001  #   otherwise set LS bit to 1

#     DISPMSN0:  DECX               # Decrement index register

#                JNZ     [DISPMSNL] # If X !=0 jump back and shift again

#                AND     %00001111  # Clear MS 4 bits (not really necessary)

#                STA     [MAINDISP] # Copy result to main display

#

#   SHL shifts left and drops the bit that falls off the top edge

#   into Carry - the opposite direction from what you'd want for

#   pulling a high nybble down to the bottom. The trick is what

#   happens right after each shift: whatever bit SHL just discarded

#   into Carry gets manually stuffed back in at the bottom via

#   JNC/OR. Trace it through with TEMP8 = $A7 (1010 0111) and this

#   becomes clear:

#

#       start:  1010 0111

#       SHL #1: 0100 1110, carry=1 -> OR sets bit0: 0100 1111

#       SHL #2: 1001 1110, carry=0 -> no OR:        1001 1110

#       SHL #3: 0011 1100, carry=1 -> OR sets bit0:  0011 1101

#       SHL #4: 0111 1010, carry=0 -> no OR:        0111 1010

#       AND $0F:                                    0000 1010  = $0A

#

#   $A7's top nybble is $A (1010) - and that's exactly what comes

#   out. Four rounds of "shift left, then immediately recycle the

#   bit that fell out back into the bottom" amounts to rotating the

#   whole byte left by four places, done one bit at a time with the

#   carry flag standing in for the "wraparound" a true rotate would

#   handle automatically. BLDX 4 / DECX / JNZ counts the four rounds,

#   the same loop-counting pattern 16b uses for its eight bits.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, press keys - results should

#   match 16c exactly for every key.

#

# Watch it happen

# -----------------

#   Step through DISPMSNL four times and watch both the accumulator

#   and the Carry flag in the Registers panel - each pass, whatever

#   bit lands in Carry gets folded straight back into bit 0.

#

# Try this next

# ---------------

#   - See 16e for the same "recycle the bit that fell out" idea done

#     with ROLC instead of a plain SHL, and 16f for the direction

#     (RORC) that needs no manual bit-recycling at all.

# ================================================================

## Lab 3a hex display using SHL instructions

#######################################################################

## Start of constant declarations                                    ##

#######################################################################

MAINDISP: .EQU     $F031      # Address of output port for main display

SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs

KEYPAD:   .EQU     $F011      # Address of input port for keypad

CLRCODE:  .EQU     $10        # Special code to clear the main display

BINMODE:  .EQU     %00000100  # LED code to indicate binary mode

DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode

HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode

#######################################################################

## End of constant declarations                                      ##

#######################################################################

.ORG     $4000      # Set program origin

#######################################################################

## Start of initialization                                           ##

#######################################################################

INIT:      LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Write clear code to main display

LDA     HEXMODE    # Load accumulator with hex mode code

STA     [SIXLEDS]  # Write to port driving six LEDs

#######################################################################

## End of initialization                                             ##

#######################################################################

#######################################################################

## Start of main program body                                        ##

#######################################################################

########## Wait for key to be pressed

GETKEY:    LDA     [KEYPAD]   # Load ACC with code from keypad

JN      [GETKEY]   # Jump back if no key pressed

STA     [TEMP8]    # Store key code in temp location

########## Prepare the main display

CLRDISP:   LDA     CLRCODE    # Load ACC with clear code

STA     [MAINDISP] # Clear main display

DISPDOLL:  LDA     $24        # Load ACC with ASCII code for '$'

STA     [MAINDISP] # Write '$' to main display

########## Extract and display the most-significant nybble

DISPMSN:   BLDX     4         # Load X reg with number of shifts

LDA     [TEMP8]    # Reload ACC with copy of key code

DISPMSNL:  SHL                # Shift left 1 bit

JNC     [DISPMSN0] # Jump if carry flag = 0

OR      %00000001  #   otherwise set LS bit to 1

DISPMSN0:  DECX               # Decrement index register

JNZ     [DISPMSNL] # If X !=0 jump back and shift again

AND     %00001111  # Clear MS 4 bits (not really necessary)

STA     [MAINDISP] # Copy result to main display

########## Extract and display the least-significant nybble

DISPLSN:   LDA     [TEMP8]    # Reload ACC with copy of key code

AND     %00001111  # Mask out (clear) MS nybble

STA     [MAINDISP] # Copy result to main display

########## Do it all again

DONE:      JMP     [GETKEY]   # Jump back and wait for new key

#######################################################################

## End of main program body                                          ##

#######################################################################

#######################################################################

## Start of subroutines                                              ##

#######################################################################

#######################################################################

## End of subroutines                                                ##

#######################################################################

#######################################################################

## Start of global data                                              ##

#######################################################################

TEMP8:    .BYTE               # 8-bit temp location to store data

#######################################################################

## End of global data                                                ##

#######################################################################

.END                # That's all folks

### 16e - Hex Nybble via ROLC

tutorial/16e_lab3a_hex_nibbles_rolc.asm

# ================================================================

# 16e_lab3a_hex_nibbles_rolc.asm

# Beboputer Hands-On Tutorial — Lab 3a (hex display, ROLC version)

#

# The same most-significant nybble as 16c/16d - this time using

# ROLC, and still forcing bit 0 by hand rather than trusting ROLC's

# own carry-in.

#

# What this program does

# -----------------------

#   Identical behavior to 16c and 16d: wait for a key, show '$',

#   then show that key's raw code as two hex digits.

#

# How it works

# -------------

#   ROLC ("rotate left through carry") is built for exactly this

#   kind of bit recycling - it shifts the accumulator left, moves the

#   old top bit into Carry, and moves the *previous* Carry flag value

#   into the new bit 0, all in one instruction. That sounds like it

#   should make 16d's manual OR unnecessary. This version, though,

#   doesn't lean on that automatic bit-0 fill at all - it overrides

#   it explicitly every single time:

#

#     DISPMSN:   BLDX     4         # Load X reg with number of shifts

#                LDA     [TEMP8]    # Reload ACC with copy of key code

#     DISPMSNL:  ROLC               # Shift left 1 bit

#                JNC     [DISPMSN0] # Jump if carry flag = 0

#     DISPMNS1:  OR      %00000001  #   otherwise set LS bit to 1

#                JMP     [DISPDECX] #   then jump to decrement X reg

#     DISPMSN0:  AND     %11111110  # Clear LS bit to 0

#     DISPDECX:  DECX               # Decrement index register

#                JNZ     [DISPMSNL] # If X !=0 jump back and shift again

#                AND     %00001111  # Clear MS 4 bits (not really necessary)

#                STA     [MAINDISP] # Copy result to main display

#

#   The JNC test here is checking the *new* Carry flag that this

#   ROLC just produced - the bit that fell off the top - exactly the

#   same bit 16d's SHL produced. Whichever way that test comes out,

#   this code then forces bit 0 to match it by hand: OR to set it,

#   AND $FE to clear it. Whatever ROLC itself had already written

#   into bit 0 (from the Carry flag's value *before* this instruction

#   ran - old, stale information left over from some earlier

#   operation) gets stomped on and replaced immediately.

#

#   Put another way: this program uses ROLC only for the "shift left

#   and capture the bit that fell out in Carry" half of what ROLC

#   does, and throws away the "fill bit 0 from the old Carry" half by

#   overwriting it a line later. It ends up doing the exact same

#   work as 16d's SHL version, with one extra instruction (AND $FE)

#   in the path where the bit was a 0, since SHL already leaves

#   bit 0 at 0 for free while ROLC does not.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, press keys - results should

#   match 16c and 16d exactly for every key.

#

# Watch it happen

# -----------------

#   Step through DISPMSNL and compare the accumulator's bit 0 right

#   after ROLC executes against what it becomes right after the

#   following OR/AND - watch the explicit override happen in real

#   time in the Registers panel.

#

# Try this next

# ---------------

#   - Remove the OR/AND overrides and see what garbage turns up in

#     the displayed nybble - Carry's value going into the very first

#     ROLC is whatever was left over from CLRDISP's LDA/STA pair, not

#     necessarily 0, so the "automatic" bit-0 fill can't be trusted

#     here without first clearing Carry.

#   - Compare against 16f, which uses RORC and needs no bit-forcing

#     at all because the final AND mask cleans up any leftover

#     carry-chain bits regardless of what they were.

# ================================================================

## Lab 3a hex display using ROLC instructions

#######################################################################

## Start of constant declarations                                    ##

#######################################################################

MAINDISP: .EQU     $F031      # Address of output port for main display

SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs

KEYPAD:   .EQU     $F011      # Address of input port for keypad

CLRCODE:  .EQU     $10        # Special code to clear the main display

BINMODE:  .EQU     %00000100  # LED code to indicate binary mode

DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode

HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode

#######################################################################

## End of constant declarations                                      ##

#######################################################################

.ORG     $4000      # Set program origin

#######################################################################

## Start of initialization                                           ##

#######################################################################

INIT:      LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Write clear code to main display

LDA     HEXMODE    # Load accumulator with hex mode code

STA     [SIXLEDS]  # Write to port driving six LEDs

#######################################################################

## End of initialization                                             ##

#######################################################################

#######################################################################

## Start of main program body                                        ##

#######################################################################

########## Wait for key to be pressed

GETKEY:    LDA     [KEYPAD]   # Load ACC with code from keypad

JN      [GETKEY]   # Jump back if no key pressed

STA     [TEMP8]    # Store key code in temp location

########## Prepare the main display

CLRDISP:   LDA     CLRCODE    # Load ACC with clear code

STA     [MAINDISP] # Clear main display

DISPDOLL:  LDA     $24        # Load ACC with ASCII code for '$'

STA     [MAINDISP] # Write '$' to main display

########## Extract and display the most-significant nybble

DISPMSN:   BLDX     4         # Load X reg with number of shifts

LDA     [TEMP8]    # Reload ACC with copy of key code

DISPMSNL:  ROLC               # Shift left 1 bit

JNC     [DISPMSN0] # Jump if carry flag = 0

DISPMNS1:  OR      %00000001  #   otherwise set LS bit to 1

JMP     [DISPDECX] #   then jump to decrement X reg

DISPMSN0:  AND     %11111110  # Clear LS bit to 0

DISPDECX:  DECX               # Decrement index register

JNZ     [DISPMSNL] # If X !=0 jump back and shift again

AND     %00001111  # Clear MS 4 bits (not really necessary)

STA     [MAINDISP] # Copy result to main display

########## Extract and display the least-significant nybble

DISPLSN:   LDA     [TEMP8]    # Reload ACC with copy of key code

AND     %00001111  # Mask out (clear) MS nybble

STA     [MAINDISP] # Copy result to main display

########## Do it all again

DONE:      JMP     [GETKEY]   # Jump back and wait for new key

#######################################################################

## End of main program body                                          ##

#######################################################################

#######################################################################

## Start of subroutines                                              ##

#######################################################################

#######################################################################

## End of subroutines                                                ##

#######################################################################

#######################################################################

## Start of global data                                              ##

#######################################################################

TEMP8:    .BYTE               # 8-bit temp location to store data

#######################################################################

## End of global data                                                ##

#######################################################################

.END                # That's all folks

### 16f - Hex Nybble via RORC

tutorial/16f_lab3a_hex_nibbles_rorc.asm

# ================================================================

# 16f_lab3a_hex_nibbles_rorc.asm

# Beboputer Hands-On Tutorial — Lab 3a (hex display, RORC version)

#

# The same most-significant nybble as 16c/16d/16e - four plain RORCs

# and a trailing mask, no bit-forcing needed anywhere.

#

# What this program does

# -----------------------

#   Identical behavior to the other three hex-display versions: wait

#   for a key, show '$', then show that key's raw code as two hex

#   digits.

#

# How it works

# -------------

#   Where 16e's ROLC needed explicit AND/OR overrides to control what

#   landed in bit 0 each pass, this version just runs RORC four times

#   in a row and masks off whatever's left afterward:

#

#     DISPMSN:   LDA     [TEMP8]    # Reload ACC with copy of key code

#                RORC               # Shift right 1 bit (= 1 bit shift)

#                RORC               # Shift right 1 bit (= 2 bit shift)

#                RORC               # Shift right 1 bit (= 3 bit shift)

#                RORC               # Shift right 1 bit (= 4 bit shift)

#                AND     %00001111  # Clear MS 4 bits (not really necessary)

#                STA     [MAINDISP] # Copy result to main display

#

#   RORC ("rotate right through carry") shifts every bit right one

#   place, moves the old bit 0 into Carry, and fills the new top bit

#   (bit 7) from whatever Carry held *before* this instruction ran.

#   That incoming Carry value is unpredictable here - it's whatever

#   was left over from earlier arithmetic, not something this code

#   bothers to clear first. So after four RORCs, the top nybble of

#   the result may have genuine leftover carry-chain bits mixed into

#   it, not necessarily zeros.

#

#   Unlike 16e, this version doesn't care. The AND $0F at the end

#   throws away the entire top nybble regardless of what ended up

#   there, keeping only the bottom nybble - which, after four

#   right-shifts, correctly holds the original top nybble of TEMP8.

#   Whatever RORC put in the bits the mask discards simply never

#   matters. That's the whole difference between this version and

#   16e's: 16e controls bit 0 on every pass because it needs the

#   *final* top nybble to already be clean; this version lets bit 7

#   be whatever it wants to be on every pass, because the final AND

#   cleans up after all four passes at once instead of after each one.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, press keys - results should

#   match 16c, 16d, and 16e exactly for every key, despite RORC's

#   top-bit fill being unpredictable along the way.

#

# Watch it happen

# -----------------

#   Step through all four RORCs and watch the accumulator's top

#   nybble in the Registers panel end up different-looking after

#   each pass depending on Carry's incoming state - then watch the

#   final AND $0F erase all of that regardless.

#

# Try this next

# ---------------

#   - Deliberately set Carry to 1 before DISPMSN runs (e.g. add a

#     CMPA that's guaranteed to set it) and confirm the displayed

#     nybble is unaffected - proof that AND $0F really does discard

#     whatever landed in the top nybble.

#   - Compare the byte count of all four hex-nybble versions (16c

#     through 16f) - SHR is the shortest, needing no loop and no

#     bit-forcing at all.

# ================================================================

## Lab 3a hex display using RORC instructions

#######################################################################

## Start of constant declarations                                    ##

#######################################################################

MAINDISP: .EQU     $F031      # Address of output port for main display

SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs

KEYPAD:   .EQU     $F011      # Address of input port for keypad

CLRCODE:  .EQU     $10        # Special code to clear the main display

BINMODE:  .EQU     %00000100  # LED code to indicate binary mode

DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode

HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode

#######################################################################

## End of constant declarations                                      ##

#######################################################################

.ORG     $4000      # Set program origin

#######################################################################

## Start of initialization                                           ##

#######################################################################

INIT:      LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Write clear code to main display

LDA     HEXMODE    # Load accumulator with hex mode code

STA     [SIXLEDS]  # Write to port driving six LEDs

#######################################################################

## End of initialization                                             ##

#######################################################################

#######################################################################

## Start of main program body                                        ##

#######################################################################

########## Wait for key to be pressed

GETKEY:    LDA     [KEYPAD]   # Load ACC with code from keypad

JN      [GETKEY]   # Jump back if no key pressed

STA     [TEMP8]    # Store key code in temp location

########## Prepare the main display

CLRDISP:   LDA     CLRCODE    # Load ACC with clear code

STA     [MAINDISP] # Clear main display

DISPDOLL:  LDA     $24        # Load ACC with ASCII code for '$'

STA     [MAINDISP] # Write '$' to main display

########## Extract and display the most-significant nybble

DISPMSN:   LDA     [TEMP8]    # Reload ACC with copy of key code

RORC               # Shift right 1 bit (= 1 bit shift)

RORC               # Shift right 1 bit (= 2 bit shift)

RORC               # Shift right 1 bit (= 3 bit shift)

RORC               # Shift right 1 bit (= 4 bit shift)

AND     %00001111  # Clear MS 4 bits (not really necessary)

STA     [MAINDISP] # Copy result to main display

########## Extract and display the least-significant nybble

DISPLSN:   LDA     [TEMP8]    # Reload ACC with copy of key code

AND     %00001111  # Mask out (clear) MS nybble

STA     [MAINDISP] # Copy result to main display

########## Do it all again

DONE:      JMP     [GETKEY]   # Jump back and wait for new key

#######################################################################

## End of main program body                                          ##

#######################################################################

#######################################################################

## Start of subroutines                                              ##

#######################################################################

#######################################################################

## End of subroutines                                                ##

#######################################################################

#######################################################################

## Start of global data                                              ##

#######################################################################

TEMP8:    .BYTE               # 8-bit temp location to store data

#######################################################################

## End of global data                                                ##

#######################################################################

.END                # That's all folks

### 16g - Accumulator Wraparound

tutorial/16g_lab3b_accumulator_wraparound.asm

# ================================================================

# 16g_lab3b_accumulator_wraparound.asm

# Beboputer Hands-On Tutorial — Lab 3b

#

# Watch an 8-bit accumulator count all the way up and wrap back

# around to zero.

#

# What this program does

# -----------------------

#   Starts the accumulator at 0, counts it up by 1 over and over,

#   and stops the instant it wraps back around to 0 again - which

#   takes exactly 255 increments, since an 8-bit register can only

#   hold values 0 through 255.

#

# How it works

# -------------

#   The entire program is four lines:

#

#     LDA         0       # Load accumulator with zero

#     LOOP:      INCA                # Increment the accumulator

#                JNZ     [LOOP]      # Jump to LOOP if ACC !=0

#                JMP     [$0000]     # Jump to address $0000

#

#   INCA adds 1 to the accumulator and sets the Zero flag exactly

#   when the result is 0. The first time through, ACC goes 0 -> 1,

#   Z is clear, JNZ ("jump if Z flag not set") loops back. This

#   keeps happening - 1, 2, 3, ... - climbing all the way up through

#   255 ($FF). The 256th increment is where it gets interesting:

#   an 8-bit register has no 257th value to hold, so 255 + 1 wraps

#   around to 0 instead of becoming 256. That's the one and only

#   time INCA's result is 0, so that's the one and only time JNZ

#   stops firing, and the loop falls through to the terminating jump.

#

#   This is the same "jump to zeroed memory" stopping trick exercise

#   09 and Lab 2a both explain (RAM below $4000 is zeroed at power-on,

#   and a zeroed byte decodes as NOP, so the CPU just spins on NOPs

#   forever once it lands there) - there's nothing displayed, no

#   keypad read, nothing but the accumulator counting and wrapping.

#   It's a stripped down, minimal way to see 8-bit overflow happen

#   with your own eyes in the debugger, without any display logic

#   getting in the way of the one concept being demonstrated.

#

# Try it

# -------

#   Assemble, Load -> CPU, and switch to Step mode (Run will finish

#   this in a flash since there's nothing slowing it down - Step is

#   how you actually watch it happen). Step through and count how

#   many times INCA runs before Z finally sets.

#

# Watch it happen

# -----------------

#   Open Registers and watch the accumulator and the Z flag as you

#   Step: ACC counts 1, 2, 3, ... up to 255 ($FF), then the next

#   INCA takes it to $00 and only then does Z flip to 1 - the loop

#   runs 255 times, not 256, because 0 was the starting value the

#   loop counted up *from*, not one it counted through again until

#   the wrap.

# ================================================================

## Lab 3b - Simple program to demonstrate the way in which

##          the program counter works

.ORG     $4000	 # Set program's origin to address $4000

LDA         0       # Load accumulator with zero

LOOP:      INCA                # Increment the accumulator

JNZ     [LOOP]      # Jump to LOOP if ACC !=0

JMP     [$0000]     # Jump to address $0000

.END                 # Terminate the program

### 16h - Binary Display, Restructured

tutorial/16h_lab3c_bin_display_restructured.asm

# ================================================================

# 16h_lab3c_bin_display_restructured.asm

# Beboputer Hands-On Tutorial — Lab 3c (binary display, restructured)

#

# The same 8-bit binary display as 16b - relabeled and reorganized,

# with the exact same logic underneath.

#

# What this program does

# -----------------------

#   Behaves identically to 16b: wait for a key, clear the display,

#   show '%', then show that key's raw code as eight binary digits.

#

# How it works

# -------------

#   Put this side by side with 16b and the two are almost the same

#   program - the same BLDX/SHL/JC/DECX/JNZ loop, the same overall

#   shape:

#

#     LOADX:     BLDX    8           # Load X reg with 8

#     LOOP:      LDA     [TEMP8]     # Reload ACC from temp location

#                SHL                 # Shift left 1 bit

#                STA     [TEMP8]     # Store new value in temp location

#                JC      [DISP_1]    # If carry = 1, jump to display a 1

#     DISP_0:    LDA     0           # ... otherwise load acc with 0

#                STA     [MAINDISP]  # ... and store it to main display

#                JMP     [DEALWX]    # ... then go and deal with the X reg

#     DISP_1:    LDA     1           # Load acc with 1

#                STA     [MAINDISP]  # ... and store it to main display

#     DEALWX:    DECX                # Decrement the index register

#                JNZ     [LOOP]      # If X not zero then do next bit

#                JMP     [GETKEY]    # Go back and wait for new key

#

#   Compare this to 16b's version of the same loop and the only real

#   differences are cosmetic: labels renamed (LOADX/DEALWX here vs

#   BLDX's bare instruction and DISPDECX in 16b), and the "go back to

#   GETKEY" jump moved to the bottom of the loop body instead of

#   living under its own DONE label. Every instruction that actually

#   runs, and the order it runs in, is identical.

#

#   This file is a good exercise in reading assembly for behavior

#   rather than appearance: two source files that don't look alike

#   line-for-line can still assemble to functionally (and here,

#   almost byte-for-byte) identical programs. Worth comparing labels

#   and structure against 16b directly to see exactly what changed

#   and what didn't.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, press keys - the display should

#   look identical to 16b's for every key you try.

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status and compare a Step-through of

#   this file against 16b's - the sequence of port writes to $F031

#   should match exactly, key for key.

#

# Try this next

# ---------------

#   - See 16i for this same loop split into two 4-bit passes with a

#     '-' separator in between, grouping the output as "XXXX-XXXX"

#     instead of one unbroken run of 8 digits.

# ================================================================

## Lab 3c - New version of binary display using index (X) reg

#######################################################################

## Start of constant declarations                                    ##

#######################################################################

MAINDISP: .EQU     $F031      # Address of output port for main display

SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs

KEYPAD:   .EQU     $F011      # Address of input port for keypad

CLRCODE:  .EQU     $10        # Special code to clear the main display

BINMODE:  .EQU     %00000100  # LED code to indicate binary mode

DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode

HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode

#######################################################################

## End of constant declarations                                      ##

#######################################################################

.ORG     $4000      # Set program origin

#######################################################################

## Start of initialization                                           ##

#######################################################################

INIT:      LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Write clear code to main display

LDA     BINMODE    # Load accumulator with bin mode code

STA     [SIXLEDS]  # Write to port driving six LEDs

#######################################################################

## End of initialization                                             ##

#######################################################################

#######################################################################

## Start of main program body                                        ##

#######################################################################

########## Get and display key codes in binary

## Wait for a key to be pressed

GETKEY:    LDA     [KEYPAD]    # Load ACC from the keypad

JN      [GETKEY]    # Jump back if no key pressed

STA     [TEMP8]     # Store ACC in temp location

## Clear display then display the '%' character

CLRDISP:   LDA     CLRCODE     # Load ACC with clear code

STA     [MAINDISP]  # Clear main display

DISPPERC:  LDA     $25         # Load ACC with ASCII code for '%'

STA     [MAINDISP]  # Write '%' to main display

## Initialize index register

LOADX:     BLDX    8           # Load X reg with 8

## Loop around writing

LOOP:      LDA     [TEMP8]     # Reload ACC from temp location

SHL                 # Shift left 1 bit

STA     [TEMP8]     # Store new value in temp location

JC      [DISP_1]    # If carry = 1, jump to display a 1

DISP_0:    LDA     0           # ... otherwise load acc with 0

STA     [MAINDISP]  # ... and store it to main display

JMP     [DEALWX]    # ... then go and deal with the X reg

DISP_1:    LDA     1           # Load acc with 1

STA     [MAINDISP]  # ... and store it to main display

DEALWX:    DECX                # Decrement the index register

JNZ     [LOOP]      # If X not zero then do next bit

JMP     [GETKEY]    # Go back and wait for new key

#######################################################################

## End of main program body                                          ##

#######################################################################

#######################################################################

## Start of subroutines                                              ##

#######################################################################

#######################################################################

## End of subroutines                                                ##

#######################################################################

#######################################################################

## Start of global data                                              ##

#######################################################################

TEMP8:    .BYTE                # Temp  8-bit (1-byte) location

#######################################################################

## End of global data                                                ##

#######################################################################

.END                # That's all folks

### 16i - Binary Display, Nybble + Dash

tutorial/16i_lab3c_bin_display_nibble_dash.asm

# ================================================================

# 16i_lab3c_bin_display_nibble_dash.asm

# Beboputer Hands-On Tutorial — Lab 3c (binary display, nybble

# groups with a dash separator)

#

# Show a key's raw code as two groups of 4 binary digits with a

# '-' between them, instead of 8 digits running together.

#

# What this program does

# -----------------------

#   Waits for a key, clears the display, shows '%', then shows the

#   key's raw code as "XXXX-XXXX" - four binary digits, a dash, then

#   four more - instead of 16h's unbroken run of eight digits.

#

# How it works

# -------------

#   This takes 16h's single 8-count loop and splits it into two

#   separate 4-count loops with a dash written in between:

#

#     LOADXA:    BLDX    4           # Load X reg with 4

#     LOOPA:     LDA     [TEMP8]     # Reload ACC from temp location

#                SHL                 # Shift left 1 bit

#                STA     [TEMP8]     # Store new value in temp location

#                JC      [DISP_1A]   # If carry = 1, jump to display a 1

#     DISP_0A:   LDA     0           # ... otherwise load acc with 0

#                STA     [MAINDISP]  # ... and store it to main display

#                JMP     [DEALWXA]   # ... then go and deal with the X reg

#     DISP_1A:   LDA     1           # Load acc with 1

#                STA     [MAINDISP]  # ... and store it to main display

#     DEALWXA:   DECX                # Decrement the index register

#                JNZ     [LOOPA]     # If X not zero then do next bit

#

#     DISPDASH:  LDA     $2D         # Load ACC with ASCII code for '-'

#                STA     [MAINDISP]  # ... and store it to main display

#

#   ... followed by a second, near-identical loop (LOOPB/DEALWXB)

#   that displays the remaining 4 bits the same way. TEMP8 is shared

#   between both halves - it isn't reset in between - so LOOPA

#   naturally consumes the top 4 bits (shifting them out via SHL just

#   like every earlier version in this series) and LOOPB picks up

#   exactly where LOOPA left off, displaying the bottom 4 bits.

#

#   Notice LOOPA and LOOPB are two separate copies of the same

#   four-line loop body, each with its own labels (the 'A' and 'B'

#   suffixes) - a small, deliberate bit of duplication rather than

#   one shared loop, presumably because writing the dash needs to

#   happen exactly once, in between the two halves, rather than

#   being something a single unified loop could express cleanly with

#   just a counter. It's a reasonable trade: two copies of a short

#   loop instead of one loop with an awkward "am I halfway done yet"

#   check bolted on.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, press keys - the display shows

#   the same 8 bits as 16h/16b would, but grouped as "XXXX-XXXX".

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status and watch $F031 receive four

#   digits, then $2D (the dash), then four more digits - compare the

#   timing against 16h, which writes eight digits with no pause.

#

# Try this next

# ---------------

#   - Try merging LOOPA and LOOPB into one loop that runs 8 times and

#     writes a dash exactly when the counter reaches 4 - is the

#     result shorter or longer than keeping two separate loops?

# ================================================================

## Lab 3c - New version of binary display using index (X) reg

##          Also display '-' between MS and LS nybbles

#######################################################################

## Start of constant declarations                                    ##

#######################################################################

MAINDISP: .EQU     $F031      # Address of output port for main display

SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs

KEYPAD:   .EQU     $F011      # Address of input port for keypad

CLRCODE:  .EQU     $10        # Special code to clear the main display

BINMODE:  .EQU     %00000100  # LED code to indicate binary mode

DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode

HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode

#######################################################################

## End of constant declarations                                      ##

#######################################################################

.ORG     $4000      # Set program origin

#######################################################################

## Start of initialization                                           ##

#######################################################################

INIT:      LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Write clear code to main display

LDA     BINMODE    # Load accumulator with bin mode code

STA     [SIXLEDS]  # Write to port driving six LEDs

#######################################################################

## End of initialization                                             ##

#######################################################################

#######################################################################

## Start of main program body                                        ##

#######################################################################

########## Get and display key codes in binary

## Wait for a key to be pressed

GETKEY:    LDA     [KEYPAD]    # Load ACC from the keypad

JN      [GETKEY]    # Jump back if no key pressed

STA     [TEMP8]     # Store ACC in temp location

## Clear display then display the '%' character

CLRDISP:   LDA     CLRCODE     # Load ACC with clear code

STA     [MAINDISP]  # Clear main display

DISPPERC:  LDA     $25         # Load ACC with ASCII code for '%'

STA     [MAINDISP]  # Write '%' to main display

## Display MS Nybble

LOADXA:    BLDX    4           # Load X reg with 4

LOOPA:     LDA     [TEMP8]     # Reload ACC from temp location

SHL                 # Shift left 1 bit

STA     [TEMP8]     # Store new value in temp location

JC      [DISP_1A]   # If carry = 1, jump to display a 1

DISP_0A:   LDA     0           # ... otherwise load acc with 0

STA     [MAINDISP]  # ... and store it to main display

JMP     [DEALWXA]   # ... then go and deal with the X reg

DISP_1A:   LDA     1           # Load acc with 1

STA     [MAINDISP]  # ... and store it to main display

DEALWXA:   DECX                # Decrement the index register

JNZ     [LOOPA]     # If X not zero then do next bit

## Display a dash character

DISPDASH:  LDA     $2D         # Load ACC with ASCII code for '-'

STA     [MAINDISP]  # ... and store it to main display

## Display MS Nybble

LOADXB:    BLDX    4           # Load X reg with 4

LOOPB:     LDA     [TEMP8]     # Reload ACC from temp location

SHL                 # Shift left 1 bit

STA     [TEMP8]     # Store new value in temp location

JC      [DISP_1B]   # If carry = 1, jump to display a 1

DISP_0B:   LDA     0           # ... otherwise load acc with 0

STA     [MAINDISP]  # ... and store it to main display

JMP     [DEALWXB]   # ... then go and deal with the X reg

DISP_1B:   LDA     1           # Load acc with 1

STA     [MAINDISP]  # ... and store it to main display

DEALWXB:   DECX                # Decrement the index register

JNZ     [LOOPB]     # If X not zero then do next bit

## Do it all again

JMP     [GETKEY]    # Go back and wait for new key

#######################################################################

## End of main program body                                          ##

#######################################################################

#######################################################################

## Start of subroutines                                              ##

#######################################################################

#######################################################################

## End of subroutines                                                ##

#######################################################################

#######################################################################

## Start of global data                                              ##

#######################################################################

TEMP8:    .BYTE                # Temp  8-bit (1-byte) location

#######################################################################

## End of global data                                                ##

#######################################################################

.END

# That's all folks

### 16j - Hex Echo, Forward

tutorial/16j_lab3c_hex_echo_forward.asm

# ================================================================

# 16j_lab3c_hex_echo_forward.asm

# Beboputer Hands-On Tutorial — Lab 3c (hex echo, forward order)

#

# Echo hex digit keys 0-F to the display, in the order they're

# pressed - the baseline this tutorial's next two files build on.

#

# What this program does

# -----------------------

#   Sits in a loop reading the keypad. Press any key with a raw code

#   of $0F or less - digits 0-9 or hex letters A-F - and it appears

#   on the display immediately, right after whatever was already

#   there. Press anything else (an operator, Clear, Sin, etc.) and

#   nothing happens.

#

# How it works

# -------------

#   This is Lab 2d's digit-only echo (exercise 15d) with one number

#   changed:

#

#     GETKEY:    LDA     [KEYPAD]   # Load ACC from the keypad

#                JN      [GETKEY]   # Jump back if no key pressed

#                CMPA    $0F        # Compare ACC to $0F

#                JC      [GETKEY]   # Jump back if ACC is bigger

#                STA     [MAINDISP] # ... else store ACC to display

#                JMP     [GETKEY]   # Go and wait for another key

#

#   15d's version compared against $09 to accept only plain digits.

#   This version compares against $0F instead, which - since digit

#   and hex-letter buttons both arrive as their raw nibble value,

#   0-15 (see exercise 10's note on tools/diy_button.py) - lets every

#   value from $00 through $0F through, meaning both the digit keys

#   (0-9) and the hex-letter keys (A-F) get displayed. Raw bytes

#   $00-$09 render as '0'-'9' and $0A-$0F render as 'A'-'F' on this

#   display (the same convention every Calculator exercise in this

#   project relies on), so no ASCII conversion step is needed - the

#   raw nibble value already *is* the character to show.

#

#   Each accepted key just gets appended to whatever's already on

#   the display - there's no clearing, no counting, no memory of

#   what came before. That simplicity is exactly why this file

#   exists: 16k and 16l both build on this same "accept 0-F, do

#   something with it" keypad loop, adding memory (an array, or the

#   Index register's own value) on top of it.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, switch to Hex mode, and type a

#   sequence like 1-2-3-A-B - the display fills up left to right in

#   exactly the order you typed, one character per keypress.

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status and watch $F011 as you press

#   digit and hex-letter keys - both kinds pass the CMPA $0F/JC test

#   and reach $F031, while an operator or Clear key's higher raw code

#   gets looped past.

#

# Try this next

# ---------------

#   - See 16k for the reverse-order version of this same idea (built

#     using an array and the Index register), and 16m for a second,

#     very different way of getting the exact same reversal using

#     the stack instead.

# ================================================================

## Lab 3c - Display '0' thru 'F' keys in the order they are entered

#######################################################################

## Start of constant declarations                                    ##

#######################################################################

MAINDISP: .EQU     $F031      # Address of output port for main display

SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs

KEYPAD:   .EQU     $F011      # Address of input port for keypad

CLRCODE:  .EQU     $10        # Special code to clear the main display

BINMODE:  .EQU     %00000100  # LED code to indicate binary mode

DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode

HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode

#######################################################################

## End of constant declarations                                      ##

#######################################################################

.ORG     $4000      # Set program origin

#######################################################################

## Start of initialization                                           ##

#######################################################################

INIT:      LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Write clear code to main display

LDA     HEXMODE    # Load accumulator with hex mode code

STA     [SIXLEDS]  # Write to port driving six LEDs

#######################################################################

## End of initialization                                             ##

#######################################################################

#######################################################################

## Start of main program body                                        ##

#######################################################################

########## Wait for key to be pressed

GETKEY:    LDA     [KEYPAD]   # Load ACC from the keypad

JN      [GETKEY]   # Jump back if no key pressed

CMPA    $0F        # Compare ACC to $0F

JC      [GETKEY]   # Jump back if ACC is bigger

STA     [MAINDISP] # ... else store ACC to display

JMP     [GETKEY]   # Go and wait for another key

#######################################################################

## End of main program body                                          ##

#######################################################################

#######################################################################

## Start of subroutines                                              ##

#######################################################################

#######################################################################

## End of subroutines                                                ##

#######################################################################

#######################################################################

## Start of global data                                              ##

#######################################################################

#######################################################################

## End of global data                                                ##

#######################################################################

.END                # That's all folks

### 16k - Hex Echo, Reverse via Array

tutorial/16k_lab3c_hex_echo_reverse_array.asm

# ================================================================

# 16k_lab3c_hex_echo_reverse_array.asm

# Beboputer Hands-On Tutorial — Lab 3c (hex echo, reverse order via

# an indexed array)

#

# Collect a sequence of hex-digit keys, then play them back in

# reverse order - the last key typed appears first.

#

# What this program does

# -----------------------

#   Reads hex-digit keys ($00-$0F, same acceptance test as 16j) and

#   stores each one instead of displaying it immediately. The moment

#   any other key is pressed (an operator, Clear, Sin - anything with

#   a raw code above $0F), the program stops collecting and instead

#   plays back everything that was typed, last-typed-first, then

#   terminates.

#

# How it works

# -------------

#   Two phases, both built around the Index register, and a small

#   fixed-size array to hold what's been typed:

#

#     BLDX    $0000       # Load index register with zero

#     GETKEY:    LDA     [KEYPAD]    # Load ACC from the keypad

#                JN      [GETKEY]    # Jump back if no key pressed

#                CMPA    $0F         # Compare ACC to $0F

#                JC      [DISPSTUF]  # Jump if ACC is bigger

#                STA     [STORE,X]   # ... else store ACC to memory

#                INCX                # ... and increment the index reg

#                JMP     [GETKEY]    # Go and wait for another key

#

#   [STORE,X] is indexed addressing (the same technique exercise 09

#   and Lab 2f's message table use for reading): "write to the byte

#   at STORE's address, plus whatever X currently holds." Each

#   accepted key gets written to STORE[0], then STORE[1], then

#   STORE[2], and so on, with INCX advancing to the next slot each

#   time. By the time a non-hex key ends this phase, X holds the

#   *count* of keys that were typed - not their sum or their last

#   value, just how many there were - and STORE[0..X-1] holds them

#   in the order they arrived.

#

#   The playback phase reuses that same count to walk backward:

#

#     DISPSTUF:  LDA     [STORE-1,X] # Load ACC with a key code

#                STA     [MAINDISP]  # Store it to the main display

#                DECX                # Decrement the index register

#                JNZ     [DISPSTUF]  # If index reg not 0 get next code

#                JMP     [$0000]     # Terminate the program

#

#   [STORE-1,X] is indexed addressing with a label-arithmetic offset

#   folded into the operand: the effective address is (STORE - 1) + X.

#   Since X still holds the count from the fill phase, the very first

#   read here lands on (STORE - 1) + count = STORE[count - 1] - the

#   *last* slot that got filled, i.e. the last key typed. DECX then

#   walks that same expression backward one slot at a time - count-2,

#   count-3, ... down to slot 0 - so the whole sequence plays back in

#   the exact opposite order from how it went in.

#

#   One edge case worth knowing about: if the very first key pressed

#   is a non-hex key (so X is still 0 when DISPSTUF begins), this

#   loop reads one byte *before* STORE, displays whatever garbage

#   happens to sit there, and then DECX takes a 16-bit X register

#   from 0 down to $FFFF - not zero, so JNZ loops again, walking

#   backward through memory indefinitely. The lab exercises in this

#   project are teaching tools rather than hardened production code,

#   and this is a good example of the kind of boundary case worth

#   noticing rather than a bug worth "fixing" in the original file -

#   simply always type at least one hex digit before pressing a

#   non-hex key to stay clear of it.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, switch to Hex mode, type a

#   short sequence like 1-2-3-A, then press any operator key - the

#   display shows A-3-2-1, the reverse of what you typed.

#

# Watch it happen

# -----------------

#   Open Registers and watch X count up 1, 2, 3, 4 while typing, then

#   count back down 4, 3, 2, 1, 0 during playback - and open Memory

#   to watch the STORE array fill up in typed order during the first

#   phase.

#

# Try this next

# ---------------

#   - Compare this array-based approach against 16m, which gets the

#     exact same last-in-first-out reversal using PUSHA/POPA and the

#     stack instead of a hand-rolled array and index arithmetic.

#   - Add a bounds check against STORE's 10-byte capacity so a very

#     long sequence can't silently run past the reserved area.

# ================================================================

## Lab 3c - Display '0' thru 'F' keys in the reverse order to that in

##          which they are entered using the index (X) register

#######################################################################

## Start of constant declarations                                    ##

#######################################################################

MAINDISP: .EQU     $F031      # Address of output port for main display

SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs

KEYPAD:   .EQU     $F011      # Address of input port for keypad

CLRCODE:  .EQU     $10        # Special code to clear the main display

BINMODE:  .EQU     %00000100  # LED code to indicate binary mode

DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode

HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode

#######################################################################

## End of constant declarations                                      ##

#######################################################################

.ORG     $4000      # Set program origin

#######################################################################

## Start of initialization                                           ##

#######################################################################

INIT:      LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Write clear code to main display

LDA     HEXMODE    # Load accumulator with hex mode code

STA     [SIXLEDS]  # Write to port driving six LEDs

#######################################################################

## End of initialization                                             ##

#######################################################################

#######################################################################

## Start of main program body                                        ##

#######################################################################

########## Initialize the index register

BLDX    $0000       # Load index register with zero

########## Wait for key to be pressed

GETKEY:    LDA     [KEYPAD]    # Load ACC from the keypad

JN      [GETKEY]    # Jump back if no key pressed

CMPA    $0F         # Compare ACC to $0F

JC      [DISPSTUF]  # Jump if ACC is bigger

STA     [STORE,X]   # ... else store ACC to memory

INCX                # ... and increment the index reg

JMP     [GETKEY]    # Go and wait for another key

########## Display the numbers

DISPSTUF:  LDA     [STORE-1,X] # Load ACC with a key code

STA     [MAINDISP]  # Store it to the main display

DECX                # Decrement the index register

JNZ     [DISPSTUF]  # If index reg not 0 get next code

JMP     [$0000]     # Terminate the program

#######################################################################

## End of main program body                                          ##

#######################################################################

#######################################################################

## Start of subroutines                                              ##

#######################################################################

#######################################################################

## End of subroutines                                                ##

#######################################################################

#######################################################################

## Start of global data                                              ##

#######################################################################

TEMP8:    .BYTE               # Temp  8-bit (1-byte) location

TEMP16:   .2BYTE              # Temp 16-bit (2-byte) location

STORE:    .BYTE    *10        # Reserve 10 x 1-byte locations

#######################################################################

## End of global data                                                ##

#######################################################################

.END                # That's all folks

### 16l - Display the X Register

tutorial/16l_lab3c_display_x_register.asm

# ================================================================

# 16l_lab3c_display_x_register.asm

# Beboputer Hands-On Tutorial — Lab 3c (display the Index register

# itself)

#

# Count hex-digit keypresses with the Index register, then display

# the register's own numeric value - as 4 hex digits - instead of

# the keys that were typed.

#

# What this program does

# -----------------------

#   Reads hex-digit keys ($00-$0F, same acceptance test as 16j/16k)

#   and counts them using the Index register - but never stores the

#   individual key values anywhere. The moment a non-hex key is

#   pressed, it shows the *count* of hex digits that were typed, as

#   a 4-digit hex number after a '$'.

#

# How it works

# -------------

#   The fill phase looks almost identical to 16k's, with one telling

#   difference - notice there's no STA [STORE,X] anywhere:

#

#     BLDX    $0000       # Load index register with zero

#     GETKEY:    LDA     [KEYPAD]    # Load ACC from the keypad

#                JN      [GETKEY]    # Jump back if no key pressed

#                CMPA    $0F         # Compare ACC to $0F

#                JC      [DISPSTUF]  # Jump if ACC is bigger

#                STA     [STORE,X]   # ... else store ACC to memory

#                INCX                # ... and increment the index reg

#                JMP     [GETKEY]    # Go and wait for another key

#

#   The keys still get written into STORE (this file borrows the

#   fill loop unchanged), but nothing ever reads STORE back out -

#   this program only cares about X's final count, not the values it

#   counted. That count can run past 255, since the Index register is

#   16 bits wide - something a plain 8-bit accumulator or TEMP8 byte

#   could never hold - and this exercise is specifically about

#   getting that 16-bit value onto an 8-bit-wide display:

#

#     DISPSTUF:  BSTX    [TEMP16]    # Store X reg into 2-byte temp location

#                LDA     $24         # Load ACC with ASCII code for '$'

#                STA     [MAINDISP]  # Write '$' to main display

#

#     DSMSNMSB:  LDA     [TEMP16]    # Load ACC with MS byte from X reg

#                SHR / SHR / SHR / SHR

#                AND     %00001111   # Clear MS 4 bits

#                STA     [MAINDISP]  # Copy result to main display

#

#     DSLSNMSB:  LDA     [TEMP16]    # Reload ACC with MS byte from X reg

#                AND     %00001111   # Mask out (clear) MS nybble

#                STA     [MAINDISP]  # Copy result to main display

#

#   BSTX ("store X register") is the write counterpart to BLDX - it

#   copies the 16-bit Index register out to a 2-byte memory location

#   in one instruction, here TEMP16. From there, showing the count as

#   hex is exactly the SHR + AND nybble-splitting technique 16c uses,

#   just repeated four times instead of twice: once each for the

#   MS and LS nybbles of TEMP16's high byte (DSMSNMSB/DSLSNMSB), then

#   again for TEMP16+1, its low byte (DSMSNLSB/DSLSNLSB) - four

#   nybbles, four hex digits, one 16-bit number.

#

#   Compare this directly against 16k: both programs count typed hex

#   keys with the exact same fill loop, but 16k displays *what* was

#   typed (in reverse), while this one displays *how many* keys were

#   typed (as a 4-digit hex count) - the same raw ingredients, put

#   to two very different uses.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, switch to Hex mode, type a

#   dozen or so hex digits, then press an operator key - the display

#   shows something like "$000C" (12 in hex), the count of digits

#   you typed, not the digits themselves.

#

# Watch it happen

# -----------------

#   Open Registers and watch the Index register count up while

#   typing, then open Memory and watch TEMP16 receive that same

#   16-bit value in one step when BSTX executes.

#

# Try this next

# ---------------

#   - Type more than 255 hex digits before stopping and confirm the

#     4-digit hex count correctly shows a value like $0105 - something

#     an 8-bit BSTX target never could have held.

# ================================================================

## Lab 3c - Demo of how to display contents of the index (X) register

#######################################################################

## Start of constant declarations                                    ##

#######################################################################

MAINDISP: .EQU     $F031      # Address of output port for main display

SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs

KEYPAD:   .EQU     $F011      # Address of input port for keypad

CLRCODE:  .EQU     $10        # Special code to clear the main display

BINMODE:  .EQU     %00000100  # LED code to indicate binary mode

DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode

HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode

#######################################################################

## End of constant declarations                                      ##

#######################################################################

.ORG     $4000      # Set program origin

#######################################################################

## Start of initialization                                           ##

#######################################################################

INIT:      LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Write clear code to main display

LDA     HEXMODE    # Load accumulator with hex mode code

STA     [SIXLEDS]  # Write to port driving six LEDs

#######################################################################

## End of initialization                                             ##

#######################################################################

#######################################################################

## Start of main program body                                        ##

#######################################################################

########## Initialize the index register

BLDX    $0000       # Load index register with zero

########## Wait for key to be pressed

GETKEY:    LDA     [KEYPAD]    # Load ACC from the keypad

JN      [GETKEY]    # Jump back if no key pressed

CMPA    $0F         # Compare ACC to $0F

JC      [DISPSTUF]  # Jump if ACC is bigger

STA     [STORE,X]   # ... else store ACC to memory

INCX                # ... and increment the index reg

JMP     [GETKEY]    # Go and wait for another key

########## Display the value in the index register

DISPSTUF:  BSTX    [TEMP16]    # Store X reg into 2-byte temp location

LDA     $24         # Load ACC with ASCII code for '$'

STA     [MAINDISP]  # Write '$' to main display

########## Extract/display MS nybble of MS byte

DSMSNMSB:  LDA     [TEMP16]    # Load ACC with MS byte from X reg

SHR                 # Shift right 1 bit (= 1 bit shift)

SHR                 # Shift right 1 bit (= 2 bit shift)

SHR                 # Shift right 1 bit (= 3 bit shift)

SHR                 # Shift right 1 bit (= 4 bit shift)

AND     %00001111   # Clear MS 4 bits

STA     [MAINDISP]  # Copy result to main display

########## Extract/display LS nybble of MS byte

DSLSNMSB:  LDA     [TEMP16]    # Reload ACC with MS byte from X reg

AND     %00001111   # Mask out (clear) MS nybble

STA     [MAINDISP]  # Copy result to main display

########## Extract/display MS nybble of LS byte

DSMSNLSB:  LDA     [TEMP16+1]  # Load ACC with LS byte from X reg

SHR                 # Shift right 1 bit (= 1 bit shift)

SHR                 # Shift right 1 bit (= 2 bit shift)

SHR                 # Shift right 1 bit (= 3 bit shift)

SHR                 # Shift right 1 bit (= 4 bit shift)

AND     %00001111   # Clear MS 4 bits

STA     [MAINDISP]  # Copy result to main display

########## Extract/display LS nybble of LS byte

DSLSNLSB:  LDA     [TEMP16+1]  # Reload ACC with LS byte from X reg

AND     %00001111   # Mask out (clear) MS nybble

STA     [MAINDISP]  # Copy result to main display

JMP     [$0000]     # Terminate the program

#######################################################################

## End of main program body                                          ##

#######################################################################

#######################################################################

## Start of subroutines                                              ##

#######################################################################

#######################################################################

## End of subroutines                                                ##

#######################################################################

#######################################################################

## Start of global data                                              ##

#######################################################################

TEMP8:    .BYTE               # Temp  8-bit (1-byte) location

TEMP16:   .2BYTE              # Temp 16-bit (2-byte) location

STORE:    .BYTE    *10        # Reserve 10 x 1-byte locations

#######################################################################

## End of global data                                                ##

#######################################################################

.END                # That's all folks

### 16m - Hex Echo, Reverse via Stack

tutorial/16m_lab3d_hex_echo_reverse_stack.asm

# ================================================================

# 16m_lab3d_hex_echo_reverse_stack.asm

# Beboputer Hands-On Tutorial — Lab 3d (hex echo, reverse order via

# the stack)

#

# The same last-typed-first reversal as 16k - this time using

# PUSHA/POPA and the CPU's stack instead of a hand-built array.

#

# What this program does

# -----------------------

#   Behaves like 16k: type a run of hex-digit keys, press any other

#   key, and the digits play back in reverse - last typed, first

#   shown. Unlike 16k, this one never stops - after playing a

#   sequence back, it's ready to collect and reverse another one.

#

# How it works

# -------------

#   16k needed a fixed-size array (STORE) and index arithmetic

#   ([STORE-1,X]) to walk backward through what was typed. This

#   version reaches for the stack instead, using PUSHA to remember

#   each digit and POPA to retrieve them - which comes back out in

#   the opposite order automatically, with no array or index math

#   needed at all:

#

#     BLDSP   $EFFF       # Load stack pointer with $EFFF

#

#     GETKEY:    NOP                 # a harmless one-cycle delay - see notes

#     LOOP:      LDA     [KEYPAD]    # Load ACC from the keypad

#                JN      [LOOP]      # Jump back if no key pressed

#                CMPA    $0F         # Compare ACC to $0F

#                JC      [DISPSTUF]  # Jump if ACC is bigger

#                PUSHA               # ... else push ACC onto the stack

#                JMP     [GETKEY]    # Go and wait for another key

#

#     DISPSTUF:  POPA                # Pop ACC off the stack

#                STA     [MAINDISP]  # Store it to the main display

#                JMP     [GETKEY]    # Go and wait for another key

#

#   BLDSP sets up the stack pointer before anything else runs, the

#   same way BLDX sets up the Index register in earlier exercises -

#   this project's stack grows downward from $EFFF. Every accepted

#   hex digit gets PUSHA'd - laid on top of whatever's already

#   there. The stack is naturally Last-In-First-Out: the most

#   recently pushed byte is always the first one a POPA retrieves.

#   So the moment a non-hex key breaks the typing loop, DISPSTUF's

#   POPA hands back the *last* digit that was typed - no counting,

#   no indexed addressing, no [STORE-1,X] arithmetic required.

#

#   And notice DISPSTUF only pops and shows *one* digit, then jumps

#   straight back to GETKEY to wait for the next key - rather than

#   looping through every pushed byte at once the way 16k's DISPSTUF

#   loop does. Practically, that means: type a run of digits, press

#   a non-digit key, and one digit (the last one typed) appears; type

#   more digits, press a non-digit key again, and the *next-most*

#   recently typed digit comes back - the stack keeps handing back

#   whatever's currently on top, one POPA at a time, for as long as

#   there's anything left on it to pop.

#

#   The leading NOP on GETKEY does nothing to the program's behavior

#   - it just burns one CPU cycle before looping back to LOOP each

#   time. The source labels it "see notes" without further comment

#   in this file; harmless either way, and left in place exactly as

#   found. (The original source's comment used curly typographic

#   quotes saved in a Windows text encoding, which don't render

#   correctly as plain UTF-8 - normalized to straight quotes here for

#   readability; the instruction itself, and everything the CPU

#   actually does, is unaffected either way.)

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, switch to Hex mode, type 1-2-3,

#   then press an operator key - '3' appears (the last digit typed).

#   Press the operator key again and nothing more appears (the stack

#   for that run is now empty); type more digits and repeat.

#

# Watch it happen

# -----------------

#   Open Registers and watch the stack pointer (SP) count down by one

#   with every PUSHA and back up by one with every POPA - and compare

#   that against 16k's Index register, which does the equivalent

#   counting job with INCX/DECX instead.

#

# Try this next

# ---------------

#   - Change DISPSTUF to loop (POPA/STA/JNZ-style, checking for an

#     empty-stack condition) so an entire typed run reverses in one

#     shot the way 16k's does, instead of one digit per non-hex

#     keypress.

#   - Compare this file directly against 16k: both reverse a typed

#     sequence, but one leans on the stack's built-in LIFO order and

#     the other builds that same ordering by hand with an array and

#     index arithmetic.

# ================================================================

## Lab 3d - Display hex chars in reverse order using the stack

#######################################################################

## Start of constant declarations                                    ##

#######################################################################

MAINDISP: .EQU     $F031      # Address of output port for main display

SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs

KEYPAD:   .EQU     $F011      # Address of input port for keypad

CLRCODE:  .EQU     $10        # Special code to clear the main display

BINMODE:  .EQU     %00000100  # LED code to indicate binary mode

DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode

HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode

#######################################################################

## End of constant declarations                                      ##

#######################################################################

.ORG     $4000      # Set program origin

#######################################################################

## Start of initialization                                           ##

#######################################################################

INIT:      LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Write clear code to main display

LDA     HEXMODE    # Load accumulator with hex mode code

STA     [SIXLEDS]  # Write to port driving six LEDs

#######################################################################

## End of initialization                                             ##

#######################################################################

#######################################################################

## Start of main program body                                        ##

#######################################################################

########## Initialize the stack pointer

BLDSP   $EFFF       # Load stack pointer with $EFFF

########## Wait for key to be pressed

GETKEY:    NOP                 # "No operation" - see notes

LOOP:      LDA     [KEYPAD]    # Load ACC from the keypad

JN      [LOOP]      # Jump back if no key pressed

CMPA    $0F         # Compare ACC to $0F

JC      [DISPSTUF]  # Jump if ACC is bigger

PUSHA               # ... else push ACC onto the stack

JMP     [GETKEY]    # Go and wait for another key

########## Display a number

DISPSTUF:  POPA                # Pop ACC off the stack

STA     [MAINDISP]  # Store it to the main display

JMP     [GETKEY]    # Go and wait for another key

#######################################################################

## End of main program body                                          ##

#######################################################################

#######################################################################

## Start of subroutines                                              ##

#######################################################################

#######################################################################

## End of subroutines                                                ##

#######################################################################

#######################################################################

## Start of global data                                              ##

#######################################################################

#######################################################################

## End of global data                                                ##

#######################################################################

.END                # That's all folks

### 16n - Nested Subroutines

tutorial/16n_lab3e_nested_subroutines.asm

# ================================================================

# 16n_lab3e_nested_subroutines.asm

# Beboputer Hands-On Tutorial — Lab 3e (nested subroutines)

#

# Display four fixed bytes as hex, using one subroutine that calls

# two more subroutines to do the actual work.

#

# What this program does

# -----------------------

#   On its own, with no keypad interaction at all, shows four

#   pre-loaded byte values ($23, $5A, $06, $4C) as hex digit pairs,

#   one after another, then stops.

#

# How it works

# -------------

#   The main body just loads each byte and calls one subroutine four

#   times:

#

#     LDA     [TEMP]     # Load 1st byte into ACC

#     JSR     [DISPBYTE] # Call display byte subroutine

#     LDA     [TEMP+1]   # Load 2nd byte into ACC

#     JSR     [DISPBYTE] # Call display byte subroutine

#     ... (and so on for TEMP+2, TEMP+3)

#     JMP     [$0000]    # Terminate the program

#

#   JSR/RTS were introduced back in exercise 12 for calling one

#   shared subroutine from three different places. This exercise

#   goes a step further: DISPBYTE isn't one flat block of code - it's

#   a short subroutine that itself calls two *more* subroutines:

#

#     DISPBYTE:  PUSHA              # Push a copy of ACC onto stack

#                JSR     [DISPMSN]  # Call sub for MS nybble

#                POPA               # Pop copy of original byte off stack

#                JSR     [DISPLSN]  # Call sub for LS nybble

#                RTS                # Return to calling location

#

#     DISPMSN:   SHR / SHR / SHR / SHR

#                AND     %00001111  # Clear MS 4 bits (not really necessary)

#                STA     [MAINDISP] # Copy result to main display

#                RTS                # Return to calling location

#

#     DISPLSN:   AND     %00001111  # Mask out (clear) MS nybble

#                STA     [MAINDISP] # Copy result to main display

#                RTS                # Return to calling location

#

#   DISPMSN's four SHRs are destructive - they consume the

#   accumulator's original value while pulling the high nybble down

#   (the same technique 16c uses). By the time DISPMSN returns, the

#   byte's original bits are gone from ACC. That's exactly why

#   DISPBYTE pushes a copy *before* calling DISPMSN, and pops it back

#   *after* - so the original, untouched byte is sitting in ACC again

#   when DISPLSN is called to extract the low nybble from it. Without

#   that PUSHA/POPA pair, DISPLSN would be masking whatever DISPMSN

#   left behind instead of the real byte.

#

#   Three levels of calling happen here: the main body calls

#   DISPBYTE, and DISPBYTE calls DISPMSN and then DISPLSN. Each JSR

#   pushes a return address on the stack (see exercise 12's note on

#   JSR/RTS), and each RTS pops the most recent one - so the calls

#   unwind in the reverse order they were made, same as any nested

#   function calls in a higher-level language.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run - the display shows "23 5A 06 4C"

#   as it processes all four bytes (no separators between pairs,

#   since DISPBYTE never writes one - watch closely, or use Step, to

#   see each pair land).

#

# Watch it happen

# -----------------

#   Open Registers and watch the stack pointer drop by 2 on each JSR

#   and rise by 2 on each RTS, and by 1 more on each PUSHA/POPA -

#   Step through one full DISPBYTE call and count exactly how deep

#   the stack gets before it's all unwound back to where it started.

#

# Try this next

# ---------------

#   - See 16o for the same net result with DISPMSN and DISPLSN

#     folded directly into DISPBYTE as fall-through labels instead of

#     separately JSR'd subroutines - same output, different internal

#     structure.

# ================================================================

## Lab 3e using a nested subroutine

#######################################################################

## Start of constant declarations                                    ##

#######################################################################

MAINDISP: .EQU     $F031      # Address of output port for main display

SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs

KEYPAD:   .EQU     $F011      # Address of input port for keypad

CLRCODE:  .EQU     $10        # Special code to clear the main display

BINMODE:  .EQU     %00000100  # LED code to indicate binary mode

DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode

HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode

#######################################################################

## End of constant declarations                                      ##

#######################################################################

.ORG     $4000      # Set program origin

#######################################################################

## Start of initialization                                           ##

#######################################################################

INIT:      LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Write clear code to main display

LDA     HEXMODE    # Load accumulator with hex mode code

STA     [SIXLEDS]  # Write to port driving six LEDs

BLDSP   $EFFF      # Load stack pointer with $EFFF

#######################################################################

## End of initialization                                             ##

#######################################################################

#######################################################################

## Start of main program body                                        ##

#######################################################################

LDA     [TEMP]     # Load 1st byte into ACC

JSR     [DISPBYTE] # Call display byte subroutine

LDA     [TEMP+1]   # Load 2nd byte into ACC

JSR     [DISPBYTE] # Call display byte subroutine

LDA     [TEMP+2]   # Load 3rd byte into ACC

JSR     [DISPBYTE] # Call display byte subroutine

LDA     [TEMP+3]   # Load 4th byte into ACC

JSR     [DISPBYTE] # Call display byte subroutine

JMP     [$0000]    # Terminate the program

#######################################################################

## End of main program body                                          ##

#######################################################################

#######################################################################

## Start of subroutines                                              ##

#######################################################################

########## Subroutine to extract/display MS and LS nybbles of byte

DISPBYTE:  PUSHA              # Push a copy of ACC onto stack

JSR     [DISPMSN]  # Call sub for MS nybble

POPA               # Pop copy of original byte off stack

JSR     [DISPLSN]  # Call sub for LS nybble

RTS                # Return to calling location

########## Subroutine to extract/display the MS nybble

DISPMSN:   SHR                # Shift right 1 bit (= 1 bit shift)

SHR                # Shift right 1 bit (= 2 bit shift)

SHR                # Shift right 1 bit (= 3 bit shift)

SHR                # Shift right 1 bit (= 4 bit shift)

AND     %00001111  # Clear MS 4 bits (not really necessary)

STA     [MAINDISP] # Copy result to main display

RTS                # Return to calling location

########## Extract and display the LS nybble

DISPLSN:   AND     %00001111  # Mask out (clear) MS nybble

STA     [MAINDISP] # Copy result to main display

RTS                # Return to calling location

#######################################################################

## End of subroutines                                                ##

#######################################################################

#######################################################################

## Start of global data                                              ##

#######################################################################

TEMP:     .BYTE $23, $5A, $06, $4C  # Just some data to play with

#######################################################################

## End of global data                                                ##

#######################################################################

.END                # That's all folks

### 16o - Flattened Subroutine

tutorial/16o_lab3e_flattened_subroutine.asm

# ================================================================

# 16o_lab3e_flattened_subroutine.asm

# Beboputer Hands-On Tutorial — Lab 3e (one flattened subroutine)

#

# The same four-byte hex display as 16n - with DISPMSN and DISPLSN

# folded into DISPBYTE as fall-through labels instead of separately

# called subroutines.

#

# What this program does

# -----------------------

#   Identical behavior to 16n: shows four pre-loaded bytes ($23,

#   $5A, $06, $4C) as hex digit pairs, then stops. The main body

#   calling DISPBYTE four times is unchanged.

#

# How it works

# -------------

#   16n's DISPBYTE made two separate JSR calls, one to DISPMSN and

#   one to DISPLSN, each with its own RTS. This version keeps the

#   same three label names, but only one of them is a real "callable"

#   entry point anymore - the other two are just waypoints inside a

#   single subroutine that falls straight through from one to the

#   next:

#

#     DISPBYTE:  PUSHA              # Push a copy of ACC onto stack

#

#     DISPMSN:   SHR                # Shift right 1 bit (= 1 bit shift)

#                SHR                # Shift right 1 bit (= 2 bit shift)

#                SHR                # Shift right 1 bit (= 3 bit shift)

#                SHR                # Shift right 1 bit (= 4 bit shift)

#                AND     %00001111  # Clear MS 4 bits (not really necessary)

#                STA     [MAINDISP] # Copy result to main display

#

#     DISPLSN:   POPA               # Pop copy of original byte off stack

#                AND     %00001111  # Mask out (clear) MS nybble

#                STA     [MAINDISP] # Copy result to main display

#                RTS                # Return from subroutine

#

#   There's no JSR anywhere in this block, and no RTS until the very

#   end. DISPMSN and DISPLSN are still labels - useful for reading

#   the code and for referring to "the bit that handles the MS

#   nybble" versus "the bit that handles the LS nybble" - but nothing

#   ever jumps to them from outside DISPBYTE. Execution simply falls

#   from the last instruction under DISPBYTE straight into DISPMSN's

#   first instruction, and from DISPMSN's last instruction straight

#   into DISPLSN's, with no jump or call involved at either boundary.

#

#   This is the same PUSHA-before/POPA-after pattern 16n uses to

#   protect the original byte across the destructive SHR chain - the

#   *data flow* is identical between the two versions. What changed

#   is purely structural: 16n expresses "do the MS nybble, then do

#   the LS nybble" as two subroutine calls from a third subroutine;

#   this version expresses the exact same two steps as one

#   subroutine with internal signposts. Both compile down to running

#   the same instructions in the same order - the only difference is

#   whether DISPMSN and DISPLSN could, in principle, be called from

#   somewhere else too (16n's version, yes; this version, no - RTS

#   only appears once, at the very end).

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run - output should be identical to

#   16n's: "23 5A 06 4C".

#

# Watch it happen

# -----------------

#   Step through one call to DISPBYTE and watch the Program Counter

#   in the Registers panel - it moves smoothly from DISPBYTE's last

#   instruction into DISPMSN's first, and from DISPMSN's last into

#   DISPLSN's first, with no jump instruction executing at either

#   handoff (contrast this with 16n, where the PC visibly jumps to a

#   JSR target and back at each of those same two points).

#

# Try this next

# ---------------

#   - Try calling DISPMSN directly (JSR [DISPMSN]) from somewhere

#     else in a copy of this file and see what goes wrong - without

#     its own RTS, it will fall straight through into DISPLSN and pop

#     a stack value that was never pushed for it, rather than

#     returning to the caller.

# ================================================================

## Lab 3e using a simple subroutine

#######################################################################

## Start of constant declarations                                    ##

#######################################################################

MAINDISP: .EQU     $F031      # Address of output port for main display

SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs

KEYPAD:   .EQU     $F011      # Address of input port for keypad

CLRCODE:  .EQU     $10        # Special code to clear the main display

BINMODE:  .EQU     %00000100  # LED code to indicate binary mode

DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode

HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode

#######################################################################

## End of constant declarations                                      ##

#######################################################################

.ORG     $4000      # Set program origin

#######################################################################

## Start of initialization                                           ##

#######################################################################

INIT:      LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Write clear code to main display

LDA     HEXMODE    # Load accumulator with hex mode code

STA     [SIXLEDS]  # Write to port driving six LEDs

BLDSP   $EFFF      # Load stack pointer with $EFFF

#######################################################################

## End of initialization                                             ##

#######################################################################

#######################################################################

## Start of main program body                                        ##

#######################################################################

LDA     [TEMP]     # Load 1st byte into ACC

JSR     [DISPBYTE] # Call display byte subroutine

LDA     [TEMP+1]   # Load 2nd byte into ACC

JSR     [DISPBYTE] # Call display byte subroutine

LDA     [TEMP+2]   # Load 3rd byte into ACC

JSR     [DISPBYTE] # Call display byte subroutine

LDA     [TEMP+3]   # Load 4th byte into ACC

JSR     [DISPBYTE] # Call display byte subroutine

JMP     [$0000]    # Terminate the program

#######################################################################

## End of main program body                                          ##

#######################################################################

#######################################################################

## Start of subroutines                                              ##

#######################################################################

########## Subroutine to extract/display MS and LS nybbles of byte

DISPBYTE:  PUSHA              # Push a copy of ACC onto stack

########## Extract and display the most-significant nybble

DISPMSN:   SHR                # Shift right 1 bit (= 1 bit shift)

SHR                # Shift right 1 bit (= 2 bit shift)

SHR                # Shift right 1 bit (= 3 bit shift)

SHR                # Shift right 1 bit (= 4 bit shift)

AND     %00001111  # Clear MS 4 bits (not really necessary)

STA     [MAINDISP] # Copy result to main display

########## Extract and display the least-significant nybble

DISPLSN:   POPA               # Pop copy of original byte off stack

AND     %00001111  # Mask out (clear) MS nybble

STA     [MAINDISP] # Copy result to main display

RTS                # Return from subroutine

#######################################################################

## End of subroutines                                                ##

#######################################################################

#######################################################################

## Start of global data                                              ##

#######################################################################

TEMP:     .BYTE $23, $5A, $06, $4C  # Just some data to play with

#######################################################################

## End of global data                                                ##

#######################################################################

.END                # That's all folks

### 16p - Recursive Factorial

tutorial/16p_lab3f_recursive_factorial.asm

# ================================================================

# 16p_lab3f_recursive_factorial.asm

# Beboputer Hands-On Tutorial — Lab 3f (recursive factorial)

#

# Press a key from 1 to 5 and see its factorial - computed by a

# subroutine that calls itself.

#

# What this program does

# -----------------------

#   Waits for a keypad key from 1 to 5 (0 and anything above 5 are

#   ignored), computes that number's factorial (1x2x3x...xn), and

#   shows the result as two hex digits. Press any key afterward to

#   clear the display and try another number.

#

# How it works

# -------------

#   This is the first genuinely *recursive* subroutine in this

#   tutorial series - FACTOR calls itself:

#

#     FACTOR:    PUSHA              # Push copy of ACC onto stack

#                CMPA     $01       # Compare ACC to 1

#                JNC     [COMEOUT]  # If ACC not > 1 start to come out

#                DECA               #   else decrement ACC

#                JSR     [FACTOR]   #   then this routine calls itself

#

#     COMEOUT:   PUSHA              # Push copy of ACC onto stack

#                JSR     [FMULT]    # Multiply two bytes on top of stack

#                POPA               # Retrieve 8-bit result

#                RTS                # End this instantiation of the routine

#

#   Call FACTOR with ACC = n. It pushes n onto the stack immediately

#   - that copy stays put through everything that follows, waiting

#   to be multiplied in later. Then CMPA $1/JNC checks whether n is

#   bigger than 1 (recall this CPU's Carry-after-CMPA convention:

#   Carry set means "strictly greater than" - see exercise 15d's

#   note on the same test). If n is 1 or less, this is the base case

#   (1! and 0! both equal 1, though this program only ever reaches n=1

#   in practice - see the note on GETNUM below) - execution jumps

#   straight to COMEOUT. Otherwise, DECA makes n-1, and FACTOR calls

#   *itself* to compute (n-1)! first - it doesn't return to its

#   caller until that inner call has finished.

#

#   Each call to FACTOR pushes its own copy of n onto the stack

#   before recursing, so five nested calls (say, computing 5!) leave

#   five values sitting on the stack - 5, 4, 3, 2, 1 - one per call,

#   each waiting for its own COMEOUT to run. When the innermost call

#   (n=1) finally hits its base case, COMEOUT pushes a second copy of

#   that same 1, then calls FMULT to multiply the two values

#   currently on top of the stack and leave the product there instead

#   - 1x1=1. POPA retrieves the product, and RTS returns to the level

#   above, which does exactly the same thing with its own n (2) and

#   the 1 that just came back: COMEOUT pushes 2, FMULT multiplies

#   2x1=2, and that 2 propagates up to the next level, and so on -

#   3x2=6, 4x6=24, 5x24=120 - one multiplication unwinding at each

#   level of the recursion, until the outermost call returns 120 (=5!)

#   in the accumulator.

#

#   Notice GETNUM (in the main body below) explicitly loops back on

#   a 0 keypress (CMPA $00 / JZ [GETNUM]) - 0 is deliberately excluded

#   even though 0! is mathematically 1, so FACTOR itself is never

#   actually invoked with n=0 by this program. And the range is

#   capped at 5 for a very practical reason: 6! is 720, which does

#   not fit in an 8-bit accumulator (max 255) - this program's whole

#   design, from the keypad range check down to FACTOR's own logic,

#   only works because every factorial from 1! through 5! (1, 2, 6,

#   24, 120) fits comfortably in one byte.

#

#   FMULT itself is not recursive - it's a self-contained 8x8-bit

#   unsigned multiply subroutine, using the classic shift-and-add

#   technique: a 2-byte result register starts as {0, multiplier},

#   and each pass through its main loop tests one bit (via the

#   Carry flag, fed by the previous pass's rotate), conditionally

#   adds the multiplicand into the result's high byte, then rotates

#   the whole 2-byte result right by one bit through Carry -

#   simultaneously shifting the running sum right and shifting the

#   next multiplier bit into position to be tested on the following

#   pass. FMULT's own header comment documents its stack-based

#   calling convention in detail; step through it slowly in the

#   emulator to watch _FM_RES build up one bit at a time.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, switch the Calculator on, and

#   press keys 1 through 5 in turn - the display shows 01, 02, 06,

#   18 ($18 = 24 decimal), and 78 ($78 = 120 decimal), i.e. 1!

#   through 5! in hex. Press any key afterward to clear and try again.

#

# Watch it happen

# -----------------

#   Open Registers and watch the stack pointer drop by 1 with every

#   PUSHA as FACTOR recurses deeper (five drops for input 5), then

#   watch it climb back up as each COMEOUT/FMULT/RTS sequence unwinds

#   one level - Step slowly through a run with input 3 or 4 to see

#   the whole push-down-then-multiply-back-up shape clearly.

#

# Try this next

# ---------------

#   - Trace by hand what stack contents look like at the deepest

#     point of recursion for input 4, before any multiplication has

#     happened yet.

#   - Work out why input 6 isn't offered as a choice, using nothing

#     but the fact that the accumulator is 8 bits wide.

# ================================================================

#######################################################################

## Start of constant declarations                                    ##

#######################################################################

MAINDISP: .EQU     $F031      # Address of output port for main display

SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs

KEYPAD:   .EQU     $F011      # Address of input port for keypad

CLRCODE:  .EQU     $10        # Special code to clear the main display

BINMODE:  .EQU     %00000100  # LED code to indicate binary mode

DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode

HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode

#######################################################################

## End of constant declarations                                      ##

#######################################################################

.ORG     $4000      # Set program origin

#######################################################################

## Start of initialization                                           ##

#######################################################################

INIT:      LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Write clear code to main display

LDA     HEXMODE    # Load accumulator with hex mode code

STA     [SIXLEDS]  # Write to port driving six LEDs

#######################################################################

## End of initialization                                             ##

#######################################################################

#######################################################################

## Start of main program body                                        ##

#######################################################################

BLDSP    $EFFF     # Initialize stack pointer

########## Wait for a key between 1 and 5

GETNUM:    LDA     [KEYPAD]   # Load ACC from keypad

CMPA     $00       # Compare to 0

JZ      [GETNUM]   # Jump back if key = 0

CMPA     $05       # Compare to 5

JC      [GETNUM]   # Jump back if key > 5

JSR     [FACTOR]   #   else call FACTOR subroutine

JSR     [DISPBYTE] #   then display value in ACC

########## Now wait for any key, then clear display and do it again

WAIT:      LDA     [KEYPAD]   # Load ACC from keypad

JN      [WAIT]     # Jump if no key pressed

CLEAR:     LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Write clear code to main display

JMP     [GETNUM]   # Jump back and wait for next number

#######################################################################

## End of main program body                                          ##

#######################################################################

#######################################################################

## Start of subroutines                                              ##

#######################################################################

########## This is the recursive subroutine called FACTOR

FACTOR:    PUSHA              # Push copy of ACC onto stack

CMPA     $01       # Compare ACC to 1

JNC     [COMEOUT]  # If ACC not > 1 start to come out

DECA               #   else decrement ACC

JSR     [FACTOR]   #   then this routine calls itself

COMEOUT:   PUSHA              # Push copy of ACC onto stack

JSR     [FMULT]    # Multiply two bytes on top of stack

POPA               # Retrieve 8-bit result

RTS                # End this instantiation of the routine

########## End of FACTOR

#---------------------------------------------------------------------#

#######################################################################

# Name:     FMULT                                                     #

#                                                                     #

# Function: Multiplies two 8-bit unsigned numbers and returns 8-bit   #

#           unsigned result (MS byte of 2-byte result is discarded.   #

#                                                                     #

# Entry:    Top of stack                                              #

#           Most-significant byte of return address                   #

#           Least-significant byte of return address                  #

#           Second 8-bit number (multiplicand)                        #

#           First 8-bit number (multiplier)                           #

#                                                                     #

# Exit:     Top of stack                                              #

#           Least-significant byte of result (product)                #

#                                                                     #

# Modifies: Accumulator                                               #

#           Index register                                            #

#                                                                     #

# Size:     Program = 63 bytes                                        #

#           Data    =  5 bytes                                        #

#######################################################################

########## Store return address

FMULT:     POPA               # Retrieve MS byte of return

STA  [_FM_RADD]    # address from stack and store it

POPA               # Retrieve LS byte of return

STA  [_FM_RADD+1]  # address from stack and store it

POPA               # Retrieve multiplicand from stack

STA  [_FM_MAND]    # and store it

POPA               # Retrieve multiplier from stack

STA  [_FM_RES+1]   # and store it in LS byte of result

LDA   0            # Load the accumulator with 0 and

STA  [_FM_RES]     # store it in the MS byte of result

########## Perform housekeeping tasks and get ready to multiply

_FM_INIT:  BLDX  9            # Load X reg no. of cycles + 1

ADD   0            # Dummy inst. to set C flag to 0

########## Do the main multiplication loop

_FM_LOOP:  LDA  [_FM_RES]     # Load MS byte of the result

#   This doesn't affect the carry flag

JNC  [_FM_SHFT]    # If carry=0, perform shift

ADD  [_FM_MAND]    #   .. else add the multiplicand

########## Shift (using rotates) 2-byte result 1 bit to the right

_FM_SHFT:  RORC               # Rotate MS byte of result 1-bit right

STA  [_FM_RES]     #   and store it

LDA  [_FM_RES+1]   # Load LS byte of result

RORC               # Rotate LS byte of result 1-bit right.

STA  [_FM_RES+1]   #   and store it

########## Test for end of multiplication

_FM_TST:   DECX               # Decrement the X register

JNZ  [_FM_LOOP]    # Jump if X reg isn't 0

########## Store LS byte of result on stack (discard MS byte)

_FM_SRES:  LDA  [_FM_RES+1]   # Load ACC with LS byte of result

PUSHA              #   and store it on the stack

########## Retrieve return address and exit routine

_FM_GRET:  LDA  [_FM_RADD+1]  # Load ACC with LS return address

PUSHA              #   and store it on the stack

LDA  [_FM_RADD]    # Load ACC with MS return address

PUSHA              #   and store it on the stack

RTS                # That's all folks (exit the routine)

########## Temp storage loactions for thsi routine

_FM_RADD: .2BYTE              # 2-byte location for return address

_FM_MAND: .BYTE               # 1-byte temp location for multiplicand

_FM_RES:  .2BYTE              # 2-byte temp location for result

########## End of FMULT

#---------------------------------------------------------------------#

########## Subroutine to extract/display MS and LS nybbles of byte

DISPBYTE:  PUSHA              # Push a copy of ACC onto stack

########## Display a dollar character

DISPDOL:   LDA   $24          # Load ACC with ASCII code for '$'

STA  [MAINDISP]    # Load ACC with MS return address

########## Extract and display the most-significant nybble

DISPMSN:   POPA               # Retrieve copy of ACC from stack

PUSHA              # And push a copy back for later

SHR                # Shift right 1 bit (= 1 bit shift)

SHR                # Shift right 1 bit (= 2 bit shift)

SHR                # Shift right 1 bit (= 3 bit shift)

SHR                # Shift right 1 bit (= 4 bit shift)

AND     %00001111  # Clear MS 4 bits (not really necessary)

STA     [MAINDISP] # Copy result to main display

########## Extract and display the least-significant nybble

DISPLSN:   POPA               # Pop copy of original byte off stack

AND     %00001111  # Mask out (clear) MS nybble

STA     [MAINDISP] # Copy result to main display

RTS                # Return from subroutine

########## End of DISPBYTE

#---------------------------------------------------------------------#

#######################################################################

## End of subroutines                                                ##

#######################################################################

#######################################################################

## Start of global data                                              ##

#######################################################################

#######################################################################

## End of global data                                                ##

#######################################################################

.END                # That's all folks

### 16q - Recursive String Reversal

tutorial/16q_lab3f_recursive_reverse.asm

# ================================================================

# 16q_lab3f_recursive_reverse.asm

# Beboputer Hands-On Tutorial - Lab 3f (recursive string reversal)

#

# Print a fixed message backward - using a subroutine that calls

# itself once per character, with no array or manual index math.

#

# What this program does

# -----------------------

#   On its own, with no keypad interaction, prints the 9-character

#   message stored in its table, in reverse order, and stops. The

#   table holds "SWAP PAWS" - and because that particular phrase

#   reads the same forwards and backwards (character 0 matches

#   character 8, character 1 matches character 7, and so on, with

#   the space sitting in the exact middle), the reversed output

#   looks identical to the original on the display. That's

#   deliberate: see the note at the end of "How it works" on why

#   watching the registers, not just the display, is how you confirm

#   the reversal is really happening.

#

# How it works

# -------------

#   This is 16m's stack-based reversal trick and 16n/16o's nested

#   subroutines combined into one elegant idea: a subroutine that

#   reverses a message by calling itself, once per character, and

#   letting the call stack itself remember the order to print them

#   back in:

#

#     REVERSE:   LDA     [PHRASE,X] # Load ACC with a character

#                JNZ     [GOIN]     # If it's not NUL jump to GO_IN

#     RETURN1:   RTS                # Otherwise return from subroutine

#

#     GOIN:      PUSHA              # Push the character onto the stack

#                INCX               # Increment the index register

#     INSIDE:    JSR     [REVERSE]  # The subroutine calls itself recursively

#

#     COMEOUT:   POPA               # Pop a character off the stack

#                STA     [MAINDISP] # Copy to main display

#     RETURN2:   RTS                # Return from subroutine

#

#   [PHRASE,X] is the same indexed-addressing message-walking idea

#   Lab 2f's "HELLO WORLD!" uses (exercise 15f) - X starts at 0 and

#   counts up through the table one character at a time. But instead

#   of displaying each character as it's read and moving on, REVERSE

#   pushes the character onto the stack, advances X, and calls

#   itself to go read the next character - before doing anything

#   at all with the one it just read.

#

#   That single JSR [REVERSE] at INSIDE is the whole trick: it means

#   nothing gets displayed until the recursion bottoms out. REVERSE

#   keeps calling itself, pushing one more character each time,

#   until LDA [PHRASE,X] finally loads the table's terminating $00 -

#   at that point JNZ doesn't fire, and this innermost call just

#   returns immediately via RETURN1, without pushing anything.

#

#   From there, every one of the outer calls resumes exactly where it

#   left off - at COMEOUT, right after its own JSR [REVERSE] - and

#   pops back the character it pushed before recursing. Since the

#   stack is Last-In-First-Out (the same property 16m's simpler

#   stack-based reversal relies on), the very last character pushed

#   (the one right before the terminator) is the very first one

#   popped - so it's the first one displayed. Each returning call

#   displays its own character and then returns to whichever call

#   invoked it, one level up, which displays its character next -

#   and so on, all the way back out to the very first call, which

#   displays the very first character in the table last. The message

#   comes out back-to-front with no array, no manually-managed

#   count, and no [STORE-1,X]-style address arithmetic anywhere -

#   the recursive call stack is the reversal mechanism.

#

#   This is the same shape as 16p's recursive FACTOR: descend first

#   (pushing something at each level), hit a base case, then do one

#   small piece of work as each level unwinds on the way back out.

#   FACTOR multiplies on the way out; REVERSE displays on the way out

#   - same underlying pattern, applied to two different problems.

#

#   One more thing worth knowing before running this: PHRASE is

#   declared as two separate .BYTE lines - $53,$57,$41,$50,$20 (that's

#   "SWAP ") followed by $50,$41,$57,$53,$00 (that's "PAWS" plus the

#   NUL terminator) - nine displayable characters in total, forming

#   the single string "SWAP PAWS". Reverse those nine characters

#   (S-W-A-P-space-P-A-W-S) end for end and you get back S-W-A-P-

#   space-P-A-W-S - the exact same sequence. The phrase was chosen

#   as a palindrome on purpose, which is why the display alone can't

#   prove the recursion reversed anything - the "Watch it happen"

#   section below is where the real evidence is.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run - the display shows "SWAP PAWS",

#   which is what the reversed message looks like for this

#   particular (palindromic) phrase - see the note above.

#

# Watch it happen

# -----------------

#   Open Registers and watch the stack pointer drop once per

#   character as REVERSE recurses in (X counting 0, 1, 2, 3, 4 along

#   the way), then watch it rise again as each level's COMEOUT pops

#   and displays its character on the way back out - Step slowly to

#   see the "all the way in, then one at a time coming back out"

#   shape play out in the Registers panel.

#

# Try this next

# ---------------

#   - Change PHRASE to a longer message (remembering the trailing

#     $00 terminator, per Lab 2f's note) and confirm it still reverses

#     correctly, however long it is.

#   - Compare this file directly against 16m: both reverse a

#     sequence using the stack's natural LIFO order, but 16m reads

#     keypad input into the stack in a loop, while this one builds

#     the same stack contents through recursive calls instead.

# ================================================================

#######################################################################

## Start of constant declarations                                    ##

#######################################################################

MAINDISP: .EQU     $F031      # Address of output port for main display

SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs

KEYPAD:   .EQU     $F011      # Address of input port for keypad

CLRCODE:  .EQU     $10        # Special code to clear the main display

BINMODE:  .EQU     %00000100  # LED code to indicate binary mode

DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode

HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode

#######################################################################

## End of constant declarations                                      ##

#######################################################################

.ORG     $4000      # Set program origin

#######################################################################

## Start of initialization                                           ##

#######################################################################

INIT:      LDA     CLRCODE    # Load accumulator with clear code

STA     [MAINDISP] # Write clear code to main display

LDA     HEXMODE    # Load accumulator with hex mode code

STA     [SIXLEDS]  # Write to port driving six LEDs

#######################################################################

## End of initialization                                             ##

#######################################################################

#######################################################################

## Start of main program body                                        ##

#######################################################################

BLDX    0          # Load index register with 0

BLDSP   $EFFF      # Load stack pointer with $EFFF

OUTSIDE:   JSR     [REVERSE]  # Call the subroutine

FINISH:    JMP     [$0000]    # Terminate the program

#######################################################################

## End of main program body                                          ##

#######################################################################

#######################################################################

## Start of subroutines                                              ##

#######################################################################

########## Recursive subroutine to display string in reverse order

REVERSE:   LDA     [PHRASE,X] # Load ACC with a character

JNZ     [GOIN]     # If it's not NUL jump to GO_IN

RETURN1:   RTS                # Otherwise return from subroutine

########## Store the character on the stack and go further in

GOIN:      PUSHA              # Push the character onto the stack

INCX               # Increment the index register

INSIDE:    JSR     [REVERSE]  # The subroutine calls itself recursively

########## Retrieve and display a character and come out

COMEOUT:   POPA               # Pop a character off the stack

STA     [MAINDISP] # Copy to main display

RETURN2:   RTS                # Return from subroutine

#######################################################################

## End of subroutines                                                ##

#######################################################################

#######################################################################

## Start of global data                                              ##

#######################################################################

PHRASE:   .BYTE $53, $57, $41, $50, $20

#      S    W    A    P  SPACE

.BYTE $50, $41, $57, $53, $00

#      P    A    W    S  NUL

#######################################################################

## End of global data                                                ##

#######################################################################

.END

# That's all folks

# Exercise: The Scientific Calculator

## What You Will Build

This tutorial adds five scientific keys to the Calculator — Sin, Cos, Tan, x², and n! — and, unlike exercise 12’s version of Sin/Cos/Tan, stores every typed value as packed BCD (binary-coded decimal) the whole way through, the same representation exercise 13’s four-function calculator uses. Type a two-digit value, press one of the five keys, and read the answer off the display:

4 5  Sin   →  shows  45Sin=0.71

4 5  Cos   →  shows  45Cos=0.71

4 5  Tan   →  shows  45Tan=1.00

9 0  Sin   →  shows  90Sin=1.00

9 0  Tan   →  shows  90Tan=Err   (tangent has no finite value at 90 degrees)

0 4  x²   →  shows  04x^2=16

1 2  x²   →  shows  12x^2=Err   (144 needs 3 digits)

0 4  n!   →  shows  04n!=24

0 5  n!   →  shows  05n!=Err   (120 needs 3 digits)

The source file is tutorial/17_bcd_scientific_calculator.asm. It reuses the keypad state machine from exercises 10 and 13, the trig lookup tables and display formatting from exercise 12, and the DADD-based BCD arithmetic from exercise 13 — combining all three into one program where x² and n! are genuine BCD arithmetic, while Sin, Cos, and Tan stay table lookups dressed up to accept BCD input.

## Before You Start

This tutorial builds directly on:

- 10_calculator_four_function.asm — the keypad state-machine pattern this program reuses, and reading/writing the Calculator’s display port.

- 12_trig_functions_sincostan.asm — the SIN_TABLE/COS_TABLE/TAN_TABLE lookup tables and FORMAT_VAL display routine, carried over here unchanged, plus why Tan’s table stops at 68 degrees.

- 13_bcd_math_routines.asm — packed BCD representation, the DADD/DSUB instructions, and why a BCD counter must be decremented with DSUB $01 instead of DECA.

Read those three tutorial documents first if any of that is unfamiliar — this exercise assumes all of it and spends its own explanation on what’s new: combining a BCD front end with a binary-indexed lookup table, and building x²/n! out of nested BCD addition loops.

## Two Kinds of “Function”: Lookup Table vs. Real BCD Arithmetic

Every scientific key on this calculator produces its answer a different way, and the program is built around that split:

- Sin, Cos, Tan — this CPU cannot compute a sine or cosine out of adds and subtracts at all (see exercise 12’s header for the full reasoning), so the answer comes from a precomputed table, addressed by a plain binary index. The typed value has to leave BCD before it can be used that way.

- x², n! — both are ordinary multiplication in disguise (x·x, and x·(x-1)·(x-2)·…·1), and multiplication is exactly what exercise 13’s DADD-based approach already builds out of repeated BCD addition. These two stay packed BCD from typed input to displayed output — no conversion, no table, ever.

That split is why the program needs a BCD-to-binary converter for one half of its keys and none at all for the other half — not an oversight, but the direct consequence of what each function actually is.

## Program Design

START clears the display before anything else runs. The display isn’t blank by default — it keeps showing whatever the previously run program last wrote — so without this, the exercise could open on a screen full of leftover characters from something else entirely:

START:  LDA     $1B             # clear code - the display isn’t blank on

STA     [DISP]          # its own; it still holds whatever the

# previously run program left showing

LDA     $00

STA     [STAGE]        # begin waiting for the tens digit

Same keypad-polling state machine as exercises 10 and 13, with one extra stage for the function key:

Entry packs the two typed digits into one BCD byte (XVAL) exactly the way exercise 13 packs each operand: SHL ×4 moves the tens digit into the top nibble, then OR merges the ones digit into the bottom nibble — no addition involved, since the digits are already sitting in the right nibble positions once shifted.

ST_TENS:

LDA     [KEYVAL]

AND     $0F              # ASCII ’0’-’9’ -> binary 0-9

SHL

SHL

SHL

SHL                      # move it into the top nibble

STA     [XVAL]

ST_ONES:

LDA     [KEYVAL]

AND     $0F

OR      [XVAL]           # merge with the tens nibble already there

STA     [XVAL]           # XVAL is now one packed-BCD byte, 00-99

## Converting BCD to a Table Index: BCD_TO_BIN

BLDX and the [TABLE,X] addressing mode need an ordinary binary offset — they have no idea XVAL’s nibbles are decimal digits, they just add whatever’s in the Index register to TABLE’s address. So before any trig lookup, BCD_TO_BIN unpacks XVAL’s two nibbles and recombines them the plain-binary way, tens×10 + ones, using the same repeated-ADD-$0A technique exercise 12 used to combine two separately typed digits — just starting from BCD nibbles (via SHR ×4 to isolate the tens digit) instead of two separate keypresses:

BCD_TO_BIN:

LDA     [XVAL]

STA     [TEMP]

SHR

SHR

SHR

SHR                      # tens digit (0-9), now in the low nibble

STA     [TCNT]

LDA     $00

STA     [BINVAL]

JMP     [B2B_TEST]

B2B_LOOP:

LDA     [BINVAL]

ADD     $0A              # +10 per tens digit (plain binary add -

STA     [BINVAL]         # BINVAL itself is never BCD)

LDA     [TCNT]

DECA

STA     [TCNT]

B2B_TEST:

LDA     [TCNT]

CMPA    $00

JNZ     [B2B_LOOP]

LDA     [TEMP]

AND     $0F              # ones digit (0-9)

ADD     [BINVAL]

STA     [BINVAL]         # BINVAL = tens*10+ones, plain binary 0-99

RTS

DO_SIN, DO_COS, and DO_TAN each JSR [BCD_TO_BIN] first, then use BINVAL exactly the way exercise 12 used its plain-binary DEG — range-checked against the table limit ($5A/90 for Sin and Cos, $44/68 for Tan), loaded into the Index register via BLDX, and looked up. SIN_TABLE, COS_TABLE, TAN_TABLE, and FORMAT_VAL are carried over from exercise 12 byte-for-byte, since a lookup table has no “BCD version” to switch to — only the path that gets a binary index into DEGIDX had to change.

## x² and n!: BCD Multiplication by Repeated Addition

x² and n! are the two keys that stay packed BCD end to end, computed with this CPU’s DADD instruction the same way exercise 13 builds a multiply operator: repeated BCD addition, with a BCD countdown decremented by DSUB $01 rather than DECA (see exercise 13’s header for why an ordinary binary decrement would corrupt a BCD counter’s low nibble on the 0→9 rollover).

DO_SQR computes XVAL×XVAL as “add XVAL to a running total, XVAL times”:

DO_SQR: LDA     $00

STA     [RESULT]

LDA     [XVAL]

STA     [MCNT]

JMP     [SQR_TEST]

SQR_LOOP:

LDA     [RESULT]

DADD    [XVAL]

STA     [RESULT]

JC      [ERR_OUT]        # running total spilled past 2 digits

LDA     [MCNT]

DSUB    $01

STA     [MCNT]

SQR_TEST:

LDA     [MCNT]

CMPA    $00

JNZ     [SQR_LOOP]

JMP     [SHOW_BCD_RESULT]

DO_FAC computes XVAL×(XVAL-1)×…×1 the same way, one level deeper: FCNT counts down from XVAL to 1, and each pass multiplies RESULT by FCNT using the identical repeated-DADD technique as an inner loop — FICNT is that inner loop’s own countdown, and FMUL_ACC holds the value being repeatedly added, since RESULT itself is rebuilt from $00 as the inner loop runs and so can’t also be the thing being added to itself:

FAC_INNER:

LDA     [FICNT]

CMPA    $00

JZ      [FAC_INNER_DONE]

LDA     [RESULT]

DADD    [FMUL_ACC]

STA     [RESULT]

JC      [ERR_OUT]         # running total spilled past 2 digits

LDA     [FICNT]

DSUB    $01

STA     [FICNT]

JMP     [FAC_INNER]

Both routines share one more consequence of staying in BCD: the Carry flag after a DADD means an honest “this digit pair overflowed 99,” so a single JC [ERR_OUT] after every DADD is enough to catch a result that needs a third digit — no separate range check needed, unlike Sin/Cos/Tan's explicit CMPA-against-the-table-limit up front. And because RESULT is already packed BCD, the result routine (SHOW_BCD_RESULT) needs no conversion loop at all — just split the nibbles and print each as a digit, the same shortcut exercise 13 uses to display its own answers.

## Where Each Key’s Range Stops, and Why

In every case, an out-of-range or overflowed result shows Err rather than a silently wrong or truncated answer — the same convention exercises 10, 12, and 13 all use for divide-by-zero and out-of-table inputs. ECHO_FUNC prints the two- or three-letter name of whichever key was pressed (“Sin,” “Cos,” “Tan,” “x^2,” or “n!”) before every result or Err, so the display always reads as a complete expression, e.g. 90Tan=Err rather than a bare =Err.

## Verified Against the Real CPU Emulator

This program was assembled with the project’s own compiler (bin/compiler_core.py — the same back end the Calculator’s Assemble / Load → CPU pipeline uses) and the resulting .ram image was loaded and run against the project’s own CPU class with simulated keypresses — not just the .asm source read and assumed correct, but the actual compiled bytecode the app loads at $4000. Every test below passed against that compiled image:

All 13 cases passed. A full breakdown (including Accumulator and flag state after each run) is in Tutorial17_BCD_Scientific_Calculator_Test_Report.xlsx alongside this document. Clear and CE were also checked directly against the emulator: Clear/CE mid-entry correctly wipes a partly-typed value, and pressing a fresh digit while an answer is showing correctly clears the display and starts the next entry.

## Try It Yourself

Assemble tutorial/17_bcd_scientific_calculator.asm, Load → CPU, click Run. Type two digits (e.g. 4 5), then press Sin, Cos, Tan, x², or n!. Try the overflow cases above (12 x², 05 n!, 90 Tan) to see the Err path, and try Clear or CE mid-entry to confirm a fresh value starts cleanly.

Open Display → Port Map Status alongside the exercise to see $F011 receive each digit and function code, and Registers to watch the Carry flag after every DADD in DO_SQR/DO_FAC — that’s the overflow test firing, exactly the moment a result needs a third digit.

## Ideas to Extend This Program

- Add Log and 1/x. Log has no cheap BCD trick either — another lookup table, using the same BCD_TO_BIN conversion this program already does for Sin/Cos/Tan. 1/x needs a genuine fractional result representation this program doesn’t have yet.

- Extend x²’s range past two digits using DADDC to chain a second BCD byte, the same extension exercise 13 suggests for its four operators.

- Add x³ by JSR-ing into DO_SQR's pattern twice, or generalize DO_SQR/DO_FAC into one “repeat DADD N times” subroutine shared by both.

## Appendix: Complete Source Listing

tutorial/17_bcd_scientific_calculator.asm

# ================================================================

# 17_bcd_scientific_calculator.asm

# Beboputer Hands-On Tutorial — Bonus Exercise (Calculator)

#

# A small scientific calculator: type a two-digit value, press Sin,

# Cos, Tan, x^2, or n!, and read the answer off the display. Unlike

# exercise 12 (which stored everything in plain binary), the typed

# value here is packed BCD throughout, the same representation

# exercise 13's four-function calculator uses - x^2 and n! are

# genuine BCD arithmetic (DADD, the same way exercise 13 multiplies),

# not table lookups.

#

# What this program does

# -----------------------

#   Type a two-digit value (always two digits - a leading zero for

#   single-digit values, e.g. 05), then press one of the five

#   scientific keys:

#       4  5   Sin    ->  shows  45Sin=0.71

#       4  5   Cos    ->  shows  45Cos=0.71

#       4  5   Tan    ->  shows  45Tan=1.00

#       9  0   Sin    ->  shows  90Sin=1.00

#       9  0   Tan    ->  shows  90Tan=Err   (out of range - see below)

#       0  4   x^2    ->  shows  04x^2=16

#       1  2   x^2    ->  shows  12x^2=Err   (144 needs 3 digits)

#       0  4   n!     ->  shows  04n!=24

#       0  5   n!     ->  shows  05n!=Err    (120 needs 3 digits)

#   Sin and Cos accept 00-90. Tan accepts 00-68 (see exercise 12 for

#   why tan's table has to stop there - the same limit applies here).

#   x^2 and n! both top out well before 99 in packed BCD (two digits,

#   00-99 only) - anything whose true answer needs a third digit

#   shows Err instead, the same convention exercise 13 uses for its

#   four operators.

#

# Why x^2 and n! are "real" BCD arithmetic, but Sin/Cos/Tan aren't

# -------------------------------------------------------------------

#   x^2 and n! are computed with this CPU's DADD instruction, exactly

#   the way exercise 13 builds multiply out of repeated BCD addition -

#   x^2 is just "add X to a running total, X times" (X*X), and n! is

#   "multiply the running total by X, X-1, X-2, ... down to 1", where

#   each of those multiplies is itself a repeated-BCD-addition loop.

#   Every intermediate value stays packed BCD from typed input to

#   displayed output - see DO_SQR and DO_FAC below.

#

#   Sine, cosine, and tangent can't be built out of adds and

#   subtracts at all (see exercise 12's header for the full

#   explanation) - the only way this CPU produces them is the same

#   precomputed lookup table exercise 12 uses, addressed by a plain

#   binary index. Since the typed value is stored as packed BCD here

#   (not plain binary, like exercise 12), BCD_TO_BIN below converts

#   it to a binary 0-99 index before every trig lookup - the table

#   itself, and the FORMAT_VAL display routine that turns a table

#   entry into "W.FF" on screen, are carried over from exercise 12

#   unchanged, since a lookup table has no "BCD version" to switch to.

#

# How it works

# -------------

#   Same keypad-polling state machine as exercises 10-13:

#

#     STAGE 0   waiting for the tens digit of X

#     STAGE 1   waiting for the ones digit of X

#     STAGE 2   waiting for Sin, Cos, Tan, x^2, or n!

#     STAGE 3   answer is showing - next digit clears and restarts

#

#   Entry packs the two typed digits into one BCD byte (XVAL) exactly

#   the way exercise 13 packs each operand - SHL x4 moves the tens

#   digit into the top nibble, then OR merges the ones digit into the

#   bottom nibble.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run. START clears the display first

#   (the display isn't blank by default - it still shows whatever the

#   last program left there), so the exercise always begins on a clean

#   screen no matter what ran before it. Type two digits, then press

#   Sin, Cos, Tan, x^2, or n!. Try the overflow cases above to see the

#   Err path, and try Clear/CE mid-entry to confirm a fresh sum starts

#   cleanly.

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status to see $F011 receive each digit

#   and function code, and Registers to watch the Carry flag after

#   each DADD in DO_SQR/DO_FAC - that's the overflow test firing.

#

# Try this next

# ---------------

#   - Add Log and 1/x. Log has no cheap BCD trick either (another

#     lookup table, same BCD_TO_BIN conversion this program already

#     does for Sin/Cos/Tan); 1/x needs a genuine fractional result

#     representation this program doesn't have yet.

#   - Extend x^2's range past two digits using DADDC to chain a

#     second BCD byte, the same way exercise 13's "Try this next"

#     suggests for its four operators.

# ================================================================

KEY:      .EQU    $F011

DISP:     .EQU    $F031

SIN_KEY:  .EQU    $3A

COS_KEY:  .EQU    $39

TAN_KEY:  .EQU    $38

SQR_KEY:  .EQU    $3D        # x^2

FAC_KEY:  .EQU    $36        # n!

.ORG    $4000

START:  LDA     $1B             # clear code - the display isn't blank on

STA     [DISP]          # its own; it still holds whatever the

# previously run program left showing

LDA     $00

STA     [STAGE]        # begin waiting for the tens digit

# ---------------------------------------------------------------

# Main loop - poll the keypad, dispatch on STAGE

# ---------------------------------------------------------------

WAIT:   LDA     [KEY]

CMPA    $FF             # $FF = idle, no key waiting

JZ      [WAIT]

STA     [KEYVAL]        # stash it - this is the ONLY read of KEY

CMPA    $10             # Clear key

JZ      [DO_CLEAR]

CMPA    $11             # CE key

JZ      [DO_CLEAR]

LDA     [STAGE]

CMPA    $00

JZ      [ST_TENS]

CMPA    $01

JZ      [ST_ONES]

CMPA    $03

JZ      [ST_NEWCALC]

JMP     [ST_FUNC]        # only remaining value is STAGE 2

# ---------------------------------------------------------------

# STAGE 3 -> 0 : answer was showing, this digit starts a new value

# ---------------------------------------------------------------

ST_NEWCALC:

LDA     $1B              # clear code - wipes the old expression

STA     [DISP]

JMP     [ST_TENS]

# ---------------------------------------------------------------

# STAGE 0 : tens digit -> top nibble of XVAL (packed BCD)

# ---------------------------------------------------------------

ST_TENS:

LDA     [KEYVAL]

STA     [DISP]           # echo the digit as typed

LDA     [KEYVAL]

AND     $0F              # ASCII '0'-'9' -> binary 0-9

SHL

SHL

SHL

SHL                      # move it into the top nibble

STA     [XVAL]

LDA     $01

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 1 : ones digit -> bottom nibble of XVAL

# ---------------------------------------------------------------

ST_ONES:

LDA     [KEYVAL]

STA     [DISP]

LDA     [KEYVAL]

AND     $0F

OR      [XVAL]           # merge with the tens nibble already there

STA     [XVAL]           # XVAL is now one packed-BCD byte, 00-99

LDA     $02

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 2 : waiting for Sin, Cos, Tan, x^2, or n! - ignore anything else

# ---------------------------------------------------------------

ST_FUNC:

LDA     [KEYVAL]

STA     [FUNCKEY]         # remembered for the display echo below

CMPA    SIN_KEY

JZ      [DO_SIN]

CMPA    COS_KEY

JZ      [DO_COS]

CMPA    TAN_KEY

JZ      [DO_TAN]

CMPA    SQR_KEY

JZ      [DO_SQR]

CMPA    FAC_KEY

JZ      [DO_FAC]

JMP     [WAIT]

# ---------------------------------------------------------------

# BCD_TO_BIN - convert packed-BCD XVAL into a plain-binary 0-99

# value in BINVAL, for use as a table index (BLDX/[TABLE,X] need an

# ordinary binary offset, not a packed-BCD byte). Same tens*10+ones

# combine exercise 12 uses, just starting from BCD nibbles instead

# of two separately-typed digits.

# ---------------------------------------------------------------

BCD_TO_BIN:

LDA     [XVAL]

STA     [TEMP]

SHR

SHR

SHR

SHR                      # tens digit (0-9), now in the low nibble

STA     [TCNT]

LDA     $00

STA     [BINVAL]

JMP     [B2B_TEST]

B2B_LOOP:

LDA     [BINVAL]

ADD     $0A              # +10 per tens digit (plain binary add -

STA     [BINVAL]         # BINVAL itself is never BCD)

LDA     [TCNT]

DECA

STA     [TCNT]

B2B_TEST:

LDA     [TCNT]

CMPA    $00

JNZ     [B2B_LOOP]

LDA     [TEMP]

AND     $0F              # ones digit (0-9)

ADD     [BINVAL]

STA     [BINVAL]         # BINVAL = tens*10+ones, plain binary 0-99

RTS

# ---------------------------------------------------------------

# Sin and Cos: valid for XVAL 00-90. Look up SIN_TABLE[BINVAL]

# (scaled x100, plain binary - same table format as exercise 12) and

# format it with FORMAT_VAL.

# ---------------------------------------------------------------

DO_SIN: JSR     [BCD_TO_BIN]

LDA     [BINVAL]

CMPA    $5A              # 90 decimal

JC      [ERR_OUT]        # BINVAL > 90: outside this table

LDA     $00

STA     [DEGIDX]

LDA     [BINVAL]

STA     [DEGIDX+1]

BLDX    [DEGIDX]

LDA     [SIN_TABLE,X]

STA     [VAL]

JSR     [FORMAT_VAL]

JMP     [RESET_STATE]

DO_COS: JSR     [BCD_TO_BIN]

LDA     [BINVAL]

CMPA    $5A              # 90 decimal

JC      [ERR_OUT]

LDA     $00

STA     [DEGIDX]

LDA     [BINVAL]

STA     [DEGIDX+1]

BLDX    [DEGIDX]

LDA     [COS_TABLE,X]

STA     [VAL]

JSR     [FORMAT_VAL]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Tan: valid for XVAL 00-68 only - same table-size limit as

# exercise 12 (tan(69)x100 = 261, too big for one byte).

# ---------------------------------------------------------------

DO_TAN: JSR     [BCD_TO_BIN]

LDA     [BINVAL]

CMPA    $44              # 68 decimal

JC      [ERR_OUT]

LDA     $00

STA     [DEGIDX]

LDA     [BINVAL]

STA     [DEGIDX+1]

BLDX    [DEGIDX]

LDA     [TAN_TABLE,X]

STA     [VAL]

JSR     [FORMAT_VAL]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# x^2:  RESULT = XVAL * XVAL, in packed BCD, via XVAL repeated BCD

# additions of XVAL - exactly exercise 13's DO_MUL pattern with both

# operands equal to XVAL. MCNT is itself packed BCD (it started as

# XVAL), so it counts down with DSUB $01, not DECA - see exercise

# 13's header note for why.

# ---------------------------------------------------------------

DO_SQR: LDA     $00

STA     [RESULT]

LDA     [XVAL]

STA     [MCNT]

JMP     [SQR_TEST]

SQR_LOOP:

LDA     [RESULT]

DADD    [XVAL]

STA     [RESULT]

JC      [ERR_OUT]        # running total spilled past 2 digits

LDA     [MCNT]

DSUB    $01

STA     [MCNT]

SQR_TEST:

LDA     [MCNT]

CMPA    $00

JNZ     [SQR_LOOP]

JMP     [SHOW_BCD_RESULT]

# ---------------------------------------------------------------

# n!:  RESULT = XVAL * (XVAL-1) * (XVAL-2) * ... * 1, in packed BCD.

#

# FCNT counts down from XVAL to 1 (BCD, DSUB $01). Each pass

# multiplies RESULT by FCNT the same way DO_SQR multiplies - by

# repeated BCD addition - using FICNT as that inner loop's own

# countdown and FMUL_ACC to hold the value being repeatedly added

# (RESULT itself is rebuilt from $00 as the inner loop runs, so it

# can't also be the thing being added).

# 0! and 1! both correctly come out as 1 (RESULT starts at 1 and the

# loop never subtracts below it for XVAL 0 or 1).

# ---------------------------------------------------------------

DO_FAC: LDA     $01

STA     [RESULT]         # 0! = 1! = 1

LDA     [XVAL]

CMPA    $00

JZ      [SHOW_BCD_RESULT]

LDA     $01

CMPA    [XVAL]

JZ      [SHOW_BCD_RESULT] # XVAL == 1: RESULT already 1, done

LDA     [XVAL]

STA     [FCNT]

FAC_TEST:

LDA     [FCNT]

CMPA    $00

JZ      [SHOW_BCD_RESULT]

LDA     [RESULT]

STA     [FMUL_ACC]        # value to add, repeatedly, below

LDA     $00

STA     [RESULT]          # rebuilt as FCNT copies of FMUL_ACC

LDA     [FCNT]

STA     [FICNT]

FAC_INNER:

LDA     [FICNT]

CMPA    $00

JZ      [FAC_INNER_DONE]

LDA     [RESULT]

DADD    [FMUL_ACC]

STA     [RESULT]

JC      [ERR_OUT]         # running total spilled past 2 digits

LDA     [FICNT]

DSUB    $01

STA     [FICNT]

JMP     [FAC_INNER]

FAC_INNER_DONE:

LDA     [FCNT]

DSUB    $01

STA     [FCNT]

JMP     [FAC_TEST]

# ---------------------------------------------------------------

# Out-of-range or overflowed result for whichever function was pressed

# ---------------------------------------------------------------

ERR_OUT:

JSR     [ECHO_FUNC]

LDA     $3D              # '='

STA     [DISP]

LDA     $45              # 'E'

STA     [DISP]

LDA     $72              # 'r'

STA     [DISP]

LDA     $72              # 'r'

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# ECHO_FUNC - print the two/three-letter name of whichever

# scientific key was pressed (FUNCKEY), so the expression on screen

# reads e.g. "45Sin=0.71" instead of just "45=0.71". Called once,

# from FORMAT_VAL/SHOW_BCD_RESULT's shared entry points below.

# ---------------------------------------------------------------

ECHO_FUNC:

LDA     [FUNCKEY]

CMPA    SIN_KEY

JZ      [EF_SIN]

CMPA    COS_KEY

JZ      [EF_COS]

CMPA    TAN_KEY

JZ      [EF_TAN]

CMPA    SQR_KEY

JZ      [EF_SQR]

JMP     [EF_FAC]         # only remaining key is n!

EF_SIN: LDA     $53              # 'S'

STA     [DISP]

LDA     $69              # 'i'

STA     [DISP]

LDA     $6E              # 'n'

STA     [DISP]

RTS

EF_COS: LDA     $43              # 'C'

STA     [DISP]

LDA     $6F              # 'o'

STA     [DISP]

LDA     $73              # 's'

STA     [DISP]

RTS

EF_TAN: LDA     $54              # 'T'

STA     [DISP]

LDA     $61              # 'a'

STA     [DISP]

LDA     $6E              # 'n'

STA     [DISP]

RTS

EF_SQR: LDA     $78              # 'x'

STA     [DISP]

LDA     $5E              # '^'

STA     [DISP]

LDA     $32              # '2'

STA     [DISP]

RTS

EF_FAC: LDA     $6E              # 'n'

STA     [DISP]

LDA     $21              # '!'

STA     [DISP]

RTS

# ---------------------------------------------------------------

# FORMAT_VAL - shared by DO_SIN/DO_COS/DO_TAN via JSR/RTS. Carried

# over from exercise 12 unchanged (see the header note on why the

# trig table/display path has no BCD version).

#

# Turns VAL (0-255, meaning 0.00-2.55) into "Fn=W.FF" on the display

# (Fn from ECHO_FUNC, then '=', then the value):

#   1. Subtract 100 from VAL while it still fits - the count (0-2)

#      is the single whole-number digit.

#   2. Subtract 10 from what's left while it still fits - the count

#      (0-9) is the tenths digit; whatever remains (0-9) is the

#      hundredths digit.

# ---------------------------------------------------------------

FORMAT_VAL:

JSR     [ECHO_FUNC]

LDA     $3D              # '='

STA     [DISP]

LDA     $00

STA     [WHOLE]

FV_100_LOOP:

LDA     [VAL]

CMPA    $64              # 100

JC      [FV_100_SUB]     # C=1: VAL > 100

JZ      [FV_100_SUB]     # Z=1: VAL == 100

JMP     [FV_100_DONE]    # VAL < 100, done

FV_100_SUB:

LDA     [VAL]

SUB     $64

STA     [VAL]

LDA     [WHOLE]

INCA

STA     [WHOLE]

JMP     [FV_100_LOOP]

FV_100_DONE:

LDA     [WHOLE]          # single digit, 0-2

OR      $30

STA     [DISP]

LDA     $2E              # '.'

STA     [DISP]

LDA     $00

STA     [FTENS]

FV_10_LOOP:

LDA     [VAL]

CMPA    $0A

JC      [FV_10_SUB]      # C=1: VAL > 10

JZ      [FV_10_SUB]      # Z=1: VAL == 10

JMP     [FV_10_DONE]     # VAL < 10, done

FV_10_SUB:

LDA     [VAL]

SUB     $0A

STA     [VAL]

LDA     [FTENS]

INCA

STA     [FTENS]

JMP     [FV_10_LOOP]

FV_10_DONE:

LDA     [FTENS]          # tenths digit - always shown, no

OR      $30              # leading-zero suppression, so the

STA     [DISP]           # fraction always prints as 2 digits

LDA     [VAL]            # hundredths digit, whatever is left

OR      $30

STA     [DISP]

RTS

# ---------------------------------------------------------------

# SHOW_BCD_RESULT - shared by DO_SQR/DO_FAC. RESULT is already

# packed BCD (00-99), so unlike FORMAT_VAL this needs no conversion

# loop at all - just split the nibbles (see exercise 13's header

# note for the full explanation of why packed BCD skips the loop).

# ---------------------------------------------------------------

SHOW_BCD_RESULT:

JSR     [ECHO_FUNC]

LDA     $3D              # '='

STA     [DISP]

LDA     [RESULT]

STA     [TEMP]           # keep a copy - SHR below destroys ACC

SHR

SHR

SHR

SHR                      # tens digit, 0-9, now in the low nibble

CMPA    $00

JZ      [SBR_ONES]       # suppress a leading zero on the tens digit

OR      $30

STA     [DISP]

SBR_ONES:

LDA     [TEMP]

AND     $0F              # ones digit

OR      $30

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Explicit Clear / CE handling

# ---------------------------------------------------------------

DO_CLEAR:

LDA     $1B

STA     [DISP]

LDA     $00

STA     [STAGE]

JMP     [WAIT]

RESET_STATE:

LDA     $03              # STAGE 3: next digit clears & restarts

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# Variables (reserved bytes - see exercise 09 for this pattern)

# ---------------------------------------------------------------

STAGE:    .BYTE

XVAL:     .BYTE       # packed BCD, 00-99, the typed value

KEYVAL:   .BYTE

FUNCKEY:  .BYTE

TEMP:     .BYTE

TCNT:     .BYTE

BINVAL:   .BYTE       # XVAL converted to plain binary, for table indexing

DEGIDX:   .2BYTE

VAL:      .BYTE       # trig table result, binary fixed-point x100

WHOLE:    .BYTE

FTENS:    .BYTE

RESULT:   .BYTE       # packed BCD, x^2/n! result

MCNT:     .BYTE

FCNT:     .BYTE

FICNT:    .BYTE

FMUL_ACC: .BYTE

# ---------------------------------------------------------------

# SIN_TABLE[d] = round(sin(d degrees) x 100), d = 0..90

# (identical to exercise 12's table - see that exercise for how it

# was generated)

# ---------------------------------------------------------------

SIN_TABLE:

.BYTE   $00, $02, $03, $05, $07, $09, $0A, $0C, $0E, $10

.BYTE   $11, $13, $15, $16, $18, $1A, $1C, $1D, $1F, $21

.BYTE   $22, $24, $25, $27, $29, $2A, $2C, $2D, $2F, $30

.BYTE   $32, $34, $35, $36, $38, $39, $3B, $3C, $3E, $3F

.BYTE   $40, $42, $43, $44, $45, $47, $48, $49, $4A, $4B

.BYTE   $4D, $4E, $4F, $50, $51, $52, $53, $54, $55, $56

.BYTE   $57, $57, $58, $59, $5A, $5B, $5B, $5C, $5D, $5D

.BYTE   $5E, $5F, $5F, $60, $60, $61, $61, $61, $62, $62

.BYTE   $62, $63, $63, $63, $63, $64, $64, $64, $64, $64

.BYTE   $64

# ---------------------------------------------------------------

# COS_TABLE[d] = round(cos(d degrees) x 100), d = 0..90

# ---------------------------------------------------------------

COS_TABLE:

.BYTE   $64, $64, $64, $64, $64, $64, $63, $63, $63, $63

.BYTE   $62, $62, $62, $61, $61, $61, $60, $60, $5F, $5F

.BYTE   $5E, $5D, $5D, $5C, $5B, $5B, $5A, $59, $58, $57

.BYTE   $57, $56, $55, $54, $53, $52, $51, $50, $4F, $4E

.BYTE   $4D, $4B, $4A, $49, $48, $47, $45, $44, $43, $42

.BYTE   $40, $3F, $3E, $3C, $3B, $39, $38, $36, $35, $34

.BYTE   $32, $30, $2F, $2D, $2C, $2A, $29, $27, $25, $24

.BYTE   $22, $21, $1F, $1D, $1C, $1A, $18, $16, $15, $13

.BYTE   $11, $10, $0E, $0C, $0A, $09, $07, $05, $03, $02

.BYTE   $00

# ---------------------------------------------------------------

# TAN_TABLE[d] = round(tan(d degrees) x 100), d = 0..68 only - see

# exercise 12's header note: tan(69)x100 = 261, too big for one byte.

# ---------------------------------------------------------------

TAN_TABLE:

.BYTE   $00, $02, $03, $05, $07, $09, $0B, $0C, $0E, $10

.BYTE   $12, $13, $15, $17, $19, $1B, $1D, $1F, $20, $22

.BYTE   $24, $26, $28, $2A, $2D, $2F, $31, $33, $35, $37

.BYTE   $3A, $3C, $3E, $41, $43, $46, $49, $4B, $4E, $51

.BYTE   $54, $57, $5A, $5D, $61, $64, $68, $6B, $6F, $73

.BYTE   $77, $7B, $80, $85, $8A, $8F, $94, $9A, $A0, $A6

.BYTE   $AD, $B4, $BC, $C4, $CD, $D6, $E1, $EC, $F8

.END    $4000

# Appendix A: Full Port and Instruction Reference

Every memory-mapped port used across the four exercises in this tutorial, in one place.

Table 12 is the full port reference for the emulator.

Table 12: Full Port Reference

| Port | Device | Direction / Notes |
| --- | --- | --- |
| $F000 | Workbench Switch Bank 1 | Input - 8 toggle switches, bit 7 = leftmost |
| $F001 | Workbench Switch Bank 2 | Input - 8 toggle switches, bit 7 = leftmost |
| $F011 | Keypad (Calculator keys AND on-screen Keyboard) | Input - last key code; idle value is $FF; auto-clears to $FF the instant your program reads a non-$FF value |
| $F021 | Workbench 7-Segment (undecoded) | Output - bits 0-6 drive segments a-g directly |
| $F022 | Workbench 8-Bit LED Bar | Output - one bit per LED |
| $F023 | Workbench 7-Segment (decoded) | Output - low nibble (0-F) selects the digit shape |
| $F024 | Workbench Dual 7-Segment (decoded) | Output - high nibble = left digit, low nibble = right digit |
| $F028 | Terminal | Output - ASCII 32-126 prints the character; $0A prints a newline; everything else is ignored |
| $F031 | Calculator Display | Output - ASCII 32-126 appends to the display; $0D, $10, $1B clear it |
| $F032 | Calculator LEDs | Output - bit 5 = leftmost LED ... bit 0 = rightmost LED |

## The $F011 Keypad Port Has Two Personalities

Both the Calculator and the on-screen Keyboard write to the same port, but they write different kinds of values:

Table 13 explains the two personalities of the $F011 keypad port.

Table 13: $F011 Keypad Port Behaviour

| Source | What lands in $F011 |
| --- | --- |
| Calculator digit keys (0-9) | Raw nibble 0-9 (translated from the button's ASCII code before writing) |
| Calculator hex keys (A-F) | Raw nibble 10-15 (translated from the button's ASCII code before writing) |
| Calculator operator/function keys (+, -, =, Clear, ...) | The button's own configured code, unmodified |
| On-screen Keyboard letters, digits, punctuation | Standard ASCII value, unmodified |
| On-screen Keyboard ENTER / BSpace / arrows | Fixed control codes ($05, $04, $07-$0A) - not standard ASCII |

Because of this, a single polling loop cannot reliably tell which physical control sent a value just by looking at the byte. Each exercise in this tutorial is written for one input device at a time to keep the examples unambiguous - Exercise 2 assumes the Calculator’s digit/hex keys, Exercise 3 assumes the on-screen Keyboard.

## Full Instruction Set Reference

Every instruction implemented by the CPU, grouped by function. Opcodes match Appendix A, Tables A-2a/A-2b of The Official DIY Calculator Data Book. Addressing-mode abbreviations (imm, abs, abs-x, ind, x-ind, ind-x) are the same ones used in the Assembler Syntax Quick Reference; — marks instructions that take no operand.

### Data Transfer

Table 14 lists the data transfer instructions.

Table 14: Data Transfer Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| LDA | imm $90 / abs $91 / abs-x $92 / ind $93 / x-ind $94 / ind-x $95 | Load the Accumulator from an immediate value or from memory, using any of the five addressing modes. Sets the Negative and Zero flags from the loaded value. |
| STA | abs $99 / abs-x $9A / ind $9B / x-ind $9C / ind-x $9D | Store the Accumulator to memory. No immediate form (there is nowhere to store to). |
| BLDX | imm $A0 / abs $A1 | Load the 16-bit Index register (IX), either from a 16-bit immediate value or from a RAM address. |
| BSTX | abs $A9 | Store the 16-bit Index register (IX) to a RAM address. |
| BLDSP | imm $50 / abs $51 | Load the 16-bit Stack Pointer (SP), either from a 16-bit immediate value or from a RAM address. |
| BSTSP | abs $59 | Store the 16-bit Stack Pointer (SP) to a RAM address. |
| BLDIV | imm $F0 / abs $F1 | Load the 16-bit Interrupt Vector, either from a 16-bit immediate value or from a RAM address. |

### Arithmetic

Table 15 lists the arithmetic instructions.

Table 15: Arithmetic Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| ADD | imm $10 / abs $11 / abs-x $12 | Add a value to the Accumulator. |
| ADDC | imm $18 / abs $19 / abs-x $1A | Add a value to the Accumulator together with the Carry flag, for chaining multi-byte addition. |
| SUB | imm $20 / abs $21 / abs-x $22 | Subtract a value from the Accumulator. |
| SUBC | imm $28 / abs $29 / abs-x $2A | Subtract a value from the Accumulator together with the Carry flag (as a borrow), for chaining multi-byte subtraction. |
| CMPA | imm $60 / abs $61 / abs-x $62 | Compare the Accumulator with a value; sets flags only, the Accumulator is unchanged. Carry is set only when the Accumulator is strictly greater than the operand. |
| INCA | — $80 | Increment the Accumulator by 1. |
| DECA | — $81 | Decrement the Accumulator by 1. |
| INCX | — $82 | Increment the Index register (IX) by 1. |
| DECX | — $83 | Decrement the Index register (IX) by 1. |

### BCD (Decimal) Arithmetic

Table 16 lists the BCD (decimal) arithmetic instructions.

Table 16: BCD (Decimal) Arithmetic Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| DADD | imm $48 / abs $49 / abs-x $4A | Add two BCD (binary-coded decimal) values, producing a correctly-adjusted decimal result. |
| DADDC | imm $68 / abs $69 / abs-x $6A | Add two BCD values together with the Carry flag, for chaining multi-byte decimal addition. |
| DSUB | imm $88 / abs $89 / abs-x $8A | Subtract two BCD values, producing a correctly-adjusted decimal result. |
| DSUBC | imm $B8 / abs $B9 / abs-x $BA | Subtract two BCD values together with the Carry flag (as a borrow), for chaining multi-byte decimal subtraction. |

### Logical (Bitwise)

Table 17 lists the logical (bitwise) instructions.

Table 17: Logical (Bitwise) Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| AND | imm $30 / abs $31 / abs-x $32 | Bitwise AND a value with the Accumulator. |
| OR | imm $38 / abs $39 / abs-x $3A | Bitwise OR a value with the Accumulator. |
| XOR | imm $40 / abs $41 / abs-x $42 | Bitwise XOR a value with the Accumulator. |

### Shift and Rotate

Table 18 lists the shift and rotate instructions.

Table 18: Shift and Rotate Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| SHL | — $70 | Shift the Accumulator left one bit. Bit 7 moves into Carry; bit 0 fills with 0. |
| SHR | — $71 | Shift the Accumulator right one bit. Bit 0 moves into Carry; bit 7 fills with 0. |
| ROLC | — $78 | Rotate the Accumulator left through Carry (a 9-bit rotate including the Carry flag). |
| RORC | — $79 | Rotate the Accumulator right through Carry (a 9-bit rotate including the Carry flag). |

### Stack Operations

Table 19 lists the stack operation instructions.

Table 19: Stack Operation Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| PSHA / PUSHA | — $B2 | Push the Accumulator onto the stack. PUSHA is an alias for PSHA. |
| POPA | — $B0 | Pop the top of the stack into the Accumulator. |
| PUSHSR | — $B3 | Push the Status Register (flags) onto the stack. |
| POPSR | — $B1 | Pop the top of the stack into the Status Register (flags). |

### Program Control — Jumps and Subroutines

Table 20 lists the jump and subroutine instructions.

Table 20: Jump and Subroutine Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| JMP | abs $C1 / abs-x $C2 / ind $C3 / x-ind $C4 / ind-x $C5 | Unconditional jump to a new address. |
| JSR | abs $C9 / abs-x $CA / ind $CB / x-ind $CC / ind-x $CD | Jump to a subroutine; pushes the return address onto the stack first. |
| RTS | — $CF | Return from a subroutine; pops the return address off the stack. |
| RTI | — $C7 | Return from an interrupt; restores the flags and Program Counter from the stack. |

### Program Control — Conditional Jumps

Table 21 lists the conditional jump instructions.

Table 21: Conditional Jump Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| JZ | abs $D1 | Jump if the Zero flag is set. |
| JNZ | abs $D6 | Jump if the Zero flag is clear. |
| JN | abs $D9 | Jump if the Negative flag is set. |
| JNN | abs $DE | Jump if the Negative flag is clear. |
| JC | abs $E1 | Jump if the Carry flag is set. |
| JNC | abs $E6 | Jump if the Carry flag is clear. |
| JO | abs $E9 | Jump if the Overflow flag is set. |
| JNO | abs $EE | Jump if the Overflow flag is clear. |

### Interrupt Control

Table 22 lists the interrupt control instructions.

Table 22: Interrupt Control Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| SETIM | — $08 | Set the Interrupt Mask bit, disabling maskable interrupts. |
| CLRIM | — $09 | Clear the Interrupt Mask bit, enabling maskable interrupts. |

### System

Table 23 lists the system instructions.

Table 23: System Instructions

| Mnemonic | Modes (Opcode) | Description |
| --- | --- | --- |
| NOP | — $00 | No operation; advances the Program Counter only. |
| HALT | — $01 | Stop the CPU. A Reset is required to resume execution. |

# Appendix B: Troubleshooting

- Program does not seem to run at all: check that the Calculator is powered on (its On/Off button) - Run, Step, and Reset all silently do nothing while it is off.

- My program vanished after I turned the Calculator on: powering on purges RAM. Always power on first, then Assemble and Load -> CPU.

- Assemble succeeded but nothing happens on Run: remember Assemble does not load the program by itself - click Load -> CPU afterward. The button is disabled until assembly succeeds.

- Reloading a second program does not clear the display/LEDs from the first one: Load -> CPU clears the program area in RAM (including the I/O port bytes) before writing your new bytes, but the on-screen displays only repaint when your program writes to them again - so a stale LED or 7-segment value can stay on screen until your new program overwrites that port.

- A key doesn’t seem to register: check WAIT loops correctly test for the $FF idle value with CMPA $FF and JZ - if the loop condition is inverted (JNZ instead of JZ, or vice-versa) it will either spin forever or fall straight through without waiting. The I/O Ports Display’s $F011 box (current + previous value) is the fastest way to confirm whether a keypress actually reached the port.

- The Terminal or Workbench displays stay blank even though my program is clearly running: confirm the panel is actually open (Display -> Terminal / Tools -> Workbench 1) and that the Calculator is powered on - both devices silently discard writes while powered off.

- Everything runs too fast to follow: lower the clock speed with Setup -> System Clock... (1 Hz is easiest for tracing a loop one pass at a time; the default is 100 Hz).

- Not sure what a program actually assembled to: use the Memory Walker’s GO field to jump to the address in question and read the DATA column directly, or double-click a DATA cell to poke a test value in by hand.

# Appendix C: ASCII Character Set

Standard 7-bit ASCII, codes $00-$7F (0-127). Codes 0-31 and 127 are non-printing control codes shown by their standard mnemonic (e.g. LF, ESC) rather than a glyph; code 32 is the printable space character, shown here as SPACE.

The emulator’s I/O ports only recognise a handful of these codes directly: the Terminal (port $F028, Appendix A) prints ASCII 32-126 as-is and treats $0A (LF) as a newline, ignoring everything else; the Calculator’s display (port $F031) appends ASCII 32-126 and clears on $0D (CR), $10, or $1B (ESC). All other control codes are accepted by ports that expect a raw byte (e.g. the Workbench’s 7-segment outputs) but have no special meaning to the emulator itself.

Table 24 is the full ASCII character set reference.

Table 24: ASCII Character Set

| Dec | Hex | Char | Dec | Hex | Char | Dec | Hex | Char | Dec | Hex | Char |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | $00 | NUL | 32 | $20 | SPACE | 64 | $40 | @ | 96 | $60 | ` |
| 1 | $01 | SOH | 33 | $21 | ! | 65 | $41 | A | 97 | $61 | a |
| 2 | $02 | STX | 34 | $22 | " | 66 | $42 | B | 98 | $62 | b |
| 3 | $03 | ETX | 35 | $23 | # | 67 | $43 | C | 99 | $63 | c |
| 4 | $04 | EOT | 36 | $24 | $ | 68 | $44 | D | 100 | $64 | d |
| 5 | $05 | ENQ | 37 | $25 | % | 69 | $45 | E | 101 | $65 | e |
| 6 | $06 | ACK | 38 | $26 | & | 70 | $46 | F | 102 | $66 | f |
| 7 | $07 | BEL | 39 | $27 | ' | 71 | $47 | G | 103 | $67 | g |
| 8 | $08 | BS | 40 | $28 | ( | 72 | $48 | H | 104 | $68 | h |
| 9 | $09 | TAB | 41 | $29 | ) | 73 | $49 | I | 105 | $69 | i |
| 10 | $0A | LF | 42 | $2A | * | 74 | $4A | J | 106 | $6A | j |
| 11 | $0B | VT | 43 | $2B | + | 75 | $4B | K | 107 | $6B | k |
| 12 | $0C | FF | 44 | $2C | , | 76 | $4C | L | 108 | $6C | l |
| 13 | $0D | CR | 45 | $2D | - | 77 | $4D | M | 109 | $6D | m |
| 14 | $0E | SO | 46 | $2E | . | 78 | $4E | N | 110 | $6E | n |
| 15 | $0F | SI | 47 | $2F | / | 79 | $4F | O | 111 | $6F | o |
| 16 | $10 | DLE | 48 | $30 | 0 | 80 | $50 | P | 112 | $70 | p |
| 17 | $11 | DC1 | 49 | $31 | 1 | 81 | $51 | Q | 113 | $71 | q |
| 18 | $12 | DC2 | 50 | $32 | 2 | 82 | $52 | R | 114 | $72 | r |
| 19 | $13 | DC3 | 51 | $33 | 3 | 83 | $53 | S | 115 | $73 | s |
| 20 | $14 | DC4 | 52 | $34 | 4 | 84 | $54 | T | 116 | $74 | t |
| 21 | $15 | NAK | 53 | $35 | 5 | 85 | $55 | U | 117 | $75 | u |
| 22 | $16 | SYN | 54 | $36 | 6 | 86 | $56 | V | 118 | $76 | v |
| 23 | $17 | ETB | 55 | $37 | 7 | 87 | $57 | W | 119 | $77 | w |
| 24 | $18 | CAN | 56 | $38 | 8 | 88 | $58 | X | 120 | $78 | x |
| 25 | $19 | EM | 57 | $39 | 9 | 89 | $59 | Y | 121 | $79 | y |
| 26 | $1A | SUB | 58 | $3A | : | 90 | $5A | Z | 122 | $7A | z |
| 27 | $1B | ESC | 59 | $3B | ; | 91 | $5B | [ | 123 | $7B | { |
| 28 | $1C | FS | 60 | $3C | < | 92 | $5C | \ | 124 | $7C | ¦ |
| 29 | $1D | GS | 61 | $3D | = | 93 | $5D | ] | 125 | $7D | } |
| 30 | $1E | RS | 62 | $3E | > | 94 | $5E | ^ | 126 | $7E | ~ |
| 31 | $1F | US | 63 | $3F | ? | 95 | $5F | _ | 127 | $7F | DEL |