# Basic BCD Math Routines: +, -, *, /

A Beboputer / PY-DIYCALCULATOR assembly-language tutorial

# 1. What You Will Build

A 4-function calculator, just like exercise 10, except every number in this version is stored as packed BCD (binary-coded decimal) instead of plain binary. Type two-digit operands, an operator, and ‘=’, and read the answer off the display:

2  3  +  1  4  =        ->  shows  23+14=37

0  9  /  0  3  =        ->  shows  09/03=3

0  5  -  1  2  =        ->  shows  05-12=Err   (negative result)

6  0  *  6  0  =        ->  shows  60*60=Err   (3600 needs 3 digits)

9  9  /  0  0  =        ->  shows  99/00=Err   (divide by zero)

The source file is tutorial/13_bcd_math_routines.asm. It reuses the keypad state machine from exercise 10 almost unchanged, but replaces every ADD/SUB with this CPU’s dedicated BCD instructions — DADD, DADDC, DSUB, DSUBC — and, as a direct result, gets to delete the decimal-conversion loop every earlier tutorial needed just to put a number on the screen.

# 2. Before You Start

This tutorial builds directly on:

- 10_calculator_four_function.asm — the keypad state-machine pattern this program reuses almost line for line, and the Carry-flag quirk of plain SUB/CMPA (see Section 5 for why BCD is different).

- 12_trig_functions_sincostan.asm — using SHL/SHR to move bits into position, reused here to pack/unpack nibbles instead of indexing a table.

# 3. Packed BCD: Two Digits, One Byte, No Conversion

Exercises 10 through 12 all stored numbers in plain binary, and all three paid for it at display time: turning a binary total back into decimal digits took a repeated-subtraction loop — subtract 10, or 100, again and again, counting how many times — because a binary byte has no idea where one decimal digit ends and the next begins.

Packed BCD avoids the conversion by never losing the digit boundary in the first place. Each byte holds exactly two decimal digits, one per nibble: the high nibble is the tens digit, the low nibble is the ones digit, and both are always 0–9. The number 47 is stored as the byte $47 — not because 47 happens to equal 0x47 in binary (that is a coincidence of how hex digits are written down), but because the nibble 4 and the nibble 7 are exactly the digits ‘47’ is made of. Reading a BCD byte back out is just splitting it into its two nibbles and turning each into an ASCII character — no loop, no subtraction, no math at all.

The catch is that ordinary ADD/SUB do not know about this packing. Adding $47 and $08 in plain binary gives $4F — a nibble of 15, not a valid decimal digit — because binary addition has no concept of “carry at 10” the way decimal addition does. This CPU has four dedicated instructions for exactly this problem:

| Instruction | What it does |
| --- | --- |
| DADD | Adds two packed-BCD bytes, carrying 1 into the next nibble whenever a digit would overflow past 9. |
| DADDC | Same as DADD, but also adds in the Carry flag — for chaining multi-byte BCD numbers together. |
| DSUB | Subtracts two packed-BCD bytes, borrowing 1 from the next nibble whenever a digit would go below 0. |
| DSUBC | Same as DSUB, but also subtracts the Carry flag — for chaining multi-byte BCD subtraction. |

$47 DADD $08 correctly gives $55 (55), not the broken $4F plain ADD would produce.

# 4. Program Design

Same keypad-polling state machine as exercise 10, just with a second digit per operand:

| STAGE | The program is... |
| --- | --- |
| 0 | Waiting for the tens digit of operand 1. |
| 1 | Waiting for the ones digit of operand 1. |
| 2 | Waiting for an operator (+ − × ÷). |
| 3 | Waiting for the tens digit of operand 2. |
| 4 | Waiting for the ones digit of operand 2. |
| 5 | Waiting for ‘=’ (or Enter). |
| 6 | Showing an answer — the next digit clears the display and starts a new one. |

Packing two typed digits into one BCD byte needs no multiply or decimal-composition trick (contrast exercise 12’s TENS×10+ONES) — a nibble is a fixed 4-bit-wide slot, so the tens digit just needs shifting into the top half with SHL, and the ones digit ORs straight into the bottom half:

LDA   [KEYVAL]

AND   $0F            # ASCII digit -> binary 0-9

SHL

SHL

SHL

SHL                  # 0-9 moved into the top nibble, bottom = 0

STA   [OP1]          # tens digit stored

...

LDA   [KEYVAL]

AND   $0F

OR    [OP1]          # ones digit merges into the bottom nibble

STA   [OP1]          # OP1 is now a single packed-BCD byte

# 5. Subtraction: DSUB’s Carry Means the Opposite of SUB/CMPA’s

This is the one flag detail worth slowing down for, because it is the exact reverse of the rule exercises 10–12 hammer home. Plain SUB/CMPA set Carry when the accumulator is strictly greater than the operand — not the usual “a borrow was needed.” DSUB does not share that quirk: its Carry means an honest, conventional “yes, a borrow was needed” — which for a single subtraction is exactly “the left-hand side was smaller than the right-hand side.” Subtracting 12 from 5 in BCD (05 DSUB 12) correctly sets Carry, because 5 is less than 12; subtracting 5 from 5 correctly leaves Carry clear, because no borrow was needed to reach zero.

That means — unlike every three-way CMPA/JC/JZ dance in exercises 10–12 — a single JC after DSUB is already the complete, correct test:

DO_SUB: LDA     [OP1]

DSUB    [OP2]

STA     [RESULT]

JC      [ERR_OUT]   # a genuine borrow: OP1 was < OP2

JMP     [SHOW_RESULT]

This program treats a borrow as “can’t show it” (Err) rather than negating the result — see Section 9 for what a full negative-number version would need.

# 6. Addition and the Overflow Check

DADD does the packed add and reports, via Carry, whether the true two-digit result needed a third digit — i.e. the sum was 100 or more, which two BCD digits cannot hold:

DO_ADD: LDA     [OP1]

DADD    [OP2]

STA     [RESULT]

JC      [ERR_OUT]   # sum >= 100 - doesn't fit in 2 digits

JMP     [SHOW_RESULT]

# 7. Multiply, and Why the Loop Counter Needs DSUB Too

Multiply is still repeated addition, exactly like exercise 10 — add OP1 to a running RESULT, OP2 times, using DADD instead of ADD so the running total stays valid packed BCD, with the same overflow check Section 6 uses:

MUL_LOOP:

LDA     [RESULT]

DADD    [OP1]

STA     [RESULT]

JC      [ERR_OUT]    # running total spilled past 2 digits

LDA     [MCNT]

DSUB    $01          # NOT DECA - see below

STA     [MCNT]

MUL_TEST:

LDA     [MCNT]

CMPA    $00

JNZ     [MUL_LOOP]

MCNT counts down from OP2 to 0, one loop per multiplication. It is tempting to count down with plain DECA, the way every counter in exercises 09–12 did — but MCNT holds a packed BCD value here (it started as OP2), and DECA is a plain binary decrement with no idea that a nibble stops at 9. $30 (BCD 30) decremented with DECA gives $2F — not $29 — because binary decrement borrows across the whole byte instead of stopping at the nibble boundary, leaving an invalid digit (F) sitting in the ones place. DSUB $01 decrements the correct, BCD-aware way, so 30 counts down 30, 29, 28, …, 1, 0 exactly as expected. Any loop counter built out of a BCD value needs DSUB (or DADD, counting up) for this same reason — plain INCA/DECA are only safe on counters that were never BCD to begin with.

# 8. Divide: DSUB Doubles as Both the Test and the Subtraction

Divide is repeated subtraction again, but DSUB’s honest borrow flag lets the loop skip the separate CMPA “does it still fit?” check exercise 10 needed. DSUB itself is the test: try the subtraction, and only keep the result if no borrow was needed:

DIV_LOOP:

LDA     [DREM]

DSUB    [OP2]

JC      [DIV_DONE]   # borrowed - DREM was < OP2, stop

STA     [DREM]       # no borrow - the subtraction stands

LDA     [RESULT]

DADD    $01          # quotient += 1 (BCD)

STA     [RESULT]

JMP     [DIV_LOOP]

DIV_DONE:

When DSUB does borrow, its result is a meaningless wrapped value (exactly like subtracting 5−8 by hand and refusing to borrow — the digits come out wrong), so it is simply never stored; DREM is left exactly as it was before that attempt, which is correct, since that attempt should not have happened.

# 9. Displaying a Packed BCD Byte: No Loop Needed At All

This is the payoff. RESULT already holds two correct decimal digits, one per nibble, so showing it is just isolating each nibble and turning it into ASCII with OR $30 — the tens digit needs shifting down into position first (SHR four times moves the top nibble into the bottom, undoing the SHL packing done at entry):

LDA     [RESULT]

STA     [TEMP]         # keep a copy - the shifts below destroy it

SHR

SHR

SHR

SHR                    # tens digit, 0-9, now in the low nibble

CMPA    $00

JZ      [SR_ONES]      # suppress a leading zero, same as exercise 10

OR      $30

STA     [DISP]

SR_ONES:

LDA     [TEMP]

AND     $0F            # ones digit

OR      $30

STA     [DISP]

Compare this to exercise 10’s SHOW_RESULT, which needed a whole repeated-subtraction TENS_LOOP to get the same two digits out of a plain binary RESULT. That loop is gone here — not simplified, just genuinely unnecessary, because the digits were never lost in the first place.

# 10. Verified Against the Real CPU Emulator

This program was assembled with the project’s own das.py and run against the project’s own CPU class with simulated keypresses — not just read and assumed correct. 62 cases were checked directly, deliberately clustered around every boundary: exact sums up to and past 99, exact and negative subtractions, multiplications that just fit and just overflow, and both exact and truncating divisions, plus divide-by-zero. A representative sample:

| Expression | Result | Why |
| --- | --- | --- |
| 23+14= | 37 | ordinary case |
| 50+50= | Err | sum is exactly 100 — needs a third digit |
| 50-49= | 1 | ordinary case (leading zero suppressed, same as exercise 10) |
| 05-12= | Err | would be negative — DSUB's Carry catches it directly |
| 60*60= | Err | 3600 needs 3 digits, nowhere close to fitting |
| 09/03= | 3 | exact division |
| 23/04= | 5 | integer quotient only, remainder 3 discarded |
| 99/00= | Err | divide by zero |

Behavior around Clear, CE, Enter, and re-entry was also checked directly against the emulator: Clear mid-entry correctly wipes a partly-typed operand; CE after an answer correctly starts a new calculation; Enter works identically to ‘=’; typing digit “1” in an operand is read correctly and not confused with CE (CE's own code was moved to $7F in an earlier fix specifically so it can never collide with a raw digit — see the 4-Function Calculator tutorial); and pressing a fresh digit while an answer is showing correctly clears the display and starts the next entry.

# 11. Try It Yourself

Assemble tutorial/13_bcd_math_routines.asm, Load → CPU, click Run. On the Calculator, type two digits, an operator, two more digits, then ‘=’ (or Enter). Try 09/03 to see exact division, 05-12 to see the borrow/Err path, 99/00 for divide-by-zero, and 60*60 to see the overflow Err path.

Open Display → Port Map Status alongside the exercise to watch each keypress arrive on $F011, and Registers to watch the Carry flag flip after each DADD/DSUB — notice it behaves like an ordinary “did this operation overflow/borrow?” flag here, the opposite of plain SUB/CMPA’s quirky “strictly greater than.”

# 12. Ideas to Extend This Program

- Add a NEG flag and a ‘−’ sign, the way exercise 10 handles a negative result, so 05-12 shows -07 instead of Err. You will need to compute OP2 DSUB OP1 instead when the first attempt borrows, exactly mirroring exercise 10’s DO_SUB.

- Extend RESULT to two packed-BCD bytes (four digits, 0000–9999) using DADDC/DSUBC to chain the carry/borrow between them — the BCD equivalent of exercise 11’s 16-bit RESULT/RESULT+1.

- Show the division remainder (DREM already holds it when DIV_DONE is reached) alongside the quotient.

# Appendix: Complete Source Listing

tutorial/13_bcd_math_routines.asm

# ================================================================

# 13_bcd_math_routines.asm

# Beboputer Hands-On Tutorial — Bonus Exercise (Calculator)

#

# A 4-function (+ - * /) calculator built on packed BCD instead of

# plain binary - same idea as exercise 10, but every number in this

# program is stored the way this CPU's DADD/DSUB instructions expect:

# two decimal digits packed into one byte, one digit per nibble.

#

# What this program does

# -----------------------

#   Same look and feel as exercise 10, just with two-digit operands

#   (always type both digits - a leading zero for single-digit

#   values, e.g. 05):

#       2  3  +  1  4  =        ->  shows  23+14=37

#       0  9  /  0  3  =        ->  shows  09/03=03

#       0  5  -  1  2  =        ->  shows  05-12=Err       (negative - see "Limitations")

#       6  0  *  6  0  =        ->  shows  60*60=Err       (3600 needs 3 digits - see "Limitations")

#       9  9  /  0  0  =        ->  shows  99/00=Err

#

# Limitations (kept simple on purpose - see "Try this next")

# ------------------------------------------------------------

#   - Operands are two BCD digits only: 00-99.

#   - A result that would need a third digit (100 or more) shows

#     Err instead of wrapping or truncating - see "Packed BCD"

#     below for why this program has no easy way to show it.

#   - Subtraction shows Err instead of a negative number - there is

#     no packed-BCD "minus sign" convention used here (see "Try this

#     next" for how a real calculator would extend this).

#   - Division shows the integer quotient only (no remainder).

#

# Packed BCD: two decimal digits, one byte, no conversion needed

# -----------------------------------------------------------------

#   Every previous exercise stored numbers in plain binary and paid

#   for it at display time: exercises 10-12 all needed a repeated-

#   subtraction loop (subtract 10, or 100, again and again, counting

#   how many times) just to turn a binary total back into decimal

#   digits, because a binary byte has no idea where one decimal

#   digit ends and the next begins.

#

#   Packed BCD fixes that by never converting in the first place.

#   Each byte holds exactly two decimal digits, one per nibble - the

#   high nibble is the tens digit, the low nibble is the ones digit,

#   both always 0-9. The number 47 is stored as the single byte

#   $47 - not because 47 happens to equal 0x47 in binary (that is a

#   coincidence of how hex digits are written), but because the

#   nibble 4 and the nibble 7 are exactly the digits "47" is made of.

#   Reading a BCD byte back out is just splitting it into its two

#   nibbles and turning each into an ASCII character - no loop, no

#   subtraction, no math at all. SHOW_RESULT below is a third the

#   size of exercise 10's SHOW_RESULT for exactly this reason.

#

#   The catch is that ordinary ADD/SUB do not understand this

#   packing - ADD $47,$08 in plain binary gives $4F (a nibble of 15,

#   not a valid digit), because binary addition has no concept of

#   "carry at 10" the way decimal addition does. This CPU has four

#   extra instructions - DADD, DADDC, DSUB, DSUBC - that add and

#   subtract packed BCD bytes correctly: whenever a nibble would

#   overflow past 9, they carry (or borrow) 1 into the other nibble

#   automatically, the same way you carry a 1 when adding 47+8 by

#   hand. $47 DADD $08 correctly gives $55 (55), not $4F.

#

# How it works

# -------------

#   Same keypad-polling state machine as exercise 10, just with a

#   second digit per operand:

#

#     STAGE 0   waiting for the tens digit of operand 1

#     STAGE 1   waiting for the ones digit of operand 1

#     STAGE 2   waiting for an operator      (+ - * /)

#     STAGE 3   waiting for the tens digit of operand 2

#     STAGE 4   waiting for the ones digit of operand 2

#     STAGE 5   waiting for '='  (or Enter)

#     STAGE 6   answer is showing - the next digit starts a new sum

#

#   Packing two typed digits into one BCD byte needs no multiply or

#   decimal-composition trick (contrast exercise 12's TENS*10+ONES) -

#   a nibble is a fixed 4-bit-wide slot, so the tens digit just needs

#   shifting into the top half with SHL, and the ones digit ORs

#   straight into the bottom half:

#

#     LDA   [KEYVAL]

#     AND   $0F            # ASCII digit -> binary 0-9

#     SHL

#     SHL

#     SHL

#     SHL                  # 0-9 moved into the top nibble, bottom = 0

#     STA   [OP1]          # tens digit stored

#     ...

#     LDA   [KEYVAL]

#     AND   $0F

#     OR    [OP1]          # ones digit merges into the bottom nibble

#     STA   [OP1]          # OP1 is now a single packed-BCD byte

#

# Addition and the overflow check

# ---------------------------------

#   DADD does the packed add and reports, via Carry, whether the

#   true two-digit result needed a third digit (i.e. the sum was

#   100 or more - two BCD digits can only hold 00-99):

#

#     DO_ADD: LDA     [OP1]

#             DADD    [OP2]

#             STA     [RESULT]

#             JC      [ERR_OUT]   # sum >= 100 - doesn't fit in 2 digits

#             JMP     [SHOW_RESULT]

#

# Subtraction: DSUB's Carry means the OPPOSITE of SUB/CMPA's

# ------------------------------------------------------------------

#   This is the one flag detail worth slowing down for, because it

#   is the exact reverse of the rule exercises 10-12 hammer home.

#   Plain SUB/CMPA set Carry when the accumulator is STRICTLY

#   GREATER THAN the operand - not the usual "a borrow was needed."

#   DSUB does NOT share that quirk: its Carry means an honest,

#   conventional "yes, a borrow was needed" - which for a single

#   subtraction is exactly "the left-hand side was smaller than the

#   right-hand side." Subtracting 12 from 5 in BCD (05 DSUB 12)

#   correctly sets Carry, because 5 is less than 12; subtracting 5

#   from 5 (an equal case) correctly leaves Carry clear, because no

#   borrow was needed to reach zero. That means - unlike every

#   three-way CMPA/JC/JZ dance in exercises 10-12 - a single JC after

#   DSUB is already the complete, correct test:

#

#     DO_SUB: LDA     [OP1]

#             DSUB    [OP2]

#             STA     [RESULT]

#             JC      [ERR_OUT]   # a genuine borrow: OP1 was < OP2

#             JMP     [SHOW_RESULT]

#

#   (This program treats that as "can't show it" rather than

#   negating it - see "Try this next" for what a full negative-

#   number version would need.)

#

# Multiply, and why the loop counter needs DSUB too

# ----------------------------------------------------

#   Multiply is still repeated addition, exactly like exercise 10 -

#   add OP1 to a running RESULT, OP2 times, using DADD instead of

#   ADD so the running total stays valid packed BCD, with the same

#   overflow check DO_ADD uses:

#

#     MUL_LOOP:

#             LDA     [RESULT]

#             DADD    [OP1]

#             STA     [RESULT]

#             JC      [ERR_OUT]    # running total spilled past 2 digits

#             LDA     [MCNT]

#             DSUB    $01          # NOT DECA - see below

#             STA     [MCNT]

#     MUL_TEST:

#             LDA     [MCNT]

#             CMPA    $00

#             JNZ     [MUL_LOOP]

#

#   MCNT counts down from OP2 to 0, one loop per multiplication. It

#   is tempting to count down with plain DECA, the way every counter

#   in exercises 09-12 did - but MCNT holds a packed BCD value here

#   (it started as OP2), and DECA is a plain binary decrement with no

#   idea that a nibble stops at 9. $30 (BCD 30) decremented with DECA

#   gives $2F - not $29 - because binary decrement borrows across the

#   whole byte instead of stopping at the nibble boundary, leaving an

#   invalid digit (F) sitting in the ones place. DSUB $01 decrements

#   the correct, BCD-aware way, so 30 counts down 30, 29, 28, ...,

#   1, 0 exactly as expected. Any loop counter built out of a BCD

#   value needs DSUB (or DADD, counting up) for this same reason -

#   plain INCA/DECA are only safe on counters that were never BCD to

#   begin with (like exercise 10's MCNT, which counted a plain binary

#   digit 0-9 and never crossed a nibble boundary).

#

# Divide: DSUB doubles as both the test and the subtraction

# ---------------------------------------------------------------

#   Divide is repeated subtraction again, but DSUB's honest borrow

#   flag lets the loop skip the separate CMPA "does it still fit?"

#   check exercise 10 needed. DSUB itself IS the test: try the

#   subtraction, and only keep the result if no borrow was needed:

#

#     DIV_LOOP:

#             LDA     [DREM]

#             DSUB    [OP2]

#             JC      [DIV_DONE]   # borrowed - DREM was < OP2, stop

#             STA     [DREM]       # no borrow - the subtraction stands

#             LDA     [RESULT]

#             DADD    $01          # quotient += 1 (BCD)

#             STA     [RESULT]

#             JMP     [DIV_LOOP]

#     DIV_DONE:

#

#   When DSUB does borrow, its result is a meaningless wrapped value

#   (exactly like subtracting 5-8 by hand and refusing to borrow -

#   the digits come out wrong) - so it is simply never stored; DREM

#   is left exactly as it was before that attempt, which is correct,

#   since that attempt should not have happened.

#

# Displaying a packed BCD byte: no loop needed at all

# -------------------------------------------------------

#   This is the payoff mentioned up top. RESULT already holds two

#   correct decimal digits, one per nibble, so showing it is just

#   isolating each nibble and turning it into ASCII with OR $30 -

#   the tens digit needs shifting down into position first (SHR four

#   times moves the top nibble into the bottom, filling the vacated

#   top bits with 0, exactly undoing the SHL packing done at entry):

#

#     LDA     [RESULT]

#     STA     [TEMP]         # keep a copy - the shifts below destroy it

#     SHR

#     SHR

#     SHR

#     SHR                    # tens digit, 0-9, now in the low nibble

#     CMPA    $00

#     JZ      [SR_ONES]      # suppress a leading zero, same as exercise 10

#     OR      $30

#     STA     [DISP]

#     SR_ONES:

#     LDA     [TEMP]

#     AND     $0F            # ones digit

#     OR      $30

#     STA     [DISP]

#

#   Compare this to exercise 10's SHOW_RESULT, which needed a whole

#   repeated-subtraction TENS_LOOP to get the same two digits out of

#   a plain binary RESULT. That loop is gone here - not simplified,

#   just genuinely unnecessary, because the digits were never lost

#   in the first place.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, then on the Calculator press:

#   two digits, an operator, two digits, then '=' (or Enter). Try

#   09/03=03 to see exact division, 05-12 to see the borrow/Err path,

#   99/00 for divide-by-zero, and something like 60*60 to see the

#   overflow Err path (60*60=3600, needs four digits - nowhere close

#   to fitting in one packed-BCD byte).

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status alongside this exercise to watch

#   each keypress arrive on $F011, and Registers to watch the Carry

#   flag flip after each DADD/DSUB - notice it behaves like an

#   ordinary "did this operation overflow/borrow?" flag here, the

#   opposite of plain SUB/CMPA's quirky "strictly greater than".

#

# Try this next

# ---------------

#   - Add a NEG flag and a '-' sign, the way exercise 10 handles a

#     negative result, so 05-12 shows -07 instead of Err. You will

#     need to compute OP2 DSUB OP1 instead when the first attempt

#     borrows, exactly mirroring exercise 10's DO_SUB.

#   - Extend RESULT to two packed-BCD bytes (four digits, 0000-9999)

#     using DADDC/DSUBC to chain the carry/borrow between them, the

#     BCD equivalent of exercise 11's 16-bit RESULT/RESULT+1.

#   - Show the division remainder (DREM already holds it when

#     DIV_DONE is reached) alongside the quotient.

# ================================================================

KEY:     .EQU    $F011

DISP:    .EQU    $F031

.ORG    $4000

START:  LDA     $00

STA     [STAGE]        # begin waiting for operand 1's tens digit

# ---------------------------------------------------------------

# Main loop - poll the keypad, dispatch on STAGE

# ---------------------------------------------------------------

WAIT:   LDA     [KEY]

CMPA    $FF             # $FF = idle, no key waiting

JZ      [WAIT]

STA     [KEYVAL]        # stash it - this is the ONLY read of KEY

CMPA    $1B             # Clear key

JZ      [DO_CLEAR]

CMPA    $7F             # CE key

JZ      [DO_CLEAR]

LDA     [STAGE]

CMPA    $00

JZ      [ST_OP1_TENS]

CMPA    $01

JZ      [ST_OP1_ONES]

CMPA    $02

JZ      [ST_OPCODE]

CMPA    $03

JZ      [ST_OP2_TENS]

CMPA    $04

JZ      [ST_OP2_ONES]

CMPA    $06

JZ      [ST_NEWCALC]

JMP     [ST_EQUALS]      # only remaining value is STAGE 5

# ---------------------------------------------------------------

# STAGE 6 -> 0 : answer was showing, this digit starts a new sum

# ---------------------------------------------------------------

ST_NEWCALC:

LDA     $1B              # clear code - wipes the old expression

STA     [DISP]

JMP     [ST_OP1_TENS]

# ---------------------------------------------------------------

# STAGE 0 : operand 1, tens digit -> top nibble of OP1

# ---------------------------------------------------------------

ST_OP1_TENS:

LDA     [KEYVAL]

STA     [DISP]           # echo the digit as typed

LDA     [KEYVAL]

AND     $0F              # ASCII '0'-'9' -> binary 0-9

SHL

SHL

SHL

SHL                      # move it into the top nibble

STA     [OP1]

LDA     $01

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 1 : operand 1, ones digit -> bottom nibble of OP1

# ---------------------------------------------------------------

ST_OP1_ONES:

LDA     [KEYVAL]

STA     [DISP]

LDA     [KEYVAL]

AND     $0F

OR      [OP1]            # merge with the tens nibble already there

STA     [OP1]            # OP1 is now one packed-BCD byte, 00-99

LDA     $02

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 2 : operator  (+ - * /)

# ---------------------------------------------------------------

ST_OPCODE:

LDA     [KEYVAL]

STA     [OPCODE]         # remember which operator was pressed

STA     [DISP]           # echo it

LDA     $03

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 3 : operand 2, tens digit -> top nibble of OP2

# ---------------------------------------------------------------

ST_OP2_TENS:

LDA     [KEYVAL]

STA     [DISP]

LDA     [KEYVAL]

AND     $0F

SHL

SHL

SHL

SHL

STA     [OP2]

LDA     $04

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 4 : operand 2, ones digit -> bottom nibble of OP2

# ---------------------------------------------------------------

ST_OP2_ONES:

LDA     [KEYVAL]

STA     [DISP]

LDA     [KEYVAL]

AND     $0F

OR      [OP2]

STA     [OP2]

LDA     $05

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 5 : waiting for '=' (or Enter) - ignore anything else

# ---------------------------------------------------------------

ST_EQUALS:

LDA     [KEYVAL]

CMPA    $3D              # '='

JZ      [DO_EQUALS]

CMPA    $0D              # Enter

JZ      [DO_EQUALS]

JMP     [WAIT]

DO_EQUALS:

LDA     $3D

STA     [DISP]           # echo the '=' sign

LDA     [OPCODE]

CMPA    $2B              # '+'

JZ      [DO_ADD]

CMPA    $2D              # '-'

JZ      [DO_SUB]

CMPA    $2A              # '*'

JZ      [DO_MUL]

CMPA    $2F              # '/'

JZ      [DO_DIV]

JMP     [WAIT]           # (shouldn't happen)

# ---------------------------------------------------------------

# Addition:  RESULT = OP1 + OP2 (packed BCD). Carry after DADD means

# the true sum needed a third digit (>= 100) - too big to show here.

# ---------------------------------------------------------------

DO_ADD: LDA     [OP1]

DADD    [OP2]

STA     [RESULT]

JC      [ERR_OUT]

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Subtraction:  RESULT = OP1 - OP2 (packed BCD). Unlike plain

# SUB/CMPA, DSUB's Carry means an ordinary "a borrow was needed" -

# OP1 was strictly less than OP2 - so one JC is the whole test (see

# the header note for the full explanation of why this is the

# opposite convention from exercises 10-12).

# ---------------------------------------------------------------

DO_SUB: LDA     [OP1]

DSUB    [OP2]

STA     [RESULT]

JC      [ERR_OUT]

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Multiply:  RESULT = OP1 * OP2, via OP2 repeated BCD additions of

# OP1. MCNT (the loop countdown) is itself a packed-BCD value, so it

# must count down with DSUB $01, not DECA - see the header note.

# ---------------------------------------------------------------

DO_MUL: LDA     $00

STA     [RESULT]

LDA     [OP2]

STA     [MCNT]

JMP     [MUL_TEST]

MUL_LOOP:

LDA     [RESULT]

DADD    [OP1]

STA     [RESULT]

JC      [ERR_OUT]        # running total spilled past 2 digits

LDA     [MCNT]

DSUB    $01              # BCD decrement - NOT DECA

STA     [MCNT]

MUL_TEST:

LDA     [MCNT]

CMPA    $00

JNZ     [MUL_LOOP]

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Divide:  RESULT = OP1 / OP2 (integer quotient), via repeated BCD

# subtraction of OP2 from a running remainder. OP2=0 -> Err. DSUB's

# own Carry flag doubles as the "does it still fit?" test - see the

# header note for why no separate CMPA is needed here.

# ---------------------------------------------------------------

DO_DIV: LDA     [OP2]

CMPA    $00

JZ      [ERR_OUT]

LDA     $00

STA     [RESULT]         # quotient, BCD, counts up

LDA     [OP1]

STA     [DREM]           # remainder, BCD, counts down

DIV_LOOP:

LDA     [DREM]

DSUB    [OP2]

JC      [DIV_DONE]       # borrowed - DREM < OP2, stop here

STA     [DREM]           # no borrow - the subtraction stands

LDA     [RESULT]

DADD    $01              # quotient += 1 (BCD)

STA     [RESULT]

JMP     [DIV_LOOP]

DIV_DONE:

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Out-of-range result for whichever operator was pressed

# ---------------------------------------------------------------

ERR_OUT:

LDA     $45              # 'E'

STA     [DISP]

LDA     $72              # 'r'

STA     [DISP]

LDA     $72              # 'r'

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Show RESULT: a packed-BCD byte needs no conversion loop, just a

# nibble split - see the header note for the full explanation.

# ---------------------------------------------------------------

SHOW_RESULT:

LDA     [RESULT]

STA     [TEMP]           # keep a copy - SHR below destroys ACC

SHR

SHR

SHR

SHR                      # tens digit, 0-9, now in the low nibble

CMPA    $00

JZ      [SR_ONES]        # suppress a leading zero on the tens digit

OR      $30

STA     [DISP]

SR_ONES:

LDA     [TEMP]

AND     $0F              # ones digit

OR      $30

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Explicit Clear / CE handling

# ---------------------------------------------------------------

DO_CLEAR:

LDA     $1B

STA     [DISP]

LDA     $00

STA     [STAGE]

JMP     [WAIT]

RESET_STATE:

LDA     $06              # STAGE 6: next digit clears & restarts

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# Variables (reserved bytes - see exercise 09 for this pattern)

# ---------------------------------------------------------------

STAGE:   .BYTE

OP1:     .BYTE

OP2:     .BYTE

OPCODE:  .BYTE

RESULT:  .BYTE

TEMP:    .BYTE

KEYVAL:  .BYTE

MCNT:    .BYTE

DREM:    .BYTE

.END    $4000