Building a 4-Function Calculator

A Beboputer / PY-DIYCALCULATOR assembly-language tutorial

# 1. What You Will Build

This tutorial adds a real, working calculator program to the Beboputer virtual 8-bit computer — one that adds, subtracts, multiplies, and divides. Type a digit, an operator, another digit, then ’=’ on the Calculator, and the answer appears right on its display, exactly like a pocket calculator:

5 + 3 =    ->   shows   5+3=8

7 * 8 =    ->   shows   7*8=56

2 - 9 =    ->   shows   2-9=-7

9 / 0 =    ->   shows   9/0=Err

The source file is tutorial/10_calculator_four_function.asm. It builds directly on ideas from exercises 05 through 09 — reading the keypad, writing to the display — and adds one new idea: doing real arithmetic on a CPU that has no multiply or divide instruction at all.

# 2. Before You Start

This tutorial assumes you have already worked through, or at least skimmed, these earlier exercises:

- 05_calculator_keypad_to_7seg.asm — how the keypad port ($F011) works as a read-clear latch.

- 06_calculator_display_text.asm — how to write characters to the Calculator’s own display ($F031).

- 09_terminal_print_message.asm — how .BYTE tables and reserved variables are laid out in a program.

Everything you need from those exercises is re-explained below, so you can also read this tutorial on its own.

# 3. How the Calculator Talks to Your Program

Two memory-mapped ports do all the work:

| Port | Direction | What it does |
| --- | --- | --- |
| $F011 | Input | The keypad. Reading it returns the code of the last key pressed, or $FF if no key is waiting. The moment you read a non-$FF value, the port snaps back to $FF — a "read-clear latch" — so the same keypress is never processed twice. |
| $F031 | Output | The Calculator’s own display. Writing a printable ASCII byte (32–126) appends that character to the display, building up an expression left to right. Writing $0D, $10, or $1B clears the display back to empty. |

Because $F031 simply appends characters, the display behaves like a tiny one-line terminal rather than a fixed set of digit slots — which is exactly what lets this program build up "5+3=8" one character at a time as each key is pressed.

# 4. Keypad Codes Used By This Program

The Calculator’s buttons send these exact byte values on $F011 (from tools/calculator.py):

| Key(s) | Code(s) | Meaning in this program |
| --- | --- | --- |
| 0–9 | $30–$39 | ASCII digit codes. AND $0F turns any of these into the plain binary value 0–9. |
| + − × ÷ | $2B $2D $2A $2F | The four operators, stored as-is and compared against later to choose which operation to run. |
| = | $3D | Triggers the calculation. Enter ($0D) does the same thing. |
| Clear / CE | $1B / $01 | Wipes the display and restarts the state machine from scratch. |

# 5. Program Design: A Small State Machine

The whole program is one loop that reads a key, decides what it means based on what has happened so far, and reacts. "What has happened so far" is tracked in one variable, STAGE:

| STAGE | The program is… |
| --- | --- |
| 0 | Waiting for the first digit (operand 1). |
| 1 | Waiting for an operator (+, −, ×, or ÷). |
| 2 | Waiting for the second digit (operand 2). |
| 3 | Waiting for ’=’ (or Enter). |
| 4 | Showing an answer — the next digit pressed clears the display and starts a brand new sum. |

Every keypress runs through the same short dispatcher: read the key once, remember it, check for Clear/CE globally, then jump to the routine for the current STAGE. Each of those routines does three things — echoes the key to the display, updates its stored value, and advances STAGE — except the STAGE-3 routine, which triggers the actual calculation instead.

# 6. Reading a Key Safely (Only Once!)

$F011 is a read-clear latch: the instant your program reads a non-$FF value from it, the port resets to $FF so the same keypress cannot be read twice. That is exactly the behavior that makes polling for a key safe — but it also means you must capture the value the first time you read it, because a second read a few instructions later will just see $FF.

This program reads $F011 exactly once per keypress, in the main WAIT loop, and immediately stores it in the KEYVAL variable:

WAIT:   LDA     [KEY]

CMPA    $FF             # $FF = idle, no key waiting

JZ      [WAIT]

STA     [KEYVAL]        # stash it - this is the ONLY read of KEY

From here on, every comparison for this keypress — "is it Clear?", "is it a digit?", "is it the operator I expect?" — reads KEYVAL instead of the port itself.

# 7. Doing the Math Without a MUL or DIV Instruction

The Beboputer CPU’s instruction set has ADD, ADDC, SUB, SUBC, compare (CMPA), shifts, and rotates — but no hardware multiply or divide. Every multiply and divide routine in this codebase, including the full 16-bit versions in Data/int-mult-2-byte.asm and Data/int-div-2-byte.asm, builds those operations out of addition and subtraction in a loop. Because this calculator only ever works with single digits (0–9), the loops here are short (at most 9 passes) and simple enough to read start to finish.

## 7.1 Addition

The simplest case — just ADD the two operands. The largest possible result is 9+9=18, which fits in a single byte with no overflow to worry about:

DO_ADD: LDA     [OP1]

ADD     [OP2]

STA     [RESULT]

## 7.2 Subtraction, and the Meaning of the Carry Flag

Subtraction introduces the one flag this whole program leans on — and it is worth being precise about it, because getting it slightly wrong caused a real, hard-to-notice bug (see the callout below). After SUB or CMPA computes "A minus M", the Carry flag on this CPU is set to 1 only when A is STRICTLY GREATER THAN M — not the more common "no borrow was needed" convention used by most CPUs, which would also cover the case where A and M are equal. When A equals M, Carry comes back 0.

That means a single "jump if carry" branch only ever catches strictly-greater; it does not catch equal. Testing "is A at least M" correctly needs a three-way check: JC for strictly-greater, JZ for exactly-equal, and only fall through to "strictly less" if neither fired. This program has no way to display a negative number directly, so when the second operand is larger than the first, it instead computes the difference the other way around and remembers that the sign should be flipped with a NEG flag:

DO_SUB: LDA     [OP1]

CMPA    [OP2]

JC      [SUB_POS]       # C=1: OP1 > OP2, no borrow needed

JZ      [SUB_POS]       # Z=1: OP1 == OP2, result is 0, no borrow

LDA     [OP2]           # OP1 < OP2: compute OP2-OP1 instead

SUB     [OP1]

STA     [RESULT]

LDA     $01

STA     [NEG]

Later, when the result is displayed, a NEG value of 1 simply prints a − sign before the digits.

## 7.2.1 A Bug This Caused, and the Fix

An earlier version of this program used only the single JC branch shown above, without the JZ line. It assembled cleanly and eight of the nine cases in the verification table below happened to still pass — but 5−5 displayed "−0" instead of "0", because CMPA saw the two equal operands, found Carry = 0 (not strictly greater), and incorrectly fell into the "OP1 < OP2" branch that computes 0 and sets the negative flag anyway. The fix is the JZ [SUB_POS] line above: exactly-equal now takes the same non-negative path as strictly-greater.

The same missing-equal-case bug also affected division and the decimal-digit conversion below, with more visible consequences: 4÷2 returned 1 instead of 2, and any answer that was an exact multiple of 10 (e.g. 2×5) printed a stray ":" character instead of "10". Both are fixed the same way — a JZ line added alongside the existing JC — and both are covered in the sections and verification table that follow.

## 7.2.2 A Second Bug: Digit "1" and the CE Button Were the Same Byte

A separate, unrelated bug turned up after the flag fix above: pressing digit "1" was silently swallowed and treated as a clear instead of entering the digit. The cause has nothing to do with the CPU’s flags — it is about what byte a button actually sends.

Every DIY button writes a fixed byte to $F011 on click, but digit and hex-letter buttons are special-cased by the button framework itself (DIYButton._execute() in tools/diy_button.py): before transmitting, it converts their ASCII code ($30–$39 for ‘0’–‘9’) down to a raw nibble (0–15). That is deliberate — it lets CPU programs read keypad digits as plain binary values without needing to mask off the ASCII bits — but it means digit "1" does not arrive as $31; it arrives as plain byte $01.

The Calculator’s CE button used to be assigned that exact byte, $01, in its default configuration (_BUTTON_DEFAULTS in tools/calculator.py). That made CE and digit "1" bit-for-bit identical on the wire: this program had no way to tell which one had actually been pressed, so its explicit CE check (CMPA $01 / JZ [DO_CLEAR]) fired every time "1" was pressed too, wiping the display instead of accepting the digit.

This is not something the program’s own logic could fix — two different buttons were sending the identical byte, so no amount of cleverness in this .asm file could distinguish them. The real fix was moving CE to a free byte outside the 0–15 nibble range, $7F (ASCII DEL), in _BUTTON_DEFAULTS, so it can never collide with any digit or hex-letter button again. This program’s CE check was updated to match ($7F instead of $01); Clear ($1B, ESC) was never affected, since ESC falls well outside the 0–15 range on its own.

## 7.3 Multiplication by Repeated Addition

With no multiply instruction, 5 × 3 is computed the way a student might do it by hand before learning the times table: add 5 to itself, 3 times. MCNT counts the remaining additions down to zero:

DO_MUL: LDA     $00

STA     [RESULT]

LDA     [OP2]

STA     [MCNT]

JMP     [MUL_TEST]

MUL_LOOP:

LDA     [RESULT]

ADD     [OP1]

STA     [RESULT]

LDA     [MCNT]

DECA

STA     [MCNT]

MUL_TEST:

LDA     [MCNT]

CMPA    $00

JNZ     [MUL_LOOP]

Checking the counter before the first addition (via the initial JMP to MUL_TEST) also means multiplying by zero correctly produces zero, with no additions performed at all.

## 7.4 Division by Repeated Subtraction

Division works the same way in reverse: how many times does the divisor (OP2) subtract out of the dividend (OP1) before what is left is smaller than the divisor? Each successful subtraction increments a quotient counter (reusing RESULT). The loop needs to keep subtracting while the remainder is greater than OR equal to the divisor, so — per the flag note in 7.2 — that is the same three-way CMPA/JC/JZ test, not a single branch:

DO_DIV: LDA     [OP2]

CMPA    $00

JZ      [DIV_ERR]

LDA     $00

STA     [RESULT]        # quotient, counts up

LDA     [OP1]

STA     [DREM]          # remainder, counts down

DIV_LOOP:

LDA     [DREM]

CMPA    [OP2]

JC      [DIV_SUB]       # C=1: DREM > OP2, still fits

JZ      [DIV_SUB]       # Z=1: DREM == OP2, still fits exactly

JMP     [DIV_DONE]      # DREM < OP2, can't subtract again

DIV_SUB:

LDA     [DREM]

SUB     [OP2]

STA     [DREM]

LDA     [RESULT]

INCA

STA     [RESULT]

JMP     [DIV_LOOP]

Dividing by zero would never stop subtracting, so OP2 is checked for zero right at the start and diverted straight to a small routine that prints "Err" instead of looping forever. Without the JZ line, this loop stopped one subtraction too early whenever OP2 divided OP1 exactly — 4÷2 subtracted once (4→2), saw the remainder equal the divisor, wrongly treated that as "too small to fit," and stopped at a quotient of 1 instead of continuing to 2.

# 8. Turning the Answer Back Into Decimal Digits

Every operation above leaves its answer, RESULT, as a plain binary byte — for example the byte value 56 after 7×8, not the two characters ’5’ and ’6’. Because RESULT never exceeds 81 (9×9, the largest possible answer here), converting it to decimal only ever needs at most a tens digit and a ones digit. The same repeated-subtraction idea from division does the conversion: subtract 10 from RESULT, counting how many times that succeeds (the tens digit), and whatever is left over afterwards is the ones digit — and, once again, "succeeds" means RESULT is greater than OR equal to 10, so the loop uses the same three-way CMPA/JC/JZ test rather than a single branch:

TENS_LOOP:

LDA     [RESULT]

CMPA    $0A

JC      [TENS_SUB]      # C=1: RESULT > 10, still fits

JZ      [TENS_SUB]      # Z=1: RESULT == 10, still fits exactly

JMP     [TENS_DONE]     # RESULT < 10, tens digit is done

TENS_SUB:

LDA     [RESULT]

SUB     $0A

STA     [RESULT]

LDA     [TENS]

INCA

STA     [TENS]

JMP     [TENS_LOOP]

Once each digit (0–9) is known, OR-ing it with $30 turns it back into the matching ASCII character — the same trick used in reverse by the AND $0F earlier, and the same one used in exercise 05. Without the JZ line, an exact multiple of 10 (e.g. 2×5=10) stopped this loop with RESULT still holding the raw value 10 (0x0A) instead of 0 — and 0x0A OR $30 produces 0x3A, the ":" character, printed in place of the ones digit.

# 9. Verified Against the Real CPU Emulator

This program was actually run against the project’s own CPU class with simulated keypresses — not just assembled and read over — and every digit press was translated through the exact same ASCII-to-raw-nibble conversion the real button framework applies (DIYButton._execute()), not sent as raw ASCII, so these results reflect what the CPU genuinely receives. The table below includes the original scenarios (every operator, a subtraction that goes negative, a divide-by-zero, a multiply-by-zero, and a fresh digit right after an answer), the equal-operand and exact-division/exact-multiple-of-10 cases that exposed the Carry-flag bug (7.2.1), and the digit-"1"/CE cases that exposed the button-code collision (7.2.2): every one of these now produces exactly the expected display text.

| Keys Pressed | Expected Display |
| --- | --- |
| 5, +, 3, = | 5+3=8 |
| 7, ×, 8, = | 7*8=56 |
| 2, −, 9, = | 2-9=-7 |
| 9, −, 2, = | 9-2=7 |
| 9, ÷, 0, = | 9/0=Err |
| 8, ÷, 3, = | 8/3=2 |
| 0, ×, 9, = | 0*9=0 |
| (after a result) 4, +, 4, = | 4+4=8 (old expression cleared first) |
| 4, ÷, 2, = | 4/2=2 (exact division — was 1 before the fix) |
| 6, ÷, 3, = | 6/3=2 (exact division — was 1 before the fix) |
| 9, ÷, 3, = | 9/3=3 (exact division — was 2 before the fix) |
| 5, −, 5, = | 5-5=0 (equal operands — was "-0" before the fix) |
| 2, ×, 5, = | 2*5=10 (exact multiple of 10 — was ":" before the fix) |
| 5, +, 5, = | 5+5=10 (exact multiple of 10 — was ":" before the fix) |
| 1, +, 3, = | 1+3=4 (digit "1" — was swallowed as Clear before the fix) |
| 9, ÷, 1, = | 9/1=9 (digit "1" as divisor) |
| (fresh) 1 | 1 (just the digit — was misread as Clear before the fix) |
| 7, CE, 2, ... | 2 (CE now $7F, clears without colliding with "1") |

# 10. Try It Yourself

Open the Workbench and Calculator, load tutorial/10_calculator_four_function.asm into the assembler, Assemble, Load → CPU, click Run, and on the Calculator press: a digit, an operator, a digit, then ’=’ (or Enter). Try all four operators, try a subtraction where the second number is bigger, and try dividing by zero. Open Display → Port Map Status alongside the exercise to watch each keypress land on $F011 and each character land on $F031 in real time.

# 11. Ideas to Extend This Program

- Show the division remainder as well as the quotient.

- Accept two-digit operands — this needs a second state per operand and a way to combine tens and ones (tens × 10 + ones) back into one binary value.

- Replace the repeated-addition/subtraction loops with the faster shift-and-add / shift-and-subtract algorithms used in Data/fmult.asm and Data/int-div-2-byte.asm — same idea, far fewer loop iterations for large numbers.

# Appendix: Complete Source Listing

tutorial/10_calculator_four_function.asm

# ================================================================

# 10_calculator_four_function.asm

# Beboputer Hands-On Tutorial — Bonus Exercise (Calculator)

#

# A working 4-function (+ - * /) calculator, built entirely from

# the primitives shown in exercises 05-09: keypad polling, the

# Calculator's own display, and simple assembly-language loops.

#

# What this program does

# -----------------------

#   Lets you type a single-digit sum on the Calculator, e.g.

#       5  +  3  =        ->  shows  5+3=8

#       7  *  8  =        ->  shows  7*8=56

#       2  -  9  =        ->  shows  2-9=-7

#       9  /  0  =        ->  shows  9/0=Err

#   Every keypress is echoed to the display as you type it, so the

#   full expression builds up on screen exactly like a real

#   calculator, then the '=' key replaces nothing - it just appends

#   the computed answer.

#

# Limitations (kept simple on purpose - see "Try this next")

# ------------------------------------------------------------

#   - Operands are single digits only (0-9), no multi-digit numbers

#     and no decimal point.

#   - Division shows the integer quotient only (no remainder).

#   - Dividing by zero shows "Err" instead of a number.

#

# How it works

# -------------

#   The program is a small state machine driven by KEY ($F011):

#

#     STAGE 0   waiting for the first digit  (operand 1)

#     STAGE 1   waiting for an operator      (+ - * /)

#     STAGE 2   waiting for the second digit (operand 2)

#     STAGE 3   waiting for '='  (or Enter)

#     STAGE 4   answer is showing - the next digit starts a new sum

#

#   $F011 is a read-clear latch (see exercise 05): each keypress is

#   read exactly once into KEYVAL and every later comparison in that

#   pass re-uses KEYVAL, since a second LDA [KEY] would just see the

#   $FF idle code.

#

#   Multiply is repeated addition (add OP1 to itself OP2 times) and

#   divide is repeated subtraction (subtract OP2 from OP1 while it

#   still fits, counting how many times) - the CPU has no hardware

#   MUL or DIV instruction, so every calculator and multiply/divide

#   routine on this machine builds the operation out of ADD/SUB in

#   a loop. Because operands here are single digits (0-9 at most),

#   the loops are at most 9 iterations - simple enough to read in

#   one sitting, unlike the 16-bit shift-add/shift-subtract versions

#   in Data/int-mult-2-byte.asm and Data/int-div-2-byte.asm, which

#   solve the same problem for much bigger numbers.

#

#   A note on this CPU's flags (read this before you adapt the

#   pattern below for your own code): SUB/CMPA set the Carry flag

#   to mean "the accumulator is STRICTLY GREATER THAN the operand"

#   - not the more usual "no borrow was needed" (which on most CPUs

#   also covers the equal case). That means a single "jump if

#   carry" or "jump if no-carry" branch only ever catches strictly-

#   greater or strictly-less; it silently gets the equal case wrong.

#   Every "is the left-hand side >= the right-hand side?" test in

#   this program - can OP1 still absorb another subtraction of OP2,

#   is the result >= 10, is OP1 >= OP2 for a same-sign subtraction -

#   therefore uses a three-way test instead of one branch: CMPA,

#   then JC for strictly-greater, JZ for exactly-equal, and only

#   falls through to "strictly less" if neither fired. This was

#   discovered (and is explained at length) in exercise 11's

#   circumference-button program; every exact-division and exact-

#   multiple-of-10 case here needed the same fix.

#

#   A second gotcha, this time about the DIY buttons themselves:

#   pressing a digit button does NOT send its ASCII code ($30-$39)

#   to $F011 - the button framework (tools/diy_button.py) silently

#   converts digit/hex-letter buttons down to a raw nibble (0-15)

#   before transmitting, so CPU programs can read keypad digits as

#   plain binary values. That means digit "1" actually arrives as

#   byte $01. The Calculator's CE button used to be assigned that

#   same byte ($01), making CE and digit "1" bit-for-bit identical

#   on the wire - pressing "1" was silently swallowed and treated as

#   a clear instead of a digit, since this program had no way to

#   tell the two keypresses apart. The real fix was to move CE to a

#   free byte outside the 0-15 nibble range ($7F) in _BUTTON_DEFAULTS

#   (tools/calculator.py), so it can never again collide with any

#   digit or hex-letter button; this program watches for that new

#   value below, alongside Clear ($1B, ESC) as the two ways to reset.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run, then on the Calculator press:

#   a digit, an operator, a digit, then '=' (the calculator's '='

#   key, or Enter). Try all four operators, try a subtraction where

#   the second number is bigger (negative result), try dividing by

#   0, and try a case where the operator divides evenly or the

#   result is an exact multiple of 10 (e.g. 4/2, 2*5) - these are

#   exactly the "equal" edge cases the flag note above covers.

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status alongside this exercise to see

#   each keypress arrive on $F011 and each character land on $F031

#   in real time.

#

# Try this next

# ---------------

#   - Show the division remainder as well as the quotient.

#   - Accept two-digit operands (you'll need a second state per

#     operand and a decimal-to-binary combine step: tens*10 + ones).

#   - Replace repeated addition/subtraction with the shift-and-add /

#     shift-and-subtract algorithms from Data/fmult.asm and

#     Data/int-div-2-byte.asm - same idea, fewer loop iterations.

# ================================================================

KEY:     .EQU    $F011

DISP:    .EQU    $F031

.ORG    $4000

START:  LDA     $00

STA     [STAGE]        # begin waiting for the first digit

# ---------------------------------------------------------------

# Main loop - poll the keypad, dispatch on STAGE

# ---------------------------------------------------------------

WAIT:   LDA     [KEY]

CMPA    $FF             # $FF = idle, no key waiting

JZ      [WAIT]

STA     [KEYVAL]        # stash it - this is the ONLY read of KEY

CMPA    $1B             # Clear key

JZ      [DO_CLEAR]

CMPA    $7F             # CE key

JZ      [DO_CLEAR]

LDA     [STAGE]

CMPA    $00

JZ      [ST_OP1]

CMPA    $01

JZ      [ST_OPCODE]

CMPA    $02

JZ      [ST_OP2]

CMPA    $04

JZ      [ST_NEWCALC]

JMP     [ST_EQUALS]     # only remaining value is STAGE 3

# ---------------------------------------------------------------

# STAGE 4 -> 0 : answer was showing, this digit starts a new sum

# ---------------------------------------------------------------

ST_NEWCALC:

LDA     $1B             # clear code - wipes the old expression

STA     [DISP]

JMP     [ST_OP1]

# ---------------------------------------------------------------

# STAGE 0 : first operand

# ---------------------------------------------------------------

ST_OP1: LDA     [KEYVAL]

STA     [DISP]          # echo the digit as typed

LDA     [KEYVAL]

AND     $0F             # ASCII '0'-'9' -> binary 0-9

STA     [OP1]

LDA     $01

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 1 : operator  (+ - * /)

# ---------------------------------------------------------------

ST_OPCODE:

LDA     [KEYVAL]

STA     [OPCODE]        # remember which operator was pressed

STA     [DISP]          # echo it

LDA     $02

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 2 : second operand

# ---------------------------------------------------------------

ST_OP2: LDA     [KEYVAL]

STA     [DISP]

LDA     [KEYVAL]

AND     $0F

STA     [OP2]

LDA     $03

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 3 : waiting for '=' (or Enter) - ignore anything else

# ---------------------------------------------------------------

ST_EQUALS:

LDA     [KEYVAL]

CMPA    $3D             # '='

JZ      [DO_EQUALS]

CMPA    $0D             # Enter

JZ      [DO_EQUALS]

JMP     [WAIT]

DO_EQUALS:

LDA     $3D

STA     [DISP]          # echo the '=' sign

LDA     [OPCODE]

CMPA    $2B             # '+'

JZ      [DO_ADD]

CMPA    $2D             # '-'

JZ      [DO_SUB]

CMPA    $2A             # '*'

JZ      [DO_MUL]

CMPA    $2F             # '/'

JZ      [DO_DIV]

JMP     [WAIT]          # (shouldn't happen)

# ---------------------------------------------------------------

# Addition:  RESULT = OP1 + OP2   (max 9+9=18, always positive)

# ---------------------------------------------------------------

DO_ADD: LDA     [OP1]

ADD     [OP2]

STA     [RESULT]

LDA     $00

STA     [NEG]

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Subtraction:  RESULT = OP1 - OP2  (negate + flag if OP1 < OP2)

#

# CMPA sets Carry only when OP1 is STRICTLY greater than OP2 (see

# the flag note up top), so both the greater-than case (C=1) and

# the equal case (Z=1, result is 0) need to reach SUB_POS - only

# OP1 strictly less than OP2 falls through to the negative branch.

# ---------------------------------------------------------------

DO_SUB: LDA     [OP1]

CMPA    [OP2]

JC      [SUB_POS]       # C=1: OP1 > OP2, no borrow needed

JZ      [SUB_POS]       # Z=1: OP1 == OP2, result is 0, no borrow

LDA     [OP2]           # OP1 < OP2: compute OP2-OP1 instead

SUB     [OP1]

STA     [RESULT]

LDA     $01

STA     [NEG]

JMP     [SHOW_RESULT]

SUB_POS:

LDA     [OP1]

SUB     [OP2]

STA     [RESULT]

LDA     $00

STA     [NEG]

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Multiply:  RESULT = OP1 * OP2, via OP2 repeated additions of OP1

# (max 9*9=81 - fits comfortably in one byte)

# ---------------------------------------------------------------

DO_MUL: LDA     $00

STA     [RESULT]

LDA     [OP2]

STA     [MCNT]

JMP     [MUL_TEST]

MUL_LOOP:

LDA     [RESULT]

ADD     [OP1]

STA     [RESULT]

LDA     [MCNT]

DECA

STA     [MCNT]

MUL_TEST:

LDA     [MCNT]

CMPA    $00

JNZ     [MUL_LOOP]

LDA     $00

STA     [NEG]

JMP     [SHOW_RESULT]

# ---------------------------------------------------------------

# Divide:  RESULT = OP1 / OP2 (integer quotient), via repeated

# subtraction of OP2 from a running remainder. OP2=0 -> "Err".

#

# Loop while DREM >= OP2 (checked as: DREM strictly greater than

# OP2, OR DREM exactly equal to OP2 - the three-way CMPA/JC/JZ

# combination from the flag note up top; a lone JNC would stop one

# subtraction too early whenever OP2 divides OP1 evenly, e.g. 4/2).

# ---------------------------------------------------------------

DO_DIV: LDA     [OP2]

CMPA    $00

JZ      [DIV_ERR]

LDA     $00

STA     [RESULT]        # quotient, counts up

LDA     [OP1]

STA     [DREM]          # remainder, counts down

DIV_LOOP:

LDA     [DREM]

CMPA    [OP2]

JC      [DIV_SUB]       # C=1: DREM > OP2, still fits

JZ      [DIV_SUB]       # Z=1: DREM == OP2, still fits exactly

JMP     [DIV_DONE]      # DREM < OP2, can't subtract again

DIV_SUB:

LDA     [DREM]

SUB     [OP2]

STA     [DREM]

LDA     [RESULT]

INCA

STA     [RESULT]

JMP     [DIV_LOOP]

DIV_DONE:

LDA     $00

STA     [NEG]

JMP     [SHOW_RESULT]

DIV_ERR:

LDA     $45             # 'E'

STA     [DISP]

LDA     $72             # 'r'

STA     [DISP]

LDA     $72             # 'r'

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Show RESULT (0-81, binary) as decimal digits, '-' first if NEG.

#

# Same three-way CMPA/JC/JZ test peels off the tens digit correctly

# even when RESULT is an exact multiple of 10 (e.g. 2*5=10) - a

# lone JNC would stop one subtraction too early and leave RESULT=10

# to be OR $30'd directly, producing ':' (0x3A) instead of "10".

# ---------------------------------------------------------------

SHOW_RESULT:

LDA     [NEG]

CMPA    $00

JZ      [SR_TENS]

LDA     $2D             # '-'

STA     [DISP]

SR_TENS:

LDA     $00

STA     [TENS]

TENS_LOOP:

LDA     [RESULT]

CMPA    $0A

JC      [TENS_SUB]      # C=1: RESULT > 10, still fits

JZ      [TENS_SUB]      # Z=1: RESULT == 10, still fits exactly

JMP     [TENS_DONE]     # RESULT < 10, tens digit is done

TENS_SUB:

LDA     [RESULT]

SUB     $0A

STA     [RESULT]

LDA     [TENS]

INCA

STA     [TENS]

JMP     [TENS_LOOP]

TENS_DONE:

LDA     [TENS]

CMPA    $00

JZ      [SR_ONES]       # no leading zero when tens digit is 0

LDA     [TENS]

OR      $30

STA     [DISP]

SR_ONES:

LDA     [RESULT]        # what's left (0-9) is the ones digit

OR      $30

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Explicit Clear / CE handling

# ---------------------------------------------------------------

DO_CLEAR:

LDA     $1B

STA     [DISP]

LDA     $00

STA     [STAGE]

JMP     [WAIT]

RESET_STATE:

LDA     $04             # STAGE 4: next digit clears & restarts

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# Variables (reserved bytes - see exercise 09 for this pattern)

# ---------------------------------------------------------------

STAGE:   .BYTE

OP1:     .BYTE

OP2:     .BYTE

OPCODE:  .BYTE

RESULT:  .BYTE

NEG:     .BYTE

KEYVAL:  .BYTE

MCNT:    .BYTE

DREM:    .BYTE

TENS:    .BYTE

.END    $4000