Beboputer Hands-On Tutorial Series

Lab 3: Loops, Nybbles, the Stack, and Recursion

This tutorial walks through all seventeen Lab 3 assembly-language exercises included with the DIY Calculator (Beboputer) project, found as tutorial/16a through tutorial/16q. Each exercise is an annotated copy of one of the Data/lab3*.asm files that ship with the project - the underlying code is unchanged from the original; only explanatory header comments have been added. Every file in this set has been verified to assemble to byte-identical machine code as its original, and to behave identically when run in the CPU emulator.

Lab 3 is the biggest lab in this tutorial series so far - seventeen small programs instead of five or six - because it covers more ground than any lab before it: looping versus unrolling, four different ways to extract a hex digit from a byte, 8-bit overflow, two different techniques for reversing a sequence (an array versus the stack), nested versus flattened subroutines, and finally genuine recursion. The seventeen files are grouped below by theme, in the order it makes sense to read them, rather than strictly alphabetically.

# 1. Unrolled vs. Looped: Binary Display (16a, 16b)

These two files show the exact same behavior - read a key, show its raw code as eight binary digits - built two different ways.

## 16a - Fully Unrolled

tutorial/16a_lab3a_bin_display_unrolled.asm writes out eight nearly-identical blocks of code, one per bit (TEST7 through TEST0), each doing the same three-step dance: SHL the working byte, test the bit that fell into Carry via JC, and write the matching character. The source file's own title calls this “one painful bit at a time” - and it is: the same three-instruction pattern, copy-pasted eight times, with only the target label changing between copies.

TEST7:     LDA     [TEMP8]    # Reload ACC with copy of key code
           SHL                # Shift left 1 bit
           STA     [TEMP8]    # Store new value in temp location
           JC      [DISP7_1]  # If carry = 1, jump to display a 1
DISP7_0:   LDA     0          # ... otherwise load acc with 0
           STA     [MAINDISP] # ... and store it to main display
           JMP     [TEST6]    # ... then go and deal with bit 6
DISP7_1:   LDA     1          # Load acc with 1
           STA     [MAINDISP] # ... and store it to main display

One small quirk worth noting: the DISP1_1 block's comment reads “Load acc with 0” even though the instruction is LDA 1 - a stale copy-paste comment. The code itself is correct; only the comment is wrong. Preserved exactly as found.

## 16b - Loop-ified

tutorial/16b_lab3a_bin_display_loop.asm replaces all eight TEST blocks with one block that runs eight times, counted down by the Index register:

BLDX    8          # Load X reg with number of bits
LOOP:      LDA     [TEMP8]    # Reload ACC with copy of key code
           SHL                # Shift left 1 bit
           STA     [TEMP8]    # Store new value in temp location
           JC      [DISP_1]   # If carry = 1, jump to display a 1
DISP_0:    LDA     0          # ... otherwise load acc with 0
           STA     [MAINDISP] # ... and store it to main display
           JMP     [DISPDECX] # ... then go and decrement the X reg
DISP_1:    LDA     1          # Load acc with 1
           STA     [MAINDISP] # ... and store it to main display
DISPDECX:  DECX               # Decrement the X reg
           JNZ     [LOOP]     # If not zero jump back for next bit

BLDX 8 loads the loop counter, and DECX/JNZ repeats the body until it reaches zero - the same counting-loop idea introduced for countdown and LED-chase exercises earlier in this series. The two files produce identical output for every key; only the assembled size differs (16a is considerably larger, since its logic is eight full copies instead of one).

# 2. Four Ways to Extract a Hex Nybble (16c-16f)

All four of these files do the exact same job - split a byte's top nybble out and show it as a hex digit - using a different shift/rotate instruction each time. Comparing them side by side is the point of this group.

| File | Instruction used | Extra bookkeeping needed |
| --- | --- | --- |
| 16c | SHR x4 | None - the trailing AND $0F is just cheap insurance. |
| 16d | SHL x4 | Manual carry reinsertion: JNC/OR after every shift, recycling the bit SHL discarded back into bit 0. |
| 16e | ROLC x4 | Same recycling trick as 16d, but done more explicitly: both an OR (bit was 1) and an AND (bit was 0) override whatever ROLC itself left in bit 0. |
| 16f | RORC x4 | None - RORC's unpredictable top-bit fill is simply discarded by the final AND $0F mask. |

16c (SHR) is the simplest: shifting right four times naturally slides the top nybble down to the bottom, and the trailing mask is just insurance.

DISPMSN:   LDA     [TEMP8]    # Reload ACC with copy of key code
           SHR                # Shift right 1 bit (= 1 bit shift)
           SHR                # Shift right 1 bit (= 2 bit shift)
           SHR                # Shift right 1 bit (= 3 bit shift)
           SHR                # Shift right 1 bit (= 4 bit shift)
           AND     %00001111  # Clear MS 4 bits (not really necessary)
           STA     [MAINDISP] # Copy result to main display

16d (SHL) has to work harder, because shifting left moves bits the wrong way for this job. Its trick: catch the bit that SHL discards into Carry, and immediately fold it back in at the bottom via JNC/OR. Four rounds of “shift left, then recycle the lost bit” amounts to rotating the whole byte by four places using the Carry flag as scratch space.

DISPMSN:   BLDX     4         # Load X reg with number of shifts
           LDA     [TEMP8]    # Reload ACC with copy of key code
DISPMSNL:  SHL                # Shift left 1 bit
           JNC     [DISPMSN0] # Jump if carry flag = 0
           OR      %00000001  #   otherwise set LS bit to 1
DISPMSN0:  DECX               # Decrement index register
           JNZ     [DISPMSNL] # If X !=0 jump back and shift again
           AND     %00001111  # Clear MS 4 bits (not really necessary)
           STA     [MAINDISP] # Copy result to main display

16e (ROLC) looks like it should need less manual work than 16d, since ROLC is built to automatically feed the Carry flag into bit 0. But this file doesn't trust that automatic fill - the incoming Carry value on the very first ROLC is whatever was left over from earlier arithmetic, not necessarily 0 - so it explicitly overrides bit 0 every single pass with an OR or an AND, exactly mirroring 16d's approach.

16f (RORC) is the cleanest of the rotate-based versions: it doesn't control bit 0 (or bit 7) at all during the four rotates. Whatever garbage ends up in the top nybble from the unpredictable carry-in is simply thrown away by the final AND $0F. Where 16c avoids the problem by shifting the correct direction in the first place, 16f embraces the “wrong” direction but cleans up only once, at the very end, instead of once per pass.

# 3. An Overflow Detour (16g)

tutorial/16g_lab3b_accumulator_wraparound.asm is the shortest program in the whole set - four lines - built purely to demonstrate 8-bit overflow with nothing else going on:

LDA         0       # Load accumulator with zero
LOOP:      INCA                # Increment the accumulator
           JNZ     [LOOP]      # Jump to LOOP if ACC !=0
           JMP     [$0000]     # Jump to address $0000

INCA counts the accumulator up by 1 each pass; JNZ keeps looping as long as the result isn't zero. Since an 8-bit register can only hold 0-255, the 256th increment (255 + 1) has nowhere to go but back around to 0 - and that's the one and only time JNZ stops firing. The loop runs exactly 255 times, not 256, since it started counting up from 0 rather than counting through it again on the way to the wrap.

# 4. A Refined Binary-Display Pair (16h, 16i)

tutorial/16h_lab3c_bin_display_restructured.asm is a relabeled, reorganized copy of 16b's loop - same logic, different label names and a slightly different layout, a useful exercise in reading for behavior rather than appearance. tutorial/16i_lab3c_bin_display_nibble_dash.asm builds on it by splitting the single 8-bit loop into two 4-bit loops with a dash written in between, so the display reads “XXXX-XXXX” instead of an unbroken run of eight digits:

LOADXA:    BLDX    4           # Load X reg with 4
LOOPA:     ...                 # (display top 4 bits, same SHL/JC pattern)
DISPDASH:  LDA     $2D          # Load ACC with ASCII code for '-'
           STA     [MAINDISP]  # ... and store it to main display
LOADXB:    BLDX    4           # Load X reg with 4
LOOPB:     ...                 # (display bottom 4 bits, same pattern again)

LOOPA and LOOPB are two separate copies of the same short loop body, rather than one shared loop with a counter - a small, deliberate trade so that the dash can be written exactly once, cleanly, between the two halves.

# 5. Hex-Digit Echo, Forward and Reversed (16j, 16k, 16m)

16j is the baseline: a keypad loop that accepts hex digits (raw codes $00-$0F) and displays each one immediately, in the order typed - the same shape as the digit-only echo from Lab 2, just widened from $09 to $0F so hex letters A-F get through too.

GETKEY:    LDA     [KEYPAD]   # Load ACC from the keypad
           JN      [GETKEY]   # Jump back if no key pressed
           CMPA    $0F        # Compare ACC to $0F
           JC      [GETKEY]   # Jump back if ACC is bigger
           STA     [MAINDISP] # ... else store ACC to display
           JMP     [GETKEY]   # Go and wait for another key

16k and 16m both reverse a typed sequence - last key typed appears first - using two very different techniques.

## 16k - Reversal via an Array and the Index Register

Every accepted key is written into a small array (STORE) using indexed addressing, with the Index register both counting how many keys were typed and tracking where the next one goes:

GETKEY:    LDA     [KEYPAD]    # Load ACC from the keypad
           ...
           STA     [STORE,X]   # ... else store ACC to memory
           INCX                # ... and increment the index reg
           JMP     [GETKEY]    # Go and wait for another key
DISPSTUF:  LDA     [STORE-1,X] # Load ACC with a key code
           STA     [MAINDISP]  # Store it to the main display
           DECX                # Decrement the index register
           JNZ     [DISPSTUF]  # If index reg not 0 get next code

[STORE-1,X] is indexed addressing with label arithmetic folded into the operand: the effective address is (STORE - 1) + X. Since X still holds the count of keys typed when playback begins, the very first read lands on the last slot filled - the last key typed - and DECX walks backward from there. One edge case is worth knowing: if the very first key pressed is a non-hex key (X still 0), this loop reads one byte before STORE and then DECX takes X from 0 to $FFFF, looping indefinitely rather than stopping - always type at least one hex digit first to avoid it.

## 16m - Reversal via the Stack

This version reaches for PUSHA/POPA instead of an array. Every accepted key gets pushed; because a stack is naturally Last-In-First-Out, the most recently pushed byte is always the first one POPA retrieves - no counting, no index arithmetic at all:

BLDSP   $EFFF       # Load stack pointer with $EFFF
GETKEY:    NOP                 # a harmless one-cycle delay
LOOP:      LDA     [KEYPAD]    # Load ACC from the keypad
           ...
           PUSHA               # ... else push ACC onto the stack
           JMP     [GETKEY]    # Go and wait for another key
DISPSTUF:  POPA                # Pop ACC off the stack
           STA     [MAINDISP]  # Store it to the main display
           JMP     [GETKEY]    # Go and wait for another key

Unlike 16k, this version shows only one digit per non-hex keypress rather than the whole sequence at once, and it never terminates - it's always ready to accept and reverse another run. Comparing 16k and 16m directly is the real point of this pair: one builds LIFO ordering by hand with an array and index math, the other gets it for free from the CPU's own stack.

# 6. Displaying the Index Register Itself (16l)

tutorial/16l_lab3c_display_x_register.asm reuses 16k's exact fill loop (hex digits go into STORE, counted by X) but never reads STORE back out. Instead, once a non-hex key stops the count, it displays X's own final value - as a 4-digit hex number - using BSTX, the store counterpart to BLDX:

DISPSTUF:  BSTX    [TEMP16]    # Store X reg into 2-byte temp location
           LDA     $24         # Load ACC with ASCII code for '$'
           STA     [MAINDISP]  # Write '$' to main display
           ... (four nybble extractions from TEMP16 and TEMP16+1,
                the same SHR + AND technique as 16c, one per hex digit)

Because the Index register is 16 bits wide, this count can run past 255 - something no single 8-bit byte could hold - which is exactly why this exercise needs four hex digits of display instead of two.

# 7. Nested vs. Flattened Subroutines (16n, 16o)

Both files display four fixed bytes ($23, $5A, $06, $4C) as hex pairs with identical output; they differ only in how the subroutine that does the work is structured internally.

## 16n - Nested

DISPBYTE calls two separate subroutines, DISPMSN and DISPLSN, each with its own JSR and RTS:

DISPBYTE:  PUSHA              # Push a copy of ACC onto stack
           JSR     [DISPMSN]  # Call sub for MS nybble
           POPA               # Pop copy of original byte off stack
           JSR     [DISPLSN]  # Call sub for LS nybble
           RTS                # Return to calling location

The PUSHA/POPA pair around the DISPMSN call exists because DISPMSN's four SHRs are destructive - they consume the byte's bits while pulling the high nybble down - so the original value has to be stashed on the stack and restored before DISPLSN can extract the low nybble from it.

## 16o - Flattened

The same two nybble-extraction blocks are folded directly into DISPBYTE as fall-through labels - no JSR, no RTS until the very end:

DISPBYTE:  PUSHA              # Push a copy of ACC onto stack
DISPMSN:   SHR / SHR / SHR / SHR
           AND     %00001111
           STA     [MAINDISP]
DISPLSN:   POPA               # Pop copy of original byte off stack
           AND     %00001111
           STA     [MAINDISP]
           RTS                # Return from subroutine

DISPMSN and DISPLSN are still labels - useful for reading - but nothing outside DISPBYTE ever jumps to them. Execution simply falls from one block into the next. Same data flow (the same PUSHA/POPA protecting the same destructive shift), same output, purely different internal structure: two callable subroutines versus one subroutine with internal signposts.

# 8. Recursion: Factorial and String Reversal (16p, 16q)

The last two files introduce genuine recursion - a subroutine calling itself - the natural extension of everything the stack-based exercises above built toward.

## 16p - Recursive Factorial

Press a key from 1 to 5 and FACTOR computes its factorial by calling itself:

FACTOR:    PUSHA              # Push copy of ACC onto stack
           CMPA     $01       # Compare ACC to 1
           JNC     [COMEOUT]  # If ACC not > 1 start to come out
           DECA               #   else decrement ACC
           JSR     [FACTOR]   #   then this routine calls itself
COMEOUT:   PUSHA              # Push copy of ACC onto stack
           JSR     [FMULT]    # Multiply two bytes on top of stack
           POPA               # Retrieve 8-bit result
           RTS                # End this instantiation of the routine

Each call pushes its own copy of n before recursing on n-1, so computing 5! leaves five values on the stack (5, 4, 3, 2, 1) - one per call. When the base case (n=1) is reached, COMEOUT multiplies the two topmost stack values via a separate, non-recursive FMULT subroutine (a classic 8x8-bit shift-and-add multiply) and the product propagates back up one level at a time: 1x1=1, 2x1=2, 3x2=6, 4x6=24, 5x24=120. The keypad range is capped at 1-5 for a concrete reason: 6! is 720, which doesn't fit in an 8-bit accumulator - every factorial this program can actually compute (1, 2, 6, 24, 120) fits in one byte by design.

## 16q - Recursive String Reversal

REVERSE prints a stored message backward by calling itself once per character and letting the call stack remember the order to print them back in:

REVERSE:   LDA     [PHRASE,X] # Load ACC with a character
           JNZ     [GOIN]     # If it's not NUL jump to GO_IN
RETURN1:   RTS                # Otherwise return from subroutine
GOIN:      PUSHA              # Push the character onto the stack
           INCX               # Increment the index register
INSIDE:    JSR     [REVERSE]  # The subroutine calls itself recursively
COMEOUT:   POPA               # Pop a character off the stack
           STA     [MAINDISP] # Copy to main display
RETURN2:   RTS                # Return from subroutine

Nothing is displayed until the recursion bottoms out at the table's NUL terminator; from there, each returning call pops and displays the one character it pushed on the way in, then returns to whichever call invoked it. The stored phrase, “SWAP PAWS”, was chosen because it's a palindrome (S-W-A-P-space-P-A-W-S reversed is the identical sequence) - which means the display alone can't prove the reversal actually happened. Watching the stack pointer and the Index register in the Registers panel while stepping through is the real evidence: it visibly descends through all nine characters before anything is displayed, then unwinds one character at a time on the way back out.

# Appendix: Full Source Listings

Complete source for all seventeen files follows. Each preserves the underlying Data/lab3*.asm logic exactly, with explanatory header comments added.

## 16a - Binary Display, Unrolled

# ================================================================
# 16a_lab3a_bin_display_unrolled.asm
# Beboputer Hands-On Tutorial — Lab 3a (version 1)
#
# Display a key's raw code as 8 binary digits - the "painful",
# fully-unrolled way.
#
# What this program does
# -----------------------
#   Waits for any key, clears the display, writes a '%' character
#   (this project's convention for "what follows is binary" - see
#   the six-LED BINMODE/DECMODE/HEXMODE codes used since exercise 07),
#   then writes out that key's raw code as eight separate '0'/'1'
#   characters, most-significant bit first. Then it waits for the
#   next key and does it all again, forever.
#
# How it works
# -------------
#   The whole bit-by-bit display is eight nearly-identical blocks of
#   code, one per bit, each doing the same three things: shift the
#   working byte left one place, check whether a 1 or a 0 just fell
#   out the top, and write the matching character - then jump to the
#   next block down for the next bit:
#
#     TEST7:     LDA     [TEMP8]    # Reload ACC with copy of key code
#                SHL                # Shift left 1 bit
#                STA     [TEMP8]    # Store new value in temp location
#                JC      [DISP7_1]  # If carry = 1, jump to display a 1
#     DISP7_0:   LDA     0          # ... otherwise load acc with 0
#                STA     [MAINDISP] # ... and store it to main display
#                JMP     [TEST6]    # ... then go and deal with bit 6
#     DISP7_1:   LDA     1          # Load acc with 1
#                STA     [MAINDISP] # ... and store it to main display
#
#   SHL shifts every bit in the accumulator one place left, and the
#   bit that falls off the top edge lands in the Carry flag - that's
#   exactly the bit this block needs to test. JC ("jump if Carry")
#   picks the DISP*_1 path when that bit was a 1, and falls through
#   to the DISP*_0 path otherwise. Either way the digit character
#   goes to the display, and the block ends by jumping to the next
#   bit's block (TEST6, then TEST5, and so on down to TEST0).
#
#   This is deliberately the "hard way" - the source file's own title
#   calls it "one painful bit at a time." Every one of the eight
#   blocks does the exact same three-step dance, differing only in
#   which bit they're working on and which block comes next. That
#   repetition is the whole point: it sets up exercise 16b, which
#   notices the pattern and replaces all eight copies with one loop.
#
#   One quirk worth a second look: TEST1's DISP1_1 block is commented
#   "Load acc with 0" even though the instruction is LDA 1 - a leftover
#   copy-paste comment from the DISP*_0 block above it. The code
#   itself is correct (it does load 1 and display a '1'); only the
#   comment is stale. Left exactly as found, since this tutorial
#   preserves the original source unchanged.
#
# Try it
# -------
#   Assemble, Load -> CPU, click Run, switch the Calculator on, and
#   press any key - its raw code appears as eight 1s and 0s after a
#   '%'. Try Hex mode's 'A'-'F' keys too (raw codes 10-15) and watch
#   the top nibble come out as all zeros.
#
# Watch it happen
# -----------------
#   Open Display -> Port Map Status and Step through one key-press -
#   watch $F011 (KEYPAD) get read once, then TEMP8 shift left eight
#   times as the eight TEST blocks run in turn, each writing one
#   character to $F031 (MAINDISP).
#
# Try this next
# ---------------
#   - Count the bytes this version assembles to, then compare against
#     16b's loop-ified version of the exact same behavior.
#   - Fix the stale "Load acc with 0" comment on DISP1_1 and see that
#     nothing about the program's behavior changes - comments don't
#     affect what the CPU does.
# ================================================================
 
## Lab 3a binary display one painful bit at a time
 
#######################################################################
## Start of constant declarations                                    ##
#######################################################################
MAINDISP: .EQU     $F031      # Address of output port for main display
SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs
KEYPAD:   .EQU     $F011      # Address of input port for keypad
CLRCODE:  .EQU     $10        # Special code to clear the main display
BINMODE:  .EQU     %00000100  # LED code to indicate binary mode
DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode
HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode
#######################################################################
## End of constant declarations                                      ##
#######################################################################
 
          .ORG     $4000      # Set program origin
 
#######################################################################
## Start of initialization                                           ##
#######################################################################
INIT:      LDA     CLRCODE    # Load accumulator with clear code
           STA     [MAINDISP] # Write clear code to main display
           LDA     BINMODE    # Load accumulator with bin mode code
           STA     [SIXLEDS]  # Write to port driving six LEDs
#######################################################################
## End of initialization                                             ##
#######################################################################
 
 
#######################################################################
## Start of main program body                                        ##
#######################################################################
 
########## Wait for key to be pressed
GETKEY:    LDA     [KEYPAD]   # Load ACC with code from keypad
           JN      [GETKEY]   # Jump back if no key pressed
           STA     [TEMP8]    # Store key code in temp location
 
########## Prepare the main display
CLRDISP:   LDA     CLRCODE    # Load ACC with clear code
           STA     [MAINDISP] # Clear main display
DISPPERC:  LDA     $25        # Load ACC with ASCII code for '%'
           STA     [MAINDISP] # Write '%' to main display
 
########## Display the binary value
## Process bit 7
TEST7:     LDA     [TEMP8]    # Reload ACC with copy of key code
           SHL                # Shift left 1 bit
           STA     [TEMP8]    # Store new value in temp location
           JC      [DISP7_1]  # If carry = 1, jump to display a 1
DISP7_0:   LDA     0          # ... otherwise load acc with 0
           STA     [MAINDISP] # ... and store it to main display
           JMP     [TEST6]    # ... then go and deal with bit 6
DISP7_1:   LDA     1          # Load acc with 1
           STA     [MAINDISP] # ... and store it to main display
 
## Process bit 6
TEST6:     LDA     [TEMP8]    # Reload ACC from temp location
           SHL                # Shift left 1 bit
           STA     [TEMP8]    # Store new value in temp location
           JC      [DISP6_1]  # If carry = 1, jump to display a 1
DISP6_0:   LDA     0          # ... otherwise load acc with 0
           STA     [MAINDISP] # ... and store it to main display
           JMP     [TEST5]    # ... then go and deal with bit 5
DISP6_1:   LDA     1          # Load acc with 1
           STA     [MAINDISP] # ... and store it to main display
 
## Process bit 5
TEST5:     LDA     [TEMP8]    # Reload ACC from temp location
           SHL                # Shift left 1 bit
           STA     [TEMP8]    # Store new value in temp location
           JC      [DISP5_1]  # If carry = 1, jump to display a 1
DISP5_0:   LDA     0          # ... otherwise load acc with 0
           STA     [MAINDISP] # ... and store it to main display
           JMP     [TEST4]    # ... then go and deal with bit 4
DISP5_1:   LDA     1          # Load acc with 1
           STA     [MAINDISP] # ... and store it to main display
 
## Process bit 4
TEST4:     LDA     [TEMP8]    # Reload ACC from temp location
           SHL                # Shift left 1 bit
           STA     [TEMP8]    # Store new value in temp location
           JC      [DISP4_1]  # If carry = 1, jump to display a 1
DISP4_0:   LDA     0          # ... otherwise load acc with 0
           STA     [MAINDISP] # ... and store it to main display
           JMP     [TEST3]    # ... then go and deal with bit 3
DISP4_1:   LDA     1          # Load acc with 1
           STA     [MAINDISP] # ... and store it to main display
 
## Process bit 3
TEST3:     LDA     [TEMP8]    # Reload ACC from temp location
           SHL                # Shift left 1 bit
           STA     [TEMP8]    # Store new value in temp location
           JC      [DISP3_1]  # If carry = 1, jump to display a 1
DISP3_0:   LDA     0          # ... otherwise load acc with 0
           STA     [MAINDISP] # ... and store it to main display
           JMP     [TEST2]    # ... then go and deal with bit 2
DISP3_1:   LDA     1          # Load acc with 1
           STA     [MAINDISP] # ... and store it to main display
 
## Process bit 2
TEST2:     LDA     [TEMP8]    # Reload ACC from temp location
           SHL                # Shift left 1 bit
           STA     [TEMP8]    # Store new value in temp location
           JC      [DISP2_1]  # If carry = 1, jump to display a 1
DISP2_0:   LDA     0          # ... otherwise load acc with 0
           STA     [MAINDISP] # ... and store it to main display
           JMP     [TEST1]    # ... then go and deal with bit 1
DISP2_1:   LDA     1          # Load acc with 1
           STA     [MAINDISP] # ... and store it to main display
 
## Process bit 1
TEST1:     LDA     [TEMP8]    # Reload ACC from temp location
           SHL                # Shift left 1 bit
           STA     [TEMP8]    # Store new value in temp location
           JC      [DISP1_1]  # If carry = 1, jump to display a 1
DISP1_0:   LDA     0          # ... otherwise load acc with 0
           STA     [MAINDISP] # ... and store it to main display
           JMP     [TEST0]    # ... then go and deal with bit 6
DISP1_1:   LDA     1          # Load acc with 0
           STA     [MAINDISP] # ... and store it to main display
 
## Process bit 0
TEST0:     LDA     [TEMP8]    # Reload ACC from temp location
           SHL                # Shift left 1 bit
           STA     [TEMP8]    # Store new value in temp location
           JC      [DISP0_1]  # If carry = 1, jump to display a 1
DISP0_0:   LDA     0          # ... otherwise load acc with 0
           STA     [MAINDISP] # ... and store it to main display
           JMP     [DONE]     # ... then go and deal with bit 6
DISP0_1:   LDA     1          # Load acc with 1
           STA     [MAINDISP] # ... and store it to main display
 
########## Do it all again
DONE:      JMP     [GETKEY]   # Jump back and wait for new key
 
#######################################################################
## End of main program body                                          ##
#######################################################################
 
 
#######################################################################
## Start of subroutines                                              ##
#######################################################################
 
#######################################################################
## End of subroutines                                                ##
#######################################################################
 
 
#######################################################################
## Start of global data                                              ##
#######################################################################
TEMP8:    .BYTE               # 8-bit temp location to store data
#######################################################################
## End of global data                                                ##
#######################################################################
 
          .END
                # That's all folks

## 16b - Binary Display, Loop

# ================================================================
# 16b_lab3a_bin_display_loop.asm
# Beboputer Hands-On Tutorial — Lab 3a (version 2)
#
# The same binary display as 16a - as one loop instead of eight
# copy-pasted blocks.
#
# What this program does
# -----------------------
#   Behaves identically to 16a: wait for a key, clear the display,
#   show '%', then show that key's raw code as eight binary digits.
#   The visible result on the display is exactly the same. What's
#   different is entirely inside the code.
#
# How it works
# -------------
#   16a's eight nearly-identical TEST blocks collapse into one block
#   that runs eight times, counted down by the Index register:
#
#     BLDX    8          # Load X reg with number of bits
#     LOOP:      LDA     [TEMP8]    # Reload ACC with copy of key code
#                SHL                # Shift left 1 bit
#                STA     [TEMP8]    # Store new value in temp location
#                JC      [DISP_1]   # If carry = 1, jump to display a 1
#     DISP_0:    LDA     0          # ... otherwise load acc with 0
#                STA     [MAINDISP] # ... and store it to main display
#                JMP     [DISPDECX] # ... then go and decrement the X reg
#     DISP_1:    LDA     1          # Load acc with 1
#                STA     [MAINDISP] # ... and store it to main display
#     DISPDECX:  DECX               # Decrement the X reg
#                JNZ     [LOOP]     # If not zero jump back for next bit
#
#   BLDX 8 loads the Index register with the loop count (8 bits to
#   show). Each pass through LOOP does the exact same SHL/JC dance
#   16a did per bit, then DECX counts the register down by one and
#   JNZ ("jump if Z flag not set") repeats LOOP as long as X hasn't
#   reached zero yet. The eighth pass leaves X at 0, JNZ stops
#   firing, and the code falls through to DONE.
#
#   This is the same "loop instead of unrolling" idea exercise 15c's
#   countdown loop and 15e's LED chase both use, just applied to a
#   fixed bit-count instead of a changing value or a shifting LED
#   pattern - one small block of code, plus a counter, replaces eight
#   copies of nearly the same thing.
#
# Try it
# -------
#   Assemble, Load -> CPU, click Run, press keys - the display should
#   look identical to 16a's for every key you try.
#
# Watch it happen
# -----------------
#   Open Registers and watch the Index register count down 8, 7, 6,
#   ... 1, 0 as LOOP repeats - compare this to 16a, which has no
#   Index register activity at all (it used the source code itself
#   as its "loop counter," unrolled eight times).
#
# Try this next
# ---------------
#   - Compare the total byte count of 16a vs 16b (Load -> CPU reports
#     size, or check the assembler's listing) - the loop version is
#     dramatically smaller for identical behavior, at the cost of a
#     few extra instructions executed per pass (DECX and JNZ, which
#     16a's unrolled version didn't need).
#   - See exercise 16h and 16i for a further-refined version of this
#     same loop idea, applied with a nybble separator added in.
# ================================================================
 
## Lab 3a binary display using the index (X) register
 
#######################################################################
## Start of constant declarations                                    ##
#######################################################################
MAINDISP: .EQU     $F031      # Address of output port for main display
SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs
KEYPAD:   .EQU     $F011      # Address of input port for keypad
CLRCODE:  .EQU     $10        # Special code to clear the main display
BINMODE:  .EQU     %00000100  # LED code to indicate binary mode
DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode
HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode
#######################################################################
## End of constant declarations                                      ##
#######################################################################
 
          .ORG     $4000      # Set program origin
 
#######################################################################
## Start of initialization                                           ##
#######################################################################
INIT:      LDA     CLRCODE    # Load accumulator with clear code
           STA     [MAINDISP] # Write clear code to main display
           LDA     BINMODE    # Load accumulator with bin mode code
           STA     [SIXLEDS]  # Write to port driving six LEDs
#######################################################################
## End of initialization                                             ##
#######################################################################
 
 
#######################################################################
## Start of main program body                                        ##
#######################################################################
 
########## Wait for key to be pressed
GETKEY:    LDA     [KEYPAD]   # Load ACC with code from keypad
           JN      [GETKEY]   # Jump back if no key pressed
           STA     [TEMP8]    # Store key code in temp location
 
########## Prepare the main display
CLRDISP:   LDA     CLRCODE    # Load ACC with clear code
           STA     [MAINDISP] # Clear main display
DISPPERC:  LDA     $25        # Load ACC with ASCII code for '%'
           STA     [MAINDISP] # Write '%' to main display
 
########## Display the binary value
           BLDX    8          # Load X reg with number of bits
LOOP:      LDA     [TEMP8]    # Reload ACC with copy of key code
           SHL                # Shift left 1 bit
           STA     [TEMP8]    # Store new value in temp location
           JC      [DISP_1]   # If carry = 1, jump to display a 1
DISP_0:    LDA     0          # ... otherwise load acc with 0
           STA     [MAINDISP] # ... and store it to main display
           JMP     [DISPDECX] # ... then go and decrement the X reg
DISP_1:    LDA     1          # Load acc with 1
           STA     [MAINDISP] # ... and store it to main display
DISPDECX:  DECX               # Decrement the X reg
           JNZ     [LOOP]     # If not zero jump back for next bit
 
########## Do it all again
DONE:      JMP     [GETKEY]   # Jump back and wait for new key
 
#######################################################################
## End of main program body                                          ##
#######################################################################
 
 
#######################################################################
## Start of subroutines                                              ##
#######################################################################
 
#######################################################################
## End of subroutines                                                ##
#######################################################################
 
 
#######################################################################
## Start of global data                                              ##
#######################################################################
TEMP8:    .BYTE               # 8-bit temp location to store data
#######################################################################
## End of global data                                                ##
#######################################################################
 
          .END
                # That's all folks

## 16c - Hex Nybble via SHR

# ================================================================
# 16c_lab3a_hex_nibbles_shr.asm
# Beboputer Hands-On Tutorial — Lab 3a (hex display, SHR version)
#
# Display a key's raw code as two hex digits, extracting the
# most-significant nybble with four SHR instructions.
#
# What this program does
# -----------------------
#   Waits for any key, clears the display, writes a '$' (this
#   project's convention for "what follows is hex," the same one
#   tutorial 14 uses), then shows that key's raw code as two hex
#   digit characters. Then it waits for the next key and repeats.
#
# How it works
# -------------
#   Two nybbles, two ways of getting them - and this version's most
#   significant nybble extraction is about as simple as it gets:
#
#     DISPMSN:   LDA     [TEMP8]    # Reload ACC with copy of key code
#                SHR                # Shift right 1 bit (= 1 bit shift)
#                SHR                # Shift right 1 bit (= 2 bit shift)
#                SHR                # Shift right 1 bit (= 3 bit shift)
#                SHR                # Shift right 1 bit (= 4 bit shift)
#                AND     %00001111  # Clear MS 4 bits (not really necessary)
#                STA     [MAINDISP] # Copy result to main display
#
#   SHR shifts every bit right one place and fills the vacated top
#   bit with 0 - do that four times and the byte's top nybble has
#   slid down into the bottom nybble, with zeros above it. The
#   trailing AND $0F is (as the comment admits) not strictly needed
#   here, since four SHRs on an 8-bit byte already leave the top
#   nybble as zero - but it's cheap insurance, and it matches the
#   pattern used for the least-significant nybble right below it:
#
#     DISPLSN:   LDA     [TEMP8]    # Reload ACC with copy of key code
#                AND     %00001111  # Mask out (clear) MS nybble
#                STA     [MAINDISP] # Copy result to main display
#
#   No shifting needed for the low nybble - it's already sitting in
#   the bottom 4 bits, so a single AND is enough to blank out the top
#   nybble before displaying it. This exact SHR + AND nybble-split is
#   the same technique tutorial 14's hex/bin conversion tutorial uses
#   to turn a 4-bit binary value back into a hex digit character.
#
#   This tutorial and the next three (16d, 16e, 16f) all extract the
#   exact same most-significant nybble, from the exact same byte,
#   and display the exact same result - only the instruction used to
#   do the shifting changes. Compare them side by side to see how
#   much (or how little) extra bookkeeping each approach costs.
#
# Try it
# -------
#   Assemble, Load -> CPU, click Run, press any key - its raw code
#   appears as two hex digits after a '$'.
#
# Watch it happen
# -----------------
#   Step through DISPMSN and watch the accumulator in the Registers
#   panel shift right one nybble's worth over the four SHRs.
#
# Try this next
# ---------------
#   - Compare against 16d (SHL + manual carry reinsertion), 16e
#     (ROLC + explicit bit0 override), and 16f (RORC, no manual
#     bit-forcing needed at all) - four different roads to the same
#     nybble.
# ================================================================
 
## Lab 3a hex display using SHR instructions
 
#######################################################################
## Start of constant declarations                                    ##
#######################################################################
MAINDISP: .EQU     $F031      # Address of output port for main display
SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs
KEYPAD:   .EQU     $F011      # Address of input port for keypad
CLRCODE:  .EQU     $10        # Special code to clear the main display
BINMODE:  .EQU     %00000100  # LED code to indicate binary mode
DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode
HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode
#######################################################################
## End of constant declarations                                      ##
#######################################################################
 
          .ORG     $4000      # Set program origin
 
#######################################################################
## Start of initialization                                           ##
#######################################################################
INIT:      LDA     CLRCODE    # Load accumulator with clear code
           STA     [MAINDISP] # Write clear code to main display
           LDA     HEXMODE    # Load accumulator with hex mode code
           STA     [SIXLEDS]  # Write to port driving six LEDs
#######################################################################
## End of initialization                                             ##
#######################################################################
 
 
#######################################################################
## Start of main program body                                        ##
#######################################################################
 
########## Wait for key to be pressed
GETKEY:    LDA     [KEYPAD]   # Load ACC with code from keypad
           JN      [GETKEY]   # Jump back if no key pressed
           STA     [TEMP8]    # Store key code in temp location
 
########## Prepare the main display
CLRDISP:   LDA     CLRCODE    # Load ACC with clear code
           STA     [MAINDISP] # Clear main display
DISPDOLL:  LDA     $24        # Load ACC with ASCII code for '$'
           STA     [MAINDISP] # Write '$' to main display
 
########## Extract and display the most-significant nybble
DISPMSN:   LDA     [TEMP8]    # Reload ACC with copy of key code
           SHR                # Shift right 1 bit (= 1 bit shift)
           SHR                # Shift right 1 bit (= 2 bit shift)
           SHR                # Shift right 1 bit (= 3 bit shift)
           SHR                # Shift right 1 bit (= 4 bit shift)
           AND     %00001111  # Clear MS 4 bits (not really necessary)
           STA     [MAINDISP] # Copy result to main display
 
########## Extract and display the least-significant nybble
DISPLSN:   LDA     [TEMP8]    # Reload ACC with copy of key code
           AND     %00001111  # Mask out (clear) MS nybble
           STA     [MAINDISP] # Copy result to main display
 
########## Do it all again
DONE:      JMP     [GETKEY]   # Jump back and wait for new key
 
#######################################################################
## End of main program body                                          ##
#######################################################################
 
 
#######################################################################
## Start of subroutines                                              ##
#######################################################################
 
#######################################################################
## End of subroutines                                                ##
#######################################################################
 
 
#######################################################################
## Start of global data                                              ##
#######################################################################
TEMP8:    .BYTE               # 8-bit temp location to store data
#######################################################################
## End of global data                                                ##
#######################################################################
 
          .END                # That's all folks

## 16d - Hex Nybble via SHL

# ================================================================
# 16d_lab3a_hex_nibbles_shl.asm
# Beboputer Hands-On Tutorial — Lab 3a (hex display, SHL version)
#
# The same most-significant nybble as 16c - extracted with SHL and a
# manual carry reinsertion trick instead of plain SHR.
#
# What this program does
# -----------------------
#   Identical behavior to 16c: wait for a key, show '$', then show
#   that key's raw code as two hex digits.
#
# How it works
# -------------
#   The least-significant nybble (DISPLSN) is unchanged from 16c -
#   a single AND does the job. The most-significant nybble is where
#   this version does something less obvious:
#
#     DISPMSN:   BLDX     4         # Load X reg with number of shifts
#                LDA     [TEMP8]    # Reload ACC with copy of key code
#     DISPMSNL:  SHL                # Shift left 1 bit
#                JNC     [DISPMSN0] # Jump if carry flag = 0
#                OR      %00000001  #   otherwise set LS bit to 1
#     DISPMSN0:  DECX               # Decrement index register
#                JNZ     [DISPMSNL] # If X !=0 jump back and shift again
#                AND     %00001111  # Clear MS 4 bits (not really necessary)
#                STA     [MAINDISP] # Copy result to main display
#
#   SHL shifts left and drops the bit that falls off the top edge
#   into Carry - the opposite direction from what you'd want for
#   pulling a high nybble down to the bottom. The trick is what
#   happens right after each shift: whatever bit SHL just discarded
#   into Carry gets manually stuffed back in at the bottom via
#   JNC/OR. Trace it through with TEMP8 = $A7 (1010 0111) and this
#   becomes clear:
#
#       start:  1010 0111
#       SHL #1: 0100 1110, carry=1 -> OR sets bit0: 0100 1111
#       SHL #2: 1001 1110, carry=0 -> no OR:        1001 1110
#       SHL #3: 0011 1100, carry=1 -> OR sets bit0:  0011 1101
#       SHL #4: 0111 1010, carry=0 -> no OR:        0111 1010
#       AND $0F:                                    0000 1010  = $0A
#
#   $A7's top nybble is $A (1010) - and that's exactly what comes
#   out. Four rounds of "shift left, then immediately recycle the
#   bit that fell out back into the bottom" amounts to rotating the
#   whole byte left by four places, done one bit at a time with the
#   carry flag standing in for the "wraparound" a true rotate would
#   handle automatically. BLDX 4 / DECX / JNZ counts the four rounds,
#   the same loop-counting pattern 16b uses for its eight bits.
#
# Try it
# -------
#   Assemble, Load -> CPU, click Run, press keys - results should
#   match 16c exactly for every key.
#
# Watch it happen
# -----------------
#   Step through DISPMSNL four times and watch both the accumulator
#   and the Carry flag in the Registers panel - each pass, whatever
#   bit lands in Carry gets folded straight back into bit 0.
#
# Try this next
# ---------------
#   - See 16e for the same "recycle the bit that fell out" idea done
#     with ROLC instead of a plain SHL, and 16f for the direction
#     (RORC) that needs no manual bit-recycling at all.
# ================================================================
 
## Lab 3a hex display using SHL instructions
 
#######################################################################
## Start of constant declarations                                    ##
#######################################################################
MAINDISP: .EQU     $F031      # Address of output port for main display
SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs
KEYPAD:   .EQU     $F011      # Address of input port for keypad
CLRCODE:  .EQU     $10        # Special code to clear the main display
BINMODE:  .EQU     %00000100  # LED code to indicate binary mode
DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode
HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode
#######################################################################
## End of constant declarations                                      ##
#######################################################################
 
          .ORG     $4000      # Set program origin
 
#######################################################################
## Start of initialization                                           ##
#######################################################################
INIT:      LDA     CLRCODE    # Load accumulator with clear code
           STA     [MAINDISP] # Write clear code to main display
           LDA     HEXMODE    # Load accumulator with hex mode code
           STA     [SIXLEDS]  # Write to port driving six LEDs
#######################################################################
## End of initialization                                             ##
#######################################################################
 
 
#######################################################################
## Start of main program body                                        ##
#######################################################################
 
########## Wait for key to be pressed
GETKEY:    LDA     [KEYPAD]   # Load ACC with code from keypad
           JN      [GETKEY]   # Jump back if no key pressed
           STA     [TEMP8]    # Store key code in temp location
 
########## Prepare the main display
CLRDISP:   LDA     CLRCODE    # Load ACC with clear code
           STA     [MAINDISP] # Clear main display
DISPDOLL:  LDA     $24        # Load ACC with ASCII code for '$'
           STA     [MAINDISP] # Write '$' to main display
 
########## Extract and display the most-significant nybble
DISPMSN:   BLDX     4         # Load X reg with number of shifts
           LDA     [TEMP8]    # Reload ACC with copy of key code
DISPMSNL:  SHL                # Shift left 1 bit
           JNC     [DISPMSN0] # Jump if carry flag = 0
           OR      %00000001  #   otherwise set LS bit to 1
DISPMSN0:  DECX               # Decrement index register
           JNZ     [DISPMSNL] # If X !=0 jump back and shift again
           AND     %00001111  # Clear MS 4 bits (not really necessary)
           STA     [MAINDISP] # Copy result to main display
 
########## Extract and display the least-significant nybble
DISPLSN:   LDA     [TEMP8]    # Reload ACC with copy of key code
           AND     %00001111  # Mask out (clear) MS nybble
           STA     [MAINDISP] # Copy result to main display
 
########## Do it all again
DONE:      JMP     [GETKEY]   # Jump back and wait for new key
 
#######################################################################
## End of main program body                                          ##
#######################################################################
 
 
#######################################################################
## Start of subroutines                                              ##
#######################################################################
 
#######################################################################
## End of subroutines                                                ##
#######################################################################
 
 
#######################################################################
## Start of global data                                              ##
#######################################################################
TEMP8:    .BYTE               # 8-bit temp location to store data
#######################################################################
## End of global data                                                ##
#######################################################################
 
          .END                # That's all folks

## 16e - Hex Nybble via ROLC

# ================================================================
# 16e_lab3a_hex_nibbles_rolc.asm
# Beboputer Hands-On Tutorial — Lab 3a (hex display, ROLC version)
#
# The same most-significant nybble as 16c/16d - this time using
# ROLC, and still forcing bit 0 by hand rather than trusting ROLC's
# own carry-in.
#
# What this program does
# -----------------------
#   Identical behavior to 16c and 16d: wait for a key, show '$',
#   then show that key's raw code as two hex digits.
#
# How it works
# -------------
#   ROLC ("rotate left through carry") is built for exactly this
#   kind of bit recycling - it shifts the accumulator left, moves the
#   old top bit into Carry, and moves the *previous* Carry flag value
#   into the new bit 0, all in one instruction. That sounds like it
#   should make 16d's manual OR unnecessary. This version, though,
#   doesn't lean on that automatic bit-0 fill at all - it overrides
#   it explicitly every single time:
#
#     DISPMSN:   BLDX     4         # Load X reg with number of shifts
#                LDA     [TEMP8]    # Reload ACC with copy of key code
#     DISPMSNL:  ROLC               # Shift left 1 bit
#                JNC     [DISPMSN0] # Jump if carry flag = 0
#     DISPMNS1:  OR      %00000001  #   otherwise set LS bit to 1
#                JMP     [DISPDECX] #   then jump to decrement X reg
#     DISPMSN0:  AND     %11111110  # Clear LS bit to 0
#     DISPDECX:  DECX               # Decrement index register
#                JNZ     [DISPMSNL] # If X !=0 jump back and shift again
#                AND     %00001111  # Clear MS 4 bits (not really necessary)
#                STA     [MAINDISP] # Copy result to main display
#
#   The JNC test here is checking the *new* Carry flag that this
#   ROLC just produced - the bit that fell off the top - exactly the
#   same bit 16d's SHL produced. Whichever way that test comes out,
#   this code then forces bit 0 to match it by hand: OR to set it,
#   AND $FE to clear it. Whatever ROLC itself had already written
#   into bit 0 (from the Carry flag's value *before* this instruction
#   ran - old, stale information left over from some earlier
#   operation) gets stomped on and replaced immediately.
#
#   Put another way: this program uses ROLC only for the "shift left
#   and capture the bit that fell out in Carry" half of what ROLC
#   does, and throws away the "fill bit 0 from the old Carry" half by
#   overwriting it a line later. It ends up doing the exact same
#   work as 16d's SHL version, with one extra instruction (AND $FE)
#   in the path where the bit was a 0, since SHL already leaves
#   bit 0 at 0 for free while ROLC does not.
#
# Try it
# -------
#   Assemble, Load -> CPU, click Run, press keys - results should
#   match 16c and 16d exactly for every key.
#
# Watch it happen
# -----------------
#   Step through DISPMSNL and compare the accumulator's bit 0 right
#   after ROLC executes against what it becomes right after the
#   following OR/AND - watch the explicit override happen in real
#   time in the Registers panel.
#
# Try this next
# ---------------
#   - Remove the OR/AND overrides and see what garbage turns up in
#     the displayed nybble - Carry's value going into the very first
#     ROLC is whatever was left over from CLRDISP's LDA/STA pair, not
#     necessarily 0, so the "automatic" bit-0 fill can't be trusted
#     here without first clearing Carry.
#   - Compare against 16f, which uses RORC and needs no bit-forcing
#     at all because the final AND mask cleans up any leftover
#     carry-chain bits regardless of what they were.
# ================================================================
 
## Lab 3a hex display using ROLC instructions
 
#######################################################################
## Start of constant declarations                                    ##
#######################################################################
MAINDISP: .EQU     $F031      # Address of output port for main display
SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs
KEYPAD:   .EQU     $F011      # Address of input port for keypad
CLRCODE:  .EQU     $10        # Special code to clear the main display
BINMODE:  .EQU     %00000100  # LED code to indicate binary mode
DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode
HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode
#######################################################################
## End of constant declarations                                      ##
#######################################################################
 
          .ORG     $4000      # Set program origin
 
#######################################################################
## Start of initialization                                           ##
#######################################################################
INIT:      LDA     CLRCODE    # Load accumulator with clear code
           STA     [MAINDISP] # Write clear code to main display
           LDA     HEXMODE    # Load accumulator with hex mode code
           STA     [SIXLEDS]  # Write to port driving six LEDs
#######################################################################
## End of initialization                                             ##
#######################################################################
 
 
#######################################################################
## Start of main program body                                        ##
#######################################################################
 
########## Wait for key to be pressed
GETKEY:    LDA     [KEYPAD]   # Load ACC with code from keypad
           JN      [GETKEY]   # Jump back if no key pressed
           STA     [TEMP8]    # Store key code in temp location
 
########## Prepare the main display
CLRDISP:   LDA     CLRCODE    # Load ACC with clear code
           STA     [MAINDISP] # Clear main display
DISPDOLL:  LDA     $24        # Load ACC with ASCII code for '$'
           STA     [MAINDISP] # Write '$' to main display
 
########## Extract and display the most-significant nybble
DISPMSN:   BLDX     4         # Load X reg with number of shifts
           LDA     [TEMP8]    # Reload ACC with copy of key code
DISPMSNL:  ROLC               # Shift left 1 bit
           JNC     [DISPMSN0] # Jump if carry flag = 0
DISPMNS1:  OR      %00000001  #   otherwise set LS bit to 1
           JMP     [DISPDECX] #   then jump to decrement X reg
DISPMSN0:  AND     %11111110  # Clear LS bit to 0
DISPDECX:  DECX               # Decrement index register
           JNZ     [DISPMSNL] # If X !=0 jump back and shift again
           AND     %00001111  # Clear MS 4 bits (not really necessary)
           STA     [MAINDISP] # Copy result to main display
 
########## Extract and display the least-significant nybble
DISPLSN:   LDA     [TEMP8]    # Reload ACC with copy of key code
           AND     %00001111  # Mask out (clear) MS nybble
           STA     [MAINDISP] # Copy result to main display
 
########## Do it all again
DONE:      JMP     [GETKEY]   # Jump back and wait for new key
 
#######################################################################
## End of main program body                                          ##
#######################################################################
 
 
#######################################################################
## Start of subroutines                                              ##
#######################################################################
 
#######################################################################
## End of subroutines                                                ##
#######################################################################
 
 
#######################################################################
## Start of global data                                              ##
#######################################################################
TEMP8:    .BYTE               # 8-bit temp location to store data
#######################################################################
## End of global data                                                ##
#######################################################################
 
          .END                # That's all folks

## 16f - Hex Nybble via RORC

# ================================================================
# 16f_lab3a_hex_nibbles_rorc.asm
# Beboputer Hands-On Tutorial — Lab 3a (hex display, RORC version)
#
# The same most-significant nybble as 16c/16d/16e - four plain RORCs
# and a trailing mask, no bit-forcing needed anywhere.
#
# What this program does
# -----------------------
#   Identical behavior to the other three hex-display versions: wait
#   for a key, show '$', then show that key's raw code as two hex
#   digits.
#
# How it works
# -------------
#   Where 16e's ROLC needed explicit AND/OR overrides to control what
#   landed in bit 0 each pass, this version just runs RORC four times
#   in a row and masks off whatever's left afterward:
#
#     DISPMSN:   LDA     [TEMP8]    # Reload ACC with copy of key code
#                RORC               # Shift right 1 bit (= 1 bit shift)
#                RORC               # Shift right 1 bit (= 2 bit shift)
#                RORC               # Shift right 1 bit (= 3 bit shift)
#                RORC               # Shift right 1 bit (= 4 bit shift)
#                AND     %00001111  # Clear MS 4 bits (not really necessary)
#                STA     [MAINDISP] # Copy result to main display
#
#   RORC ("rotate right through carry") shifts every bit right one
#   place, moves the old bit 0 into Carry, and fills the new top bit
#   (bit 7) from whatever Carry held *before* this instruction ran.
#   That incoming Carry value is unpredictable here - it's whatever
#   was left over from earlier arithmetic, not something this code
#   bothers to clear first. So after four RORCs, the top nybble of
#   the result may have genuine leftover carry-chain bits mixed into
#   it, not necessarily zeros.
#
#   Unlike 16e, this version doesn't care. The AND $0F at the end
#   throws away the entire top nybble regardless of what ended up
#   there, keeping only the bottom nybble - which, after four
#   right-shifts, correctly holds the original top nybble of TEMP8.
#   Whatever RORC put in the bits the mask discards simply never
#   matters. That's the whole difference between this version and
#   16e's: 16e controls bit 0 on every pass because it needs the
#   *final* top nybble to already be clean; this version lets bit 7
#   be whatever it wants to be on every pass, because the final AND
#   cleans up after all four passes at once instead of after each one.
#
# Try it
# -------
#   Assemble, Load -> CPU, click Run, press keys - results should
#   match 16c, 16d, and 16e exactly for every key, despite RORC's
#   top-bit fill being unpredictable along the way.
#
# Watch it happen
# -----------------
#   Step through all four RORCs and watch the accumulator's top
#   nybble in the Registers panel end up different-looking after
#   each pass depending on Carry's incoming state - then watch the
#   final AND $0F erase all of that regardless.
#
# Try this next
# ---------------
#   - Deliberately set Carry to 1 before DISPMSN runs (e.g. add a
#     CMPA that's guaranteed to set it) and confirm the displayed
#     nybble is unaffected - proof that AND $0F really does discard
#     whatever landed in the top nybble.
#   - Compare the byte count of all four hex-nybble versions (16c
#     through 16f) - SHR is the shortest, needing no loop and no
#     bit-forcing at all.
# ================================================================
 
## Lab 3a hex display using RORC instructions
 
#######################################################################
## Start of constant declarations                                    ##
#######################################################################
MAINDISP: .EQU     $F031      # Address of output port for main display
SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs
KEYPAD:   .EQU     $F011      # Address of input port for keypad
CLRCODE:  .EQU     $10        # Special code to clear the main display
BINMODE:  .EQU     %00000100  # LED code to indicate binary mode
DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode
HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode
#######################################################################
## End of constant declarations                                      ##
#######################################################################
 
          .ORG     $4000      # Set program origin
 
#######################################################################
## Start of initialization                                           ##
#######################################################################
INIT:      LDA     CLRCODE    # Load accumulator with clear code
           STA     [MAINDISP] # Write clear code to main display
           LDA     HEXMODE    # Load accumulator with hex mode code
           STA     [SIXLEDS]  # Write to port driving six LEDs
#######################################################################
## End of initialization                                             ##
#######################################################################
 
 
#######################################################################
## Start of main program body                                        ##
#######################################################################
 
########## Wait for key to be pressed
GETKEY:    LDA     [KEYPAD]   # Load ACC with code from keypad
           JN      [GETKEY]   # Jump back if no key pressed
           STA     [TEMP8]    # Store key code in temp location
 
########## Prepare the main display
CLRDISP:   LDA     CLRCODE    # Load ACC with clear code
           STA     [MAINDISP] # Clear main display
DISPDOLL:  LDA     $24        # Load ACC with ASCII code for '$'
           STA     [MAINDISP] # Write '$' to main display
 
########## Extract and display the most-significant nybble
DISPMSN:   LDA     [TEMP8]    # Reload ACC with copy of key code
           RORC               # Shift right 1 bit (= 1 bit shift)
           RORC               # Shift right 1 bit (= 2 bit shift)
           RORC               # Shift right 1 bit (= 3 bit shift)
           RORC               # Shift right 1 bit (= 4 bit shift)
           AND     %00001111  # Clear MS 4 bits (not really necessary)
           STA     [MAINDISP] # Copy result to main display
 
########## Extract and display the least-significant nybble
DISPLSN:   LDA     [TEMP8]    # Reload ACC with copy of key code
           AND     %00001111  # Mask out (clear) MS nybble
           STA     [MAINDISP] # Copy result to main display
 
########## Do it all again
DONE:      JMP     [GETKEY]   # Jump back and wait for new key
 
#######################################################################
## End of main program body                                          ##
#######################################################################
 
 
#######################################################################
## Start of subroutines                                              ##
#######################################################################
 
#######################################################################
## End of subroutines                                                ##
#######################################################################
 
 
#######################################################################
## Start of global data                                              ##
#######################################################################
TEMP8:    .BYTE               # 8-bit temp location to store data
#######################################################################
## End of global data                                                ##
#######################################################################
 
          .END                # That's all folks

## 16g - Accumulator Wraparound

# ================================================================
# 16g_lab3b_accumulator_wraparound.asm
# Beboputer Hands-On Tutorial — Lab 3b
#
# Watch an 8-bit accumulator count all the way up and wrap back
# around to zero.
#
# What this program does
# -----------------------
#   Starts the accumulator at 0, counts it up by 1 over and over,
#   and stops the instant it wraps back around to 0 again - which
#   takes exactly 255 increments, since an 8-bit register can only
#   hold values 0 through 255.
#
# How it works
# -------------
#   The entire program is four lines:
#
#     LDA         0       # Load accumulator with zero
#     LOOP:      INCA                # Increment the accumulator
#                JNZ     [LOOP]      # Jump to LOOP if ACC !=0
#                JMP     [$0000]     # Jump to address $0000
#
#   INCA adds 1 to the accumulator and sets the Zero flag exactly
#   when the result is 0. The first time through, ACC goes 0 -> 1,
#   Z is clear, JNZ ("jump if Z flag not set") loops back. This
#   keeps happening - 1, 2, 3, ... - climbing all the way up through
#   255 ($FF). The 256th increment is where it gets interesting:
#   an 8-bit register has no 257th value to hold, so 255 + 1 wraps
#   around to 0 instead of becoming 256. That's the one and only
#   time INCA's result is 0, so that's the one and only time JNZ
#   stops firing, and the loop falls through to the terminating jump.
#
#   This is the same "jump to zeroed memory" stopping trick exercise
#   09 and Lab 2a both explain (RAM below $4000 is zeroed at power-on,
#   and a zeroed byte decodes as NOP, so the CPU just spins on NOPs
#   forever once it lands there) - there's nothing displayed, no
#   keypad read, nothing but the accumulator counting and wrapping.
#   It's a stripped down, minimal way to see 8-bit overflow happen
#   with your own eyes in the debugger, without any display logic
#   getting in the way of the one concept being demonstrated.
#
# Try it
# -------
#   Assemble, Load -> CPU, and switch to Step mode (Run will finish
#   this in a flash since there's nothing slowing it down - Step is
#   how you actually watch it happen). Step through and count how
#   many times INCA runs before Z finally sets.
#
# Watch it happen
# -----------------
#   Open Registers and watch the accumulator and the Z flag as you
#   Step: ACC counts 1, 2, 3, ... up to 255 ($FF), then the next
#   INCA takes it to $00 and only then does Z flip to 1 - the loop
#   runs 255 times, not 256, because 0 was the starting value the
#   loop counted up *from*, not one it counted through again until
#   the wrap.
# ================================================================
 
## Lab 3b - Simple program to demonstrate the way in which
##          the program counter works
 
          .ORG     $4000         # Set program's origin to address $4000
           LDA         0       # Load accumulator with zero
LOOP:      INCA                # Increment the accumulator
           JNZ     [LOOP]      # Jump to LOOP if ACC !=0
           JMP     [$0000]     # Jump to address $0000
          .END                 # Terminate the program

## 16h - Binary Display, Restructured

# ================================================================
# 16h_lab3c_bin_display_restructured.asm
# Beboputer Hands-On Tutorial — Lab 3c (binary display, restructured)
#
# The same 8-bit binary display as 16b - relabeled and reorganized,
# with the exact same logic underneath.
#
# What this program does
# -----------------------
#   Behaves identically to 16b: wait for a key, clear the display,
#   show '%', then show that key's raw code as eight binary digits.
#
# How it works
# -------------
#   Put this side by side with 16b and the two are almost the same
#   program - the same BLDX/SHL/JC/DECX/JNZ loop, the same overall
#   shape:
#
#     LOADX:     BLDX    8           # Load X reg with 8
#     LOOP:      LDA     [TEMP8]     # Reload ACC from temp location
#                SHL                 # Shift left 1 bit
#                STA     [TEMP8]     # Store new value in temp location
#                JC      [DISP_1]    # If carry = 1, jump to display a 1
#     DISP_0:    LDA     0           # ... otherwise load acc with 0
#                STA     [MAINDISP]  # ... and store it to main display
#                JMP     [DEALWX]    # ... then go and deal with the X reg
#     DISP_1:    LDA     1           # Load acc with 1
#                STA     [MAINDISP]  # ... and store it to main display
#     DEALWX:    DECX                # Decrement the index register
#                JNZ     [LOOP]      # If X not zero then do next bit
#                JMP     [GETKEY]    # Go back and wait for new key
#
#   Compare this to 16b's version of the same loop and the only real
#   differences are cosmetic: labels renamed (LOADX/DEALWX here vs
#   BLDX's bare instruction and DISPDECX in 16b), and the "go back to
#   GETKEY" jump moved to the bottom of the loop body instead of
#   living under its own DONE label. Every instruction that actually
#   runs, and the order it runs in, is identical.
#
#   This file is a good exercise in reading assembly for behavior
#   rather than appearance: two source files that don't look alike
#   line-for-line can still assemble to functionally (and here,
#   almost byte-for-byte) identical programs. Worth comparing labels
#   and structure against 16b directly to see exactly what changed
#   and what didn't.
#
# Try it
# -------
#   Assemble, Load -> CPU, click Run, press keys - the display should
#   look identical to 16b's for every key you try.
#
# Watch it happen
# -----------------
#   Open Display -> Port Map Status and compare a Step-through of
#   this file against 16b's - the sequence of port writes to $F031
#   should match exactly, key for key.
#
# Try this next
# ---------------
#   - See 16i for this same loop split into two 4-bit passes with a
#     '-' separator in between, grouping the output as "XXXX-XXXX"
#     instead of one unbroken run of 8 digits.
# ================================================================
 
## Lab 3c - New version of binary display using index (X) reg
 
#######################################################################
## Start of constant declarations                                    ##
#######################################################################
MAINDISP: .EQU     $F031      # Address of output port for main display
SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs
KEYPAD:   .EQU     $F011      # Address of input port for keypad
CLRCODE:  .EQU     $10        # Special code to clear the main display
BINMODE:  .EQU     %00000100  # LED code to indicate binary mode
DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode
HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode
#######################################################################
## End of constant declarations                                      ##
#######################################################################
 
          .ORG     $4000      # Set program origin
 
#######################################################################
## Start of initialization                                           ##
#######################################################################
INIT:      LDA     CLRCODE    # Load accumulator with clear code
           STA     [MAINDISP] # Write clear code to main display
           LDA     BINMODE    # Load accumulator with bin mode code
           STA     [SIXLEDS]  # Write to port driving six LEDs
#######################################################################
## End of initialization                                             ##
#######################################################################
 
 
#######################################################################
## Start of main program body                                        ##
#######################################################################
 
########## Get and display key codes in binary
## Wait for a key to be pressed
GETKEY:    LDA     [KEYPAD]    # Load ACC from the keypad
           JN      [GETKEY]    # Jump back if no key pressed
           STA     [TEMP8]     # Store ACC in temp location
 
## Clear display then display the '%' character
CLRDISP:   LDA     CLRCODE     # Load ACC with clear code
           STA     [MAINDISP]  # Clear main display
DISPPERC:  LDA     $25         # Load ACC with ASCII code for '%'
           STA     [MAINDISP]  # Write '%' to main display
 
## Initialize index register
LOADX:     BLDX    8           # Load X reg with 8
 
## Loop around writing
LOOP:      LDA     [TEMP8]     # Reload ACC from temp location
           SHL                 # Shift left 1 bit
           STA     [TEMP8]     # Store new value in temp location
           JC      [DISP_1]    # If carry = 1, jump to display a 1
DISP_0:    LDA     0           # ... otherwise load acc with 0
           STA     [MAINDISP]  # ... and store it to main display
           JMP     [DEALWX]    # ... then go and deal with the X reg
DISP_1:    LDA     1           # Load acc with 1
           STA     [MAINDISP]  # ... and store it to main display
DEALWX:    DECX                # Decrement the index register
           JNZ     [LOOP]      # If X not zero then do next bit
           JMP     [GETKEY]    # Go back and wait for new key
 
#######################################################################
## End of main program body                                          ##
#######################################################################
 
 
#######################################################################
## Start of subroutines                                              ##
#######################################################################
 
#######################################################################
## End of subroutines                                                ##
#######################################################################
 
 
#######################################################################
## Start of global data                                              ##
#######################################################################
 
TEMP8:    .BYTE                # Temp  8-bit (1-byte) location
 
#######################################################################
## End of global data                                                ##
#######################################################################
 
          .END                # That's all folks

## 16i - Binary Display, Nybble + Dash

# ================================================================
# 16i_lab3c_bin_display_nibble_dash.asm
# Beboputer Hands-On Tutorial — Lab 3c (binary display, nybble
# groups with a dash separator)
#
# Show a key's raw code as two groups of 4 binary digits with a
# '-' between them, instead of 8 digits running together.
#
# What this program does
# -----------------------
#   Waits for a key, clears the display, shows '%', then shows the
#   key's raw code as "XXXX-XXXX" - four binary digits, a dash, then
#   four more - instead of 16h's unbroken run of eight digits.
#
# How it works
# -------------
#   This takes 16h's single 8-count loop and splits it into two
#   separate 4-count loops with a dash written in between:
#
#     LOADXA:    BLDX    4           # Load X reg with 4
#     LOOPA:     LDA     [TEMP8]     # Reload ACC from temp location
#                SHL                 # Shift left 1 bit
#                STA     [TEMP8]     # Store new value in temp location
#                JC      [DISP_1A]   # If carry = 1, jump to display a 1
#     DISP_0A:   LDA     0           # ... otherwise load acc with 0
#                STA     [MAINDISP]  # ... and store it to main display
#                JMP     [DEALWXA]   # ... then go and deal with the X reg
#     DISP_1A:   LDA     1           # Load acc with 1
#                STA     [MAINDISP]  # ... and store it to main display
#     DEALWXA:   DECX                # Decrement the index register
#                JNZ     [LOOPA]     # If X not zero then do next bit
#
#     DISPDASH:  LDA     $2D         # Load ACC with ASCII code for '-'
#                STA     [MAINDISP]  # ... and store it to main display
#
#   ... followed by a second, near-identical loop (LOOPB/DEALWXB)
#   that displays the remaining 4 bits the same way. TEMP8 is shared
#   between both halves - it isn't reset in between - so LOOPA
#   naturally consumes the top 4 bits (shifting them out via SHL just
#   like every earlier version in this series) and LOOPB picks up
#   exactly where LOOPA left off, displaying the bottom 4 bits.
#
#   Notice LOOPA and LOOPB are two separate copies of the same
#   four-line loop body, each with its own labels (the 'A' and 'B'
#   suffixes) - a small, deliberate bit of duplication rather than
#   one shared loop, presumably because writing the dash needs to
#   happen exactly once, in between the two halves, rather than
#   being something a single unified loop could express cleanly with
#   just a counter. It's a reasonable trade: two copies of a short
#   loop instead of one loop with an awkward "am I halfway done yet"
#   check bolted on.
#
# Try it
# -------
#   Assemble, Load -> CPU, click Run, press keys - the display shows
#   the same 8 bits as 16h/16b would, but grouped as "XXXX-XXXX".
#
# Watch it happen
# -----------------
#   Open Display -> Port Map Status and watch $F031 receive four
#   digits, then $2D (the dash), then four more digits - compare the
#   timing against 16h, which writes eight digits with no pause.
#
# Try this next
# ---------------
#   - Try merging LOOPA and LOOPB into one loop that runs 8 times and
#     writes a dash exactly when the counter reaches 4 - is the
#     result shorter or longer than keeping two separate loops?
# ================================================================
 
## Lab 3c - New version of binary display using index (X) reg
##          Also display '-' between MS and LS nybbles
 
#######################################################################
## Start of constant declarations                                    ##
#######################################################################
MAINDISP: .EQU     $F031      # Address of output port for main display
SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs
KEYPAD:   .EQU     $F011      # Address of input port for keypad
CLRCODE:  .EQU     $10        # Special code to clear the main display
BINMODE:  .EQU     %00000100  # LED code to indicate binary mode
DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode
HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode
#######################################################################
## End of constant declarations                                      ##
#######################################################################
 
          .ORG     $4000      # Set program origin
 
#######################################################################
## Start of initialization                                           ##
#######################################################################
INIT:      LDA     CLRCODE    # Load accumulator with clear code
           STA     [MAINDISP] # Write clear code to main display
           LDA     BINMODE    # Load accumulator with bin mode code
           STA     [SIXLEDS]  # Write to port driving six LEDs
#######################################################################
## End of initialization                                             ##
#######################################################################
 
 
#######################################################################
## Start of main program body                                        ##
#######################################################################
 
########## Get and display key codes in binary
## Wait for a key to be pressed
GETKEY:    LDA     [KEYPAD]    # Load ACC from the keypad
           JN      [GETKEY]    # Jump back if no key pressed
           STA     [TEMP8]     # Store ACC in temp location
 
## Clear display then display the '%' character
CLRDISP:   LDA     CLRCODE     # Load ACC with clear code
           STA     [MAINDISP]  # Clear main display
DISPPERC:  LDA     $25         # Load ACC with ASCII code for '%'
           STA     [MAINDISP]  # Write '%' to main display
 
## Display MS Nybble
LOADXA:    BLDX    4           # Load X reg with 4
LOOPA:     LDA     [TEMP8]     # Reload ACC from temp location
           SHL                 # Shift left 1 bit
           STA     [TEMP8]     # Store new value in temp location
           JC      [DISP_1A]   # If carry = 1, jump to display a 1
DISP_0A:   LDA     0           # ... otherwise load acc with 0
           STA     [MAINDISP]  # ... and store it to main display
           JMP     [DEALWXA]   # ... then go and deal with the X reg
DISP_1A:   LDA     1           # Load acc with 1
           STA     [MAINDISP]  # ... and store it to main display
DEALWXA:   DECX                # Decrement the index register
           JNZ     [LOOPA]     # If X not zero then do next bit
 
## Display a dash character
DISPDASH:  LDA     $2D         # Load ACC with ASCII code for '-'
           STA     [MAINDISP]  # ... and store it to main display
 
## Display MS Nybble
LOADXB:    BLDX    4           # Load X reg with 4
LOOPB:     LDA     [TEMP8]     # Reload ACC from temp location
           SHL                 # Shift left 1 bit
           STA     [TEMP8]     # Store new value in temp location
           JC      [DISP_1B]   # If carry = 1, jump to display a 1
DISP_0B:   LDA     0           # ... otherwise load acc with 0
           STA     [MAINDISP]  # ... and store it to main display
           JMP     [DEALWXB]   # ... then go and deal with the X reg
DISP_1B:   LDA     1           # Load acc with 1
           STA     [MAINDISP]  # ... and store it to main display
DEALWXB:   DECX                # Decrement the index register
           JNZ     [LOOPB]     # If X not zero then do next bit
 
## Do it all again
           JMP     [GETKEY]    # Go back and wait for new key
 
#######################################################################
## End of main program body                                          ##
#######################################################################
 
 
#######################################################################
## Start of subroutines                                              ##
#######################################################################
 
#######################################################################
## End of subroutines                                                ##
#######################################################################
 
 
#######################################################################
## Start of global data                                              ##
#######################################################################
 
TEMP8:    .BYTE                # Temp  8-bit (1-byte) location
 
#######################################################################
## End of global data                                                ##
#######################################################################
 
          .END
                # That's all folks

## 16j - Hex Echo, Forward

# ================================================================
# 16j_lab3c_hex_echo_forward.asm
# Beboputer Hands-On Tutorial — Lab 3c (hex echo, forward order)
#
# Echo hex digit keys 0-F to the display, in the order they're
# pressed - the baseline this tutorial's next two files build on.
#
# What this program does
# -----------------------
#   Sits in a loop reading the keypad. Press any key with a raw code
#   of $0F or less - digits 0-9 or hex letters A-F - and it appears
#   on the display immediately, right after whatever was already
#   there. Press anything else (an operator, Clear, Sin, etc.) and
#   nothing happens.
#
# How it works
# -------------
#   This is Lab 2d's digit-only echo (exercise 15d) with one number
#   changed:
#
#     GETKEY:    LDA     [KEYPAD]   # Load ACC from the keypad
#                JN      [GETKEY]   # Jump back if no key pressed
#                CMPA    $0F        # Compare ACC to $0F
#                JC      [GETKEY]   # Jump back if ACC is bigger
#                STA     [MAINDISP] # ... else store ACC to display
#                JMP     [GETKEY]   # Go and wait for another key
#
#   15d's version compared against $09 to accept only plain digits.
#   This version compares against $0F instead, which - since digit
#   and hex-letter buttons both arrive as their raw nibble value,
#   0-15 (see exercise 10's note on tools/diy_button.py) - lets every
#   value from $00 through $0F through, meaning both the digit keys
#   (0-9) and the hex-letter keys (A-F) get displayed. Raw bytes
#   $00-$09 render as '0'-'9' and $0A-$0F render as 'A'-'F' on this
#   display (the same convention every Calculator exercise in this
#   project relies on), so no ASCII conversion step is needed - the
#   raw nibble value already *is* the character to show.
#
#   Each accepted key just gets appended to whatever's already on
#   the display - there's no clearing, no counting, no memory of
#   what came before. That simplicity is exactly why this file
#   exists: 16k and 16l both build on this same "accept 0-F, do
#   something with it" keypad loop, adding memory (an array, or the
#   Index register's own value) on top of it.
#
# Try it
# -------
#   Assemble, Load -> CPU, click Run, switch to Hex mode, and type a
#   sequence like 1-2-3-A-B - the display fills up left to right in
#   exactly the order you typed, one character per keypress.
#
# Watch it happen
# -----------------
#   Open Display -> Port Map Status and watch $F011 as you press
#   digit and hex-letter keys - both kinds pass the CMPA $0F/JC test
#   and reach $F031, while an operator or Clear key's higher raw code
#   gets looped past.
#
# Try this next
# ---------------
#   - See 16k for the reverse-order version of this same idea (built
#     using an array and the Index register), and 16m for a second,
#     very different way of getting the exact same reversal using
#     the stack instead.
# ================================================================
 
## Lab 3c - Display '0' thru 'F' keys in the order they are entered
 
#######################################################################
## Start of constant declarations                                    ##
#######################################################################
MAINDISP: .EQU     $F031      # Address of output port for main display
SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs
KEYPAD:   .EQU     $F011      # Address of input port for keypad
CLRCODE:  .EQU     $10        # Special code to clear the main display
BINMODE:  .EQU     %00000100  # LED code to indicate binary mode
DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode
HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode
#######################################################################
## End of constant declarations                                      ##
#######################################################################
 
          .ORG     $4000      # Set program origin
 
#######################################################################
## Start of initialization                                           ##
#######################################################################
INIT:      LDA     CLRCODE    # Load accumulator with clear code
           STA     [MAINDISP] # Write clear code to main display
           LDA     HEXMODE    # Load accumulator with hex mode code
           STA     [SIXLEDS]  # Write to port driving six LEDs
#######################################################################
## End of initialization                                             ##
#######################################################################
 
 
#######################################################################
## Start of main program body                                        ##
#######################################################################
 
########## Wait for key to be pressed
GETKEY:    LDA     [KEYPAD]   # Load ACC from the keypad
           JN      [GETKEY]   # Jump back if no key pressed
           CMPA    $0F        # Compare ACC to $0F
           JC      [GETKEY]   # Jump back if ACC is bigger
           STA     [MAINDISP] # ... else store ACC to display
           JMP     [GETKEY]   # Go and wait for another key
 
#######################################################################
## End of main program body                                          ##
#######################################################################
 
 
#######################################################################
## Start of subroutines                                              ##
#######################################################################
 
#######################################################################
## End of subroutines                                                ##
#######################################################################
 
 
#######################################################################
## Start of global data                                              ##
#######################################################################
 
#######################################################################
## End of global data                                                ##
#######################################################################
 
          .END                # That's all folks

## 16k - Hex Echo, Reverse via Array

# ================================================================
# 16k_lab3c_hex_echo_reverse_array.asm
# Beboputer Hands-On Tutorial — Lab 3c (hex echo, reverse order via
# an indexed array)
#
# Collect a sequence of hex-digit keys, then play them back in
# reverse order - the last key typed appears first.
#
# What this program does
# -----------------------
#   Reads hex-digit keys ($00-$0F, same acceptance test as 16j) and
#   stores each one instead of displaying it immediately. The moment
#   any other key is pressed (an operator, Clear, Sin - anything with
#   a raw code above $0F), the program stops collecting and instead
#   plays back everything that was typed, last-typed-first, then
#   terminates.
#
# How it works
# -------------
#   Two phases, both built around the Index register, and a small
#   fixed-size array to hold what's been typed:
#
#     BLDX    $0000       # Load index register with zero
#     GETKEY:    LDA     [KEYPAD]    # Load ACC from the keypad
#                JN      [GETKEY]    # Jump back if no key pressed
#                CMPA    $0F         # Compare ACC to $0F
#                JC      [DISPSTUF]  # Jump if ACC is bigger
#                STA     [STORE,X]   # ... else store ACC to memory
#                INCX                # ... and increment the index reg
#                JMP     [GETKEY]    # Go and wait for another key
#
#   [STORE,X] is indexed addressing (the same technique exercise 09
#   and Lab 2f's message table use for reading): "write to the byte
#   at STORE's address, plus whatever X currently holds." Each
#   accepted key gets written to STORE[0], then STORE[1], then
#   STORE[2], and so on, with INCX advancing to the next slot each
#   time. By the time a non-hex key ends this phase, X holds the
#   *count* of keys that were typed - not their sum or their last
#   value, just how many there were - and STORE[0..X-1] holds them
#   in the order they arrived.
#
#   The playback phase reuses that same count to walk backward:
#
#     DISPSTUF:  LDA     [STORE-1,X] # Load ACC with a key code
#                STA     [MAINDISP]  # Store it to the main display
#                DECX                # Decrement the index register
#                JNZ     [DISPSTUF]  # If index reg not 0 get next code
#                JMP     [$0000]     # Terminate the program
#
#   [STORE-1,X] is indexed addressing with a label-arithmetic offset
#   folded into the operand: the effective address is (STORE - 1) + X.
#   Since X still holds the count from the fill phase, the very first
#   read here lands on (STORE - 1) + count = STORE[count - 1] - the
#   *last* slot that got filled, i.e. the last key typed. DECX then
#   walks that same expression backward one slot at a time - count-2,
#   count-3, ... down to slot 0 - so the whole sequence plays back in
#   the exact opposite order from how it went in.
#
#   One edge case worth knowing about: if the very first key pressed
#   is a non-hex key (so X is still 0 when DISPSTUF begins), this
#   loop reads one byte *before* STORE, displays whatever garbage
#   happens to sit there, and then DECX takes a 16-bit X register
#   from 0 down to $FFFF - not zero, so JNZ loops again, walking
#   backward through memory indefinitely. The lab exercises in this
#   project are teaching tools rather than hardened production code,
#   and this is a good example of the kind of boundary case worth
#   noticing rather than a bug worth "fixing" in the original file -
#   simply always type at least one hex digit before pressing a
#   non-hex key to stay clear of it.
#
# Try it
# -------
#   Assemble, Load -> CPU, click Run, switch to Hex mode, type a
#   short sequence like 1-2-3-A, then press any operator key - the
#   display shows A-3-2-1, the reverse of what you typed.
#
# Watch it happen
# -----------------
#   Open Registers and watch X count up 1, 2, 3, 4 while typing, then
#   count back down 4, 3, 2, 1, 0 during playback - and open Memory
#   to watch the STORE array fill up in typed order during the first
#   phase.
#
# Try this next
# ---------------
#   - Compare this array-based approach against 16m, which gets the
#     exact same last-in-first-out reversal using PUSHA/POPA and the
#     stack instead of a hand-rolled array and index arithmetic.
#   - Add a bounds check against STORE's 10-byte capacity so a very
#     long sequence can't silently run past the reserved area.
# ================================================================
 
## Lab 3c - Display '0' thru 'F' keys in the reverse order to that in
##          which they are entered using the index (X) register
 
#######################################################################
## Start of constant declarations                                    ##
#######################################################################
MAINDISP: .EQU     $F031      # Address of output port for main display
SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs
KEYPAD:   .EQU     $F011      # Address of input port for keypad
CLRCODE:  .EQU     $10        # Special code to clear the main display
BINMODE:  .EQU     %00000100  # LED code to indicate binary mode
DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode
HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode
#######################################################################
## End of constant declarations                                      ##
#######################################################################
 
          .ORG     $4000      # Set program origin
 
#######################################################################
## Start of initialization                                           ##
#######################################################################
INIT:      LDA     CLRCODE    # Load accumulator with clear code
           STA     [MAINDISP] # Write clear code to main display
           LDA     HEXMODE    # Load accumulator with hex mode code
           STA     [SIXLEDS]  # Write to port driving six LEDs
#######################################################################
## End of initialization                                             ##
#######################################################################
 
 
#######################################################################
## Start of main program body                                        ##
#######################################################################
 
########## Initialize the index register
           BLDX    $0000       # Load index register with zero
 
########## Wait for key to be pressed
GETKEY:    LDA     [KEYPAD]    # Load ACC from the keypad
           JN      [GETKEY]    # Jump back if no key pressed
           CMPA    $0F         # Compare ACC to $0F
           JC      [DISPSTUF]  # Jump if ACC is bigger
           STA     [STORE,X]   # ... else store ACC to memory
           INCX                # ... and increment the index reg
           JMP     [GETKEY]    # Go and wait for another key
 
########## Display the numbers
DISPSTUF:  LDA     [STORE-1,X] # Load ACC with a key code
           STA     [MAINDISP]  # Store it to the main display
           DECX                # Decrement the index register
           JNZ     [DISPSTUF]  # If index reg not 0 get next code
           JMP     [$0000]     # Terminate the program
 
#######################################################################
## End of main program body                                          ##
#######################################################################
 
 
#######################################################################
## Start of subroutines                                              ##
#######################################################################
 
#######################################################################
## End of subroutines                                                ##
#######################################################################
 
 
#######################################################################
## Start of global data                                              ##
#######################################################################
 
TEMP8:    .BYTE               # Temp  8-bit (1-byte) location
TEMP16:   .2BYTE              # Temp 16-bit (2-byte) location
STORE:    .BYTE    *10        # Reserve 10 x 1-byte locations
 
#######################################################################
## End of global data                                                ##
#######################################################################
 
          .END                # That's all folks

## 16l - Display the X Register

# ================================================================
# 16l_lab3c_display_x_register.asm
# Beboputer Hands-On Tutorial — Lab 3c (display the Index register
# itself)
#
# Count hex-digit keypresses with the Index register, then display
# the register's own numeric value - as 4 hex digits - instead of
# the keys that were typed.
#
# What this program does
# -----------------------
#   Reads hex-digit keys ($00-$0F, same acceptance test as 16j/16k)
#   and counts them using the Index register - but never stores the
#   individual key values anywhere. The moment a non-hex key is
#   pressed, it shows the *count* of hex digits that were typed, as
#   a 4-digit hex number after a '$'.
#
# How it works
# -------------
#   The fill phase looks almost identical to 16k's, with one telling
#   difference - notice there's no STA [STORE,X] anywhere:
#
#     BLDX    $0000       # Load index register with zero
#     GETKEY:    LDA     [KEYPAD]    # Load ACC from the keypad
#                JN      [GETKEY]    # Jump back if no key pressed
#                CMPA    $0F         # Compare ACC to $0F
#                JC      [DISPSTUF]  # Jump if ACC is bigger
#                STA     [STORE,X]   # ... else store ACC to memory
#                INCX                # ... and increment the index reg
#                JMP     [GETKEY]    # Go and wait for another key
#
#   The keys still get written into STORE (this file borrows the
#   fill loop unchanged), but nothing ever reads STORE back out -
#   this program only cares about X's final count, not the values it
#   counted. That count can run past 255, since the Index register is
#   16 bits wide - something a plain 8-bit accumulator or TEMP8 byte
#   could never hold - and this exercise is specifically about
#   getting that 16-bit value onto an 8-bit-wide display:
#
#     DISPSTUF:  BSTX    [TEMP16]    # Store X reg into 2-byte temp location
#                LDA     $24         # Load ACC with ASCII code for '$'
#                STA     [MAINDISP]  # Write '$' to main display
#
#     DSMSNMSB:  LDA     [TEMP16]    # Load ACC with MS byte from X reg
#                SHR / SHR / SHR / SHR
#                AND     %00001111   # Clear MS 4 bits
#                STA     [MAINDISP]  # Copy result to main display
#
#     DSLSNMSB:  LDA     [TEMP16]    # Reload ACC with MS byte from X reg
#                AND     %00001111   # Mask out (clear) MS nybble
#                STA     [MAINDISP]  # Copy result to main display
#
#   BSTX ("store X register") is the write counterpart to BLDX - it
#   copies the 16-bit Index register out to a 2-byte memory location
#   in one instruction, here TEMP16. From there, showing the count as
#   hex is exactly the SHR + AND nybble-splitting technique 16c uses,
#   just repeated four times instead of twice: once each for the
#   MS and LS nybbles of TEMP16's high byte (DSMSNMSB/DSLSNMSB), then
#   again for TEMP16+1, its low byte (DSMSNLSB/DSLSNLSB) - four
#   nybbles, four hex digits, one 16-bit number.
#
#   Compare this directly against 16k: both programs count typed hex
#   keys with the exact same fill loop, but 16k displays *what* was
#   typed (in reverse), while this one displays *how many* keys were
#   typed (as a 4-digit hex count) - the same raw ingredients, put
#   to two very different uses.
#
# Try it
# -------
#   Assemble, Load -> CPU, click Run, switch to Hex mode, type a
#   dozen or so hex digits, then press an operator key - the display
#   shows something like "$000C" (12 in hex), the count of digits
#   you typed, not the digits themselves.
#
# Watch it happen
# -----------------
#   Open Registers and watch the Index register count up while
#   typing, then open Memory and watch TEMP16 receive that same
#   16-bit value in one step when BSTX executes.
#
# Try this next
# ---------------
#   - Type more than 255 hex digits before stopping and confirm the
#     4-digit hex count correctly shows a value like $0105 - something
#     an 8-bit BSTX target never could have held.
# ================================================================
 
## Lab 3c - Demo of how to display contents of the index (X) register
 
#######################################################################
## Start of constant declarations                                    ##
#######################################################################
MAINDISP: .EQU     $F031      # Address of output port for main display
SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs
KEYPAD:   .EQU     $F011      # Address of input port for keypad
CLRCODE:  .EQU     $10        # Special code to clear the main display
BINMODE:  .EQU     %00000100  # LED code to indicate binary mode
DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode
HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode
#######################################################################
## End of constant declarations                                      ##
#######################################################################
 
          .ORG     $4000      # Set program origin
 
#######################################################################
## Start of initialization                                           ##
#######################################################################
INIT:      LDA     CLRCODE    # Load accumulator with clear code
           STA     [MAINDISP] # Write clear code to main display
           LDA     HEXMODE    # Load accumulator with hex mode code
           STA     [SIXLEDS]  # Write to port driving six LEDs
#######################################################################
## End of initialization                                             ##
#######################################################################
 
 
#######################################################################
## Start of main program body                                        ##
#######################################################################
 
########## Initialize the index register
           BLDX    $0000       # Load index register with zero
 
########## Wait for key to be pressed
GETKEY:    LDA     [KEYPAD]    # Load ACC from the keypad
           JN      [GETKEY]    # Jump back if no key pressed
           CMPA    $0F         # Compare ACC to $0F
           JC      [DISPSTUF]  # Jump if ACC is bigger
           STA     [STORE,X]   # ... else store ACC to memory
           INCX                # ... and increment the index reg
           JMP     [GETKEY]    # Go and wait for another key
 
########## Display the value in the index register
DISPSTUF:  BSTX    [TEMP16]    # Store X reg into 2-byte temp location
           LDA     $24         # Load ACC with ASCII code for '$'
           STA     [MAINDISP]  # Write '$' to main display
 
########## Extract/display MS nybble of MS byte
DSMSNMSB:  LDA     [TEMP16]    # Load ACC with MS byte from X reg
           SHR                 # Shift right 1 bit (= 1 bit shift)
           SHR                 # Shift right 1 bit (= 2 bit shift)
           SHR                 # Shift right 1 bit (= 3 bit shift)
           SHR                 # Shift right 1 bit (= 4 bit shift)
           AND     %00001111   # Clear MS 4 bits
           STA     [MAINDISP]  # Copy result to main display
 
########## Extract/display LS nybble of MS byte
DSLSNMSB:  LDA     [TEMP16]    # Reload ACC with MS byte from X reg
           AND     %00001111   # Mask out (clear) MS nybble
           STA     [MAINDISP]  # Copy result to main display
 
########## Extract/display MS nybble of LS byte
DSMSNLSB:  LDA     [TEMP16+1]  # Load ACC with LS byte from X reg
           SHR                 # Shift right 1 bit (= 1 bit shift)
           SHR                 # Shift right 1 bit (= 2 bit shift)
           SHR                 # Shift right 1 bit (= 3 bit shift)
           SHR                 # Shift right 1 bit (= 4 bit shift)
           AND     %00001111   # Clear MS 4 bits
           STA     [MAINDISP]  # Copy result to main display
 
########## Extract/display LS nybble of LS byte
DSLSNLSB:  LDA     [TEMP16+1]  # Reload ACC with LS byte from X reg
           AND     %00001111   # Mask out (clear) MS nybble
           STA     [MAINDISP]  # Copy result to main display
 
           JMP     [$0000]     # Terminate the program
 
#######################################################################
## End of main program body                                          ##
#######################################################################
 
 
#######################################################################
## Start of subroutines                                              ##
#######################################################################
 
#######################################################################
## End of subroutines                                                ##
#######################################################################
 
 
#######################################################################
## Start of global data                                              ##
#######################################################################
 
TEMP8:    .BYTE               # Temp  8-bit (1-byte) location
TEMP16:   .2BYTE              # Temp 16-bit (2-byte) location
STORE:    .BYTE    *10        # Reserve 10 x 1-byte locations
 
#######################################################################
## End of global data                                                ##
#######################################################################
 
          .END                # That's all folks

## 16m - Hex Echo, Reverse via Stack

# ================================================================
# 16m_lab3d_hex_echo_reverse_stack.asm
# Beboputer Hands-On Tutorial — Lab 3d (hex echo, reverse order via
# the stack)
#
# The same last-typed-first reversal as 16k - this time using
# PUSHA/POPA and the CPU's stack instead of a hand-built array.
#
# What this program does
# -----------------------
#   Behaves like 16k: type a run of hex-digit keys, press any other
#   key, and the digits play back in reverse - last typed, first
#   shown. Unlike 16k, this one never stops - after playing a
#   sequence back, it's ready to collect and reverse another one.
#
# How it works
# -------------
#   16k needed a fixed-size array (STORE) and index arithmetic
#   ([STORE-1,X]) to walk backward through what was typed. This
#   version reaches for the stack instead, using PUSHA to remember
#   each digit and POPA to retrieve them - which comes back out in
#   the opposite order automatically, with no array or index math
#   needed at all:
#
#     BLDSP   $EFFF       # Load stack pointer with $EFFF
#
#     GETKEY:    NOP                 # a harmless one-cycle delay - see notes
#     LOOP:      LDA     [KEYPAD]    # Load ACC from the keypad
#                JN      [LOOP]      # Jump back if no key pressed
#                CMPA    $0F         # Compare ACC to $0F
#                JC      [DISPSTUF]  # Jump if ACC is bigger
#                PUSHA               # ... else push ACC onto the stack
#                JMP     [GETKEY]    # Go and wait for another key
#
#     DISPSTUF:  POPA                # Pop ACC off the stack
#                STA     [MAINDISP]  # Store it to the main display
#                JMP     [GETKEY]    # Go and wait for another key
#
#   BLDSP sets up the stack pointer before anything else runs, the
#   same way BLDX sets up the Index register in earlier exercises -
#   this project's stack grows downward from $EFFF. Every accepted
#   hex digit gets PUSHA'd - laid on top of whatever's already
#   there. The stack is naturally Last-In-First-Out: the most
#   recently pushed byte is always the first one a POPA retrieves.
#   So the moment a non-hex key breaks the typing loop, DISPSTUF's
#   POPA hands back the *last* digit that was typed - no counting,
#   no indexed addressing, no [STORE-1,X] arithmetic required.
#
#   And notice DISPSTUF only pops and shows *one* digit, then jumps
#   straight back to GETKEY to wait for the next key - rather than
#   looping through every pushed byte at once the way 16k's DISPSTUF
#   loop does. Practically, that means: type a run of digits, press
#   a non-digit key, and one digit (the last one typed) appears; type
#   more digits, press a non-digit key again, and the *next-most*
#   recently typed digit comes back - the stack keeps handing back
#   whatever's currently on top, one POPA at a time, for as long as
#   there's anything left on it to pop.
#
#   The leading NOP on GETKEY does nothing to the program's behavior
#   - it just burns one CPU cycle before looping back to LOOP each
#   time. The source labels it "see notes" without further comment
#   in this file; harmless either way, and left in place exactly as
#   found. (The original source's comment used curly typographic
#   quotes saved in a Windows text encoding, which don't render
#   correctly as plain UTF-8 - normalized to straight quotes here for
#   readability; the instruction itself, and everything the CPU
#   actually does, is unaffected either way.)
#
# Try it
# -------
#   Assemble, Load -> CPU, click Run, switch to Hex mode, type 1-2-3,
#   then press an operator key - '3' appears (the last digit typed).
#   Press the operator key again and nothing more appears (the stack
#   for that run is now empty); type more digits and repeat.
#
# Watch it happen
# -----------------
#   Open Registers and watch the stack pointer (SP) count down by one
#   with every PUSHA and back up by one with every POPA - and compare
#   that against 16k's Index register, which does the equivalent
#   counting job with INCX/DECX instead.
#
# Try this next
# ---------------
#   - Change DISPSTUF to loop (POPA/STA/JNZ-style, checking for an
#     empty-stack condition) so an entire typed run reverses in one
#     shot the way 16k's does, instead of one digit per non-hex
#     keypress.
#   - Compare this file directly against 16k: both reverse a typed
#     sequence, but one leans on the stack's built-in LIFO order and
#     the other builds that same ordering by hand with an array and
#     index arithmetic.
# ================================================================
 
## Lab 3d - Display hex chars in reverse order using the stack
 
#######################################################################
## Start of constant declarations                                    ##
#######################################################################
MAINDISP: .EQU     $F031      # Address of output port for main display
SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs
KEYPAD:   .EQU     $F011      # Address of input port for keypad
CLRCODE:  .EQU     $10        # Special code to clear the main display
BINMODE:  .EQU     %00000100  # LED code to indicate binary mode
DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode
HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode
#######################################################################
## End of constant declarations                                      ##
#######################################################################
 
          .ORG     $4000      # Set program origin
 
#######################################################################
## Start of initialization                                           ##
#######################################################################
INIT:      LDA     CLRCODE    # Load accumulator with clear code
           STA     [MAINDISP] # Write clear code to main display
           LDA     HEXMODE    # Load accumulator with hex mode code
           STA     [SIXLEDS]  # Write to port driving six LEDs
#######################################################################
## End of initialization                                             ##
#######################################################################
 
 
#######################################################################
## Start of main program body                                        ##
#######################################################################
 
########## Initialize the stack pointer
           BLDSP   $EFFF       # Load stack pointer with $EFFF
 
########## Wait for key to be pressed
GETKEY:    NOP                 # "No operation" - see notes
LOOP:      LDA     [KEYPAD]    # Load ACC from the keypad
           JN      [LOOP]      # Jump back if no key pressed
           CMPA    $0F         # Compare ACC to $0F
           JC      [DISPSTUF]  # Jump if ACC is bigger
           PUSHA               # ... else push ACC onto the stack
           JMP     [GETKEY]    # Go and wait for another key
 
########## Display a number
DISPSTUF:  POPA                # Pop ACC off the stack
           STA     [MAINDISP]  # Store it to the main display
           JMP     [GETKEY]    # Go and wait for another key
 
#######################################################################
## End of main program body                                          ##
#######################################################################
 
 
#######################################################################
## Start of subroutines                                              ##
#######################################################################
 
#######################################################################
## End of subroutines                                                ##
#######################################################################
 
 
#######################################################################
## Start of global data                                              ##
#######################################################################
 
#######################################################################
## End of global data                                                ##
#######################################################################
 
          .END                # That's all folks

## 16n - Nested Subroutines

# ================================================================
# 16n_lab3e_nested_subroutines.asm
# Beboputer Hands-On Tutorial — Lab 3e (nested subroutines)
#
# Display four fixed bytes as hex, using one subroutine that calls
# two more subroutines to do the actual work.
#
# What this program does
# -----------------------
#   On its own, with no keypad interaction at all, shows four
#   pre-loaded byte values ($23, $5A, $06, $4C) as hex digit pairs,
#   one after another, then stops.
#
# How it works
# -------------
#   The main body just loads each byte and calls one subroutine four
#   times:
#
#     LDA     [TEMP]     # Load 1st byte into ACC
#     JSR     [DISPBYTE] # Call display byte subroutine
#     LDA     [TEMP+1]   # Load 2nd byte into ACC
#     JSR     [DISPBYTE] # Call display byte subroutine
#     ... (and so on for TEMP+2, TEMP+3)
#     JMP     [$0000]    # Terminate the program
#
#   JSR/RTS were introduced back in exercise 12 for calling one
#   shared subroutine from three different places. This exercise
#   goes a step further: DISPBYTE isn't one flat block of code - it's
#   a short subroutine that itself calls two *more* subroutines:
#
#     DISPBYTE:  PUSHA              # Push a copy of ACC onto stack
#                JSR     [DISPMSN]  # Call sub for MS nybble
#                POPA               # Pop copy of original byte off stack
#                JSR     [DISPLSN]  # Call sub for LS nybble
#                RTS                # Return to calling location
#
#     DISPMSN:   SHR / SHR / SHR / SHR
#                AND     %00001111  # Clear MS 4 bits (not really necessary)
#                STA     [MAINDISP] # Copy result to main display
#                RTS                # Return to calling location
#
#     DISPLSN:   AND     %00001111  # Mask out (clear) MS nybble
#                STA     [MAINDISP] # Copy result to main display
#                RTS                # Return to calling location
#
#   DISPMSN's four SHRs are destructive - they consume the
#   accumulator's original value while pulling the high nybble down
#   (the same technique 16c uses). By the time DISPMSN returns, the
#   byte's original bits are gone from ACC. That's exactly why
#   DISPBYTE pushes a copy *before* calling DISPMSN, and pops it back
#   *after* - so the original, untouched byte is sitting in ACC again
#   when DISPLSN is called to extract the low nybble from it. Without
#   that PUSHA/POPA pair, DISPLSN would be masking whatever DISPMSN
#   left behind instead of the real byte.
#
#   Three levels of calling happen here: the main body calls
#   DISPBYTE, and DISPBYTE calls DISPMSN and then DISPLSN. Each JSR
#   pushes a return address on the stack (see exercise 12's note on
#   JSR/RTS), and each RTS pops the most recent one - so the calls
#   unwind in the reverse order they were made, same as any nested
#   function calls in a higher-level language.
#
# Try it
# -------
#   Assemble, Load -> CPU, click Run - the display shows "23 5A 06 4C"
#   as it processes all four bytes (no separators between pairs,
#   since DISPBYTE never writes one - watch closely, or use Step, to
#   see each pair land).
#
# Watch it happen
# -----------------
#   Open Registers and watch the stack pointer drop by 2 on each JSR
#   and rise by 2 on each RTS, and by 1 more on each PUSHA/POPA -
#   Step through one full DISPBYTE call and count exactly how deep
#   the stack gets before it's all unwound back to where it started.
#
# Try this next
# ---------------
#   - See 16o for the same net result with DISPMSN and DISPLSN
#     folded directly into DISPBYTE as fall-through labels instead of
#     separately JSR'd subroutines - same output, different internal
#     structure.
# ================================================================
 
## Lab 3e using a nested subroutine
 
#######################################################################
## Start of constant declarations                                    ##
#######################################################################
MAINDISP: .EQU     $F031      # Address of output port for main display
SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs
KEYPAD:   .EQU     $F011      # Address of input port for keypad
CLRCODE:  .EQU     $10        # Special code to clear the main display
BINMODE:  .EQU     %00000100  # LED code to indicate binary mode
DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode
HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode
#######################################################################
## End of constant declarations                                      ##
#######################################################################
 
          .ORG     $4000      # Set program origin
 
#######################################################################
## Start of initialization                                           ##
#######################################################################
INIT:      LDA     CLRCODE    # Load accumulator with clear code
           STA     [MAINDISP] # Write clear code to main display
           LDA     HEXMODE    # Load accumulator with hex mode code
           STA     [SIXLEDS]  # Write to port driving six LEDs
           BLDSP   $EFFF      # Load stack pointer with $EFFF
#######################################################################
## End of initialization                                             ##
#######################################################################
 
 
#######################################################################
## Start of main program body                                        ##
#######################################################################
 
           LDA     [TEMP]     # Load 1st byte into ACC
           JSR     [DISPBYTE] # Call display byte subroutine
           LDA     [TEMP+1]   # Load 2nd byte into ACC
           JSR     [DISPBYTE] # Call display byte subroutine
           LDA     [TEMP+2]   # Load 3rd byte into ACC
           JSR     [DISPBYTE] # Call display byte subroutine
           LDA     [TEMP+3]   # Load 4th byte into ACC
           JSR     [DISPBYTE] # Call display byte subroutine
           JMP     [$0000]    # Terminate the program
 
#######################################################################
## End of main program body                                          ##
#######################################################################
 
 
#######################################################################
## Start of subroutines                                              ##
#######################################################################
 
########## Subroutine to extract/display MS and LS nybbles of byte
DISPBYTE:  PUSHA              # Push a copy of ACC onto stack
           JSR     [DISPMSN]  # Call sub for MS nybble
           POPA               # Pop copy of original byte off stack
           JSR     [DISPLSN]  # Call sub for LS nybble
           RTS                # Return to calling location
 
########## Subroutine to extract/display the MS nybble
DISPMSN:   SHR                # Shift right 1 bit (= 1 bit shift)
           SHR                # Shift right 1 bit (= 2 bit shift)
           SHR                # Shift right 1 bit (= 3 bit shift)
           SHR                # Shift right 1 bit (= 4 bit shift)
           AND     %00001111  # Clear MS 4 bits (not really necessary)
           STA     [MAINDISP] # Copy result to main display
           RTS                # Return to calling location
 
########## Extract and display the LS nybble
DISPLSN:   AND     %00001111  # Mask out (clear) MS nybble
           STA     [MAINDISP] # Copy result to main display
           RTS                # Return to calling location
 
#######################################################################
## End of subroutines                                                ##
#######################################################################
 
 
#######################################################################
## Start of global data                                              ##
#######################################################################
 
TEMP:     .BYTE $23, $5A, $06, $4C  # Just some data to play with
 
#######################################################################
## End of global data                                                ##
#######################################################################
 
          .END                # That's all folks

## 16o - Flattened Subroutine

# ================================================================
# 16o_lab3e_flattened_subroutine.asm
# Beboputer Hands-On Tutorial — Lab 3e (one flattened subroutine)
#
# The same four-byte hex display as 16n - with DISPMSN and DISPLSN
# folded into DISPBYTE as fall-through labels instead of separately
# called subroutines.
#
# What this program does
# -----------------------
#   Identical behavior to 16n: shows four pre-loaded bytes ($23,
#   $5A, $06, $4C) as hex digit pairs, then stops. The main body
#   calling DISPBYTE four times is unchanged.
#
# How it works
# -------------
#   16n's DISPBYTE made two separate JSR calls, one to DISPMSN and
#   one to DISPLSN, each with its own RTS. This version keeps the
#   same three label names, but only one of them is a real "callable"
#   entry point anymore - the other two are just waypoints inside a
#   single subroutine that falls straight through from one to the
#   next:
#
#     DISPBYTE:  PUSHA              # Push a copy of ACC onto stack
#
#     DISPMSN:   SHR                # Shift right 1 bit (= 1 bit shift)
#                SHR                # Shift right 1 bit (= 2 bit shift)
#                SHR                # Shift right 1 bit (= 3 bit shift)
#                SHR                # Shift right 1 bit (= 4 bit shift)
#                AND     %00001111  # Clear MS 4 bits (not really necessary)
#                STA     [MAINDISP] # Copy result to main display
#
#     DISPLSN:   POPA               # Pop copy of original byte off stack
#                AND     %00001111  # Mask out (clear) MS nybble
#                STA     [MAINDISP] # Copy result to main display
#                RTS                # Return from subroutine
#
#   There's no JSR anywhere in this block, and no RTS until the very
#   end. DISPMSN and DISPLSN are still labels - useful for reading
#   the code and for referring to "the bit that handles the MS
#   nybble" versus "the bit that handles the LS nybble" - but nothing
#   ever jumps to them from outside DISPBYTE. Execution simply falls
#   from the last instruction under DISPBYTE straight into DISPMSN's
#   first instruction, and from DISPMSN's last instruction straight
#   into DISPLSN's, with no jump or call involved at either boundary.
#
#   This is the same PUSHA-before/POPA-after pattern 16n uses to
#   protect the original byte across the destructive SHR chain - the
#   *data flow* is identical between the two versions. What changed
#   is purely structural: 16n expresses "do the MS nybble, then do
#   the LS nybble" as two subroutine calls from a third subroutine;
#   this version expresses the exact same two steps as one
#   subroutine with internal signposts. Both compile down to running
#   the same instructions in the same order - the only difference is
#   whether DISPMSN and DISPLSN could, in principle, be called from
#   somewhere else too (16n's version, yes; this version, no - RTS
#   only appears once, at the very end).
#
# Try it
# -------
#   Assemble, Load -> CPU, click Run - output should be identical to
#   16n's: "23 5A 06 4C".
#
# Watch it happen
# -----------------
#   Step through one call to DISPBYTE and watch the Program Counter
#   in the Registers panel - it moves smoothly from DISPBYTE's last
#   instruction into DISPMSN's first, and from DISPMSN's last into
#   DISPLSN's first, with no jump instruction executing at either
#   handoff (contrast this with 16n, where the PC visibly jumps to a
#   JSR target and back at each of those same two points).
#
# Try this next
# ---------------
#   - Try calling DISPMSN directly (JSR [DISPMSN]) from somewhere
#     else in a copy of this file and see what goes wrong - without
#     its own RTS, it will fall straight through into DISPLSN and pop
#     a stack value that was never pushed for it, rather than
#     returning to the caller.
# ================================================================
 
## Lab 3e using a simple subroutine
 
#######################################################################
## Start of constant declarations                                    ##
#######################################################################
MAINDISP: .EQU     $F031      # Address of output port for main display
SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs
KEYPAD:   .EQU     $F011      # Address of input port for keypad
CLRCODE:  .EQU     $10        # Special code to clear the main display
BINMODE:  .EQU     %00000100  # LED code to indicate binary mode
DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode
HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode
#######################################################################
## End of constant declarations                                      ##
#######################################################################
 
          .ORG     $4000      # Set program origin
 
#######################################################################
## Start of initialization                                           ##
#######################################################################
INIT:      LDA     CLRCODE    # Load accumulator with clear code
           STA     [MAINDISP] # Write clear code to main display
           LDA     HEXMODE    # Load accumulator with hex mode code
           STA     [SIXLEDS]  # Write to port driving six LEDs
           BLDSP   $EFFF      # Load stack pointer with $EFFF
#######################################################################
## End of initialization                                             ##
#######################################################################
 
 
#######################################################################
## Start of main program body                                        ##
#######################################################################
 
           LDA     [TEMP]     # Load 1st byte into ACC
           JSR     [DISPBYTE] # Call display byte subroutine
           LDA     [TEMP+1]   # Load 2nd byte into ACC
           JSR     [DISPBYTE] # Call display byte subroutine
           LDA     [TEMP+2]   # Load 3rd byte into ACC
           JSR     [DISPBYTE] # Call display byte subroutine
           LDA     [TEMP+3]   # Load 4th byte into ACC
           JSR     [DISPBYTE] # Call display byte subroutine
           JMP     [$0000]    # Terminate the program
 
#######################################################################
## End of main program body                                          ##
#######################################################################
 
 
#######################################################################
## Start of subroutines                                              ##
#######################################################################
 
########## Subroutine to extract/display MS and LS nybbles of byte
DISPBYTE:  PUSHA              # Push a copy of ACC onto stack
 
########## Extract and display the most-significant nybble
DISPMSN:   SHR                # Shift right 1 bit (= 1 bit shift)
           SHR                # Shift right 1 bit (= 2 bit shift)
           SHR                # Shift right 1 bit (= 3 bit shift)
           SHR                # Shift right 1 bit (= 4 bit shift)
           AND     %00001111  # Clear MS 4 bits (not really necessary)
           STA     [MAINDISP] # Copy result to main display
 
########## Extract and display the least-significant nybble
DISPLSN:   POPA               # Pop copy of original byte off stack
           AND     %00001111  # Mask out (clear) MS nybble
           STA     [MAINDISP] # Copy result to main display
           RTS                # Return from subroutine
 
#######################################################################
## End of subroutines                                                ##
#######################################################################
 
 
#######################################################################
## Start of global data                                              ##
#######################################################################
 
TEMP:     .BYTE $23, $5A, $06, $4C  # Just some data to play with
 
#######################################################################
## End of global data                                                ##
#######################################################################
 
          .END                # That's all folks

## 16p - Recursive Factorial

# ================================================================
# 16p_lab3f_recursive_factorial.asm
# Beboputer Hands-On Tutorial — Lab 3f (recursive factorial)
#
# Press a key from 1 to 5 and see its factorial - computed by a
# subroutine that calls itself.
#
# What this program does
# -----------------------
#   Waits for a keypad key from 1 to 5 (0 and anything above 5 are
#   ignored), computes that number's factorial (1x2x3x...xn), and
#   shows the result as two hex digits. Press any key afterward to
#   clear the display and try another number.
#
# How it works
# -------------
#   This is the first genuinely *recursive* subroutine in this
#   tutorial series - FACTOR calls itself:
#
#     FACTOR:    PUSHA              # Push copy of ACC onto stack
#                CMPA     $01       # Compare ACC to 1
#                JNC     [COMEOUT]  # If ACC not > 1 start to come out
#                DECA               #   else decrement ACC
#                JSR     [FACTOR]   #   then this routine calls itself
#
#     COMEOUT:   PUSHA              # Push copy of ACC onto stack
#                JSR     [FMULT]    # Multiply two bytes on top of stack
#                POPA               # Retrieve 8-bit result
#                RTS                # End this instantiation of the routine
#
#   Call FACTOR with ACC = n. It pushes n onto the stack immediately
#   - that copy stays put through everything that follows, waiting
#   to be multiplied in later. Then CMPA $1/JNC checks whether n is
#   bigger than 1 (recall this CPU's Carry-after-CMPA convention:
#   Carry set means "strictly greater than" - see exercise 15d's
#   note on the same test). If n is 1 or less, this is the base case
#   (1! and 0! both equal 1, though this program only ever reaches n=1
#   in practice - see the note on GETNUM below) - execution jumps
#   straight to COMEOUT. Otherwise, DECA makes n-1, and FACTOR calls
#   *itself* to compute (n-1)! first - it doesn't return to its
#   caller until that inner call has finished.
#
#   Each call to FACTOR pushes its own copy of n onto the stack
#   before recursing, so five nested calls (say, computing 5!) leave
#   five values sitting on the stack - 5, 4, 3, 2, 1 - one per call,
#   each waiting for its own COMEOUT to run. When the innermost call
#   (n=1) finally hits its base case, COMEOUT pushes a second copy of
#   that same 1, then calls FMULT to multiply the two values
#   currently on top of the stack and leave the product there instead
#   - 1x1=1. POPA retrieves the product, and RTS returns to the level
#   above, which does exactly the same thing with its own n (2) and
#   the 1 that just came back: COMEOUT pushes 2, FMULT multiplies
#   2x1=2, and that 2 propagates up to the next level, and so on -
#   3x2=6, 4x6=24, 5x24=120 - one multiplication unwinding at each
#   level of the recursion, until the outermost call returns 120 (=5!)
#   in the accumulator.
#
#   Notice GETNUM (in the main body below) explicitly loops back on
#   a 0 keypress (CMPA $00 / JZ [GETNUM]) - 0 is deliberately excluded
#   even though 0! is mathematically 1, so FACTOR itself is never
#   actually invoked with n=0 by this program. And the range is
#   capped at 5 for a very practical reason: 6! is 720, which does
#   not fit in an 8-bit accumulator (max 255) - this program's whole
#   design, from the keypad range check down to FACTOR's own logic,
#   only works because every factorial from 1! through 5! (1, 2, 6,
#   24, 120) fits comfortably in one byte.
#
#   FMULT itself is not recursive - it's a self-contained 8x8-bit
#   unsigned multiply subroutine, using the classic shift-and-add
#   technique: a 2-byte result register starts as {0, multiplier},
#   and each pass through its main loop tests one bit (via the
#   Carry flag, fed by the previous pass's rotate), conditionally
#   adds the multiplicand into the result's high byte, then rotates
#   the whole 2-byte result right by one bit through Carry -
#   simultaneously shifting the running sum right and shifting the
#   next multiplier bit into position to be tested on the following
#   pass. FMULT's own header comment documents its stack-based
#   calling convention in detail; step through it slowly in the
#   emulator to watch _FM_RES build up one bit at a time.
#
# Try it
# -------
#   Assemble, Load -> CPU, click Run, switch the Calculator on, and
#   press keys 1 through 5 in turn - the display shows 01, 02, 06,
#   18 ($18 = 24 decimal), and 78 ($78 = 120 decimal), i.e. 1!
#   through 5! in hex. Press any key afterward to clear and try again.
#
# Watch it happen
# -----------------
#   Open Registers and watch the stack pointer drop by 1 with every
#   PUSHA as FACTOR recurses deeper (five drops for input 5), then
#   watch it climb back up as each COMEOUT/FMULT/RTS sequence unwinds
#   one level - Step slowly through a run with input 3 or 4 to see
#   the whole push-down-then-multiply-back-up shape clearly.
#
# Try this next
# ---------------
#   - Trace by hand what stack contents look like at the deepest
#     point of recursion for input 4, before any multiplication has
#     happened yet.
#   - Work out why input 6 isn't offered as a choice, using nothing
#     but the fact that the accumulator is 8 bits wide.
# ================================================================
 
#######################################################################
## Start of constant declarations                                    ##
#######################################################################
MAINDISP: .EQU     $F031      # Address of output port for main display
SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs
KEYPAD:   .EQU     $F011      # Address of input port for keypad
CLRCODE:  .EQU     $10        # Special code to clear the main display
BINMODE:  .EQU     %00000100  # LED code to indicate binary mode
DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode
HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode
#######################################################################
## End of constant declarations                                      ##
#######################################################################
 
          .ORG     $4000      # Set program origin
 
#######################################################################
## Start of initialization                                           ##
#######################################################################
INIT:      LDA     CLRCODE    # Load accumulator with clear code
           STA     [MAINDISP] # Write clear code to main display
           LDA     HEXMODE    # Load accumulator with hex mode code
           STA     [SIXLEDS]  # Write to port driving six LEDs
#######################################################################
## End of initialization                                             ##
#######################################################################
 
 
#######################################################################
## Start of main program body                                        ##
#######################################################################
           BLDSP    $EFFF     # Initialize stack pointer
 
########## Wait for a key between 1 and 5
GETNUM:    LDA     [KEYPAD]   # Load ACC from keypad
           CMPA     $00       # Compare to 0
           JZ      [GETNUM]   # Jump back if key = 0
           CMPA     $05       # Compare to 5
           JC      [GETNUM]   # Jump back if key > 5
           JSR     [FACTOR]   #   else call FACTOR subroutine
           JSR     [DISPBYTE] #   then display value in ACC
 
########## Now wait for any key, then clear display and do it again
WAIT:      LDA     [KEYPAD]   # Load ACC from keypad
           JN      [WAIT]     # Jump if no key pressed
CLEAR:     LDA     CLRCODE    # Load accumulator with clear code
           STA     [MAINDISP] # Write clear code to main display
           JMP     [GETNUM]   # Jump back and wait for next number
 
#######################################################################
## End of main program body                                          ##
#######################################################################
 
 
#######################################################################
## Start of subroutines                                              ##
#######################################################################
 
########## This is the recursive subroutine called FACTOR
FACTOR:    PUSHA              # Push copy of ACC onto stack
           CMPA     $01       # Compare ACC to 1
           JNC     [COMEOUT]  # If ACC not > 1 start to come out
           DECA               #   else decrement ACC
           JSR     [FACTOR]   #   then this routine calls itself
 
COMEOUT:   PUSHA              # Push copy of ACC onto stack
           JSR     [FMULT]    # Multiply two bytes on top of stack
           POPA               # Retrieve 8-bit result
           RTS                # End this instantiation of the routine
########## End of FACTOR
#---------------------------------------------------------------------#
 
 
#######################################################################
# Name:     FMULT                                                     #
#                                                                     #
# Function: Multiplies two 8-bit unsigned numbers and returns 8-bit   #
#           unsigned result (MS byte of 2-byte result is discarded.   #
#                                                                     #
# Entry:    Top of stack                                              #
#           Most-significant byte of return address                   #
#           Least-significant byte of return address                  #
#           Second 8-bit number (multiplicand)                        #
#           First 8-bit number (multiplier)                           #
#                                                                     #
# Exit:     Top of stack                                              #
#           Least-significant byte of result (product)                #
#                                                                     #
# Modifies: Accumulator                                               #
#           Index register                                            #
#                                                                     #
# Size:     Program = 63 bytes                                        #
#           Data    =  5 bytes                                        #
#######################################################################
 
########## Store return address
FMULT:     POPA               # Retrieve MS byte of return
           STA  [_FM_RADD]    # address from stack and store it
           POPA               # Retrieve LS byte of return
           STA  [_FM_RADD+1]  # address from stack and store it
 
           POPA               # Retrieve multiplicand from stack
           STA  [_FM_MAND]    # and store it
           POPA               # Retrieve multiplier from stack
           STA  [_FM_RES+1]   # and store it in LS byte of result
           LDA   0            # Load the accumulator with 0 and
           STA  [_FM_RES]     # store it in the MS byte of result
 
########## Perform housekeeping tasks and get ready to multiply
_FM_INIT:  BLDX  9            # Load X reg no. of cycles + 1
           ADD   0            # Dummy inst. to set C flag to 0
 
########## Do the main multiplication loop
_FM_LOOP:  LDA  [_FM_RES]     # Load MS byte of the result
                              #   This doesn't affect the carry flag
           JNC  [_FM_SHFT]    # If carry=0, perform shift
           ADD  [_FM_MAND]    #   .. else add the multiplicand
 
########## Shift (using rotates) 2-byte result 1 bit to the right
_FM_SHFT:  RORC               # Rotate MS byte of result 1-bit right
           STA  [_FM_RES]     #   and store it
           LDA  [_FM_RES+1]   # Load LS byte of result
           RORC               # Rotate LS byte of result 1-bit right.
           STA  [_FM_RES+1]   #   and store it
 
########## Test for end of multiplication
_FM_TST:   DECX               # Decrement the X register
           JNZ  [_FM_LOOP]    # Jump if X reg isn't 0
 
########## Store LS byte of result on stack (discard MS byte)
_FM_SRES:  LDA  [_FM_RES+1]   # Load ACC with LS byte of result
           PUSHA              #   and store it on the stack
 
########## Retrieve return address and exit routine
_FM_GRET:  LDA  [_FM_RADD+1]  # Load ACC with LS return address
           PUSHA              #   and store it on the stack
           LDA  [_FM_RADD]    # Load ACC with MS return address
           PUSHA              #   and store it on the stack
           RTS                # That's all folks (exit the routine)
 
########## Temp storage loactions for thsi routine
_FM_RADD: .2BYTE              # 2-byte location for return address
_FM_MAND: .BYTE               # 1-byte temp location for multiplicand
_FM_RES:  .2BYTE              # 2-byte temp location for result
 
########## End of FMULT
#---------------------------------------------------------------------#
 
 
########## Subroutine to extract/display MS and LS nybbles of byte
DISPBYTE:  PUSHA              # Push a copy of ACC onto stack
 
########## Display a dollar character
DISPDOL:   LDA   $24          # Load ACC with ASCII code for '$'
           STA  [MAINDISP]    # Load ACC with MS return address
 
########## Extract and display the most-significant nybble
DISPMSN:   POPA               # Retrieve copy of ACC from stack
           PUSHA              # And push a copy back for later
           SHR                # Shift right 1 bit (= 1 bit shift)
           SHR                # Shift right 1 bit (= 2 bit shift)
           SHR                # Shift right 1 bit (= 3 bit shift)
           SHR                # Shift right 1 bit (= 4 bit shift)
           AND     %00001111  # Clear MS 4 bits (not really necessary)
           STA     [MAINDISP] # Copy result to main display
 
########## Extract and display the least-significant nybble
DISPLSN:   POPA               # Pop copy of original byte off stack
           AND     %00001111  # Mask out (clear) MS nybble
           STA     [MAINDISP] # Copy result to main display
           RTS                # Return from subroutine
 
########## End of DISPBYTE
#---------------------------------------------------------------------#
 
#######################################################################
## End of subroutines                                                ##
#######################################################################
 
 
#######################################################################
## Start of global data                                              ##
#######################################################################
 
#######################################################################
## End of global data                                                ##
#######################################################################
 
          .END                # That's all folks

## 16q - Recursive String Reversal

# ================================================================
# 16q_lab3f_recursive_reverse.asm
# Beboputer Hands-On Tutorial - Lab 3f (recursive string reversal)
#
# Print a fixed message backward - using a subroutine that calls
# itself once per character, with no array or manual index math.
#
# What this program does
# -----------------------
#   On its own, with no keypad interaction, prints the 9-character
#   message stored in its table, in reverse order, and stops. The
#   table holds "SWAP PAWS" - and because that particular phrase
#   reads the same forwards and backwards (character 0 matches
#   character 8, character 1 matches character 7, and so on, with
#   the space sitting in the exact middle), the reversed output
#   looks identical to the original on the display. That's
#   deliberate: see the note at the end of "How it works" on why
#   watching the registers, not just the display, is how you confirm
#   the reversal is really happening.
#
# How it works
# -------------
#   This is 16m's stack-based reversal trick and 16n/16o's nested
#   subroutines combined into one elegant idea: a subroutine that
#   reverses a message by calling itself, once per character, and
#   letting the call stack itself remember the order to print them
#   back in:
#
#     REVERSE:   LDA     [PHRASE,X] # Load ACC with a character
#                JNZ     [GOIN]     # If it's not NUL jump to GO_IN
#     RETURN1:   RTS                # Otherwise return from subroutine
#
#     GOIN:      PUSHA              # Push the character onto the stack
#                INCX               # Increment the index register
#     INSIDE:    JSR     [REVERSE]  # The subroutine calls itself recursively
#
#     COMEOUT:   POPA               # Pop a character off the stack
#                STA     [MAINDISP] # Copy to main display
#     RETURN2:   RTS                # Return from subroutine
#
#   [PHRASE,X] is the same indexed-addressing message-walking idea
#   Lab 2f's "HELLO WORLD!" uses (exercise 15f) - X starts at 0 and
#   counts up through the table one character at a time. But instead
#   of displaying each character as it's read and moving on, REVERSE
#   pushes the character onto the stack, advances X, and calls
#   itself to go read the next character - before doing anything
#   at all with the one it just read.
#
#   That single JSR [REVERSE] at INSIDE is the whole trick: it means
#   nothing gets displayed until the recursion bottoms out. REVERSE
#   keeps calling itself, pushing one more character each time,
#   until LDA [PHRASE,X] finally loads the table's terminating $00 -
#   at that point JNZ doesn't fire, and this innermost call just
#   returns immediately via RETURN1, without pushing anything.
#
#   From there, every one of the outer calls resumes exactly where it
#   left off - at COMEOUT, right after its own JSR [REVERSE] - and
#   pops back the character it pushed before recursing. Since the
#   stack is Last-In-First-Out (the same property 16m's simpler
#   stack-based reversal relies on), the very last character pushed
#   (the one right before the terminator) is the very first one
#   popped - so it's the first one displayed. Each returning call
#   displays its own character and then returns to whichever call
#   invoked it, one level up, which displays its character next -
#   and so on, all the way back out to the very first call, which
#   displays the very first character in the table last. The message
#   comes out back-to-front with no array, no manually-managed
#   count, and no [STORE-1,X]-style address arithmetic anywhere -
#   the recursive call stack is the reversal mechanism.
#
#   This is the same shape as 16p's recursive FACTOR: descend first
#   (pushing something at each level), hit a base case, then do one
#   small piece of work as each level unwinds on the way back out.
#   FACTOR multiplies on the way out; REVERSE displays on the way out
#   - same underlying pattern, applied to two different problems.
#
#   One more thing worth knowing before running this: PHRASE is
#   declared as two separate .BYTE lines - $53,$57,$41,$50,$20 (that's
#   "SWAP ") followed by $50,$41,$57,$53,$00 (that's "PAWS" plus the
#   NUL terminator) - nine displayable characters in total, forming
#   the single string "SWAP PAWS". Reverse those nine characters
#   (S-W-A-P-space-P-A-W-S) end for end and you get back S-W-A-P-
#   space-P-A-W-S - the exact same sequence. The phrase was chosen
#   as a palindrome on purpose, which is why the display alone can't
#   prove the recursion reversed anything - the "Watch it happen"
#   section below is where the real evidence is.
#
# Try it
# -------
#   Assemble, Load -> CPU, click Run - the display shows "SWAP PAWS",
#   which is what the reversed message looks like for this
#   particular (palindromic) phrase - see the note above.
#
# Watch it happen
# -----------------
#   Open Registers and watch the stack pointer drop once per
#   character as REVERSE recurses in (X counting 0, 1, 2, 3, 4 along
#   the way), then watch it rise again as each level's COMEOUT pops
#   and displays its character on the way back out - Step slowly to
#   see the "all the way in, then one at a time coming back out"
#   shape play out in the Registers panel.
#
# Try this next
# ---------------
#   - Change PHRASE to a longer message (remembering the trailing
#     $00 terminator, per Lab 2f's note) and confirm it still reverses
#     correctly, however long it is.
#   - Compare this file directly against 16m: both reverse a
#     sequence using the stack's natural LIFO order, but 16m reads
#     keypad input into the stack in a loop, while this one builds
#     the same stack contents through recursive calls instead.
# ================================================================
 
#######################################################################
## Start of constant declarations                                    ##
#######################################################################
MAINDISP: .EQU     $F031      # Address of output port for main display
SIXLEDS:  .EQU     $F032      # Address of output port for six LEDs
KEYPAD:   .EQU     $F011      # Address of input port for keypad
CLRCODE:  .EQU     $10        # Special code to clear the main display
BINMODE:  .EQU     %00000100  # LED code to indicate binary mode
DECMODE:  .EQU     %00000010  # LED code to indicate decimal mode
HEXMODE:  .EQU     %00000001  # LED code to indicate hexadecimal mode
#######################################################################
## End of constant declarations                                      ##
#######################################################################
 
          .ORG     $4000      # Set program origin
 
#######################################################################
## Start of initialization                                           ##
#######################################################################
INIT:      LDA     CLRCODE    # Load accumulator with clear code
           STA     [MAINDISP] # Write clear code to main display
           LDA     HEXMODE    # Load accumulator with hex mode code
           STA     [SIXLEDS]  # Write to port driving six LEDs
#######################################################################
## End of initialization                                             ##
#######################################################################
 
 
#######################################################################
## Start of main program body                                        ##
#######################################################################
 
           BLDX    0          # Load index register with 0
           BLDSP   $EFFF      # Load stack pointer with $EFFF
OUTSIDE:   JSR     [REVERSE]  # Call the subroutine
FINISH:    JMP     [$0000]    # Terminate the program
 
#######################################################################
## End of main program body                                          ##
#######################################################################
 
 
#######################################################################
## Start of subroutines                                              ##
#######################################################################
 
########## Recursive subroutine to display string in reverse order
REVERSE:   LDA     [PHRASE,X] # Load ACC with a character
           JNZ     [GOIN]     # If it's not NUL jump to GO_IN
RETURN1:   RTS                # Otherwise return from subroutine
 
########## Store the character on the stack and go further in
GOIN:      PUSHA              # Push the character onto the stack
           INCX               # Increment the index register
INSIDE: