Scientific Functions in BCD: Sin, Cos, Tan, x², and n!

A Beboputer / PY-DIYCALCULATOR assembly-language tutorial

# 1. What You Will Build

This tutorial adds five scientific keys to the Calculator — Sin, Cos, Tan, x², and n! — and, unlike exercise 12's version of Sin/Cos/Tan, stores every typed value as packed BCD (binary-coded decimal) the whole way through, the same representation exercise 13's four-function calculator uses. Type a two-digit value, press one of the five keys, and read the answer off the display:

4 5  Sin   →  shows  45Sin=0.71

4 5  Cos   →  shows  45Cos=0.71

4 5  Tan   →  shows  45Tan=1.00

9 0  Sin   →  shows  90Sin=1.00

9 0  Tan   →  shows  90Tan=Err   (tangent has no finite value at 90 degrees)

0 4  x²   →  shows  04x^2=16

1 2  x²   →  shows  12x^2=Err   (144 needs 3 digits)

0 4  n!   →  shows  04n!=24

0 5  n!   →  shows  05n!=Err   (120 needs 3 digits)

The source file is tutorial/17_bcd_scientific_calculator.asm. It reuses the keypad state machine from exercises 10 and 13, the trig lookup tables and display formatting from exercise 12, and the DADD-based BCD arithmetic from exercise 13 — combining all three into one program where x² and n! are genuine BCD arithmetic, while Sin, Cos, and Tan stay table lookups dressed up to accept BCD input.

# 2. Before You Start

This tutorial builds directly on:

- 10_calculator_four_function.asm — the keypad state-machine pattern this program reuses, and reading/writing the Calculator's display port.

- 12_trig_functions_sincostan.asm — the SIN_TABLE/COS_TABLE/TAN_TABLE lookup tables and FORMAT_VAL display routine, carried over here unchanged, plus why Tan's table stops at 68 degrees.

- 13_bcd_math_routines.asm — packed BCD representation, the DADD/DSUB instructions, and why a BCD counter must be decremented with DSUB $01 instead of DECA.

Read those three tutorial documents first if any of that is unfamiliar — this exercise assumes all of it and spends its own explanation on what's new: combining a BCD front end with a binary-indexed lookup table, and building x²/n! out of nested BCD addition loops.

# 3. Two Kinds of “Function”: Lookup Table vs. Real BCD Arithmetic

Every scientific key on this calculator produces its answer a different way, and the program is built around that split:

- Sin, Cos, Tan — this CPU cannot compute a sine or cosine out of adds and subtracts at all (see exercise 12's header for the full reasoning), so the answer comes from a precomputed table, addressed by a plain binary index. The typed value has to leave BCD before it can be used that way.

- x², n! — both are ordinary multiplication in disguise (x·x, and x·(x-1)·(x-2)·…·1), and multiplication is exactly what exercise 13's DADD-based approach already builds out of repeated BCD addition. These two stay packed BCD from typed input to displayed output — no conversion, no table, ever.

That split is why the program needs a BCD-to-binary converter for one half of its keys and none at all for the other half — not an oversight, but the direct consequence of what each function actually is.

# 4. Program Design

START clears the display before anything else runs. The display isn't blank by default — it keeps showing whatever the previously run program last wrote — so without this, the exercise could open on a screen full of leftover characters from something else entirely:

START:  LDA     $1B             # clear code - the display isn't blank on

STA     [DISP]          # its own; it still holds whatever the

# previously run program left showing

LDA     $00

STA     [STAGE]        # begin waiting for the tens digit

Same keypad-polling state machine as exercises 10 and 13, with one extra stage for the function key:

| Stage | The program is… |
| --- | --- |
| 0 | Waiting for the tens digit of the value (X). |
| 1 | Waiting for the ones digit of X. |
| 2 | Waiting for Sin, Cos, Tan, x², or n!. |
| 3 | Showing an answer — the next digit clears the display and starts a new entry. |

Entry packs the two typed digits into one BCD byte (XVAL) exactly the way exercise 13 packs each operand: SHL ×4 moves the tens digit into the top nibble, then OR merges the ones digit into the bottom nibble — no addition involved, since the digits are already sitting in the right nibble positions once shifted.

ST_TENS:

LDA     [KEYVAL]

AND     $0F              # ASCII '0'-'9' -> binary 0-9

SHL

SHL

SHL

SHL                      # move it into the top nibble

STA     [XVAL]

ST_ONES:

LDA     [KEYVAL]

AND     $0F

OR      [XVAL]           # merge with the tens nibble already there

STA     [XVAL]           # XVAL is now one packed-BCD byte, 00-99

# 5. Converting BCD to a Table Index: BCD_TO_BIN

BLDX and the [TABLE,X] addressing mode need an ordinary binary offset — they have no idea XVAL's nibbles are decimal digits, they just add whatever's in the Index register to TABLE's address. So before any trig lookup, BCD_TO_BIN unpacks XVAL's two nibbles and recombines them the plain-binary way, tens×10 + ones, using the same repeated-ADD-$0A technique exercise 12 used to combine two separately typed digits — just starting from BCD nibbles (via SHR ×4 to isolate the tens digit) instead of two separate keypresses:

BCD_TO_BIN:

LDA     [XVAL]

STA     [TEMP]

SHR

SHR

SHR

SHR                      # tens digit (0-9), now in the low nibble

STA     [TCNT]

LDA     $00

STA     [BINVAL]

JMP     [B2B_TEST]

B2B_LOOP:

LDA     [BINVAL]

ADD     $0A              # +10 per tens digit (plain binary add -

STA     [BINVAL]         # BINVAL itself is never BCD)

LDA     [TCNT]

DECA

STA     [TCNT]

B2B_TEST:

LDA     [TCNT]

CMPA    $00

JNZ     [B2B_LOOP]

LDA     [TEMP]

AND     $0F              # ones digit (0-9)

ADD     [BINVAL]

STA     [BINVAL]         # BINVAL = tens*10+ones, plain binary 0-99

RTS

DO_SIN, DO_COS, and DO_TAN each JSR [BCD_TO_BIN] first, then use BINVAL exactly the way exercise 12 used its plain-binary DEG — range-checked against the table limit ($5A/90 for Sin and Cos, $44/68 for Tan), loaded into the Index register via BLDX, and looked up. SIN_TABLE, COS_TABLE, TAN_TABLE, and FORMAT_VAL are carried over from exercise 12 byte-for-byte, since a lookup table has no “BCD version” to switch to — only the path that gets a binary index into DEGIDX had to change.

# 6. x² and n!: BCD Multiplication by Repeated Addition

x² and n! are the two keys that stay packed BCD end to end, computed with this CPU's DADD instruction the same way exercise 13 builds a multiply operator: repeated BCD addition, with a BCD countdown decremented by DSUB $01 rather than DECA (see exercise 13's header for why an ordinary binary decrement would corrupt a BCD counter's low nibble on the 0→9 rollover).

DO_SQR computes XVAL×XVAL as “add XVAL to a running total, XVAL times”:

DO_SQR: LDA     $00

STA     [RESULT]

LDA     [XVAL]

STA     [MCNT]

JMP     [SQR_TEST]

SQR_LOOP:

LDA     [RESULT]

DADD    [XVAL]

STA     [RESULT]

JC      [ERR_OUT]        # running total spilled past 2 digits

LDA     [MCNT]

DSUB    $01

STA     [MCNT]

SQR_TEST:

LDA     [MCNT]

CMPA    $00

JNZ     [SQR_LOOP]

JMP     [SHOW_BCD_RESULT]

DO_FAC computes XVAL×(XVAL-1)×…×1 the same way, one level deeper: FCNT counts down from XVAL to 1, and each pass multiplies RESULT by FCNT using the identical repeated-DADD technique as an inner loop — FICNT is that inner loop's own countdown, and FMUL_ACC holds the value being repeatedly added, since RESULT itself is rebuilt from $00 as the inner loop runs and so can't also be the thing being added to itself:

FAC_INNER:

LDA     [FICNT]

CMPA    $00

JZ      [FAC_INNER_DONE]

LDA     [RESULT]

DADD    [FMUL_ACC]

STA     [RESULT]

JC      [ERR_OUT]         # running total spilled past 2 digits

LDA     [FICNT]

DSUB    $01

STA     [FICNT]

JMP     [FAC_INNER]

Both routines share one more consequence of staying in BCD: the Carry flag after a DADD means an honest “this digit pair overflowed 99,” so a single JC [ERR_OUT] after every DADD is enough to catch a result that needs a third digit — no separate range check needed, unlike Sin/Cos/Tan's explicit CMPA-against-the-table-limit up front. And because RESULT is already packed BCD, the result routine (SHOW_BCD_RESULT) needs no conversion loop at all — just split the nibbles and print each as a digit, the same shortcut exercise 13 uses to display its own answers.

# 7. Where Each Key's Range Stops, and Why

| Key | Valid input | Why it stops there |
| --- | --- | --- |
| Sin / Cos | 00–90 | SIN_TABLE/COS_TABLE only hold entries for 0–90 degrees. |
| Tan | 00–68 | tan(69°)×100 = 261, too large for a single byte — see exercise 12. |
| x² | any XVAL whose square ≤ 99 | packed BCD is two digits only; a 3-digit result (e.g. 12²=144) sets Carry out of DADD and reports Err. |
| n! | 0! through 4! | 5! = 120 needs a third digit; the same DADD-Carry check reports Err. |

In every case, an out-of-range or overflowed result shows Err rather than a silently wrong or truncated answer — the same convention exercises 10, 12, and 13 all use for divide-by-zero and out-of-table inputs. ECHO_FUNC prints the two- or three-letter name of whichever key was pressed (“Sin,” “Cos,” “Tan,” “x^2,” or “n!”) before every result or Err, so the display always reads as a complete expression, e.g. 90Tan=Err rather than a bare =Err.

# 8. Verified Against the Real CPU Emulator

This program was assembled with the project's own compiler (bin/compiler_core.py — the same back end the Calculator's Assemble / Load → CPU pipeline uses) and the resulting .ram image was loaded and run against the project's own CPU class with simulated keypresses — not just the .asm source read and assumed correct, but the actual compiled bytecode the app loads at $4000. Every test below passed against that compiled image:

| # | Test | Keys pressed | Result | Status |
| --- | --- | --- | --- | --- |
| 1 | 45 Sin | 0x04 0x05 0x3a | 45Sin=0.71 | PASS |
| 2 | 45 Cos | 0x04 0x05 0x39 | 45Cos=0.71 | PASS |
| 3 | 45 Tan | 0x04 0x05 0x38 | 45Tan=1.00 | PASS |
| 4 | 90 Sin | 0x09 0x00 0x3a | 90Sin=1.00 | PASS |
| 5 | 90 Tan (Err) | 0x09 0x00 0x38 | 90Tan=Err | PASS |
| 6 | 04 x² | 0x00 0x04 0x3d | 04x^2=16 | PASS |
| 7 | 12 x² (Err) | 0x01 0x02 0x3d | 12x^2=Err | PASS |
| 8 | 04 n! | 0x00 0x04 0x36 | 04n!=24 | PASS |
| 9 | 05 n! (Err) | 0x00 0x05 0x36 | 05n!=Err | PASS |
| 10 | 00 n! | 0x00 0x00 0x36 | 00n!=1 | PASS |
| 11 | 01 n! | 0x00 0x01 0x36 | 01n!=1 | PASS |
| 12 | 03 n! | 0x00 0x03 0x36 | 03n!=6 | PASS |
| 13 | CE mid-entry | 0x05 0x11 0x00 0x04 0x3d | 5[CLR]04x^2=16 | PASS |

All 13 cases passed. A full breakdown (including Accumulator and flag state after each run) is in Tutorial17_BCD_Scientific_Calculator_Test_Report.xlsx alongside this document. Clear and CE were also checked directly against the emulator: Clear/CE mid-entry correctly wipes a partly-typed value, and pressing a fresh digit while an answer is showing correctly clears the display and starts the next entry.

# 9. Try It Yourself

Assemble tutorial/17_bcd_scientific_calculator.asm, Load → CPU, click Run. Type two digits (e.g. 4 5), then press Sin, Cos, Tan, x², or n!. Try the overflow cases above (12 x², 05 n!, 90 Tan) to see the Err path, and try Clear or CE mid-entry to confirm a fresh value starts cleanly.

Open Display → Port Map Status alongside the exercise to see $F011 receive each digit and function code, and Registers to watch the Carry flag after every DADD in DO_SQR/DO_FAC — that's the overflow test firing, exactly the moment a result needs a third digit.

# 10. Ideas to Extend This Program

- Add Log and 1/x. Log has no cheap BCD trick either — another lookup table, using the same BCD_TO_BIN conversion this program already does for Sin/Cos/Tan. 1/x needs a genuine fractional result representation this program doesn't have yet.

- Extend x²'s range past two digits using DADDC to chain a second BCD byte, the same extension exercise 13 suggests for its four operators.

- Add x³ by JSR-ing into DO_SQR's pattern twice, or generalize DO_SQR/DO_FAC into one “repeat DADD N times” subroutine shared by both.

# Appendix: Complete Source Listing

tutorial/17_bcd_scientific_calculator.asm

# ================================================================

# 17_bcd_scientific_calculator.asm

# Beboputer Hands-On Tutorial — Bonus Exercise (Calculator)

#

# A small scientific calculator: type a two-digit value, press Sin,

# Cos, Tan, x^2, or n!, and read the answer off the display. Unlike

# exercise 12 (which stored everything in plain binary), the typed

# value here is packed BCD throughout, the same representation

# exercise 13's four-function calculator uses - x^2 and n! are

# genuine BCD arithmetic (DADD, the same way exercise 13 multiplies),

# not table lookups.

#

# What this program does

# -----------------------

#   Type a two-digit value (always two digits - a leading zero for

#   single-digit values, e.g. 05), then press one of the five

#   scientific keys:

#       4  5   Sin    ->  shows  45Sin=0.71

#       4  5   Cos    ->  shows  45Cos=0.71

#       4  5   Tan    ->  shows  45Tan=1.00

#       9  0   Sin    ->  shows  90Sin=1.00

#       9  0   Tan    ->  shows  90Tan=Err   (out of range - see below)

#       0  4   x^2    ->  shows  04x^2=16

#       1  2   x^2    ->  shows  12x^2=Err   (144 needs 3 digits)

#       0  4   n!     ->  shows  04n!=24

#       0  5   n!     ->  shows  05n!=Err    (120 needs 3 digits)

#   Sin and Cos accept 00-90. Tan accepts 00-68 (see exercise 12 for

#   why tan's table has to stop there - the same limit applies here).

#   x^2 and n! both top out well before 99 in packed BCD (two digits,

#   00-99 only) - anything whose true answer needs a third digit

#   shows Err instead, the same convention exercise 13 uses for its

#   four operators.

#

# Why x^2 and n! are "real" BCD arithmetic, but Sin/Cos/Tan aren't

# -------------------------------------------------------------------

#   x^2 and n! are computed with this CPU's DADD instruction, exactly

#   the way exercise 13 builds multiply out of repeated BCD addition -

#   x^2 is just "add X to a running total, X times" (X*X), and n! is

#   "multiply the running total by X, X-1, X-2, ... down to 1", where

#   each of those multiplies is itself a repeated-BCD-addition loop.

#   Every intermediate value stays packed BCD from typed input to

#   displayed output - see DO_SQR and DO_FAC below.

#

#   Sine, cosine, and tangent can't be built out of adds and

#   subtracts at all (see exercise 12's header for the full

#   explanation) - the only way this CPU produces them is the same

#   precomputed lookup table exercise 12 uses, addressed by a plain

#   binary index. Since the typed value is stored as packed BCD here

#   (not plain binary, like exercise 12), BCD_TO_BIN below converts

#   it to a binary 0-99 index before every trig lookup - the table

#   itself, and the FORMAT_VAL display routine that turns a table

#   entry into "W.FF" on screen, are carried over from exercise 12

#   unchanged, since a lookup table has no "BCD version" to switch to.

#

# How it works

# -------------

#   Same keypad-polling state machine as exercises 10-13:

#

#     STAGE 0   waiting for the tens digit of X

#     STAGE 1   waiting for the ones digit of X

#     STAGE 2   waiting for Sin, Cos, Tan, x^2, or n!

#     STAGE 3   answer is showing - next digit clears and restarts

#

#   Entry packs the two typed digits into one BCD byte (XVAL) exactly

#   the way exercise 13 packs each operand - SHL x4 moves the tens

#   digit into the top nibble, then OR merges the ones digit into the

#   bottom nibble.

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run. START clears the display first

#   (the display isn't blank by default - it still shows whatever the

#   last program left there), so the exercise always begins on a clean

#   screen no matter what ran before it. Type two digits, then press

#   Sin, Cos, Tan, x^2, or n!. Try the overflow cases above to see the

#   Err path, and try Clear/CE mid-entry to confirm a fresh sum starts

#   cleanly.

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status to see $F011 receive each digit

#   and function code, and Registers to watch the Carry flag after

#   each DADD in DO_SQR/DO_FAC - that's the overflow test firing.

#

# Try this next

# ---------------

#   - Add Log and 1/x. Log has no cheap BCD trick either (another

#     lookup table, same BCD_TO_BIN conversion this program already

#     does for Sin/Cos/Tan); 1/x needs a genuine fractional result

#     representation this program doesn't have yet.

#   - Extend x^2's range past two digits using DADDC to chain a

#     second BCD byte, the same way exercise 13's "Try this next"

#     suggests for its four operators.

# ================================================================

KEY:      .EQU    $F011

DISP:     .EQU    $F031

SIN_KEY:  .EQU    $3A

COS_KEY:  .EQU    $39

TAN_KEY:  .EQU    $38

SQR_KEY:  .EQU    $3D        # x^2

FAC_KEY:  .EQU    $36        # n!

.ORG    $4000

START:  LDA     $1B             # clear code - the display isn't blank on

STA     [DISP]          # its own; it still holds whatever the

# previously run program left showing

LDA     $00

STA     [STAGE]        # begin waiting for the tens digit

# ---------------------------------------------------------------

# Main loop - poll the keypad, dispatch on STAGE

# ---------------------------------------------------------------

WAIT:   LDA     [KEY]

CMPA    $FF             # $FF = idle, no key waiting

JZ      [WAIT]

STA     [KEYVAL]        # stash it - this is the ONLY read of KEY

CMPA    $10             # Clear key

JZ      [DO_CLEAR]

CMPA    $11             # CE key

JZ      [DO_CLEAR]

LDA     [STAGE]

CMPA    $00

JZ      [ST_TENS]

CMPA    $01

JZ      [ST_ONES]

CMPA    $03

JZ      [ST_NEWCALC]

JMP     [ST_FUNC]        # only remaining value is STAGE 2

# ---------------------------------------------------------------

# STAGE 3 -> 0 : answer was showing, this digit starts a new value

# ---------------------------------------------------------------

ST_NEWCALC:

LDA     $1B              # clear code - wipes the old expression

STA     [DISP]

JMP     [ST_TENS]

# ---------------------------------------------------------------

# STAGE 0 : tens digit -> top nibble of XVAL (packed BCD)

# ---------------------------------------------------------------

ST_TENS:

LDA     [KEYVAL]

STA     [DISP]           # echo the digit as typed

LDA     [KEYVAL]

AND     $0F              # ASCII '0'-'9' -> binary 0-9

SHL

SHL

SHL

SHL                      # move it into the top nibble

STA     [XVAL]

LDA     $01

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 1 : ones digit -> bottom nibble of XVAL

# ---------------------------------------------------------------

ST_ONES:

LDA     [KEYVAL]

STA     [DISP]

LDA     [KEYVAL]

AND     $0F

OR      [XVAL]           # merge with the tens nibble already there

STA     [XVAL]           # XVAL is now one packed-BCD byte, 00-99

LDA     $02

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 2 : waiting for Sin, Cos, Tan, x^2, or n! - ignore anything else

# ---------------------------------------------------------------

ST_FUNC:

LDA     [KEYVAL]

STA     [FUNCKEY]         # remembered for the display echo below

CMPA    SIN_KEY

JZ      [DO_SIN]

CMPA    COS_KEY

JZ      [DO_COS]

CMPA    TAN_KEY

JZ      [DO_TAN]

CMPA    SQR_KEY

JZ      [DO_SQR]

CMPA    FAC_KEY

JZ      [DO_FAC]

JMP     [WAIT]

# ---------------------------------------------------------------

# BCD_TO_BIN - convert packed-BCD XVAL into a plain-binary 0-99

# value in BINVAL, for use as a table index (BLDX/[TABLE,X] need an

# ordinary binary offset, not a packed-BCD byte). Same tens*10+ones

# combine exercise 12 uses, just starting from BCD nibbles instead

# of two separately-typed digits.

# ---------------------------------------------------------------

BCD_TO_BIN:

LDA     [XVAL]

STA     [TEMP]

SHR

SHR

SHR

SHR                      # tens digit (0-9), now in the low nibble

STA     [TCNT]

LDA     $00

STA     [BINVAL]

JMP     [B2B_TEST]

B2B_LOOP:

LDA     [BINVAL]

ADD     $0A              # +10 per tens digit (plain binary add -

STA     [BINVAL]         # BINVAL itself is never BCD)

LDA     [TCNT]

DECA

STA     [TCNT]

B2B_TEST:

LDA     [TCNT]

CMPA    $00

JNZ     [B2B_LOOP]

LDA     [TEMP]

AND     $0F              # ones digit (0-9)

ADD     [BINVAL]

STA     [BINVAL]         # BINVAL = tens*10+ones, plain binary 0-99

RTS

# ---------------------------------------------------------------

# Sin and Cos: valid for XVAL 00-90. Look up SIN_TABLE[BINVAL]

# (scaled x100, plain binary - same table format as exercise 12) and

# format it with FORMAT_VAL.

# ---------------------------------------------------------------

DO_SIN: JSR     [BCD_TO_BIN]

LDA     [BINVAL]

CMPA    $5A              # 90 decimal

JC      [ERR_OUT]        # BINVAL > 90: outside this table

LDA     $00

STA     [DEGIDX]

LDA     [BINVAL]

STA     [DEGIDX+1]

BLDX    [DEGIDX]

LDA     [SIN_TABLE,X]

STA     [VAL]

JSR     [FORMAT_VAL]

JMP     [RESET_STATE]

DO_COS: JSR     [BCD_TO_BIN]

LDA     [BINVAL]

CMPA    $5A              # 90 decimal

JC      [ERR_OUT]

LDA     $00

STA     [DEGIDX]

LDA     [BINVAL]

STA     [DEGIDX+1]

BLDX    [DEGIDX]

LDA     [COS_TABLE,X]

STA     [VAL]

JSR     [FORMAT_VAL]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Tan: valid for XVAL 00-68 only - same table-size limit as

# exercise 12 (tan(69)x100 = 261, too big for one byte).

# ---------------------------------------------------------------

DO_TAN: JSR     [BCD_TO_BIN]

LDA     [BINVAL]

CMPA    $44              # 68 decimal

JC      [ERR_OUT]

LDA     $00

STA     [DEGIDX]

LDA     [BINVAL]

STA     [DEGIDX+1]

BLDX    [DEGIDX]

LDA     [TAN_TABLE,X]

STA     [VAL]

JSR     [FORMAT_VAL]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# x^2:  RESULT = XVAL * XVAL, in packed BCD, via XVAL repeated BCD

# additions of XVAL - exactly exercise 13's DO_MUL pattern with both

# operands equal to XVAL. MCNT is itself packed BCD (it started as

# XVAL), so it counts down with DSUB $01, not DECA - see exercise

# 13's header note for why.

# ---------------------------------------------------------------

DO_SQR: LDA     $00

STA     [RESULT]

LDA     [XVAL]

STA     [MCNT]

JMP     [SQR_TEST]

SQR_LOOP:

LDA     [RESULT]

DADD    [XVAL]

STA     [RESULT]

JC      [ERR_OUT]        # running total spilled past 2 digits

LDA     [MCNT]

DSUB    $01

STA     [MCNT]

SQR_TEST:

LDA     [MCNT]

CMPA    $00

JNZ     [SQR_LOOP]

JMP     [SHOW_BCD_RESULT]

# ---------------------------------------------------------------

# n!:  RESULT = XVAL * (XVAL-1) * (XVAL-2) * ... * 1, in packed BCD.

#

# FCNT counts down from XVAL to 1 (BCD, DSUB $01). Each pass

# multiplies RESULT by FCNT the same way DO_SQR multiplies - by

# repeated BCD addition - using FICNT as that inner loop's own

# countdown and FMUL_ACC to hold the value being repeatedly added

# (RESULT itself is rebuilt from $00 as the inner loop runs, so it

# can't also be the thing being added).

# 0! and 1! both correctly come out as 1 (RESULT starts at 1 and the

# loop never subtracts below it for XVAL 0 or 1).

# ---------------------------------------------------------------

DO_FAC: LDA     $01

STA     [RESULT]         # 0! = 1! = 1

LDA     [XVAL]

CMPA    $00

JZ      [SHOW_BCD_RESULT]

LDA     $01

CMPA    [XVAL]

JZ      [SHOW_BCD_RESULT] # XVAL == 1: RESULT already 1, done

LDA     [XVAL]

STA     [FCNT]

FAC_TEST:

LDA     [FCNT]

CMPA    $00

JZ      [SHOW_BCD_RESULT]

LDA     [RESULT]

STA     [FMUL_ACC]        # value to add, repeatedly, below

LDA     $00

STA     [RESULT]          # rebuilt as FCNT copies of FMUL_ACC

LDA     [FCNT]

STA     [FICNT]

FAC_INNER:

LDA     [FICNT]

CMPA    $00

JZ      [FAC_INNER_DONE]

LDA     [RESULT]

DADD    [FMUL_ACC]

STA     [RESULT]

JC      [ERR_OUT]         # running total spilled past 2 digits

LDA     [FICNT]

DSUB    $01

STA     [FICNT]

JMP     [FAC_INNER]

FAC_INNER_DONE:

LDA     [FCNT]

DSUB    $01

STA     [FCNT]

JMP     [FAC_TEST]

# ---------------------------------------------------------------

# Out-of-range or overflowed result for whichever function was pressed

# ---------------------------------------------------------------

ERR_OUT:

JSR     [ECHO_FUNC]

LDA     $3D              # '='

STA     [DISP]

LDA     $45              # 'E'

STA     [DISP]

LDA     $72              # 'r'

STA     [DISP]

LDA     $72              # 'r'

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# ECHO_FUNC - print the two/three-letter name of whichever

# scientific key was pressed (FUNCKEY), so the expression on screen

# reads e.g. "45Sin=0.71" instead of just "45=0.71". Called once,

# from FORMAT_VAL/SHOW_BCD_RESULT's shared entry points below.

# ---------------------------------------------------------------

ECHO_FUNC:

LDA     [FUNCKEY]

CMPA    SIN_KEY

JZ      [EF_SIN]

CMPA    COS_KEY

JZ      [EF_COS]

CMPA    TAN_KEY

JZ      [EF_TAN]

CMPA    SQR_KEY

JZ      [EF_SQR]

JMP     [EF_FAC]         # only remaining key is n!

EF_SIN: LDA     $53              # 'S'

STA     [DISP]

LDA     $69              # 'i'

STA     [DISP]

LDA     $6E              # 'n'

STA     [DISP]

RTS

EF_COS: LDA     $43              # 'C'

STA     [DISP]

LDA     $6F              # 'o'

STA     [DISP]

LDA     $73              # 's'

STA     [DISP]

RTS

EF_TAN: LDA     $54              # 'T'

STA     [DISP]

LDA     $61              # 'a'

STA     [DISP]

LDA     $6E              # 'n'

STA     [DISP]

RTS

EF_SQR: LDA     $78              # 'x'

STA     [DISP]

LDA     $5E              # '^'

STA     [DISP]

LDA     $32              # '2'

STA     [DISP]

RTS

EF_FAC: LDA     $6E              # 'n'

STA     [DISP]

LDA     $21              # '!'

STA     [DISP]

RTS

# ---------------------------------------------------------------

# FORMAT_VAL - shared by DO_SIN/DO_COS/DO_TAN via JSR/RTS. Carried

# over from exercise 12 unchanged (see the header note on why the

# trig table/display path has no BCD version).

#

# Turns VAL (0-255, meaning 0.00-2.55) into "Fn=W.FF" on the display

# (Fn from ECHO_FUNC, then '=', then the value):

#   1. Subtract 100 from VAL while it still fits - the count (0-2)

#      is the single whole-number digit.

#   2. Subtract 10 from what's left while it still fits - the count

#      (0-9) is the tenths digit; whatever remains (0-9) is the

#      hundredths digit.

# ---------------------------------------------------------------

FORMAT_VAL:

JSR     [ECHO_FUNC]

LDA     $3D              # '='

STA     [DISP]

LDA     $00

STA     [WHOLE]

FV_100_LOOP:

LDA     [VAL]

CMPA    $64              # 100

JC      [FV_100_SUB]     # C=1: VAL > 100

JZ      [FV_100_SUB]     # Z=1: VAL == 100

JMP     [FV_100_DONE]    # VAL < 100, done

FV_100_SUB:

LDA     [VAL]

SUB     $64

STA     [VAL]

LDA     [WHOLE]

INCA

STA     [WHOLE]

JMP     [FV_100_LOOP]

FV_100_DONE:

LDA     [WHOLE]          # single digit, 0-2

OR      $30

STA     [DISP]

LDA     $2E              # '.'

STA     [DISP]

LDA     $00

STA     [FTENS]

FV_10_LOOP:

LDA     [VAL]

CMPA    $0A

JC      [FV_10_SUB]      # C=1: VAL > 10

JZ      [FV_10_SUB]      # Z=1: VAL == 10

JMP     [FV_10_DONE]     # VAL < 10, done

FV_10_SUB:

LDA     [VAL]

SUB     $0A

STA     [VAL]

LDA     [FTENS]

INCA

STA     [FTENS]

JMP     [FV_10_LOOP]

FV_10_DONE:

LDA     [FTENS]          # tenths digit - always shown, no

OR      $30              # leading-zero suppression, so the

STA     [DISP]           # fraction always prints as 2 digits

LDA     [VAL]            # hundredths digit, whatever is left

OR      $30

STA     [DISP]

RTS

# ---------------------------------------------------------------

# SHOW_BCD_RESULT - shared by DO_SQR/DO_FAC. RESULT is already

# packed BCD (00-99), so unlike FORMAT_VAL this needs no conversion

# loop at all - just split the nibbles (see exercise 13's header

# note for the full explanation of why packed BCD skips the loop).

# ---------------------------------------------------------------

SHOW_BCD_RESULT:

JSR     [ECHO_FUNC]

LDA     $3D              # '='

STA     [DISP]

LDA     [RESULT]

STA     [TEMP]           # keep a copy - SHR below destroys ACC

SHR

SHR

SHR

SHR                      # tens digit, 0-9, now in the low nibble

CMPA    $00

JZ      [SBR_ONES]       # suppress a leading zero on the tens digit

OR      $30

STA     [DISP]

SBR_ONES:

LDA     [TEMP]

AND     $0F              # ones digit

OR      $30

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Explicit Clear / CE handling

# ---------------------------------------------------------------

DO_CLEAR:

LDA     $1B

STA     [DISP]

LDA     $00

STA     [STAGE]

JMP     [WAIT]

RESET_STATE:

LDA     $03              # STAGE 3: next digit clears & restarts

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# Variables (reserved bytes - see exercise 09 for this pattern)

# ---------------------------------------------------------------

STAGE:    .BYTE

XVAL:     .BYTE       # packed BCD, 00-99, the typed value

KEYVAL:   .BYTE

FUNCKEY:  .BYTE

TEMP:     .BYTE

TCNT:     .BYTE

BINVAL:   .BYTE       # XVAL converted to plain binary, for table indexing

DEGIDX:   .2BYTE

VAL:      .BYTE       # trig table result, binary fixed-point x100

WHOLE:    .BYTE

FTENS:    .BYTE

RESULT:   .BYTE       # packed BCD, x^2/n! result

MCNT:     .BYTE

FCNT:     .BYTE

FICNT:    .BYTE

FMUL_ACC: .BYTE

# ---------------------------------------------------------------

# SIN_TABLE[d] = round(sin(d degrees) x 100), d = 0..90

# (identical to exercise 12's table - see that exercise for how it

# was generated)

# ---------------------------------------------------------------

SIN_TABLE:

.BYTE   $00, $02, $03, $05, $07, $09, $0A, $0C, $0E, $10

.BYTE   $11, $13, $15, $16, $18, $1A, $1C, $1D, $1F, $21

.BYTE   $22, $24, $25, $27, $29, $2A, $2C, $2D, $2F, $30

.BYTE   $32, $34, $35, $36, $38, $39, $3B, $3C, $3E, $3F

.BYTE   $40, $42, $43, $44, $45, $47, $48, $49, $4A, $4B

.BYTE   $4D, $4E, $4F, $50, $51, $52, $53, $54, $55, $56

.BYTE   $57, $57, $58, $59, $5A, $5B, $5B, $5C, $5D, $5D

.BYTE   $5E, $5F, $5F, $60, $60, $61, $61, $61, $62, $62

.BYTE   $62, $63, $63, $63, $63, $64, $64, $64, $64, $64

.BYTE   $64

# ---------------------------------------------------------------

# COS_TABLE[d] = round(cos(d degrees) x 100), d = 0..90

# ---------------------------------------------------------------

COS_TABLE:

.BYTE   $64, $64, $64, $64, $64, $64, $63, $63, $63, $63

.BYTE   $62, $62, $62, $61, $61, $61, $60, $60, $5F, $5F

.BYTE   $5E, $5D, $5D, $5C, $5B, $5B, $5A, $59, $58, $57

.BYTE   $57, $56, $55, $54, $53, $52, $51, $50, $4F, $4E

.BYTE   $4D, $4B, $4A, $49, $48, $47, $45, $44, $43, $42

.BYTE   $40, $3F, $3E, $3C, $3B, $39, $38, $36, $35, $34

.BYTE   $32, $30, $2F, $2D, $2C, $2A, $29, $27, $25, $24

.BYTE   $22, $21, $1F, $1D, $1C, $1A, $18, $16, $15, $13

.BYTE   $11, $10, $0E, $0C, $0A, $09, $07, $05, $03, $02

.BYTE   $00

# ---------------------------------------------------------------

# TAN_TABLE[d] = round(tan(d degrees) x 100), d = 0..68 only - see

# exercise 12's header note: tan(69)x100 = 261, too big for one byte.

# ---------------------------------------------------------------

TAN_TABLE:

.BYTE   $00, $02, $03, $05, $07, $09, $0B, $0C, $0E, $10

.BYTE   $12, $13, $15, $17, $19, $1B, $1D, $1F, $20, $22

.BYTE   $24, $26, $28, $2A, $2D, $2F, $31, $33, $35, $37

.BYTE   $3A, $3C, $3E, $41, $43, $46, $49, $4B, $4E, $51

.BYTE   $54, $57, $5A, $5D, $61, $64, $68, $6B, $6F, $73

.BYTE   $77, $7B, $80, $85, $8A, $8F, $94, $9A, $A0, $A6

.BYTE   $AD, $B4, $BC, $C4, $CD, $D6, $E1, $EC, $F8

.END    $4000