# Sin, Cos, and Tan: Trig Functions on the Beboputer

A Beboputer / PY-DIYCALCULATOR assembly-language tutorial

# 1. What You Will Build

This tutorial makes the Calculator’s Sin, Cos, and Tan buttons actually do something. Type a two-digit angle in degrees, press one of the three trig buttons, and read the answer straight off the display:

4  5   Sin    ->  shows  45=0.71

4  5   Cos    ->  shows  45=0.71

4  5   Tan    ->  shows  45=1.00

9  0   Sin    ->  shows  90=1.00

9  0   Tan    ->  shows  90=Err   (tangent has no finite value at 90 degrees)

The source file is tutorial/12_trig_functions_sincostan.asm. It reuses the state-machine and display techniques from the earlier calculator tutorials, and introduces two new ideas: computing a function this CPU cannot compute directly by looking answers up in a table instead, and sharing code between three near-identical buttons with a real subroutine call (JSR/RTS).

# 2. Before You Start

This tutorial builds directly on:

- 10_calculator_four_function.asm — the keypad state-machine pattern, and reading/writing the Calculator’s display port.

- 11_circle_circumference_button.asm — fixed-point decimal math (scaling by 100 to fake a decimal point), 16-bit arithmetic, and this CPU’s Carry-flag quirk after SUB/CMPA.

- The 4-Function Calculator and Circumference Button tutorial documents — port map, keypad codes, and why a lone JNC/JC branch is not enough to test “greater than or equal to.”

# 3. Sin, Cos, and Tan Are Already Buttons

Exercise 11 had to claim a blank button from the left-hand reserve bank, because Circumference isn’t a function the Calculator ships with. Sin, Cos, and Tan are different — they’re real buttons in the scientific column already, wired up in _BUTTON_DEFAULTS (tools/calculator.py) exactly like every digit and operator key:

"Sin":  (0x73, "Sine of x")

"Cos":  (0x63, "Cosine of x")

"Tan":  (0x74, "Tangent of x")

Nothing currently listens for those three bytes on port $F011, so today the buttons light up and do nothing. There is no Configure Button Attributes step for this exercise — just assemble the program below, Load → CPU, click Run, and the existing Sin/Cos/Tan buttons come alive.

# 4. Program Design

The angle is always entered as two digits (type a leading zero for single-digit angles, e.g. 05 for 5 degrees), so the state machine needs one more stage than exercise 11’s single-digit radius entry:

| STAGE | The program is... |
| --- | --- |
| 0 | Waiting for the tens digit of the angle. |
| 1 | Waiting for the ones digit of the angle. |
| 2 | Waiting for Sin, Cos, or Tan. |
| 3 | Showing an answer — the next digit clears the display and starts a new one. |

Combining the two typed digits into one angle (0–99) is the same repeated-addition idea exercise 11 used for multiplying by a constant, just 8-bit and much shorter — add 10 to a running total once per unit of the tens digit, then add the ones digit once:

MUL10_LOOP:

LDA     [DEG]

ADD     $0A              # +10 per tens digit, no carry needed:

STA     [DEG]            # DEG never exceeds 99 here

LDA     [CNT]

DECA

STA     [CNT]

MUL10_TEST:

LDA     [CNT]

CMPA    $00

JNZ     [MUL10_LOOP]

LDA     [DEG]

ADD     [ONES]

STA     [DEG]

# 5. Why Sine Needs a Different Trick: Lookup Tables

This CPU has no MUL or DIV instruction and no concept of a fraction — every value in it is a plain 0–255 whole number. Exercises 10 and 11 got by with ADD and repeated subtraction, because addition, subtraction, and multiplying/dividing by a fixed amount (like 2π) can all be built out of a short loop of those. Sine and cosine cannot — there is no short loop of adds and subtracts that produces “the sine of 37 degrees” starting from nothing.

The way around this is to not compute it at all. A table of every answer, worked out ahead of time and assembled straight into the program as data, is just as valid a “function” as a formula — looking a value up is exactly what a function does, and a table lookup is something this CPU is very good at. SIN_TABLE holds 91 bytes: entry 0 is sin(0°)×100 rounded to the nearest whole number, entry 1 is sin(1°)×100, and so on up to entry 90 = sin(90°)×100 = 100. (The ×100 scaling is the same fixed-point idea exercise 11 used for π — it turns 0.71 into the whole number 71, which this CPU can actually store.) COS_TABLE is the same idea for cosine. Reading the angle straight out of memory and using it as an index into these tables — byte SIN_TABLE+37 IS sin(37°)×100, no arithmetic required — is the entire “calculation.”

# 6. Using the Index Register for Table Lookups

Table lookups need the Index register (IX), which exercise 09 introduced for walking a message character by character. The same [TABLE,X] addressing mode — “read the byte at TABLE’s address, plus whatever is currently in IX” — is exactly a table lookup once IX holds the angle instead of a walking counter. BLDX loads IX from a 2-byte memory location (the same big-endian, high-byte-first layout as RESULT in exercise 11), so DEGIDX exists purely to hold the angle as a 2-byte value — high byte always $00, low byte the angle 0–99 — so BLDX has somewhere 16-bit to load it from:

LDA     $00

STA     [DEGIDX]         # high byte of the 16-bit index = 0

LDA     [DEG]

STA     [DEGIDX+1]       # low byte = the angle itself

BLDX    [DEGIDX]

LDA     [SIN_TABLE,X]

STA     [VAL]

# 7. Sharing Code with JSR/RTS

Sin, Cos, and Tan all finish exactly the same way: take whatever byte the table lookup produced (0–255, meaning 0.00–2.55) and turn it into “=W.FF” on the display. Rather than writing that formatting logic three times, this exercise introduces one instruction the earlier tutorials never needed: JSR / RTS.

JSR pushes the address of the instruction right after it onto the stack, then jumps to a subroutine; RTS pops that address back off and returns there. That is why DO_SIN, DO_COS, and DO_TAN can all JSR [FORMAT_VAL] — the same shared routine — and each correctly comes back to its own JMP [RESET_STATE] afterwards, instead of all three ending up in the same place:

DO_SIN:

LDA     [DEG]

CMPA    $5A              # 90 decimal

JC      [ERR_OUT]        # DEG > 90: outside this table

LDA     $00

STA     [DEGIDX]

LDA     [DEG]

STA     [DEGIDX+1]

BLDX    [DEGIDX]

LDA     [SIN_TABLE,X]

STA     [VAL]

JSR     [FORMAT_VAL]

JMP     [RESET_STATE]

FORMAT_VAL itself is exercise 11’s repeated-subtraction decimal conversion again, run twice: subtract 100 from VAL while it still fits (the count becomes the one whole-number digit, 0–2), then subtract 10 from what is left while it still fits (the count becomes the tenths digit, 0–9, and whatever remains after that, also 0–9, is the hundredths digit). Both loops use the same CMPA/JC/JZ “greater-than-or-equal” pair the earlier tutorials rely on throughout, for the same reason: this CPU’s Carry flag alone only ever means strictly-greater-than, so a lone JNC would miss the case where VAL lands exactly on 100 or exactly on 10.

# 8. Why Tan Stops at 68 Degrees

Tangent is built from the same kind of table (TAN_TABLE), but it cannot cover the same 0–90 range sine and cosine do. Mathematically, tan(x) grows without bound as x approaches 90 — tan(89°) is already about 57.3 — and this table can only hold whole numbers 0–255 per byte, which caps it at 2.55 once scaled ×100. tan(68°)×100 rounds to 248, right up against that ceiling; tan(69°)×100 would be 261, which no longer fits in a byte.

So TAN_TABLE stops at 68 entries, and DO_TAN checks the angle against that limit the same way exercise 10’s DO_DIV checks for a zero divisor before it would produce a nonsense answer — out of range means Err, not a wrong number silently truncated to fit:

DO_TAN:

LDA     [DEG]

CMPA    $44              # 68 decimal

JC      [ERR_OUT]        # DEG > 68: table stops here

...

ERR_OUT:

LDA     $3D              # '='

STA     [DISP]

LDA     $45              # 'E'

STA     [DISP]

LDA     $72              # 'r'

STA     [DISP]

LDA     $72              # 'r'

STA     [DISP]

JMP     [RESET_STATE]

The same Err convention exercise 10 uses for divide-by-zero is reused here — a wrong-looking answer is worse than an honest “this exercise cannot compute that,” and Err says exactly that.

# 9. Verified Against the Real CPU Emulator

This program was assembled with the project’s own das.py and run against the project’s own CPU class with simulated keypresses — not just read and assumed correct. Every one of the 91 possible Sin and Cos angles (0–90°) and all 69 valid Tan angles (0–68°) was checked against Python’s math.sin/cos/tan, along with every out-of-range angle that should show Err. A representative sample:

| Angle | Sin | Cos | Tan |
| --- | --- | --- | --- |
| 0° | 0=0.00 | 0=1.00 | 0=0.00 |
| 15° | 15=0.26 | 15=0.97 | 15=0.27 |
| 30° | 30=0.50 | 30=0.87 | 30=0.58 |
| 45° | 45=0.71 | 45=0.71 | 45=1.00 |
| 60° | 60=0.87 | 60=0.50 | 60=1.73 |
| 68° | 68=0.93 | 68=0.37 | 68=2.48 (Tan's last valid angle) |
| 69° | 69=0.93 | 69=0.36 | 69=Err (out of Tan's range) |
| 90° | 90=1.00 | 90=0.00 | 90=Err (undefined) |

Behavior around Clear, CE, and re-entry was also checked directly against the emulator: Clear mid-entry correctly wipes a partly-typed angle; CE after an answer correctly starts a new calculation; typing digit “1” as part of an angle is correctly read as the digit 1 and not confused with CE (CE’s own code was moved to $7F in an earlier fix specifically so it can never collide with a raw digit — see the 4-Function Calculator tutorial for the full story); and pressing a fresh digit while an answer is showing correctly clears the display and starts the next entry.

# 10. Try It Yourself

Assemble tutorial/12_trig_functions_sincostan.asm, Load → CPU, click Run. On the Calculator, type two digits (e.g. 3 0 for 30 degrees), then press Sin, Cos, or Tan. Check the answer against any calculator’s sin/cos/tan in degree mode — it should match to two decimal places (Tan will be slightly rounder near its table limit, since a single byte only has so much room).

Open Display → Port Map Status alongside the exercise to see $F011 receive $73/$63/$74 the instant you press Sin/Cos/Tan, and Registers to watch IX load with the angle right before each table lookup.

# 11. Ideas to Extend This Program

- Extend TAN_TABLE’s usable range by scaling it ×10 instead of ×100 (less precision, but tan(x) up to about 25 would then fit in a byte), or by storing it as a 2-byte table the way RESULT was 16-bit in exercise 11.

- Add an ASin/ACos/ATan button that runs SIN_TABLE “backwards” — scan it for the closest match to a typed value and report the index (angle) that produced it.

- Accept a flexible one-or-two-digit angle instead of always requiring two digits, the same extension suggested for the radius in exercise 11.

# Appendix: Complete Source Listing

tutorial/12_trig_functions_sincostan.asm

# ================================================================

# 12_trig_functions_sincostan.asm

# Beboputer Hands-On Tutorial — Bonus Exercise (Calculator)

#

# Type an angle in degrees, press Sin, Cos, or Tan, and read the

# answer straight off the display.

#

# Before you assemble this: no button setup needed

# -------------------------------------------------

#   Exercise 11 had to claim a blank button because Circumference

#   isn't a button the Calculator already has. Sin, Cos, and Tan are

#   different - they're real buttons in the scientific column on the

#   left of the keypad already, wired up in _BUTTON_DEFAULTS same as

#   every other key:

#       Sin -> Code $73   Cos -> Code $63   Tan -> Code $74

#   Nothing currently listens for those three bytes on port $F011,

#   so today they light up and do nothing. This exercise is what

#   makes them do something - just assemble, load, and run; there is

#   no Configure Button Attributes step to do first.

#

# What this program does

# -----------------------

#   Type a two-digit angle in degrees (always two digits - type a

#   leading zero for single-digit angles, e.g. 05 for 5 degrees),

#   then press Sin, Cos, or Tan. The display fills in "=" followed

#   by the answer to two decimal places:

#       4  5   Sin    ->  shows  =0.71

#       4  5   Cos    ->  shows  =0.71

#       4  5   Tan    ->  shows  =1.00

#       9  0   Sin    ->  shows  =1.00

#       0  0   Cos    ->  shows  =1.00

#       9  0   Tan    ->  shows  =Err   (tangent has no finite value

#                                        at 90 degrees)

#   Sin and Cos accept any angle from 00 to 90. Tan only goes up to

#   68 - see "How it works" for why - anything past that (including

#   90) shows Err, the same way exercise 10 shows Err for divide by

#   zero rather than a wrong number.

#

# How it works

# -------------

#   Same keypad-polling state machine as exercises 10 and 11, just

#   with one more stage for the second digit:

#

#     STAGE 0   waiting for the tens digit of the angle

#     STAGE 1   waiting for the ones digit of the angle

#     STAGE 2   waiting for Sin, Cos, or Tan

#     STAGE 3   answer is showing - next digit clears and restarts

#

#   The interesting part is how the actual sine/cosine/tangent gets

#   computed, because this CPU has no MUL or DIV instruction and no

#   idea what a fraction is - every value in it is a plain 0-255

#   whole number. Exercises 10 and 11 got by with ADD and repeated

#   subtraction because + and - (and even x and / by a fixed amount

#   like 2xpi) can be built out of those. Sine and cosine can't -

#   there's no short loop of adds and subtracts that produces

#   "the sine of 37 degrees" from scratch.

#

#   The trick is to not compute it at all. A table of every answer,

#   worked out ahead of time and assembled straight into the

#   program as data, is just as valid a "function" as a formula -

#   looking a value up is exactly what a function does, and a table

#   lookup is something this CPU is very good at. So SIN_TABLE below

#   holds 91 bytes: entry 0 is sin(0 degrees) x100 rounded to the

#   nearest whole number, entry 1 is sin(1 degree) x100, and so on

#   up to entry 90 = sin(90 degrees) x100 = 100. (The x100 scaling

#   is the same fixed-point idea exercise 11 used for pi - it turns

#   "0.71" into the whole number 71, which this CPU can actually

#   store.) COS_TABLE is the same idea for cosine. Reading the

#   angle straight out of memory and using it as an index into

#   these tables - byte SIN_TABLE+37 IS sin(37 degrees)x100, no

#   arithmetic required - is the entire "calculation".

#

#   Doing this needs the Index register (IX), which exercise 09

#   introduced for walking a message character by character. The

#   same [TABLE,X] addressing mode - "read the byte at TABLE's

#   address, plus whatever is currently in IX" - is exactly a table

#   lookup once IX holds the angle instead of a walking counter.

#   BLDX loads IX from a 2-byte memory location (the same

#   big-endian, high-byte-first layout as RESULT in exercise 11),

#   so DEGIDX below exists purely to hold the angle as a 2-byte

#   value (high byte always $00, low byte the angle 0-99) so BLDX

#   has somewhere 16-bit to load it from.

#

#   Tangent is built from the exact same kind of table (TAN_TABLE),

#   but it can't cover the same 0-90 range. Mathematically tan(x)

#   grows without bound as x approaches 90 - tan(89) is already

#   about 57.3 - and this table can only hold whole numbers 0-255

#   per byte (max representable is 2.55 once scaled x100). tan(68)

#   x100 rounds to 248, right up against that ceiling; tan(69)x100

#   would be 261, which no longer fits in a byte. So TAN_TABLE stops

#   at 68 entries, and DO_TAN checks the angle against that limit

#   the same way exercise 10's DO_DIV checks for a zero divisor

#   before it would produce a nonsense answer - out of range means

#   Err, not a wrong number silently truncated to fit.

#

#   One new instruction shows up here: JSR / RTS. Sin, Cos, and Tan

#   all finish the exact same way - take whatever byte the table

#   lookup produced (0-255, meaning 0.00-2.55) and turn it into

#   "=W.FF" on the display - so that display-formatting code is

#   written once, as a subroutine (FORMAT_VAL), and JSR [FORMAT_VAL]

#   calls it from all three places. JSR pushes the return address on

#   the stack and jumps there; RTS at the end of FORMAT_VAL pops

#   that address back off and returns to right after the JSR that

#   called it - which is why DO_SIN, DO_COS, and DO_TAN can each

#   JSR the same routine and each correctly come back to their own

#   JMP [RESET_STATE] afterwards, instead of all three ending up in

#   the same place.

#

#   FORMAT_VAL itself is just exercise 11's repeated-subtraction

#   decimal conversion again, run twice: subtract 100 from VAL while

#   it still fits (the count becomes the one whole-number digit,

#   0-2), then subtract 10 from what's left while it still fits (the

#   count becomes the tenths digit, 0-9, and whatever remains after

#   that, also 0-9, is the hundredths digit) - both loops use the

#   same CMPA/JC/JZ "greater-than-or-equal" pair exercises 10 and 11

#   rely on throughout, since this CPU's Carry flag alone only means

#   strictly-greater-than (see either of those exercises for the

#   full explanation if you haven't read it yet).

#

# Try it

# -------

#   Assemble, Load -> CPU, click Run. On the Calculator, type two

#   digits (e.g. 3 0 for 30 degrees), then press Sin, Cos, or Tan.

#   Check the answer against any calculator's sin/cos/tan in degree

#   mode - it should match to two decimal places (tan will be

#   slightly rounder near its table limit, since a single byte only

#   has so much room).

#

# Watch it happen

# -----------------

#   Open Display -> Port Map Status alongside this exercise to see

#   $F011 receive $73/$63/$74 the instant you press Sin/Cos/Tan, and

#   Registers to watch IX load with the angle right before each

#   table lookup.

#

# Try this next

# ---------------

#   - Extend TAN_TABLE's usable range by scaling it x10 instead of

#     x100 (less precision, but tan(x) up to about 25 would then fit

#     in a byte) or by storing it as a 2-byte table the way RESULT

#     was 16-bit in exercise 11.

#   - Add an ASin/ACos/ATan button that runs SIN_TABLE "backwards" -

#     scan it for the closest match to a typed value and report the

#     index (angle) that produced it.

#   - Accept a flexible one-or-two-digit angle instead of always

#     requiring two digits, the same extension suggested for the

#     radius in exercise 11.

# ================================================================

KEY:      .EQU    $F011

DISP:     .EQU    $F031

SIN_KEY:  .EQU    $73        # code the Sin button already sends

COS_KEY:  .EQU    $63        # code the Cos button already sends

TAN_KEY:  .EQU    $74        # code the Tan button already sends

.ORG    $4000

START:  LDA     $00

STA     [STAGE]        # begin waiting for the tens digit

# ---------------------------------------------------------------

# Main loop - poll the keypad, dispatch on STAGE

# ---------------------------------------------------------------

WAIT:   LDA     [KEY]

CMPA    $FF             # $FF = idle, no key waiting

JZ      [WAIT]

STA     [KEYVAL]        # stash it - this is the ONLY read of KEY

CMPA    $1B             # Clear key

JZ      [DO_CLEAR]

CMPA    $7F             # CE key

JZ      [DO_CLEAR]

LDA     [STAGE]

CMPA    $00

JZ      [ST_TENS]

CMPA    $01

JZ      [ST_ONES]

CMPA    $03

JZ      [ST_NEWCALC]

JMP     [ST_FUNC]        # only remaining value is STAGE 2

# ---------------------------------------------------------------

# STAGE 3 -> 0 : answer was showing, this digit starts a new one

# ---------------------------------------------------------------

ST_NEWCALC:

LDA     $1B              # clear code - wipes the old expression

STA     [DISP]

JMP     [ST_TENS]

# ---------------------------------------------------------------

# STAGE 0 : tens digit of the angle

# ---------------------------------------------------------------

ST_TENS:

LDA     [KEYVAL]

STA     [DISP]           # echo the digit as typed

LDA     [KEYVAL]

AND     $0F              # ASCII '0'-'9' -> binary 0-9

STA     [TENS]

LDA     $01

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 1 : ones digit of the angle -> combine into DEG = TENS*10+ONES

# ---------------------------------------------------------------

ST_ONES:

LDA     [KEYVAL]

STA     [DISP]           # echo the digit as typed

LDA     [KEYVAL]

AND     $0F

STA     [ONES]

LDA     $00

STA     [DEG]

LDA     [TENS]

STA     [CNT]

JMP     [MUL10_TEST]

MUL10_LOOP:

LDA     [DEG]

ADD     $0A              # +10 per tens digit, no carry needed:

STA     [DEG]            # DEG never exceeds 99 here

LDA     [CNT]

DECA

STA     [CNT]

MUL10_TEST:

LDA     [CNT]

CMPA    $00

JNZ     [MUL10_LOOP]

LDA     [DEG]

ADD     [ONES]

STA     [DEG]

LDA     $02

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# STAGE 2 : waiting for Sin, Cos, or Tan - ignore anything else

# ---------------------------------------------------------------

ST_FUNC:

LDA     [KEYVAL]

CMPA    SIN_KEY

JZ      [DO_SIN]

CMPA    COS_KEY

JZ      [DO_COS]

CMPA    TAN_KEY

JZ      [DO_TAN]

JMP     [WAIT]

# ---------------------------------------------------------------

# Sin and Cos: valid for DEG 0-90. Look up SIN_TABLE[DEG] (already

# scaled x100) and format it.

# ---------------------------------------------------------------

DO_SIN:

LDA     [DEG]

CMPA    $5A              # 90 decimal

JC      [ERR_OUT]        # DEG > 90: outside this table

LDA     $00

STA     [DEGIDX]         # high byte of the 16-bit index = 0

LDA     [DEG]

STA     [DEGIDX+1]       # low byte = the angle itself

BLDX    [DEGIDX]

LDA     [SIN_TABLE,X]

STA     [VAL]

JSR     [FORMAT_VAL]

JMP     [RESET_STATE]

DO_COS:

LDA     [DEG]

CMPA    $5A              # 90 decimal

JC      [ERR_OUT]        # DEG > 90: outside this table

LDA     $00

STA     [DEGIDX]

LDA     [DEG]

STA     [DEGIDX+1]

BLDX    [DEGIDX]

LDA     [COS_TABLE,X]

STA     [VAL]

JSR     [FORMAT_VAL]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Tan: only valid for DEG 0-68 here - see the header note on why

# TAN_TABLE has to stop there. 69 and up (including 90) show Err.

# ---------------------------------------------------------------

DO_TAN:

LDA     [DEG]

CMPA    $44              # 68 decimal

JC      [ERR_OUT]        # DEG > 68: table stops here

LDA     $00

STA     [DEGIDX]

LDA     [DEG]

STA     [DEGIDX+1]

BLDX    [DEGIDX]

LDA     [TAN_TABLE,X]

STA     [VAL]

JSR     [FORMAT_VAL]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# Out-of-range angle for whichever function was pressed

# ---------------------------------------------------------------

ERR_OUT:

LDA     $3D              # '='

STA     [DISP]

LDA     $45              # 'E'

STA     [DISP]

LDA     $72              # 'r'

STA     [DISP]

LDA     $72              # 'r'

STA     [DISP]

JMP     [RESET_STATE]

# ---------------------------------------------------------------

# FORMAT_VAL - shared by DO_SIN/DO_COS/DO_TAN via JSR/RTS.

#

# Turns VAL (0-255, meaning 0.00-2.55) into "=W.FF" on the display:

#   1. Subtract 100 from VAL while it still fits - the count (0-2)

#      is the single whole-number digit.

#   2. Subtract 10 from what's left while it still fits - the count

#      (0-9) is the tenths digit; whatever remains (0-9) is the

#      hundredths digit.

# Both loops use the CMPA/JC/JZ "greater-than-or-equal" pair, same

# reason as exercises 10 and 11: a lone JNC would miss the case

# where VAL lands exactly on 100 or exactly on 10.

# ---------------------------------------------------------------

FORMAT_VAL:

LDA     $3D              # '='

STA     [DISP]

LDA     $00

STA     [WHOLE]

FV_100_LOOP:

LDA     [VAL]

CMPA    $64              # 100

JC      [FV_100_SUB]     # C=1: VAL > 100

JZ      [FV_100_SUB]     # Z=1: VAL == 100

JMP     [FV_100_DONE]    # VAL < 100, done

FV_100_SUB:

LDA     [VAL]

SUB     $64

STA     [VAL]

LDA     [WHOLE]

INCA

STA     [WHOLE]

JMP     [FV_100_LOOP]

FV_100_DONE:

LDA     [WHOLE]          # single digit, 0-2

OR      $30

STA     [DISP]

LDA     $2E              # '.'

STA     [DISP]

LDA     $00

STA     [FTENS]

FV_10_LOOP:

LDA     [VAL]

CMPA    $0A

JC      [FV_10_SUB]      # C=1: VAL > 10

JZ      [FV_10_SUB]      # Z=1: VAL == 10

JMP     [FV_10_DONE]     # VAL < 10, done

FV_10_SUB:

LDA     [VAL]

SUB     $0A

STA     [VAL]

LDA     [FTENS]

INCA

STA     [FTENS]

JMP     [FV_10_LOOP]

FV_10_DONE:

LDA     [FTENS]          # tenths digit - always shown, no

OR      $30              # leading-zero suppression, so the

STA     [DISP]           # fraction always prints as 2 digits

LDA     [VAL]            # hundredths digit, whatever is left

OR      $30

STA     [DISP]

RTS

# ---------------------------------------------------------------

# Explicit Clear / CE handling

# ---------------------------------------------------------------

DO_CLEAR:

LDA     $1B

STA     [DISP]

LDA     $00

STA     [STAGE]

JMP     [WAIT]

RESET_STATE:

LDA     $03              # STAGE 3: next digit clears & restarts

STA     [STAGE]

JMP     [WAIT]

# ---------------------------------------------------------------

# Variables (reserved bytes - see exercise 09 for this pattern)

# ---------------------------------------------------------------

STAGE:    .BYTE

TENS:     .BYTE

ONES:     .BYTE

DEG:      .BYTE

CNT:      .BYTE

KEYVAL:   .BYTE

DEGIDX:   .2BYTE

VAL:      .BYTE

WHOLE:    .BYTE

FTENS:    .BYTE

# ---------------------------------------------------------------

# SIN_TABLE[d] = round(sin(d degrees) x 100), d = 0..90

# ---------------------------------------------------------------

SIN_TABLE:

.BYTE   $00, $02, $03, $05, $07, $09, $0A, $0C, $0E, $10

.BYTE   $11, $13, $15, $16, $18, $1A, $1C, $1D, $1F, $21

.BYTE   $22, $24, $25, $27, $29, $2A, $2C, $2D, $2F, $30

.BYTE   $32, $34, $35, $36, $38, $39, $3B, $3C, $3E, $3F

.BYTE   $40, $42, $43, $44, $45, $47, $48, $49, $4A, $4B

.BYTE   $4D, $4E, $4F, $50, $51, $52, $53, $54, $55, $56

.BYTE   $57, $57, $58, $59, $5A, $5B, $5B, $5C, $5D, $5D

.BYTE   $5E, $5F, $5F, $60, $60, $61, $61, $61, $62, $62

.BYTE   $62, $63, $63, $63, $63, $64, $64, $64, $64, $64

.BYTE   $64

# ---------------------------------------------------------------

# COS_TABLE[d] = round(cos(d degrees) x 100), d = 0..90

# ---------------------------------------------------------------

COS_TABLE:

.BYTE   $64, $64, $64, $64, $64, $64, $63, $63, $63, $63

.BYTE   $62, $62, $62, $61, $61, $61, $60, $60, $5F, $5F

.BYTE   $5E, $5D, $5D, $5C, $5B, $5B, $5A, $59, $58, $57

.BYTE   $57, $56, $55, $54, $53, $52, $51, $50, $4F, $4E

.BYTE   $4D, $4B, $4A, $49, $48, $47, $45, $44, $43, $42

.BYTE   $40, $3F, $3E, $3C, $3B, $39, $38, $36, $35, $34

.BYTE   $32, $30, $2F, $2D, $2C, $2A, $29, $27, $25, $24

.BYTE   $22, $21, $1F, $1D, $1C, $1A, $18, $16, $15, $13

.BYTE   $11, $10, $0E, $0C, $0A, $09, $07, $05, $03, $02

.BYTE   $00

# ---------------------------------------------------------------

# TAN_TABLE[d] = round(tan(d degrees) x 100), d = 0..68 only - see

# the header note: tan(69)x100 = 261, too big for one byte.

# ---------------------------------------------------------------

TAN_TABLE:

.BYTE   $00, $02, $03, $05, $07, $09, $0B, $0C, $0E, $10

.BYTE   $12, $13, $15, $17, $19, $1B, $1D, $1F, $20, $22

.BYTE   $24, $26, $28, $2A, $2D, $2F, $31, $33, $35, $37

.BYTE   $3A, $3C, $3E, $41, $43, $46, $49, $4B, $4E, $51

.BYTE   $54, $57, $5A, $5D, $61, $64, $68, $6B, $6F, $73

.BYTE   $77, $7B, $80, $85, $8A, $8F, $94, $9A, $A0, $A6

.BYTE   $AD, $B4, $BC, $C4, $CD, $D6, $E1, $EC, $F8

.END    $4000